# Title:	Test tolerance methodology for 1Rx RedCap CSI-RS-based L1-RSRP measurement tests

# Source:	Mediatek

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for 1Rx RedCap CSI-RS-based L1-RSRP measurement tests.

The test cases in question, from TS 38.533, are:

- 16.6.4.5, NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE

- 16.6.4.7, NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.14.0.

#### A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE

##### A.16.6.4.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of L1-RSRP measurement. This test will partly verify the L1-RSRP measurement requirements in clause 9.5B.4.2, with the testing configurations for NR cells in Table A.16.6.4.5.1-1.

Table A.16.6.4.5.1-1: Applicable NR configurations for FR1 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

##### A.16.6.4.5.2	Test parameters

There is one cells in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.16.6.4.5.2-1 and Table A.16.6.4.5.2-2 below.

In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the CSI-RS and report aperiodically. The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. UE is also configured to measure L1-RSRP based on SSB. After 80ms from the beginning of the test, the DCI trigger comes in slot n (0 for Config 1,2 and 8 for Config 3) of a frame and UE provides the report back based on the reporting configuration as defined in Table A.16.6.4.5.2-1.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.16.6.4.5.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~4 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
|  | 4 |  | HD-FDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1,4 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 20: NRB,c = 51 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1,4 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.2 Redcap FR1 |
| CSI-RS configuration | 1,4 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~4 |  | OP.1 |
| TRS Configuration | 1,4 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1,2,4 |  | SMTC.1 Redcap |
|  | 3 |  | SMTC.1 Redcap |
| DRX configuration | 1~4 |  | Off |
| reportConfigType | 1~4 |  | aperiodic |
| reportQuantity | 1~4 |  | cri-RSRP |
| Number of reported RS | 1~4 |  | 2 |
| qcl-Info | 1~4 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~4 | slots | 8 |
| T1 | 1~4 | s | 5 |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~4 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.16.6.4.5.2-2: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~4 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2,4 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~4 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2,4 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2,4 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/18.36 MHz | -60.77 | -59.02 |
|  | 1~4 | dB | 0 | 3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.16.6.4.5.3	Test Requirements

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement in clause 10.1.19.2.1 and relative accuracy requirement in clause 10.1.19.2.2.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 3. 	Discussion

The test case consists of 4 test configurations. To analyse the test case, it is helpful to show the values of RSRP for CSI-RS#0 and CSI-RS#1 versus time on (Figure 1).

Figure 1 - RSRP for CSI-RS#0 and CSI-RS#1

In the accompanying spreadsheet only configurations 1 and 3 are considered. For other configurations, critical test parameters are identical with test configuration 1 and 3, thus the same level uncertainties and method of deciding test tolerances can be used.

Observations

The test consist of one time period T1. During T1, for all configurations 1, 2, 3 and 4, both CSI-RS#0 and CSI-RS#1 are detected. CSI-RS#0 with the nominal Ês/Iot value of 0 dB and CSI-RS#1 with the nominal Ês/Iot value of 3 dB. The UE shall send L1-RSRP report at slot 8 from the reception of DCI trigger. The report shall contain L1-RSRP of both CSI-RS#0 and CSI-RS#1 while meeting the absolute accuracy requirement for the stronger CSI-RS#1 and relative accuracy requirement for the weaker CSI-RS#0. The effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

A detailed analysis of the test case is given in section 4 of this document.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.16.6.4.5.1-1, A.16.6.4.5.2-1 and A.16.6.4.5.2-2 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case the UE is expected to send L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1.

CSI-RS#1 which is nominally 3dB stronger than CSI-RS#0 is measured with the absolute accuracy as defined in TS 38.133 10.1A.14.2.1 and reported using mapping table 10.1.6.1-1. CSI-RS#0 is measured with the relative accuracy as defined in TS 38.133 10.1A.14.2.2 and reported using differential mapping table 10.1.6.1-2.

c) Uncertainties

The SS provides AWGN, 1 E-UTRA Cell and 1 NR cell containing two CSI-RSs to the UE. We propose to control the following parameters:

- AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- AWGN absolute power, Noc ±1.5 dB for PRBs #RB0-RB275

- Ratio of signal / AWGN for CSI-RS#0, Ês0 / Noc ±0.3 dB averaged over BWConfig

- Ratio of signal / AWGN, for CSI-RS#0, Ês0 / Noc ±0.3 dB for PRBs #RB0-RB275

- Ratio of signal / AWGN for CSI-RS#1, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of signal / AWGN, for CSI-RS#1, Ês1 / Noc ±0.3 dB for PRBs #RB0-RB275

In this test the UE measures the CSI-RSRP over specific Physical Resource Block (PRB) numbers # RB0 to # RB275. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE L1-RSRP absolute and relative measurement accuracies need to be considered, because when the UE makes the L1-RSRP measurements it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 38.133:

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements

10.1A.14.2.1	Absolute Accuracy

The accuracy requirements in clause 10.1.19.2.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1A.19.2.1-1.

Table 10.1A.14.2.1-1: CSI-RS based L1-RSRP absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
| ±8.0 | ±12.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
| ±11.5 | ±14.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

10.1A.14.2.2	Relative Accuracy

The accuracy requirements in clause 10.1.19.2.2 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.14.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3I of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to Annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 10.1A.14.2.2-1.

Table 10.1A.14.2.2-1: CSI-RS based L1-RSRP relative accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±6 | ±7 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of CSI-RS resources to which the requirement applies. NOTE 3: Void NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

To ensure this absolute L1-RSRP accuracy, the conditions of Io, CSI-RS Es/Iot in Table 10.1A.14.2.1-1, and conditions of CSI-RS_RP and CSI-RS Es/Iot in section Annex B.2.4.2 need to be fulfilled.

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

The CSI-RS Ês/Iot side condition is ≥ -3dB in both tables. Band group NR_FDD_FR1_H gives the highest CSI-RS_RP side condition of -120.5dBm/15kHz for configuration 1, 2, 4 and -117.5dBm/30kHz for configuration 3.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (L1-RSRP reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each CSI-RS they are the Es/Iot range, the CSI-RS_RP power range, and the Io power range, over which the UE meets the specified CSI-RS based L1-RSRP reporting accuracy. The 8 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Analysis is performed for configuration 1 (BW 10MHz, SCS 15kHz, 52 RBs) and configuration 3 (BW 20MHz, SCS 30kHz, 51 RBs), the shaded cells, in the spreadsheet, show what changes with configuration.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. In this RRM test case there is usually a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the CSI-RS#0 CSI-RS_RP, so the sensitivity factor is 1.000. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to CSI-RS#0 Es/Iot, because it is specified relative to Noc, so the sensitivity factor is zero.

The overall power Io is an exception, as the effect of Es/Noc uncertainty is diluted depending on the linear fraction of CSI-RS power making up the total Io. The sensitivity factor is made equal to the linear fraction of power which was calculated considering the RB allocation.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE L1-RSRP relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE CSI-RSRP accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE CSI-RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

We observe that the Es/Noc of both CSI-RS#0 and CSI-RS#1 is well above -3dB with nominal conditions. The test equipment uncertainties will not take the UE outside the allowed range, so there is no need for adding any offset to Es/Noc for that particular reason. However, Es/Noc of CSI-RS#1 was increased by 3.5dB to ensure the CSI-RS#1 is stronger than CSI-RS#0 considering the MU. This is to ensure that relative accuracy and differential mapping table can be applied to weaker CSI-RS#0.

We also observe that the nominal Io for both resources is well within the limit of the +/-11.5 dB UE reporting accuracy range, and that the test equipment uncertainties will not take the UE outside the allowed range, so no offset is required.

The RSRP values for both CSI-RSs remain inside the allowed range and are checked in step g).

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/Iot, CSI-RS_RP and Io). The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and test tolerances proposed, all the requirements are met.

For this test, the verdict is based on the CSI-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (CSI-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of CSI-RSRP values that a conformant UE could report.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 16.6.4.5 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- CSI-RS RSRP minimum requirement, section g) and associated Source/Reference

# 6. Treatment of other test cases

TS 38.533 also contains a group of 1Rx RedCap CSI-RS-based L1-RSRP measurement test cases. Since the test parameters critical for CSI-RS-based L1-RSRP measurements are identical with that of 16.6.4.5, the same level uncertainties and method of deciding Test Tolerances can be used.