3GPP TSG-RAN5 Meeting #104	R5-244530r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1071 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test case 6.4A.1.5 Frequency Error for NR CA for 6CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update Tx test case according to WP for RAN5 work item: 5GS_NR_LTE-UEConTest |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of test case 6.4A.1.5 Frequency error for CA (6UL CA) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR CA will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.522 CR 0452 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 : Removed and corrected the descriptions in Editor’s note and Test description. |  |  |  |  |  |  |  |  |

{Start of changes}

6.4A	Transmit signal quality for CA

6.4A.1	Frequency error for CA

{Unchanged parts skipped}

6.4A.1.5	Frequency error for CA (6UL CA)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz is TBD.

- Measurement Uncertainties and Test Tolerances are TBDFFS for power class 1, 2 and 4.

6.4A.1.5.1	Test purpose

This test verifies the ability of both, the receiver and the transmitter, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

6.4A.1.5.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 6UL CA.

6.4A.1.5.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.1.0.

6.4A.1.5.4	Test description

Same as in clause 6.4A.1.1.4 with following exceptions:

-	Instead of Table 6.4A.1.1.4.1-1 use Table 6.4A.1.5.4-1.

-	Instead of Table 6.4A.1.1.5-1 use Table 6.4A.1.5.5-1.

Table 6.4A.1.5.4-1: Test Configuration Table

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  |  |  | Mid range |  |  |
| Test CC Combination setting (aggregated BW of the CA configuration) as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  | Highest aggregated BW of the CA configuration |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| CA Configuration / Aggregated BW |  |  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC & Mapping (NOTE 4) | CBW (MHz) | Modulation | RB allocation |  | Modulation | RB allocation (NOTE 1) |
| 1 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
|  | SCC/CC5 |  | - | - |  | - | - |
|  | SCC/CC6 |  | - | - |  | - | - |
| 2 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
|  | SCC/CC5 |  | - | - |  | - | - |
|  | SCC/CC6 |  | - | - |  | - | - |
| 3 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC4 |  | - | - |  | - | - |
|  | SCC/CC5 |  | - | - |  | - | - |
|  | SCC/CC6 |  | - | - |  | - | - |
| 4 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC5 |  | - | - |  | - | - |
|  | SCC/CC6 |  | - | - |  | - | - |
| 5 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
|  | SCC/CC5 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
|  | SCC/CC6 |  | - | - |  | - | - |
| 6 | PCC/CC1 | Default | CP-OFDM QPSK | Full RB (NOTE 1) |  | - | - |
|  | SCC/CC2 |  | - | - |  | - | - |
|  | SCC/CC3 |  | - | - |  | - | - |
|  | SCC/CC4 |  | - | - |  | - | - |
|  | SCC/CC5 |  | - | - |  | - | - |
|  | SCC/CC6 |  | - | - |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |  |  |

6.4A.1.5.5	Test Requirements

The 10 frequency error Δf results for the θ-polarization or the 10 frequency error Δf results for the φ-polarization must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 0.005 PPM),  (for Aggregated BW ≤ 400MHz)

{End of changes}