3GPP TSG-RAN5 Meeting #104	R5-244574r1

# Maastricht, The Netherlands,  19th August 2024 - 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3275 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to default 5GMM messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New Information Elements of the following 5GMM messages have been added in the core specifications: - AUTHENTICATION RESULT - REGISTRATION REQUEST - REGISTRATION ACCEPT - REGISTRATION REJECT - UL NAS TRANSPORT - De-registration request (UE originating de-registration) - CONFIGURATION UPDATE COMMAND - SECURITY MODE COMMAND The default message definitions have to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default message definitions of the the 5GMM messages listed above have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated table formatting. Added non-cleartext condition for Deregistration Request message. |  |  |  |  |  |  |  |  |

### 4.7.1	Contents of 5GMM messages

##### <<<< TEXT SKIPPED >>>>

#### –	Authentication result

Table 4.7.1-3: AUTHENTICATION RESULT

| Derivation Path: 24.501 clause 8.2.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication result message identity |  | ‘0101 1010’B |  |  |
| ngKSI |  | The same value as the last AUTHENTICATION REQUEST message |  |  |
| Spare half octet |  | '0000'B |  |  |
| EAP message |  | EAP-Success | See Table 4.7.3.2-03 |  |
| ABBA |  | ‘0000 0000 0000 0000’B |  |  |
| AUN3 device security key |  | Not present |  |  |

NOTE:	The security protection of this message is the same as the previous AUTHENTICATION REQUEST message.

##### <<<< TEXT SKIPPED >>>>

#### –	Registration request

Table 4.7.1-6: REGISTRATION REQUEST

| Derivation Path: 24.501 clause 8.2.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Registration request message identity |  | ‘0100 0001’B |  |  |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B | Initial registration |  |
|  |  | ‘010’B |  | MOBILITY |
|  |  | ‘011’B |  | PERIODIC |
|  |  | ‘100’B |  | EMERGENCY |
|  |  | '101'B |  | SNPN_ONBOARDING |
| FOR |  | Present but contents not checked |  |  |
|  |  | '1'B | Follow-on request pending | EMERGENCY |
| ngKSI |  | Present but contents not checked |  |  |
| 5GS mobile identity |  | Present but contents not checked |  |  |
| Non-current native NAS key set identifier |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GMM capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE security capability |  | If present: contents not checked |  |  |
| Requested NSSAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Last visited registered TAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| S1 UE network capability |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Uplink data status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| MICO indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE status |  | If present: contents not checked |  |  |
| Additional GUTI |  | If present: contents not checked |  |  |
| Allowed PDU session status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE's usage setting |  | If present: contents not checked |  | NOT pc_IMS AND NON_CLEARTEXT_IE |
|  |  | Present but contents not checked |  | NON_CLEARTEXT_IE |
| Requested DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| EPS NAS message container |  | If present: contents not checked |  |  |
| LADN indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Payload container type |  | If present: ‘0101’B | UE policy container | NON_CLEARTEXT_IE |
| Payload container |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Network slicing indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| 5GS update type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Mobile station classmark 2 |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Supported codecs |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NAS message container |  | The complete, ciphered, REGISTRATION REQUEST message including all IEs. |  | CIPHERED_MESSAGE |
| EPS bearer context status |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested extended DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| T3324 value |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE radio capability ID |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested mapped NSSAI |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Additional information requested |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested WUS assistance information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| N5GC indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested NB-N1 mode DRX parameters |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| UE request type |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Paging restriction |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Service-level-AA container |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NID |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| MS determined PLMN with disaster condition |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested PEIPS assistance information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Requested T3512 value |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Unavailability information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| Non-3GPP path switching information |  | If present: contents not checked |  | NON_CLEARTEXT_IE |
| AUN3 indication |  | If present: contents not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |
| MOBILITY | Mobility registration updating |
| PERIODIC | Periodic registration updating |
| EMERGENCY | Emergency registration |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext and shall only be included in the complete REGISTRATION REQUEST message in the NAS message container IE. |
| CIPHERED_MESSAGE | If any of the IEs marked with the condition NON_CLEARTEXT_IE is present, and the UE has a valid 5G NAS security context, this condition applies. |
| SNPN_ONBOARDING | SNPN onboarding registration |

NOTE:	This message is sent integrity protected when a valid security context exists otherwise sent without integrity protection, including only cleartext IEs.

#### –	Registration accept

Table 4.7.1-7: REGISTRATION ACCEPT

| Derivation Path: 24.501 clause 8.2.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Extended protocol discriminator | ‘0111 1110’B | 5GS mobility management messages |  |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |  |
| Spare half octet | '0000'B |  |  |  |
| Registration accept message identity | ‘0100 0010’B |  |  |  |
| 5GS registration result |  |  |  |  |
| 5GS registration result value | ‘001’B | 3GPP access |  |  |
| SMS allowed | ‘0’B | SMS over NAS not allowed |  |  |
| 5G-GUTI | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| Equivalent PLMNs | Not Present |  |  |  |
| TAI list |  |  |  |  |
| Length of tracking area identity list contents | '0000 0111'B | 7 octets |  |  |
| Partial tracking area identity list 1 |  |  |  |  |
| Number of elements | '0 0000'B | 1 element |  |  |
| Type of list | '00'B | list of TACs belonging to one PLMN, with non-consecutive TAC values |  |  |
| MCC | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| MNC | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| TAC 1 | See Table 4.4.2-3 | For 5GC NAS test cases see Table 6.3.2.2-1 |  |  |
| Allowed NSSAI |  |  |  |  |
| Length of NSSAI contents | 4 entries | Equal to the number of S-NSSAI values included |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |  |
| SST | ‘0000 0001’B | SST value 1 (eMBB) |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |  |
| SST | ‘0000 0010’B | SST value 2 (URLLC) |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |  |
| SST | ‘0000 0011’B | SST value 3 (MIoT) |  |  |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents | ‘0000 0001’B | SST |  |  |
| SST | ‘0000 0100’B | SST value 4 (V2X) |  |  |
| Rejected NSSAI | Not Present |  |  |  |
| Configured NSSAI | Not Present |  |  |  |
| 5GS network feature support | ‘0000 1101 0000 0000 0000 0000’B | IMS voice over PS session supported over 3GPP access, Emergency services supported in NR connected to 5GCN and E-UTRA connected to 5GCN. All other features set to "not supported" including the 'Interworking without N26 interface not supported'. |  |  |
| 5GS network feature support | Not Present |  | SST_URLLC OR SST_V2X OR SST_MIOT |  |
| PDU session status | The same value as the PDU session status IE of the most recently received REGISTRATION REQUEST message |  |  |  |
| PDU session reactivation result | Not Present |  |  |  |
| PDU session reactivation result error cause | Not Present |  |  |  |
| LADN information | Not Present |  |  |  |
| MICO indication | Not Present |  |  |  |
| Network slicing indication | Not Present |  |  |  |
| Service area list | Not Present |  |  |  |
| T3512 value |  |  | INITIAL |  |
| Timer value | '0 0000'B |  |  |  |
| Unit | '111'B | value indicates that the timer is deactivated |  |  |
| T3512 value | Not Present |  |  |  |
| Non-3GPP de-registration timer value | Not Present |  |  |  |
| T3502 value | Not Present |  |  |  |
| Emergency number list | Not Present |  |  |  |
| Extended emergency number list | Not Present |  |  |  |
| SOR Transparent container | Not Present |  |  |  |
| EAP message | Not Present |  |  |  |
| NSSAI inclusion mode | Not Present |  |  |  |
| Operator-defined access category definitions | Not Present |  |  |  |
| Negotiated DRX parameters | Not Present |  |  |  |
| Non-3GPP NW policies | Not Present |  |  |  |
| EPS bearer context status | The same value as the EPS bearer context status IE of the most recently received REGISTRATION REQUEST message |  |  |  |
| Negotiated extended DRX parameters | Not Present |  |  |  |
| T3447 value | Not Present |  |  |  |
| T3448 value | Not Present |  |  |  |
| T3324 value | Not Present |  |  |  |
| UE radio capability ID | Not Present |  |  |  |
| UE radio capability ID | The same value as received in UE radio capability ID; if any of the REGISTRATION REQUEST message |  | pc_5GC_RACS |  |
| UE radio capability ID deletion indication | Not Present |  |  |  |
| Pending NSSAI | Not Present |  |  |  |
| Ciphering key data | Not Present |  |  |  |
| CAG information list | Not Present |  |  |  |
| Truncated 5G-S-TMSI configuration | Not Present |  |  |  |
| Negotiated WUS assistance information | Not Present |  |  |  |
| Negotiated NB-N1 mode DRX parameters | Not Present |  |  |  |
| Extended rejected NSSAI | Not Present |  |  |  |
| Service-level-AA container | Not Present |  |  |  |
| Negotiated PEIPS assistance information | Not Present |  |  |
| Negotiated PEIPS assistance information |  |  | pc_PEIPS_assistance_information |  |
| Length of tracking area identity list contents | '0000 0001'B | 1 octet |  |
| PEIPS assistance information type 1 |  |  |  |
| Type of information | '000'B | Paging subgroup ID |  |
| Paging subgroup ID value | '00000'B |  |  |
| 5GS additional request result | Not Present |  |  |  |
| NSSRG information | Not Present |  |  |  |
| Disaster roaming wait range | Not Present |  |  |  |
| Disaster return wait range | Not Present |  |  |  |
| List of PLMNs to be used in disaster condition | Not Present |  |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | Not Present |  |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | Not Present |  |  |  |
| Extended CAG information list | Not Present |  |  |  |
| NSAG information | Not Present |  |  |  |
| Equivalent SNPNs | Not Present |  |  |  |
| NID | Not Present |  |  |  |
| Registration accept type 6 IE container | Not Present |  |  |  |
| RAN timing synchronization | Not Present |  |  |  |
| Alternative NSSAI | Not Present |  |  |  |
| Maximum time offset | Not Present |  |  |  |
| S-NSSAI time validity information | Not Present |  |  |  |
| Unavailability configuration | Not Present |  |  |  |
| Feature authorization indication | Not Present |  |  |  |
| On-demand NSSAI | Not Present |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL | Initial registration |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

##### <<<< TEXT SKIPPED >>>>

#### –	Registration reject

Table 4.7.1-9: REGISTRATION REJECT

| Derivation Path: 24.501 clause 8.2.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| Registration reject message identity | ‘0100 0100’B |  |  |
| 5GMM cause | Set according to specific message content |  |  |
| T3346 value | Not Present |  |  |
| T3502 value | Not Present |  |  |
| EAP message | Not Present |  |  |
| Rejected NSSAI | Not Present |  |  |
| CAG information list | Not Present |  |  |
| Extended rejected NSSAI | Not Present |  |  |
| Disaster return wait range | Not Present |  |  |
| Extended CAG information list | Not Present |  |  |
| Lower bound timer value | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for roaming" | Not Present |  |  |
| Forbidden TAI(s) for the list of "5GS forbidden tracking areas for regional provision of service" | Not Present |  |  |
| N3IWF identifier | Not Present |  |  |
| TNAN information | Not Present |  |  |

NOTE:	This message is sent without integrity protection before the secure exchange of NAS messages has been established and sent within SECURITY PROTECTED 5GS NAS MESSAGE message after the secure exchange of NAS messages has been established.

#### –	UL NAS transport

Table 4.7.1-10: UL NAS TRANSPORT

| Derivation Path: 24.501 clause 8.2.10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| UL NAS TRANSPORT message identity |  | ‘0110 0111’B |  |  |
| Payload container type |  | Set according to specific message content |  |  |
| Payload container type |  | ‘0001’B | N1 SM information | INITIAL_PDU_REQUEST |
| Payload container type |  | ‘0100’B | SOR transparent container | 5GS_SOR_CMCI |
| Spare half octet |  | '0000'B |  |  |
| Payload container |  | Set according to specific message content |  |  |
| Payload container |  | Present | The SOR transparent container in the payload container IE carries steering of roaming information. | 5GS_SOR_CMCI |
| Length of payload container contents |  |  | 2 octet |  |
| Payload container contents |  |  |  |  |
| SOR transparent container |  | Present | The SOR transparent container carries steering of roaming information. |  |
| Length of SOR transparent container contents |  |  | 2 octet |  |
| SOR header |  |  | 1 octet |  |
| SOR data type |  | 1 | The SOR transparent container carries acknowledgement of successful reception of the steering of roaming information. |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | If present: contents not checked |  |  |
| ME support of SOR-CMCI indicator (MSSI) value |  | 1 | SOR-CMCI supported by the ME |  |
| ME support of SOR-SNPN-SI indicator (MSSNPNSI) value |  | If present: contents not checked |  |  |
| SOR-MAC-IAUSF |  | Set to match the calculated SoR-MAC-IAUS as the way defined in TS 33.501 A.17 |  |  |
| Payload container |  | PDU SESSION ESTABLISHMENT REQUEST message |  | INITIAL_PDU_REQUEST |
| PDU session ID |  | If present: contents not checked |  |  |
| PDU session ID |  | Same PDU session ID as defined in the PDU SESSION ESTABLISHMENT REQUEST message in the Payload container |  | INITIAL_PDU_REQUEST |
| Old PDU session ID |  | If present: contents not checked |  |  |
| Request type |  | If present: contents not checked |  |  |
| Request type |  | ‘001’B | initial request | INITIAL_PDU_REQUEST |
| S-NSSAI |  | If present: contents not checked |  |  |
| DNN |  | If present: contents not checked | (NOTE 1) |  |
| Additional information |  | If present: contents not checked |  |  |
| MA PDU session information |  | If present: contents not checked |  |  |
| Release assistance indication |  | If present: contents not checked |  |  |
| Non-3GPP access path switching indication |  | If present: contents not checked |  |  |
| Alternative S-NSSAI |  | If present: contents not checked |  |  |
| Payload container information |  | If present: contents not checked |  |  |
| NOTE 1: Although the contents of the IE is not required to be verified for PASS/FAIL purposes, the provided information shall be taken into account e.g. for the building the content of messages and allowing for specific UE behaviour as specified in Table 4.8.4-1. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INITIAL_PDU_REQUEST | The UL NAS TRANSPORT message is used to transport a PDU SESSION ESTABLISHMENT REQUEST message to establish a new PDU session. |
| 5GS_SOR_CMCI | Used in Rel-17 SOR-CMCI test cases (TS 38.523-1 [12]) |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

##### <<<< TEXT SKIPPED >>>>

#### –	De-registration request (UE originating de-registration)

Table 4.7.1-12: DEREGISTRATION REQUEST_1

| Derivation Path: 24.501 clause 8.2.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| De-registration request message identity | ‘0100 0101’B |  |  |
| De-registration type |  |  |  |
| Switch off | ‘0’B |  | NORMAL |
|  | ‘1’B |  | SWITCH_OFF |
| Re-registration required | ‘0’B |  |  |
| Access type | ‘01’B | 3GPP access |  |
| ngKSI | Present but contents not checked |  |  |
| 5GS mobile identity | Present but contents not checked |  |  |
| Unavailability information | If present: contents not checked |  | NON_CLEARTEXT_IE |
| NAS message container | If present: contents not checked |  | NON_CLEARTEXT_IE |

| Condition | Explanation |
| --- | --- |
| NORMAL | Normal de-registration |
| SWITCH_OFF | Switch off |
| NON_CLEARTEXT_IE | An information element that is not allowed to be sent in cleartext. |

NOTE:	If this message is sent as an initial NAS message, it is sent with integrity protection but without ciphering. Otherwise it is sent without integrity protection and ciphering before SS has started the ciphering and integrity and ciphered protected after SS has started the ciphering.

##### <<<< TEXT SKIPPED >>>>

#### –	Configuration update command

Table 4.7.1-19: CONFIGURATION UPDATE COMMAND

| Derivation Path: 24.501 clause 8.2.19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| Configuration update command message identity | ‘0101 0100’B |  |  |
| Configuration update indication | Not Present |  |  |
| 5G-GUTI | Not Present |  |  |
| TAI list | Not Present |  |  |
| Allowed NSSAI | Not Present |  |  |
| Service area list | Not Present |  |  |
| Full name for network | Not Present |  |  |
| Short name for network | Not Present |  |  |
| Local time zone | Not Present |  |  |
| Universal time and local time zone | Not Present |  |  |
| Network daylight saving time | Not Present |  |  |
| LADN information | Not Present |  |  |
| MICO indication | Not Present |  |  |
| Network slicing indication | Not Present |  |  |
| Configured NSSAI | Not Present |  |  |
| Rejected NSSAI | Not Present |  |  |
| Operator-defined access category definitions | Not Present |  |  |
| SMS indication | Not Present |  |  |
| T3447 value | Not Present |  |  |
| CAG information list | Not Present |  |  |
| UE radio capability ID | Not Present |  |  |
| UE radio capability ID deletion indication | Not Present |  |  |
| 5GS registration result | Not Present |  |  |
| Truncated 5G-S-TMSI configuration | Not Present |  |  |
| Additional configuration indication | Not Present |  |  |
| Extended rejected NSSAI | Not Present |  |  |
| Service-level-AA container | Not Present |  |  |
| NSSRG information | Not Present |  |  |
| Disaster roaming wait range | Not Present |  |  |
| Disaster return wait range | Not Present |  |  |
| List of PLMNs to be used in disaster condition | Not Present |  |  |
| Extended CAG information list | Not Present |  |  |
| Updated PEIPS assistance information | Not Present |  |  |
| NSAG information | Not Present |  |  |
| Priority indicator | Not Present |  |  |
| RAN timing synchronization | Not Present |  |  |
| Extended LADN information | Not Present |  |  |
| Alternative NSSAI | Not Present |  |  |
| S-NSSAI location validity information | Not Present |  |  |
| S-NSSAI time validity information | Not Present |  |  |
| Maximum time offset | Not Present |  |  |
| Partially allowed NSSAI | Not Present |  |  |
| Partially rejected NSSAI | Not Present |  |  |
| Feature authorization indication | Not Present |  |  |
| On-demand NSSAI | Not Present |  |  |

NOTE:	This message is always sent within SECURITY PROTECTED 5GS NAS MESSAGE message.

##### <<<< TEXT SKIPPED >>>>

#### –	Security mode command

Table 4.7.1-25: SECURITY MODE COMMAND

| Derivation Path: 24.501 clause 8.2.25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Extended protocol discriminator | ‘0111 1110’B | 5GS mobility management messages |  |
| Security header type | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet | '0000'B |  |  |
| Security mode command message identity | ‘0101 1101’B |  |  |
| Selected NAS security algorithms |  |  |  |
| Type of ciphering algorithm | Set according to PIXIT px_NAS_5GC_CipheringAlgorithm for default ciphering algorithm |  |  |
| Type of ciphering algorithm | ‘0000’B | 5G encryption algorithm 5G EA0 (null ciphering algorithm) | For RF |
| Type of integrity protection algorithm | Set according to PIXIT px_NAS_5GC_IntegrityProtAlgorithm for default integrity protection algorithm | This value should not be equal to the null integrity algorithm. |  |
| ngKSI |  |  |  |
| NAS key set identifier | The valid NAS key set identifier |  |  |
| TSC | '0'B | native security context (for KSIAMF) |  |
| Spare half octet | '0000'B |  |  |
| Replayed UE security capabilities | Set according to the received UE security capabilities |  |  |
| IMEISV request | Not Present |  |  |
| Selected EPS NAS security algorithms | Not Present |  |  |
| Selected EPS NAS security algorithms |  |  | UE_S1_SUPPORTED |
| Type of ciphering algorithm | Set according to PIXIT px_NAS_CipheringAlgorithm for default ciphering algorithm | The px_NAS_CipheringAlgorithm PIXIT is defined in TS 36.523-3 [x] |  |
| Type of integrity protection algorithm | Set according to PIXIT px_NAS_IntegrityProtAlgorithmfor default integrity protection algorithm | The px_NAS_IntegrityProtAlgorithm is defined in TS 36.523-3 [x] |  |
| Additional 5G security information | Not Present |  |  |
| Additional 5G security information |  |  | NO_VALID_SS_SECURITY_CONTEXT |
| RINMR | ‘1’B | Retransmission of the initial NAS message requested |  |
| HDP | ‘0’B | KAMF derivation is not required |  |
| EAP message | Not Present |  |  |
| EAP message | EAP-Success | See Table 4.7.3.2-03 | EAP-AKA |
| ABBA | ‘0000 0000 0000 0000’B |  | EAP-AKA |
| Replayed S1 UE security capabilities | Not Present |  |  |
| Replayed S1 UE security capabilities | Set according to the received UE security capabilities in the last REGISTRATION REQUEST message |  | UE_S1_SUPPORTED |
| AUN3 device security key | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| NO_VALID_SS_SECURITY_CONTEXT | If the SS doesn’t have a valid security context |
| EAP_AKA | EAP based primary authentication and key agreement procedure |
| For RF | Used for RF/RRM test cases |
| UE_S1_SUPPORTED | The UE indicated support of S1 in the last REGISTRATION REQUEST message |

NOTE:	This message is always sent integrity protected with new 5GS NAS security context.

##### <<<< TEXT SKIPPED >>>>