3GPP TSG-RAN5 Meeting #104	R5-244581r1

# Maastricht, The Netherlands,  19th August 2024 - 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0377 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MCPTT TC 6.1.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The core requirements for TC 6.1.1.5 are from Rel-13 but the TC applicability is Rel-14 so the core requirements needs to be updated to Rel-14 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Core requirements were updated to Rel-14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requiremens will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated WIC. |  |  |  |  |  |  |  |  |

#### 6.1.1.5	On-network / Pre-arranged Group Call using pre-established session / Client originated Pre-established Session Release with associated MCPTT session / Client Originated (CO)

6.1.1.5.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service and having a pre-established session with the MCPTT Server }

ensure that {

when { the MCPTT User requests the establishment of an MCPTT On-demand Pre-arranged Group Call  using a pre-established session requesting Automatic Commencement Mode at the invited MCPTT client(s) and implicit floor control }

then { UE (MCPTT Client) requests On-demand Pre-arranged Group Call using a pre-established session establishment by sending a SIP REFER message and responds to the SS with correct MCPC messages }

}

(2)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call using a pre-established session }

ensure that {

when { the MCPTT User (MCPTT Client) wants to terminate the ongoing MCPTT Group Call but keep the pre-established session }

then { UE (MCPTT Client) sends a SIP REFER request and leaves the MCPTT session }

}

6.1.1.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379, clauses 10.1.1.2.2.1, 10.1.1.2.3.2, 6.2.4.2,TS 24.380, clauses 9.2.2.2.2, 9.2.2.3.2, 9.2.2.4.6. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-14 requirements.

[TS 24.379, clause 10.1.1.2.2.1]

Upon receiving a request from an MCPTT user to establish an MCPTT group session using an MCPTT group identity identifying a prearranged MCPTT group within the pre-established session, the MCPTT client shall generate a SIP REFER request as specified in IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], and in accordance with the UE procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client shall follow the procedures specified in clause 10.1.2.2.2.1 with the clarification in step 3) of clause 10.1.2.2.2.1 that:

1)	the <entry> element in the application/resource-lists MIME body shall contain a "uri" attribute set to the prearranged MCPTT group identity;

2)	the <session-type> element of the application/vnd.3gpp.mcptt-info MIME body in the hname "body" URI header field shall be set to a value of "prearranged"; and

3)	if the MCPTT user has requested the origination of a broadcast group call, the MCPTT client shall comply with the procedures in clause 6.2.8.2.

[TS 24.379, clause 10.1.1.2.3.2]

When an MCPTT client wants to leave the MCPTT session within a pre-established session, the MCPTT client shall follow the procedures as specified in clause 6.2.4.2.

[TS 24.379, clause 6.2.4.2]

Upon receiving a request from an MCPTT user to leave an MCPTT session within a pre-established session, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate an initial SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27];

3)	shall set the Request-URI of the SIP REFER request to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

4)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

5)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

6)	shall set the Refer-To header field of the SIP REFER request to the MCPTT session identity to leave;

7)	shall include the "method" SIP URI parameter with the value "BYE" in the URI in the Refer-To header field;

8)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

Upon receiving a SIP 2xx response to the SIP REFER request, the MCPTT client shall interact with media plane as specified in 3GPP TS 24.380 [5].

[TS 24.380, clause 9.2.2.2.2]

When a pre-established session is created between the MCPTT client and the participating MCPTT function, as specified in 3GPP TS 24.379 [2], the MCPTT client:

1.	shall initialize any needed user plane resources for the pre-established session as specified in 3GPP TS 24.379 [2]; and

2.	shall enter the 'U: Pre-established session not in use' state.

[TS 24.380, clause 9.2.2.3.2]

Upon reception of a Connect message:

1.	if the MCPTT client accepts the incoming call the MCPTT client:

a.	shall send the Acknowledgement message with Reason Code field set to 'Accepted';

b.	shall use only the media streams of the pre-established session which are indicated as used in the associated call session Media Streams field, if the Connect contains a Media Streams field;

c.	shall create an instance of the 'Floor participant state transition diagram for basic operation' as specified in subclause 6.2.4; and

d. shall enter the 'U: Pre-established session in use' state; or

2.	Otherwise the MCPTT client:

a.	shall send the Acknowledgement message with the Reason Code field set to 'Busy' or 'Not Accepted'; and

b. 	shall remain in 'U: Pre-established session not in use' state.

[TS 24.380, clause 9.2.2.4.6]

Upon receiving a 2xx response to the sent SIP REFER request as described in 3GPP TS 24.379 [2] when the call is released, but the Pre-established Session is kept alive the MCPTT client:

1.	shall enter the 'U: Pre-established session not in use' state; and

2.	 shall terminate the instance of 'Floor participant state transition diagram for basic operation' state machine as specified in subclause 6.2.4.

6.1.1.5.3	Test description

6.1.1.5.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	The UE has performed procedure 'MCX pre-established session establishment' as described in TS 36.579-1 [2] clause 5.3.3.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

-	A pre-established session is established.

6.1.1.5.3.2	Test procedure sequence

Table 6.1.1.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a group call using the pre-established session with automatic commencement mode and implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] table 5.3A.3.3-1 to establish a group call with automatic commencement mode? | - | - | 1 | P |
| 2A | The SS (MCPTT server) sends a Floor Granted message. | <-- | Floor Granted | - | - |
| 3-6 | Void | - | - | - | - |
| 7 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] table 5.3A.4.3-1 to release the call? | - | - | 2 | P |
| 9 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.1.5.3.3	Specific message contents

Table 6.1.1.5.3.3-1: SIP REFER from the UE (step 2, Table 6.1.1.5.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3A.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1, condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource list |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.1.5.3.3-3 |  |  |  |

Table 6.1.1.5.3.3-2: Void

Table 6.1.1.5.3.3-3: Resource-lists in SIP REFER (Table 6.1.1.5.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1, condition PRE-ESTABLISH, GROUP-CALL with the uri attribute of the entry extended with the SIP URI header fields as specified in Table 6.1.1.5.3.3-3A

Table 6.1.1.5.3.3-3A: SIP header fields extending the uri attribute of the resource-lists' single entry
(Table 6.1.1.5.3.3-3)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-2, condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/sdp” |  |  |  |
| MIME-part-body | SDP Message as described in Table 6.1.1.5.3.3-3B |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.1.5.3.3-3C |  |  |  |

Table 6.1.1.5.3.3-3B: SDP Message in SIP header fields (Table 6.1.1.5.3.3-3A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition SDP_OFFER, PRE_ESTABLISHED_SESSION, IMPLICIT_GRANT_REQUESTED

Table 6.1.1.5.3.3-3C: MCPTT-Info in SIP header fields (Table 6.1.1.5.3.3-3A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE-REFER

Table 6.1.1.5.3.3-4: SIP 200 (OK) from the SS (step 2, Table 6.1.1.5.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3A.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition REFER-RSP

Table 6.1.1.5.3.3-4A: Void

Table 6.1.1.5.3.3-5: Connect (step 2, Table 6.1.1.5.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3A.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.12-1, condition GROUP-CALL, ACK