3GPP TSG-RAN WG5 Meeting #104                                           R5-244679r1

# Maastricht, Netherlands, 19 - 23 August 2024

Agenda Item:	4.2.3

Source: 	CAICT

Title: 	Discussion on capabilities mandatory with or without signaling

Document for:	Decision

Introduction

There are two kinds of mandatory capabilities defined in 3GPP core specs, mandatory with signaling and mandatory without signaling. This paper is for the discussion about whether RAN5 conformance test shall allow UE not to support capabilities which are mandatory with/without signaling.

Discussion

It is obvious that RAN5 conformance test shall not allow UE not to support capabilities which are mandatory without signaling, otherwise there will be interoperability issues between UE and network.

But it is not quite clear whether there is common understanding in RAN5 about whether RAN5 conformance test shall allow UE not to support capabilities which are mandatory with signaling.

The following is an example in TS 38.508-1 which allows UE not to support capabilities which are mandatory with signaling.

Conclusion

The following is the common understanding in RAN5 about whether RAN5 conformance test shall allow UE not to support capabilities which are mandatory with/without signaling:

Conclusion 1: RAN5 conformance test shall not allow UE not to support capabilities which are mandatory without signaling.

Conclusion 2: RAN5 conformance test shall not allow UE not to support capabilities which are mandatory with signaling if the capability spec (e.g. 36.306 or 38.306) emphasizes the signaling parameter shall be set to the mandatory value.

Conclusion 3A: RAN5 conformance test shall allow UE not to support capabilities which are mandatory with signaling if the capability spec (e.g. 36.306 or 38.306) does not emphasize the signaling parameter shall be set to the mandatory value.

Conclusion 3B: Unless otherwise stated, RAN5 conformance test shall not allow UE not to support capabilities which are mandatory with signaling although the capability spec (e.g. 36.306 or 38.306) does not emphasize the signaling parameter shall be set to the mandatory value.

# References

- TS 38.508-1 v18.3.0: Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Common test environment