3GPP TSG-RAN5 Meeting #104	R5-244742r2

# Maastricht, Netherlands, 19th Aug 2024 – 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4595 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of DAPS test case 8.1.4.3.4 and 8.1.4.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | interFreqDAPS-r16 is per BC UE capability, and it is reported in CA-parametersNR-v1610. UE may only report interFreqDAPS-r16 in Intra-band contiguous BC or only report it in Intra-band non-contiguous BC or report it in both Intra-band contiguous BC and Intra-band non-contiguous BC. If UE only support interFreqDAPS-r16 in Intra-band contiguous BC, this kind of UE support interFreqDAPS-r16 but cannot test any of the inter-frequency DAPS test case (i.e 7.1.3.4.4/8.1.4.3.4/8.1.4.3.5), because those test case configure source NR Cell in Low Range test frequency (NRf1) and target NR Cell in Mid-Low Range test frequency(NRf2) (i.e Intra-band non-contiguous configuration).Therefore, this kind of UE may skip or fail the inter-frequency DAPS test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Split interfreq DAPS test case. 8.1.4.3.4.1/8.1.4.3.5.1 test on intra-band non-contiguous test frequcencies. 8.1.4.3.4.2/8.1.4.3.5.2 test on intra-band contiguous test frequcencies. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may skip or fail the inter-frequency DAPS test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3.4, 8.1.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 38.508-2 CR 0501 0703 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Update the title for 8.1.4.3.4/8.1.4.3.5 to include Intra-band non-contiguous - Create new test case 8.1.4.3.4a/8.1.4.3.5a for Intra-band contiguous test environment 2nd revision: Update the test case number to 8.1.4.3.4.1/2 and 8.1.4.3.5.1/2 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.4.3.4	DAPS handover with key change / Success / Inter-frequency

8.1.4.3.4.1	DAPS handover with key change / Success / Inter-frequency / Intra-band non-contiguous

8.1.4.3.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for handover with key change and dapsConfig is configured for a DRB }
    then { UE performs DAPS handover to the target cell and keeps DL/UL reception/transmission with the source gNB for the DRB configured with dapsConfig }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when {  UE has performed random access procedure to the target cell successfully }
    then { UE transmits an RRCReconfigurationComplete message to the target gNB }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when { UE receives an RRCReconfiguration message including a daps-SourceRelease }
    then { UE releases the radio link with the source cell }

}

(4)

with { UE in NR RRC_CONNECTED state and supporting DAPS handover and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for any DRB }

ensure that {

when { UE has performed random access procedure to the target cell successfully }
    then { UE continues the downlink user data reception from the source gNB until releasing the source cell }

}

8.1.4.3.4.1.2	Conformance requirements

Same as test case 8.1.4.3.1.

8.1.4.3.4.1.3	Test description

8.1.4.3.4.1.3.1	Pre-test conditions

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.4.3.4.1.3.2	Test procedure sequence

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.4.1.3.3	Specific message contents

Same as test case 8.1.4.3.1 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.4.2	DAPS handover with key change / Success / Inter-frequency / Intra-band contiguous

8.1.4.3.4.2.1	Test Purpose (TP)

Same as test case 8.1.4.3.4.1.

8.1.4.3.4.2.2	Conformance requirements

Same as test case 8.1.4.3.1.

8.1.4.3.4.2.3	Test description

8.1.4.3.4.2.3.1	Pre-test conditions

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

-	Test frequency of NR Cell 1 and NR Cell 3 is configured according to TS 38.508-1[4] Table 6.2.3.4-1.

8.1.4.3.4.2.3.2	Test procedure sequence

Same as test case 8.1.4.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.4.2.3.3	Specific message contents

Same as test case 8.1.4.3.1 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.5	DAPS handover / HO Failure and source link available / HO Success and RLF in source / Inter-frequency

8.1.4.3.5.1	DAPS handover / HO Failure and source link available / HO Success and RLF in source / Inter-frequency / Intra-band non-contiguous

8.1.4.3.5.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for a DRB }

ensure that {

when { T304 of the MCG expires and the source link has not been released }

then { UE falls back to source cell configuration, resumes the connection with source cell, and reports DAPS HO failure via the source without triggering RRC connection re-establishment }

}

(2)

with { UE in NR RRC_CONNECTED state and having received an RRCReconfiguration message including a reconfigurationWithSync for handover to the neighbour cell and a dapsConfig configured for a DRB }

ensure that {

when { Radio link failure is detected for the source MCG }

then { UE suspends all DRBs in the source, releases the source connection and continues the handover procedure to target }

}

8.1.4.3.5.1.2	Conformance requirements

Same as test case 8.1.4.3.2.

8.1.4.3.5.1.3	Test description

8.1.4.3.5.1.3.1	Pre-test conditions

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.4.3.5.1.3.2	Test procedure sequence

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.5.1.3.3	Specific message contents

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.5.2	DAPS handover / HO Failure and source link available / HO Success and RLF in source / Inter-frequency / Intra-band contiguous

8.1.4.3.5.2.1	Test Purpose (TP)

Same as test case 8.1.4.3.5.1.

8.1.4.3.5.2.2	Conformance requirements

Same as test case 8.1.4.3.2.

8.1.4.3.5.2.3	Test description

8.1.4.3.5.2.3.1	Pre-test conditions

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

-	Test frequency of NR Cell 1 and NR Cell 3 is configured according to TS 38.508-1[4] Table 6.2.3.4-1.

8.1.4.3.5.2.3.2	Test procedure sequence

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.4.3.5.2.3.3	Specific message contents

Same as test case 8.1.4.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

<End of modified section 1>