3GPP TSG-RAN5 Meeting #104 	R5-244834r1

# Maastricht, Netherlands, 19th Aug. 2024 - 23rd Aug. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3366 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to title of 16.7.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - “for 1 Rx UE” is missing in the title of section 16.7.1.1. - Section title for 16.7.1.2 is missing. - “-FR1” is missing in the title of test cases 16.7.1.3.x and 16.7.1.4.x. - Editorial errors in test parameter tables. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added “for 1 Rx UE” to the title of 16.7.1.1. - Added section title for 16.7.1.2. - Added “-FR1” to the titles of 16.7.1.3.x and 16.7.1.4.x. - Editorial corrections in Table 16.7.1.2.1.5-1, Table 16.7.1.2.2.5-1, Table 16.7.1.3.1.5-1, Table 16.7.1.4.1.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Titles will be still incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.1.1, 16.7.1.2, 16.7.1.3, 16.7.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.522 CR 0468 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related CR 0468 for the change #3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Editorial correction in Table 16.7.1.2.2.5-1 was reverted to solve overlap with R5-245275. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

16.7.1.1	Intra-frequency measurements for RedCap for 1 Rx UE

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE

16.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.1.

16.7.1.1.1.4	Test description

16.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.1.1.4.1-1.

Table 16.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.1.1.4.1-2.

Table 16.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.1.4.2 with following exceptions:

-	Table 6.7.1.1.1.5-1 is replaced by Table 16.7.1.1.1.5-1.

-	Table 6.7.1.1.1.5-2 is replaced by Table 16.7.1.1.1.5-2.

16.7.1.1.1.4.3	Message contents

Same as the message contents given in clause 6.7.1.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.1.1-1 applies to test configuration 16.7.1.1.1-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.1.1.1-3

16.7.1.1.1.5	Test requirement

Table 16.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.1.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.1.1.5-3 for test configuration 3.

Table 16.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1, 4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1, 4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1, 4 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1, 4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1, 4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1, 4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1, 4 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |
| Time offset with Cell 1 |  | Config 1, 4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1, 4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -107.5 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -115.20 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -112.20 + ΔBG_offset |  |
|  |  |  | dB | 2.23 | -5.57 | 2.23 | -5.57 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -101.50 | -106.10 | -82 | -86.60 | -111.00 + ΔBG_offset | -114.00 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -89.60 | -109.20 + ΔBG_offset | -112.2+ ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.09 |  | -80.20 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.91 |  | -75.13 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x1 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 16.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 43 | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 35 |
|  |  |  | Bands NR_FDD_FR1_B | 36 |
|  |  |  | Bands NR_TDD_FR1_C | 36 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 37 |
|  |  |  | NR_FDD_FR1_F | 38 |
|  |  |  | Bands NR_FDD_FR1_G | 38 |
|  |  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 57 | 80 | NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | NR_FDD_FR1_B | 50 |
|  |  |  | NR_TDD_FR1_C | 51 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 52 |
|  |  |  | NR_FDD_FR1_F | 52 |
|  |  |  | NR_FDD_FR1_G | 53 |
|  |  |  | NR_FDD_FR1_H | 53 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 39 | 56 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  |  | Bands NR_FDD_FR1_B | 31 |
|  |  |  | Bands NR_TDD_FR1_C | 32 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 33 |
|  |  |  | NR_FDD_FR1_F | 33 |
|  |  |  | Bands NR_FDD_FR1_G | 34 |
|  |  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 62 | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 54 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 55 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 56 |
|  |  |  | NR_FDD_FR1_F | 57 |
|  |  |  | Bands NR_FDD_FR1_G | 57 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 16.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 56 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 37 |
|  |  |  | Bands NR_FDD_FR1_B | 38 |
|  |  |  | Bands NR_TDD_FR1_C | 38 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 39 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 39 |
|  |  |  | NR_FDD_FR1_F | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 40 |
|  |  |  | Bands NR_FDD_FR1_H | 41 |
| Highest reported value (Cell 2) | N/A | 77 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 51 |
|  |  |  | Bands NR_FDD_FR1_B | 52 |
|  |  |  | Bands NR_TDD_FR1_C | 52 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 53 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 53 |
|  |  |  | NR_FDD_FR1_F | 54 |
|  |  |  | Bands NR_FDD_FR1_G | 54 |
|  |  |  | Bands NR_FDD_FR1_H | 55 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 53 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_FDD_FR1_B | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 35 |
|  |  |  | NR_FDD_FR1_F | 35 |
|  |  |  | Bands NR_FDD_FR1_G | 36 |
|  |  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | N/A | 80 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 56 |
|  |  |  | Bands NR_FDD_FR1_B | 56 |
|  |  |  | Bands NR_TDD_FR1_C | 57 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 57 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 58 |
|  |  |  | NR_FDD_FR1_F | 58 |
|  |  |  | Bands NR_FDD_FR1_G | 59 |
|  |  |  | Bands NR_FDD_FR1_H | 59 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE

16.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.1.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.1.

16.7.1.1.2.4	Test description

16.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.1.2.4.1-1.

Table 16.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.1.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.1.2.4.1-2.

Table 16.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.1.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.1.1.2.5-1 is replaced by Table 16.7.1.1.2.5-1.

-	Table 6.7.1.1.2.5-2 is replaced by Table 16.7.1.1.2.5-2.

16.7.1.1.2.4.3	Message contents

Message contents are same as in clause 16.7.1.1.1.4.3.

16.7.1.1.2.5	Test requirement

Table 16.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.1.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.1.2.5-2.

Table 16.7.1.1.2.5-1: Same as Table 16.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -114 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 1.88 | -4.97 | 1.88 | -4.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 2 | 6 | 2 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -100 | -104 | -82 | -86.00 | -111.00 + ΔBG_offset | -114.00 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -89 | -108 + ΔBG_offset | -111+ ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -70.05 |  | -52.05 |  | -80.20 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -58.05 |  | -80.20 + ΔBG_offset |  |

Table 16.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.2	Intra-frequency measurements for RedCap for 2 Rx UE

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE

16.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.1.4	Test description

16.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.1.4.1-1.

Table 16.7.1.2.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.1.4.1-2.

Table 16.7.1.2.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.1.4.2 with following exceptions:

-	Table 6.7.1.1.1.5-1 is replaced by Table 16.7.1.2.1.5-1.

-	Table 6.7.1.1.1.5-2 is replaced by Table 16.7.1.2.1.5-2.

16.7.1.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.1.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.1.1-1 applies to test configuration 16.7.1.2.1-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.1.2.1-3

16.7.1.2.1.5	Test requirement

Table 16.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.2.1.5-3 for test configuration 3.

Table 16.7.1.2.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| BWP BW |  | Config 1,4 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1,4 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB. 1 RedCap FR1 | SSB.1 RedCap FR1 | SSB.1 RedCap FR1 |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -107.50 |  | -88 |  | -116 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -115.20 + ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -107.50 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -112.20 + ΔBG_offset |  |
|  |  |  | dB | 2.23 | -5.57 | 2.23 | -5.57 | 0.37 | -5.56 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -101.50 | -106.10 | -82 | -86.60 | -113.00 + ΔBG_offset | -116.80+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -89.60 | -109.20 + ΔBG_offset | -112.20+ ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -82.39 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.91 |  | -75.13 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 7: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 16.7.1.2.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 35 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 36 |
|  |  |  | Bands NR_FDD_FR1_G | 37 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  |  | NR_FDD_FR1_B | 46 |
|  |  |  | NR_TDD_FR1_C | 47 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  |  | NR_FDD_FR1_G | 49 |
|  |  |  | NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  |  | Bands NR_FDD_FR1_B | 30 |
|  |  |  | Bands NR_TDD_FR1_C | 30 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 31 |
|  |  |  | Bands NR_FDD_FR1_G | 32 |
|  |  |  | Bands NR_FDD_FR1_H | 33 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 51 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 54 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 16.7.1.2.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 38 |
|  |  |  | Bands NR_FDD_FR1_B | 39 |
|  |  |  | Bands NR_TDD_FR1_C | 39 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 40 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 41 |
|  |  |  | Bands NR_FDD_FR1_H | 42 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 51 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 54 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 35 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 36 |
|  |  |  | Bands NR_FDD_FR1_G | 37 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 55 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 56 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 57 |
|  |  |  | Bands NR_FDD_FR1_G | 58 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE

16.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.2.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.2.

16.7.1.2.2.4	Test description

16.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.2.2.4.1-1.

Table 16.7.1.2.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.2.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.2.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.2.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.2.2.4.1-2.

Table 16.7.1.2.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.2.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.1.2.4.2 with following exceptions:

-	Table 6.7.1.1.2.5-1 is replaced by Table 16.7.1.2.2.5-1.

-	Table 6.7.1.1.2.5-2 is replaced by Table 16.7.1.2.2.5-2.

16.7.1.2.2.4.3	Message contents

Message contents are same as in clause 16.7.1.2.1.4.3.

16.7.1.2.2.5	Test requirement

Table 16.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.2.2.5-2.

Table 16.7.1.2.2.5-1: Same as Table 16.7.1.2.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Note2 | Config 1,2,4 | Depending on band group | dBm/15KhZ | -107.50 |  | -88 |  | -116 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -94 |  | -114+ ΔBG_offset |  |
| Note2 | Config 1,2,4 |  | dBm/SCS | -107.50 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 |  | -91 |  | -111 + ΔBG_offset |  |
|  |  |  | dB | 2.23 | -5.57 | 1.83 | -4.97 | -0.01 | -4.76 |
|  |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2,4 | Depending on band group | dBm/SCS | -101.50 | -106.10 | -82 | --86.60 | -113.00 + ΔBG_offset | -116.80+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -89 | -108 + ΔBG_offset | -111+ ΔBG_offset |
| IoNote3 | Config 1,2,4 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -82.39 + ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | Not applicableNote 5- |  | -58.05 |  | -80.20 + ΔBG_offset |  |

Table 16.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x – 8 | RSRP_x – 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x – 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x – 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.3	Inter-frequency measurements for RedCap for 1 Rx UE

16.7.1.3.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

16.7.1.3.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.3.

16.7.1.3.1.4	Test description

16.7.1.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.3.1.4.1-1.

Table 16.7.1.3.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.3.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.3.1.4.1-2.

Table 16.7.1.3.1.4.1-2: Initial conditions for SS-RSRP inter frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.3.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the neighbour cell on a different frequency than the PCell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.2.1.4.2 with following exceptions:

-	Table 6.7.1.2.1.5-1 is replaced by Table 16.7.1.3.1.5-1.

-	Table 6.7.1.2.1.5-2 is replaced by Table 16.7.1.3.1.5-2.

16.7.1.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.1.2.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.1.2.1-1 applies to test configuration 16.7.2.2-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.1.3.1-3

16.7.1.3.1.5	Test requirement

Table 16.7.1.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.3.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.3.1.5-3 for test configuration 3.

Table 16.7.1.3.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  | 20: NRB,c = 51 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  | FDD |  |  |
|  |  | Config 2 |  | TDD |  |  |  | TDD |  |  |
|  |  | Config 3 |  | TDD |  |  |  | TDD |  |  |
|  |  | Config 4 |  | HD-FDD |  |  |  | HD-FDD |  |  |
| TDD configuration |  | Config 1 |  | N/A |  |  |  | N/A |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  |  | Config 3 |  | CR.2.1 TDD |  |  | - | CR.2.1 TDD |  | - |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | Config 1,4 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Config 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  | SSB.1 RedCap FR1 |  |  |
| OCNG Patterns |  | Config 1~4 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | Config 1,4 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | Config 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | Config 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | Config 1,4 | ms | - | 3 |  |  | - | 3 |  |
|  |  | Config 2,3 | s | - | 3 |  |  | - | 3 |  |
| SMTC configuration |  | Config 1,4 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | Config 1~4 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | Depending on band group | Config 1~4 | dBm/15kHz | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115+ ΔBG_offset |
| Note2 | Depending on band group | Config 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115+ ΔBG_offset |
|  | Depending on band group | Config 3 |  | -91.65 |  |  |  | ( for Channel 2 +8dB) |  | -112.00+ ΔBG_offset |
|  |  | Config 1~4 | dB | 10+TT |  |  | 10+TT | 13+TT |  | -3+TT |
| SS-RSRPNote3 | Depending on band group | Config 1,2,4 | dBm/SCS | -84.65+TT |  |  |  | (RSRP for Cell 2 +25dB) |  | -118.00+ ΔBG_offset+TT |
|  | Depending on band group | Config 3 |  | -84.65+TT |  |  |  | (RSRP for Cell 2 +25dB) |  | -115.00+ ΔBG_offset+TT |
| IoNote3 | Depending on band group | Config 1,2,4 | dBm/ 9.36MHz | -56.28 |  |  |  | (Io for Channel 2 +19.75dB) |  | -85.28+ ΔBG_offset |
|  | Depending on band group | Config 3 | dBm/ 18.36MHz | -53.33 |  |  |  | (Io for Channel 2 +19.75dB) |  | -82.35+ ΔBG_offset |
|  |  | Config 1~4 | dB | 10 |  |  | 10 | 13 |  | -3 |
| Propagation condition |  | Config 1~4 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Config 1~4 |  | 1x1 low |  |  |  | 1x1 low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |  |

Table 16.7.1.3.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | NR_FDD_FR1_B | FFS |
|  |  | NR_TDD_FR1_C | FFS |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | FFS |
|  |  | NR_FDD_FR1_G | FFS |
|  |  | NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |

Table 16.7.1.3.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.3.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 1 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

16.7.1.3.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.3.2.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.7.1.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.3.

16.7.1.3.2.4	Test description

16.7.1.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.3.2.4.1-1.

Table 16.7.1.3.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.3.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.3.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.3.2.4.1-2.

Table 16.7.1.3.2.4.1-2: Initial conditions for SS-RSRP inter frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.3.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the neighbour cell on a different frequency than the PCell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.3.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.2.2.4.2 with following exceptions:

-	Table 6.7.1.2.2.5-1 is replaced by Table 16.7.1.3.2.5-1.

-	Table 6.7.1.2.2.5-2 is replaced by Table 16.7.1.3.2.5-2.

16.7.1.3.2.4.3	Message contents

Message contents are same as in clause 16.7.1.3.1.4.3.

16.7.1.3.2.5	Test requirement

Table 16.7.1.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.3.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.3.2.5-2.

Table 16.7.1.3.2.5-1: Same as Table 16.7.1.3.1.5-1 with the following exceptions:

TBD

Table 16.7.1.3.2.5-2: SS-RSRP Inter frequency relative accuracy requirements for the reported values

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.4	Inter-frequency measurements for RedCap for 2 Rx UE

16.7.1.4.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.

16.7.1.4.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

16.7.1.4.1.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.1.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.4.

16.7.1.4.1.4	Test description

16.7.1.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.4.1.4.1-1.

Table 16.7.1.4.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.4.1-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.4.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.4.1.4.1-2.

Table 16.7.1.4.1.4.1-2: Initial conditions for SS-RSRP inter frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.4.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the neighbour cell on a different frequency than the PCell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.4.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.2.1.4.2 with following exceptions:

-	Table 6.7.1.2.1.5-1 is replaced by Table 16.7.1.4.1.5-1.

-	Table 6.7.1.2.1.5-2 is replaced by Table 16.7.1.4.1.5-2.

16.7.1.4.1.4.3	Message contents

Same as the message contents given in clause 6.7.1.2.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.1.2.1-1 applies to test configuration 16.7.1.4.1-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.1.4.1-3

16.7.1.4.1.5	Test requirement

Table 16.7.1.4.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.4.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 16.7.1.4.1.5-2 for test configurations 1, 2 and 4 and the corresponding absolute accuracy requirements in Table 16.7.1.4.1.5-3 for test configuration 3.

Table 16.7.1.4.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  | Config | Unit | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 |  |  | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~4 |  | freq1 |  |  | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 20: NRB,c = 51 |  |  |  | 20: NRB,c = 51 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  |  | TDD |  |  |
|  |  | 4 |  | HD-FDD |  |  |  | HD-FDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |  |  | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD |  |  | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 TDD |  |  | - | CR.2.1 TDD |  | - |
| Dedicated CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD |  |  | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD |  |  | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.1 RedCap FR1 |  |  |  | SSB.1 RedCap FR1 |  |  |
| OCNG Patterns |  | 1~4 |  | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1,4 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,4 | ms | - | 3 |  |  | - | 3 |  |
|  |  | 2,3 | s | - | 3 |  |  | - | 3 |  |
| SMTC configuration |  | 1,4 |  | SMTC.2 |  |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  |  | SMTC.1 |  |  |
| EPRE ratio of PSS to SSS |  | 1~4 | dB | 0 |  |  | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~4 | dBm/15kHz | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115+ ΔBG_offset |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4 | dBm/SSB SCS | -94.65 |  |  |  | ( for Channel 2 +8dB) |  | -115+ ΔBG_offset |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  |  |  | ( for Channel 2 +8dB) |  | -112.00+ ΔBG_offset |
|  |  | 1~4 | dB | 10+TT |  |  | 10+TT | 13+TT |  | -3+TT |
| SS-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4 | dBm/SCS | -84.65+TT |  |  |  | (RSRP for Cell 2 +25dB) |  | -118.00+ ΔBG_offset+TT |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -84.65+TT |  |  |  | (RSRP for Cell 2 +25dB) |  | -115.00+ ΔBG_offset+TT |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4 | dBm/ 9.36MHz | -56.28 |  |  |  | (Io for Channel 2 +19.75dB) |  | -85.28+ ΔBG_offset |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 18.36MHz | -53.33 |  |  |  | (Io for Channel 2 +19.75dB) |  | -82.35+ ΔBG_offset |
|  |  | 1~4 | dB | 10 |  |  | 10 | 13 |  | -3 |
| Propagation condition |  | 1~4 | - | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | 1~4 |  | 1x2 |  |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |  |

Table 16.7.1.4.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2 and 4

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | NR_FDD_FR1_B | FFS |
|  |  | NR_TDD_FR1_C | FFS |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | FFS |
|  |  | NR_FDD_FR1_G | FFS |
|  |  | NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | FFS | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |

Table 16.7.1.4.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Highest reported value (Cell 2) | N/A | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | FFS |
|  |  | Bands NR_FDD_FR1_B | FFS |
|  |  | Bands NR_TDD_FR1_C | FFS |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | FFS |
|  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | FFS |
|  |  | Bands NR_FDD_FR1_G | FFS |
|  |  | Bands NR_FDD_FR1_H | FFS |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

16.7.1.4.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy for 2 Rx UE

Editor’s note: This test case is incomplete in following aspects:

-	TT analysis is missing.r

16.7.1.4.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

16.7.1.4.2.2	Test applicability

This test applies to all types of NR RedCap UE with 2 Rx from Release 17 onwards.

16.7.1.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.1.4.

16.7.1.4.2.4	Test description

16.7.1.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.7.1.4.2.4.1-1.

Table 16.7.1.4.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 16.7.1.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.1.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.1.4.2-3 | NR 30kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.1.4.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 16.7.1.4.2.4.1-2.

Table 16.7.1.4.2.4.1-2: Initial conditions for SS-RSRP inter frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.1.4.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 16.7.1.4.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the neighbour cell on a different frequency than the PCell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

16.7.1.4.2.4.2	Test procedure

Same as the test procedure given in clause 6.7.1.2.2.4.2 with following exceptions:

-	Table 6.7.1.2.2.5-1 is replaced by Table 16.7.1.4.2.5-1.

-	Table 6.7.1.2.2.5-2 is replaced by Table 16.7.1.4.2.5-2.

16.7.1.4.2.4.3	Message contents

Message contents are same as in clause 16.7.1.4.1.4.3.

16.7.1.4.2.5	Test requirement

Table 16.7.1.4.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 16.7.1.4.2.5-1 shall meet the corresponding relative accuracy requirements in Table 16.7.1.4.2.5-2.

Table 16.7.1.4.2.5-1: Same as Table 16.7.1.4.1.5-1 with the following exceptions:

TBD

Table 16.7.1.4.2.5-2: SS-RSRP Inter frequency relative accuracy requirements for the reported values

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<<End of change>>