3GPP TSG-RAN5 Meeting #104	R5-244905r1

# Maastricht, Netherlands, 19th – 23rd August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2934 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of maximum input level test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Note with TT reference is missing in Table 7.4.5-1. The test requirement and minimum conformance requirement tables are not aligned for test case 7.4A.1. Tables 7.4A.1.5-2, 7.4A.1.5-3, 7.4A.2.5-2, 7.4A.3.5-2, 7.4A.4.5-2 are not in alignment with Table 7.4.5-1. The inter-band CA test requirement tables do not consider that for inter-band CA a channel bandwidth smaller 5 MHz is possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note with TT reference has been added to Table 7.4.5-1. Table 7.4A.1.5-1 has been aligned to Table 7.4A.0.1-1 by removing NR CA bandwidth Class E. Content of Tables 7.4A.1.5-2, 7.4A.1.5-3, 7.4A.2.5-2, 7.4A.3.5-2, 7.4A.4.5-2 has been replaced by a reference to Table 7.4.5-1 in order to reduce future maintainability effort. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.5, 7.4A.1.5, 7.4A.2.5, 7.4A.3.5, 7.4A.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: considered TT in Tables 7.4A.1.5-1, 7.4A.2.5-1, 7.4A.3.5-1 for the power in each other CC, voided superfluous TT tables updated the test requirement to clarify subblocks consisting of more than one component carrier |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

7.4.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 and A.3.3 with parameters specified in Tables 7.4.5-1.

Table 7.4.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10, 15, 20 | 25, 30, 35, 40, 45, 50 | 60, 70, 80, 90, 100 |
| Power in Transmission Bandwidth Configuration4 | dBm | -252 -TT | -25 + 10log10(BWChannel /20)Note 2 -TT | -202 -TT |
|  |  | -273,5 -TT | -27 + 10log10(BWChannel /20)Note 3,5 -TT | -223,5 -TT |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: 10log10(x) is rounded to the nearest 0.5dB value. NOTE 5: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 6: TT for each frequency is specified in Table 7.4.5-3. |  |  |  |  |

Table 7.4.5-2: Void

Table 7.4.5-3: Test Tolerance (Maximum input level)

| f ≤ 3.0GHz | 3.0GHz < f ≤6.0GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

< Unchanged sections omitted >

## 7.4A	Maximum input level for CA

### 7.4A.0	Minimum conformance requirements

#### 7.4A.0.1	Maximum input level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation maximum input level is defined as the maximum mean power received at the UE antenna port, over the Transmission bandwidth configuration of each CC.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4A.0.1-1 for each component carrier.

Table 7.4A.0.1-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232 | -232 | -252 |  |
|  |  | -253, 4 | -253, 4 | -273, 4 |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c as defined in subclause 6.2.4.3. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. |  |  |  |  |  |

#### 7.4A.0.2	Maximum input level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the maximum input level requirements are defined with the uplink configuration in accordance with 7.3A.0.2.2-1. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified subclause 7.4.3 and Table 7.4A.0.1-1 for one component carrier and two component carriers per sub-block, respectively. The throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the specified reference measurement channel as specified in Annex A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1 and A.5.2.1. The requirements apply with all downlink carriers active.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.4A.

#### 7.4A.0.3	Maximum input level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an

unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in subclause 7.4.3 for each component carrier while all downlink carriers are active.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) for each component carrier.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.4A.

### 7.4A.1	Maximum input level for CA (2DL CA)

7.4A.1.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2DL CA.

7.4A.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.1.4	Test description

7.4A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.1.4.1-1 or 7.4A.1.4.1-2. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.1.4.1-1: Test configuration table for Intra-band contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-1. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |

Table 7.4A.1.4.1-2: Test configuration table for Inter-band CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 3 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 4 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-2. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.1.4.1-2. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 4: Test CC Combination setting is set as defined in Table 7.3A.1.4.1-2. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 5: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |

Table 7.4A.1.4.1-3: Test configuration table for Intra-band non-contiguous CA

| Default Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1 |  |  |
| Test CC Combination setting (NRB_agg) as specified in Table 5.5A.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 3 |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |  |
| Test Parameters |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC RB allocation | CC Mod'n | PCC RB allocation |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.1.4.1-3. NOTE 2: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 4: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.1.4.1-1, Table 7.4A.1.4.1-2 or Table 7.4A.1.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.1.4.3.

7.4A.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.1.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.1.4.1-1 for intra-band contiguous CA, 7.4A.1.4.1-2 for inter-band CA or 7.4A.1.4.1-3 for intra-band non-contiguous CA on both PCC and SCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4A.1.4.1-1 for intra-band contiguous CA, 7.4A.1.4.1-2 for inter-band CA or 7.4A.1.4.1-3 for intra-band non-contiguous CA. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCC to the value as defined in Table 7.4A.1.5-1 for intra-band contiguous CA, Table 7.4A.1.5-2 for inter-band CA or Table 7.4A.1.5-3 for intra-band non-contiguous CA. Send uplink power control commands to the UE using 1dB step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.1.5-1 for intra-band contiguous CA, Table 7.4A.1.5-2 for inter-band CA or 7.4A.1.5-3 for intra-band non-contiguous CA for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	For intra-band contiguous and non-contiguous CA: measure the average throughput of each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

For inter-band CA: measure the average throughput of SCC for a duration sufficient to achieve statistical significance according to Annex H.2A.

8.	For Inter-band CA only: Repeat steps from 1 to 7 setting the original PCell as SCell and the original SCell as PCell in the corresponding CA configuration, except for operating bands without uplink band.

7.4A.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4A.1.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4A.1.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.1.5-1 for intra-band contiguous CA or Table 7.4A.1.5-2 for inter-band CA.

Table 7.4A.1.5-1: Maximum input level for Intra-band contiguous CA

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253, 4-TT | -253, 4-TT | -273, 4-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}-TT |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2.3-3 with PCMAX_L,f,c as defined in subclause 6.2.4.3. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.1.5-5. |  |  |  |  |  |

Table 7.4A.1.5-2: Maximum input level for inter-band

see Table 7.4.5-1 for each component carrier

Table 7.4A.1.5-3: Maximum input level for intra-band non-contiguous

see Table 7.4.5-1 for each sub-block larger than or equal to 5 MHz

Table 7.4A.1.5-4: Void

Table 7.4A.1.5-5: Void

### 7.4A.2	Maximum input level for CA (3DL CA)

7.4A.2.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 3DL CA.

7.4A.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.2.4	Test description

7.4A.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.2.4.1-1, 7.4A.2.4.1-2 or 7.4A.2.4.1-3. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.2.4.1-1: Test Configuration Table for 3DL CA

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Intra-band contiguous: Mid range for PCC and SCCs Inter-band: NOTE 1, NOTE 5 Intra-band contiguous + Inter-band: NOTE 1, NOTE 5 Intra-band non-contiguous + Inter-band, Intra-band non-contiguous, Intra-band contiguous + Intra-band non-contiguous: NOTE 1 with Wgap for intra-band non-contiguous defined in table 7.3A.2.4.1-1 (NOTE 5) |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 6 |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXD Configuration (Intra-band contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(3A) Configuration (Intra-band non-contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(A-C) and CA_nX(A-B) Configuration (Intra-band contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.2.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: X,Y,Z correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n8A, X=1, Y=3, Z=8; Intra-band contiguous + Inter-band: X,Y correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, X=1,Y=3; Intra-band non-contiguous + Inter-band: X and Y correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, X=1, Y =8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.2.4.1-1. For NR band n28, 30MHz test channel bandwidth is tested with Low range test frequencies. NOTE 6: Test CC Combination setting is set as defined in Table 7.3A.1.4.1-2. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 7: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |  |

Table 7.4A.2.4.1-2: Void

Table7.4A.2.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.2.4.3.

7.4A.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.2.4.1-1 to Table 7.4A.2.4.1-3 as appropriate for PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4A.2.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value as defined in Table 7.4A.2.5-1 and Table 7.4A.2.5-2 according to the type of CA. Send uplink power control commands to the UE using 1dB step size to ensure that the PCC output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.2.5-1 or Table 7.4A.2.5-2 as appropriate for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	Measure the average throughput for the carrier(s) indicated in table 7.4A.2.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.2.4.2-1.

Table 7.4A.2.4.2-1: Test repetition and measurement configuration

| CA configuration | Test ID (NOTE1) | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- | --- |
| Intra-band contiguous | 1, 2, 3 | 15 | PCC, SCC1, SCC2 | Table 7.4A.2.5-1 |
| Inter-band | 1, 2, 3 | 12, 22, 32 | SCC1, SCC2 | Table 7.4A.2.5-2 |
| Intra-band contiguous + Inter-band | 1, 2, 3 | 13 | SCC2 | Table 7.4A.2.5-2 |
|  |  | 23 | SCC1, SCC2 | Table 7.4A.2.5-1 |
| Intra-band non-contiguous + Inter-band | 1, 2, 3 | 14 | SCC2 | Table 7.4A.2.5-2 |
|  |  | 24 | SCC1, SCC2 | Table 7.4A.2.5-2 |
| Intra-band non-contiguous | 1, 2, 3 | 16 | PCC, SCC1, SCC2 | Table 7.4A.2.5-2 |
| Intra-band contiguous + Intra-band non-contiguous | 1, 2, 3 | 17 | PCC, SCC1, SCC2 | Table 7.4A.2.5-1 Table 7.4A.2.5-2 |
| NOTE 1: Refers to Test IDs in Table 7.4A.2.4.1-1 NOTE 2: CA configuration ID as defined in “Default Test Settings for a CA_nXA-nYA-nZA Configuration (Inter-band)” in table 7.3A.2.4.1-11. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nXC-YA, CA_nYA-nXC, CA_nYA-nXB and CA_nXB-nYA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.2.4.1-2. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.2.4.1-1. NOTE 5: CA configuration ID as defined in “Default Test Settings for a CA_nXD Configuration (Intra-band contiguous)” in table 7.3A.2.4.1-1. NOTE 6: CA configuration ID as defined in “Default Test Settings for a CA_nX(3A) Configuration (Intra-band non-contiguous)” in table 7.3A.2.4.1-1. NOTE 7: CA configuration ID as defined in “Default Test Settings for a CA_nX(A-C) and CA_nX(A-B) Configuration (Intra-band contiguous + Intra-band non-contiguous)” in table 7.3A.2.4.1-1. NOTE 8: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |

7.4A.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4A.2.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4A.2.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.2.5-1 and Table 7.4A.2.5-2 as applicable.

Table 7.4A.2.5-1: Maximum input level for 3DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253, 4-TT | -253, 4-TT | -273, 4-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}-TT |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.2.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.2.5-2: Maximum input level for 3DL CA (Intra-band non-contiguous, Inter-band)

See Table 7.4.5-1 for each inter-band component carrier. For intra-band non-contiguous CA and for each sub-block larger than or equal to 5 MHz, see Table 7.4.5-1 and Table 7.4A.1.5-1 for one component carrier and more than one component carriers per sub-block, respectively.

Table 7.4A.2.5-3: Void

### 7.4A.3	Maximum input level for CA (4DL CA)

7.4A.3.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 4DL CA.

7.4A.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.3.4	Test description

7.4A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.3.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.3.4.1-1: Test Configuration Table for 4DL CA

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | NOTE 1, NOTE 6 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | NOTE 1, NOTE 5 |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest for PCC and SCCs |  |  |  |  |
| Network signalling value |  |  | NS_01 by default |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
|  | Downlink Configuration |  |  |  |  | Uplink Configuration |  |
| Test ID | CC Mod'n | PCC RB allocation | SCC1 RB allocation | SCC2 RB allocation | SCC3 RB allocation | CC Mod'n | PCC RB allocation |
| Default Test Settings for a CA_nXE Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXA-nYA-nZA-nRA Configuration (Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intraband non-contiguous) |  |  |  |  |  |  |  |
| 1 | CP-OFDM 64QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 2 | CP-OFDM 256QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| 3 | CP-OFDM 1024QAM | NOTE 1 | NOTE 1 |  |  | DFT-s-OFDM QPSK | NOTE 1 |
| NOTE 1: The specific configuration of uplink and downlink are defined in Table 7.3A.3.4.1-1. NOTE 2: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. NOTE 3: Inter-band: nX, nY, nZ,nR correspond to the different bands in the CA Configuration. E.g. for CA_n66A-n70A-n71A-n77A, nX=n66, nY=n70, nZ=n71 ,nR=n77; Intra-band contiguous + Inter-band: nX, nY correspond to the different bands in the CA Configuration, e.g. for CA_1C-3A, nX=n1, nY=n3; Intra-band non-contiguous + Inter-band: nX and nY correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n1A-n8A, nX=n1, nY =n8. NOTE 4: In a band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected and 4Rx REFSENS requirement (Table 7.3.2.5-2) is used in the test requirements. NOTE 5: Test CC Combination setting is set as defined in Table 7.3A.3.4.1-1. If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. NOTE 6: Test frequency is set to Mid Range for PCC and SCC with exceptions defined in Table 7.3A.3.4.1-1. NOTE 7: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.3.4.3.

7.4A.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.3.4.1-1 as appropriate for PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Tables 7.4A.3.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value as defined in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 according to the type of CA. Send uplink power control commands to the UE using 1dB step size to ensure that the PCC output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.3.5-1 or Table 7.4A.3.5-2 as appropriate for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	Measure the average throughput for the component carrier(s) indicated in table 7.4A.3.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.3.4.2-1.

Table 7.4A.3.4.2-1: Test repetition and measurement configuration

| CA configuration | Test ID (NOTE1) | CA configuration ID in REFSENS | Throughput measured on | Table with test parameters to select |
| --- | --- | --- | --- | --- |
| Intra-band contiguous | 1, 2, 3 | 15 | PCC,SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Inter-band | 1, 2, 3 | 12, 22, 32, 42 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
| Intra-band contiguous + Inter-band | 1, 2, 3 | 13 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 23 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
|  |  | 33 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-1 |
| Intra-band non-contiguous + Inter-band | 1, 2, 3 | 14 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 24 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 34 | SCC1, SCC2, SCC3 | Table 7.4A.3.5-2 |
| Intra-band non-contiguous + Intra-band non-contiguous | 1, 2, 3 | 1 | SCC2, SCC3 | Table 7.4A.3.5-2 |
|  |  | 2 | SCC2, SCC3 | Table 7.4A.3.5-2 |
| NOTE 1: Refers to Test IDs in Table 7.4A.3.4.1-1 NOTE 2: CA configuration ID as defined in table 7.3A.3.4.1-1. NOTE 3: CA configuration ID as defined in “Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 4: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous + Inter-band)” in table 7.3A.3.4.1-1. NOTE 5: CA configuration ID as defined in “Default Test Settings for a CA_nXE Configuration (Intra-band contiguous)” in table 7.3A.3.4.1-1. NOTE 6: CA configuration ID as defined in “Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous)” in table 7.3A.3.4.1-1. NOTE 7: Testing for 1024 QAM is applicable for UE supporting capabilities 1024QAM (pdsch-1024QAM-FR1). |  |  |  |  |

7.4A.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4A.3.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4A.3.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.3.5-1 and Table 7.4A.3.5-2 as applicable.

Table 7.4A.3.5-1: Maximum input level for 4DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253, 4-TT | -253, 4-TT | -273, 4-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}-TT |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.3.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.3.5-2: Maximum input level for 4DL CA (Intra-band non-contiguous, Inter-band)

See Table 7.4.5-1 for each inter-band component carrier. For intra-band non-contiguous CA and for each sub-block larger than or equal to 5 MHz, see Table 7.4.5-1 and Table 7.4A.1.5-1 for one component carrier and more than one component carriers per sub-block, respectively.

Table 7.4A.3.5-3: Void

### 7.4A.4	Maximum input level for CA (5DL CA)

Editor’s Note:

- Test Configuration Table and test repetition and measurement configuration are TBD.

- RefSens for 5DL CA is FFS

7.4A.4.1	Test purpose

The same test purpose as defined in 7.4.1.

7.4A.4.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 5DL CA.

7.4A.4.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.4A.0.

7.4A.4.4	Test description

7.4A.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configurations specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in table 7.4A.4.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4A.4.4.1-1: Test Configuration Table for 5DL CA[TBD]

[TBD]

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The DL and UL Reference Measurement Channels are set according to Table 7.4A.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.4A.4.4.3.

7.4A.4.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 7.4A.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.4A.4.4.1-1 as appropriate for PCC and SCCs. The SS sends downlink MAC padding bits on the DL RMC.

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.4A.4.4.1-1. Since the UE has no payload data and no loopback data to send, the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level for PCC and SCCs to the value as defined in Table 7.4A.4.5-1 and Table 7.4A.4.5-2 according to the type of CA. Send uplink power control commands to the UE using 1dB step size to ensure that the PCC output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.4A.4.5-1 or Table 7.4A.4.5-2 as appropriate for at least the duration of the Throughput measurement ,where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

7.	Measure the average throughput for the component carrier(s) indicated in table 7.4A.4.4.2-1 for duration sufficient to achieve statistical significance according to Annex H.2A.

8.	Repeat steps 6 to 7 for all component carriers indicated in Table 7.4A.4.4.2-1.

Table 7.4A.4.4.2-1: Test repetition and measurement configuration

[TBD]

7.4A.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

The following exception applies to 1024QAM testing:

Table 7.4A.4.4.3-1: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table-r17 | qam1024 |  |  |
| } |  |  |  |

7.4A.4.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3 with parameters specified in Table 7.4A.4.5-1 and Table 7.4A.4.5-2 as applicable.

Table 7.4A.4.5-1: Maximum input level for 5DL CA (Intra-band contiguous)

| Rx Parameter | Units | NR CA Bandwidth Class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | B | C | D |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -232-TT | -232-TT | -252-TT |  |
|  |  | -253, 4-TT | -253, 4-TT | -273, 4-TT |  |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}-TT |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. NOTE 4: Reference measurement channel is A.3.2.5 or A.3.3.5 for 1024 QAM. NOTE 5: TT for each frequency is specified in Table 7.4A.4.5-3 for each CC. |  |  |  |  |  |

Table 7.4A.4.5-2: Maximum input level for 5DL CA (Intra-band non-contiguous, Inter-band)

See Table 7.4.5-1 for each inter-band component carrier. For intra-band non-contiguous CA and for each sub-block larger than or equal to 5 MHz, see Table 7.4.5-1 and Table 7.4A.1.5-1 for one component carrier and more than one component carriers per sub-block, respectively.

Table 7.4A.4.5-3: Void

< End of changes >