3GPP TSG-RAN5 Meeting #104	R5-244974r2

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3310 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of the content of sidelink information elements and 5G ProSe discovery messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the content of sidelink information elements and 5G ProSe discovery messages, Some IEs haven’t been defined according to TS 38.331 and TS 24.554 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating the content of sidelink information elements and 5G ProSe discovery messages according to TS 38.331 and TS 24.554 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.6, 4.7C.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated according TF160 comments R2: Merged with R5-244101, removed some tables. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.6.6	Sidelink information elements

–	SL-BWP-ConfigCommon

Table 4.6.6-2: SL-BWP-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-ConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |  |
| sl-BWP-r16 |  | BWP |  |  |
| sl-LengthSymbols-r16 |  | sym14 | All symbols in slot are used for SL |  |
|  |  | sym12 | All symbols in slot are used for SL | EXTENDED |
| sl-StartSymbol-r16 |  | sym0 |  |  |
| sl-PSBCH-Config-r16 |  | Not present |  |  |
| sl-TxDirectCurrentLocation-r16 |  | Not present |  |  |
| } |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon with RXPOOL and SELECTED |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EXTENDED | When cyclicPrefix is configured in sl-BWP-r16, i.e. extended CP is used |

–	SL-BWP-DiscPoolConfig

Table 4.6.6-2A: SL-BWP-DiscPoolConfig

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-BWP-DiscPoolConfig-r17 ::= SEQUENCE { |  |  |  |
| sl-DiscRxPool-r17 | Not present |  |  |
| sl-DiscTxPoolSelected-r17 | Not present |  |  |
| sl-DiscTxPoolSelected-r17 SEQUENCE { |  |  | DISC_SELECTED |
| sl-PoolToReleaseList-r16 | Not present |  |  |
| sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-ResourcePoolID-r16 | 1 | Index of the resource pool for normal case |  |
| sl-ResourcePool-r16 | SL-ResourcePool |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-DiscTxPoolScheduling-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DISC_SELECTED | To configure NR sidelink discovery Tx resource pool |

–	SL-BWP-DiscPoolConfigCommon

Table 4.6.6-2B: SL-BWP-DiscPoolConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-DiscPoolConfigCommon-r17 ::= SEQUENCE { |  |  |  |  |
| sl-DiscRxPool-r17 |  | Not present |  |  |
| sl-DiscRxPool-r17 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  | DISC_RXPOOL |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool | entry 1 |  |
| } |  |  |  |  |
| sl-DiscTxPoolSelected-r17 |  | Not present |  |  |
| sl-DiscTxPoolSelected-r17 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  | DISC_SELECTED |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePoolID-r16 |  | 1 | Index of the resource pool for normal case |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DISC_RXPOOL | To configure NR sidelink discovery Rx resource pool |
| DISC_SELECTED | To configure NR sidelink discovery Tx resource pool |

<Unchanged Skip>

–	SL-MeasObjectList

Table 4.6.6-16: SL-MeasObjectList

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-MeasObjectList-r16 ::= SEQUENCE (SIZE (1..maxNrofSL-ObjectId-r16)) OF SL-MeasObjectInfo-r16 { |  | 1 entry |  |  |
| SL-MeasObjectInfo-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MeasObjectId-r16 |  | 1 |  |  |
| sl-MeasObject-r16 SEQUENCE { |  |  |  |  |
| frequencyInfoSL-r16 |  | ARFCN-ValueNR with condition SL_MeasFrequencyInfo | For signalling test cases, set as subclause 6.2.3.7. Otherwise, set as subclause 4.3.1.8. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

–	SL-PDCP-Config

Table 4.6.6-17: SL-PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PDCP-Config-r16 ::= SEQUENCE { |  |  |  |  |
| sl-DiscardTimer-r16 |  | infinity |  |  |
| sl-PDCP-SN-Size-r16 |  | len12bits |  |  |
| sl-OutOfOrderDelivery |  | Not present |  |  |
| } |  |  |  |  |

–	SL-PSBCH-Config

Table 4.6.6-18: SL-PSBCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PSBCH-Config-r16 ::= SEQUENCE { |  |  |  |  |
| dl-P0-PSBCH-r16 |  | 0 |  |  |
| dl-Alpha-PSBCH-r16 |  | Not present | Default value 1 is used |  |
| dl-P0-PSBCH-r17 |  | 0 |  |  |
| } |  |  |  |  |

<Unchanged Skip>

–	SL-ResourcePool

Table 4.6.6-25: SL-ResourcePool

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSCCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-TimeResourcePSCCH-r16 |  | n2 |  |  |
| sl-FreqResourcePSCCH-r16 |  | n10 |  |  |
| sl-DMRS-ScrambleID-r16 |  | 0 |  |  |
| sl-NumReservedBits-r16 |  | 2 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSSCH-Config-r16 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| sl-PSSCH-DMRS-TimePatternList-r16 SEQUENCE (SIZE (1..3)) OF INTEGER (2..4) { |  | 2 entries |  |  |
| INTEGER[1] |  | 2 | entry 1 |  |
| INTEGER[2] |  | 4 | entry 2 |  |
| } |  |  |  |  |
| sl-BetaOffsets2ndSCI-r16 SEQUENCE (SIZE (4)) OF SL-BetaOffsets-r16 { |  | 4 entries |  |  |
| SL-BetaOffsets-r16[k, k=1..4] |  | 1 | entry k |  |
| } |  |  |  |  |
| sl-Scaling-r16 |  | f1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-PSFCH-Config-r16 |  | Not present |  |  |
| sl-PSFCH-Config-r16 CHOICE { |  |  |  | SL_HARQ |
| setup SEQUENCE { |  |  |  |  |
| sl-PSFCH-Period-r16 |  | sl4 |  |  |
| sl-PSFCH-RB-Set-r16 |  | bitstring of length n, The leftmost p*floor(m/p)*floor(n/m) bits are set to "1" and the rest are set to "0" | Note 1, Note 2, Note 3 |  |
| sl-NumMuxCS-Pair-r16 |  | n2 |  |  |
| sl-MinTimeGapPSFCH-r16 |  | sl2 |  |  |
| sl-PSFCH-HopID-r16 |  | Not present | Default frequency hopping ID 0 is used |  |
| sl-PSFCH-CandidateResourceType-r16 |  | startSubCH |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-SyncAllowed-r16 SEQUENCE { |  |  |  |  |
| gnss-Sync-r16 |  | true |  |  |
| gnbEnb-Sync-r16 |  | true |  |  |
| ue-Sync-r16 |  | true |  |  |
| } |  |  |  |  |
| sl-SubchannelSize-r16 |  | n10 |  |  |
| dummy |  | Not present |  |  |
| sl-StartRB-Subchannel-r16 |  | 0 | start RB of BWP |  |
| sl-NumSubchannel-r16 |  | floor(n/m) | Note 1, Note 2 |  |
| sl-Additional-MCS-Table-r16 |  | Not present |  |  |
| sl-ThreshS-RSSI-CBR-r16 |  | 0 | actual value is -112 dBm |  |
| sl-TimeWindowSizeCBR-r16 |  | slot100 |  |  |
| sl-TimeWindowSizeCR-r16 |  | slot1000 |  |  |
| sl-PTRS-Config-r16 |  | Not present |  |  |
| sl-UE-SelectedConfigRP-r16 SEQUENCE { |  |  |  |  |
| sl-CBR-PriorityTxConfigList-r16 |  | SL-CBR-PriorityTxConfigList |  |  |
| sl-Thres-RSRP-List-r16 |  | SL-Thres-RSRP-List |  |  |
| sl-MultiReserveResource-r16 |  | Not present |  |  |
| sl-MaxNumPerReserve-r16 |  | n2 |  |  |
| sl-SensingWindow-r16 |  | ms100 |  |  |
| sl-SelectionWindowList-r16 SEQUENCE (SIZE (8)) OF SL-SelectionWindowConfig-r16 { |  | 8 entries |  |  |
| SL-SelectionWindowConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-SelectionWindow-r16 |  | n10 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-ResourceReservePeriodList-r16 |  | Not present |  |  |
| sl-RS-ForSensing-r16 |  | pssch |  |  |
| sl-CBR-PriorityTxConfigList-v1650 |  | Not present |  |  |
| } |  |  |  |  |
| sl-RxParametersNcell-r16 |  | Not present |  |  |
| sl-ZoneConfigMCR-List-r16 |  | Not present |  |  |
| sl-FilterCoefficient-r16 |  | fc4 |  |  |
| sl-RB-Number-r16 |  | 10*floor(n/10) | Note 1 |  |
| sl-PreemptionEnable-r16 |  | Not present |  |  |
| sl-PriorityThreshold-UL-URLLC-r16 |  | Not present |  |  |
| sl-PriorityThreshold-r16 |  | Not present |  |  |
| sl-X-Overhead-r16 |  | Not present | Default value n0 is used |  |
| sl-PowerControl-r16 |  | Not present |  |  |
| sl-TxPercentageList-r16 SEQUENCE (SIZE (8)) OF SL-TxPercentageConfig-r16 { |  | 8 entries |  |  |
| SL-TxPercentageConfig-r16[k, k=1..8] SEQUENCE { |  |  | entry k |  |
| sl-Priority-r16 |  | k |  |  |
| sl-TxPercentage-r16 |  | p50 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-MinMaxMCS-List-r16 |  | Not present |  |  |
| sl-TimeResource-r16 |  | 1111111100 | First 8 of every 10 logical slots |  |
|  |  | 0000000011 | Last 2 of every 10 logical slots | EXCEPTIONAL |
| sl-PBPS-CPS-Config-r17 CHOICE { |  |  |  |  |
| setup |  | SL-PBPS-CPS-Config |  |  |
| } |  |  |  |  |
| sl-InterUE-CoordinationConfig-r17 |  | Not present |  |  |
| } |  |  |  |  |
| Note 1: n is the bandwidth of active SL BWP (in RB). n is determined in Table 4.3.1.0D-1 and Table 4.3.1.0D-2 for FR1 and FR2 respectively. Note 2: m is subchannel size configured by sl-SubchannelSize-r16. Note 3: p is the PSFCH period configured by sl-PSFCH-Period-r16 |  |  |  |  |

<Unchanged Skip>

4.7C.4	Contents of 5G ProSe discovery and security messages over PC3a

–	DISCOVERY_REQUEST

This message is sent by the UE to the SS.

Table 4.7C.4-1: DISCOVERY_REQUEST

| Derivation path: 24.554 [58], clause 10.5.4.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| discovery-request [1] { |  |  |  |
| transaction-ID | Any allowed value |  |  |
| command | 1 |  | Announce |
|  | 2 |  | Monitor |
|  | 3 | Query | Discoverer |
|  | 4 | Response | Discoveree |
| ProSe-Application-ID | Any allowed value |  | Open |
| RPAUID | Any allowed value |  | Restricted |
| application-identity | Any allowed value |  |  |
| discovery-type | 2 |  | Restricted |
| announcing-type | 0 |  | Restricted |
| discovery-model | 1 |  | Model_A AND Restricted |
|  | 2 |  | Model_B AND Restricted |
| Discovery-Entry-ID | Any allowed value |  | Open |
| discovery-entry-id | Any allowed value |  | Restricted |
| } |  |  |  |
| ... |  |  |  |
| discovery-request[N] {} | Any allowed value if present | A UE can include one or multiple transactions in one DISCOVERY_REQUEST message for one or more ProSe Application IDs/ RPAUIDs |  |

| Condition | Explanation |
| --- | --- |
| Announce | Announce request procedure for open or restricted 5G ProSe direct discovery. |
| Monitor | Monitor request procedure for open or restricted 5G ProSe direct discovery. |
| Discoveree | Discoveree request procedure for restricted 5G ProSe direct discovery. |
| Discoverer | Discoverer request procedure for restricted 5G ProSe direct discovery. |
| Open | 5G ProSe direct discovery is set to open. |
| Restricted | 5G ProSe direct discovery is set to restricted. |
| Model_A | Direct discovery model A is used. |
| Model_B | Direct discovery model B is used. |

-	DISCOVERY_RESPONSE

This message is sent by the SS to the UE.

Table 4.7C.4-2: DISCOVERY_RESPONSE

| Derivation path: 24.554 [58], clause 10.5.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Current-Time | Current UTC time |  |  |
| Max-Offset | 32 | 32 seconds |  |
| response-announce [1] { |  |  | Announce AND Open |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Application-Code [1] { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | country-specific scope |  |
| Spare | '0' |  |  |
| E | '1' | MCC and MNC included |  |
| MCC | The MCC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| MNC | The MNC part of the The PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| validity-timer-T5060 | 5 | Indicating the time during which the Application Code is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-key | 1000100000001000 0100010000001000 0010001000001000 0001000100001000 0000100010001000 0000010001001000 0000001000101000 0000000100011000 | 128-bit As defined in TS 33.303 [60] and TS 33.220 [61] |  |
| discovery-entry-ID | the discovery-entry-ID received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-announce-response [1] { |  |  | Announce AND Restricted |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Restricted-Code [1] { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case | The PLMN ID of the ProSe Function that assigned the ProSe Restricted Code. |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | A temporary identity that corresponds to the RPAUID. |  |
| } |  |  |  |
| validity-timer-T5062 | 3 | Indicating the time during which the Application Code is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| response-monitor [1] { |  |  | Monitor AND Open |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| discovery-filter [1] { |  |  |  |
| ProSe Application code { |  | 184 bits See TS 23.003 [2], section 24.3. |  |
| PLMN ID { |  | 24 bits The PLMN ID of the ProSe Function that assigned the ProSe Application Code, i.e. Mobile Country Code (MCC) and Mobile Network Code (MNC). |  |
| Scope | '10' | country-specific scope |  |
| Spare | '0' |  |  |
| E | '1' | MCC and MNC included |  |
| MCC | The MCC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| MNC | The MNC part of the PLMN ID of the ProSe Function that assigned the ProSe Application Code (the relevant PLMN will be set in the TC) |  |  |
| } |  |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 | 160 bits - see setting of bit 'E' above. A temporary identity that corresponds to the ProSe Application ID Name. |  |
| } |  |  |  |
| ProSe application mask [1] | 0000111111111100 00000000 1111111111111111 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 | 184 bits - The length of the ProSe Application mask is the same as the length of ProSe Application code Full matching for the MCC part is required, as well as for the first 16 bits of the Temporary ID in order a received ProSe Application code to result in a match |  |
| ProSe application mask [2] | 0000111111111100 00000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 1111111111111111 | 184 bits - The length of the ProSe Application mask is as same as the length of ProSe Application code Full matching for the MCC part is required, as well as for the last 16 bits of the Temporary ID |  |
| TTLTimer T5064 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| } |  |  |  |
| } |  |  |  |
| restricted-discoveree-response [1] { |  |  | Monitor AND Restricted |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| restricted-discovery-filter [1] { |  |  |  |
| filter | Set according to specific message content |  |  |
| TTL Timer T5066 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| RPAUID | Set according to specific message content |  |  |
| metadata indicator | 0 |  |  |
| metadata | Not present |  |  |
| } |  |  |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-discoveree-response [1] { |  |  | Discoveree |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| ProSe-Response-Code { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| } |  |  |  |
| validity-timer-T5068 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| restricted-discoverer-response [1] { |  |  | Discoverer |
| transaction-ID | the transaction ID received in the DISCOVERY_REQUEST message |  |  |
| subquery-result [1] { |  |  |  |
| ProSe query code { |  |  |  |
| PLMN ID | Set to the PLMN value used in the test case |  |  |
| TemporaryID | 1111111100000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000000000000 0000000011111111 |  |  |
| } |  |  |  |
| discovery response filter | Set according to specific message content |  |  |
| validity timer T5070 | 3 | Indicating the time during which the filter is valid Integer in the 1-525600 range representing the timer value in unit of minutes |  |
| code-sending security parameter | Set according to specific message content |  |  |
| code-receiving security parameter | Set according to specific message content |  |  |
| RPAUID | Set according to specific message content |  |  |
| metadata | Not present |  |  |
| } |  |  |  |
| discovery-entry-id | the discovery-entry-id received in the DISCOVERY_REQUEST message |  |  |
| } |  |  |  |
| response-reject | Not Present |  |  |

| Condition | Explanation |
| --- | --- |
| Announce | Announce request procedure for open or restricted 5G ProSe direct discovery. |
| Monitor | Monitor request procedure for open or restricted 5G ProSe direct discovery. |
| Discoveree | Discoveree request procedure for restricted 5G ProSe direct discovery. |
| Discoverer | Discoverer request procedure for restricted 5G ProSe direct discovery. |
| Open | 5G ProSe direct direct discovery is set to open. |
| Restricted | 5G ProSe direct direct discovery is set to restricted. |

<End of Changed Section