3GPP TSG-RAN5 Meeting #104	 R5-245043r1

# Maastricht, Netherlands, August 19th - August 23rd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3381 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR-U ENDC FR1 RSSI measurement accuracy on PSCell under DL CCA including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There are several open aspects of NR-U ENDC FR1 RSSI measurement accuracy on PSCell under DL CCA TCs 10.5.5.1,10.5.5.2 and 10.5.5.3 that need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test cases has been corrrected with following updates. - Test tolerance is added. - Editor’s note for Message contents, Test procedure and Test requirement have been removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases 10.5.5.1, 10.5.5.2 and 10.5.5.3 will remain incomplete. WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.5.1, 10.5.5.2 and 10.5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependent: R4-2413369 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1:corrected I0 value in TC 10.5.5.1 and 10.5.5.2 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 10.5.5	RSSI

#### 10.5.5.0	Minimum Conformance Requirements

<< Skipped sections >>

#### 10.5.5.1	EN-DC FR1 RSSI measurement accuracy on PSCC with CCA

10.5.5.1.1	Test purpose

The purpose of this test is to verify that RSSI measurement accuracy for PSCC is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in clause 10.5.5.0.1 when the PSCC is subject to CCA.

10.5.5.1.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting RRM and RSSI measurements on shared channel access.

10.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 10.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.5.1.

10.5.5.1.4	Test description

10.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.5.1.4.1-1.

Table 10.5.5.1.4.1-1: EN-DC FR1 RSSI measurement accuracy supported test configurations for PSCC under CCA

| Test Case ID | Description |
| --- | --- |
| 10.5.5.1-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.5.5.1-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| NOTE: The UE is only required to pass in one of the supported test configurations above. |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.5.1.4.1-2.

Table 10.5.5.1.4.1-2: Initial conditions for EN-DC FR1 RSSI measurement accuracy on PSCC under CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.5.1.5-1.

2.	Message contents are defined in clause 10.5.5.1.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the PSCell and is the target cell for RSSI measurements. The connection setup is done according to the settings in clause C.1.2 and C.1.3.

10.5.5.1.4.2	Test procedure

In this set of test cases there are two cells, E-UTRAN PCell (Cell 1), FR1 PSCell under CCA (Cell 2). Cell 2 operates on a carrier frequency with CCA and transmits SSBs in DBT window according to DL CCA model. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.5.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform RSSI measurements in DBT windows as specified in section 10.5.5.1.4.3.

4.	The UE shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 10.5.5.1.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the RSSI reported values in the periodic MeasurementReport. The RSSI value of Cell 2 reported by the UE is compared to the expected RSSI. If the value is outside the limits in Table 10.5.5.1.5-3 or the UE fails to report the measurement value for Cell 2 , the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Annex G clause G.2.7] is achieved.

10.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.5.1.4.3-1: Common Exception messages for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access Table H.3.10-2 with Condition SMTC.1 |

Table 10.5.5.1.4.3-2: MeasResults for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| Derivation Path: TS 38.508 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResultForRSSI-r16 | MeasResultForRSSI |  | SharedSpectrum |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.5.5.1.4.3-3: MeasResultForRSSI for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| Derivation Path: TS 38.508-1[14], Table 4.6.3-78AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | measResultRSSI | RSSI-Range-r16, integer (0..76) | RSSI |

10.5.5.1.5	Test requirement

Table 10.5.5.1.5-1 defines the primary level settings including test tolerances for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA. Each RSSI measurement report shall meet the corresponding accuracy requirements in Table 10.5.5.1.5-3.

Table 10.5.5.1.5-1: Cell specific test parameters for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| Parameter |  | Configurations | Unit | Test 1 |
| --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |
| RF Channel Number |  |  |  | 1 |
| BWchannel |  |  | MHz | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA |
| PCCA_DL |  |  |  | 0.9375 (Note 1, 3) |
| PCCA_UL |  |  |  | 0.75 / 0.75 (Note 2, 3) |
| DL CCA model |  |  |  | As specified in D.7.2.1 |
| UL CCA model |  |  |  | As specified in D.7.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |
| Channel access bandwidth |  |  | MHz | 20 |
| DRX Cycle configuration |  |  | ms | Not Applicable |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -77.96 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -58.96 |
| Propagation condition |  |  | - | AWGN |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |

Table 10.5.5.1.5-2: EN-DC FR1 RSSI RMTC parameters for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

Table 10.5.5.1.5-3: Requirements for the reported values for EN-DC FR1 RSSI measurement accuracy for PSCC under CCA

| Normal Conditions | Test 1 All bands |
| --- | --- |
| Lowest RSSI reported value | 35 |
| Highest RSSI reported value | 49 |
| Extreme Conditions | Test 1 All bands |
| Lowest RSSI reported value | 32 |
| Highest RSSI reported value | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1 |  |

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

#### 10.5.5.2	EN-DC FR1 RSSI measurement accuracy on SCC with CCA

10.5.5.2.1	Test purpose

The purpose of this test is to verify that RSSI measurement accuracy for SCC is within the specified limits. This test will partially verify the RSSI measurement accuracy requirements in clause 10.5.5.0.1 when SCC is subject to CCA.

10.5.5.2.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting RRM and RSSI measurements on shared channel access.

10.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A. 10.5.5.2.

10.5.5.2.4	Test description

10.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.5.2.4.1-1.

Table 10.5.5.2.4.1-1: Supported test configurations for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| Configuration | Description |
| --- | --- |
| 10.5.5.2-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.5.5.2-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.5.2.4.1-2.

Table 10.5.5.2.4.1-2: Initial conditions for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.5.2.5-1.

2.	Message contents are defined in clause 10.5.5.2.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR cell (SCC) also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.5.2.4.2	Test procedure

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 SCC cell (Cell 3) on a different frequency than the PSCell. Both Cell 2 and Cell 3 are subject to CCA and transmit SSBs in DBT windows according to DL CCA model. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5. Both Cell 2 (PCell) and Cell 3 (SCell) are configured on the UE.

2.	Set the parameters according to Table 10.5.5.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform RSSI measurements in DBT windows in the target SCC frequency, as specified in section 10.5.5.2.4.3.

4.	The UE shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 10.5.5.2.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the RSSI reported values in the periodic MeasurementReport. The RSSI value of Cell 3 reported by the UE is compared to the expected RSSI. If the value is outside the limits in Table 10.5.5.2.5-3 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Annex G clause G. 2.7 is achieved.

10.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.5.2.4.3-1: Common Exception messages for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition INTER-FREQ MO Table H.3.1-5 Table H.3.1-7 Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access H.3.10-2 with Condition SMTC.1 |

Table 10.5.5.2.4.3-2: MeasResults for EN-DC FR1 RSSI measurement accuracy on SCC under CCA

| Derivation Path: TS 38.508 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResultForRSSI-r16 | MeasResultForRSSI |  | SharedSpectrum |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.5.5.2.4.3-3: MeasResultForRSSI for EN-DC FR1 RSSI measurement accuracy on SCC under CCA

| Derivation Path: TS 38.508-1[14], Table 4.6.3-78AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | measResultRSSI | RSSI-Range-r16, integer (0..76) | RSSI |

10.5.5.2.5	Test requirements

Table 10.5.5.2.5-1 defines the primary level settings including test tolerances for EN-DC FR1 RSSI measurement accuracy on SCC with CCA. Each RSSI measurement report shall meet the corresponding accuracy requirements in Table 10.5.5.2.5-3.

Table 10.5.5.2.5-1: Cell specific test parameters for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | 0.9375 (Note 1, 3) |
| PCCA_UL |  |  |  | 1 | 0.75 / 0.75 (Note 2, 3) |
| DL CCA model |  |  |  | N/A | As specified in D.7.2.1 |
| UL CCA model |  |  |  | N/A | As specified in D.7.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 CCA | SR.1.1 CCA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 CCA | CR.1.1 CCA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 CCA | CCR.1.1 CCA |
| OCNG Patterns |  |  |  | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -77.96 | -77.96 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -77.96 | -58.96 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table 10.5.5.2.5-2: EN-DC FR1 RSSI RMTC parameters for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

Table 10.5.5.2.5-3: Requirements for the reported values for EN-DC FR1 RSSI measurement accuracy on SCC with CCA

| Normal Conditions | Test 1 All bands |
| --- | --- |
| Lowest RSSI reported value | 35 |
| Highest RSSI reported value | 49 |
| Extreme Conditions | Test 1 All bands |
| Lowest RSSI reported value | 32 |
| Highest RSSI reported value | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1 |  |

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

#### 10.5.5.3		EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

10.5.5.3.1	Test purpose

The purpose of this test is to verify that the RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clause 10.5.5.0.2 when both the serving cell and the target cell are subject to CCA.

10.5.5.3.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC with a shared spectrum NR carrier and supporting inter-frequency RRM and RSSI measurements on shared channel access.

10.5.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 10.5.5.0.2 for absolute accuracy.

The normative reference for this requirement is TS 38.133 [6] clause A.10.5.5.3.

10.5.5.3.4	Test description

Supported test configurations are shown in Table 10.5.5.3.4.1-1. Absolute accuracy of RSSI inter-frequency measurements are tested by using the parameters in Table 10.5.5.3.5-1. The configuration of cell 1 (E-UTRA PCell) is specified in clause A.6B. In all test cases, Cell 2 is the PSCell, and Cell 3 is the target cell. The inter-frequency measurements are supported by a measurement gap.

10.5.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 10.5.5.3.4.1-1.

Table 10.5.5.3.4.1-1: Supported test configurations for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| Configuration | Description |
| --- | --- |
| 10.5.5.3-1 | LTE FDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| 10.5.5.3-2 | LTE TDD; NR: TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations for each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 10.5.5.3.4.1-2.

Table 10.5.5.3.4.1-2: Initial conditions for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.8-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 10.5.5.3.5-1.

2.	Message contents are defined in clause 10.5.5.3.4.3.

3.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6B. Cell 2 is the NR cell (PSCell) with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 3 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

10.5.5.3.4.2	Test procedure

In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. Both Cell 2 and Cell 3 are subject to CCA and transmit SSBs in DBT windows according to DL CCA model. The CCA model (both DL and UL) is disabled (i.e. PCCA_DL_i= PCCA_UL=1) at the start of the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 10.5.5.3.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfiguration message) on Cell 1 configuring the UE to perform RSSI measurements in DBT windows in the target neighbouring frequency, as specified in section 10.5.5.3.4.3.

4.	The UE shall transmit an RRCReconfiguration message (embedded in RRCConnectionReconfigurationComplete message) to acknowledge the configuration. Afterwards, the SS shall enable DL and UL CCA model according to Table 10.5.5.3.5-1.

5.	The UE shall transmit periodical MeasurementReport messages.

6.	After 10s wait from Step 5, the SS shall check the RSSI reported values in the periodic MeasurementReport. The RSSI value of Cell 3 reported by the UE is compared to the expected RSSI. If the value is outside the limits in Table 10.5.5.3.5-3 or the UE fails to report the measurement value for Cell 3, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Annex G clause G. 2.7is achieved.

10.5.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 10.5.5.3.4.3-1: Common Exception messages for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.4-1 Table H.3.4-1a Table H.3.4-2 Table H.3.4-4 with Condition gapUE Table H.3.4-5 with Condition Pattern#0 Table H.3.10-1 with Condition SSB.3 CCA and SemiStatic, for semi-static channel access; or Condition SSB.4 CCA and Dynamic, for dynamic channel access H.3.10-2 with Condition SMTC.1 |

Table 10.5.5.3.4.3-2: MeasResults for EN-DC FR1 RSSI measurement accuracy on a carrier under CCA

| Derivation Path: TS 38.508 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResultForRSSI-r16 | MeasResultForRSSI |  | SharedSpectrum |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 10.5.5.3.4.3-3: MeasResultForRSSI for EN-DC FR1 RSSI measurement accuracy on a carrier under CCA

| Derivation Path: TS 38.508-1[14], Table 4.6.3-78AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | measResultRSSI | RSSI-Range-r16, integer (0..76) | RSSI |

10.5.5.3.5	Test requirements

Table 10.5.5.3.5-1 defines the primary level settings including test tolerances for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA. Each RSSI measurement report shall meet the corresponding accuracy requirements in Table 10.5.5.3.5-3.

Table 10.5.5.3.5-1: Cell specific test parameters for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| Parameter |  | Configurations | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |
| RF Channel Number |  |  |  | 1 | 2 |
| BWchannel |  |  | MHz | 40 | 40 |
| SSB configuration | Semi-static channel access Note 1, 3 | 1,2 |  | SSB.1 CCA | SSB.1 CCA |
|  | Dynamic channel access Note 2, 3 | 1,2 |  | SSB.2 CCA | SSB.2 CCA |
| PCCA_DL |  |  |  | 1 | 0.9375 (Note 1, 3) |
| PCCA_UL |  |  |  | 1 | 0.75 / 0.75 (Note 2, 3) |
| DL CCA model |  |  |  | N/A | As specified in D.7.2.1 |
| UL CCA model |  |  |  | N/A | As specified in D.7.2.2 |
| Measurement bandwidth |  |  |  | Same as channel access bandwidth |  |
| Channel access bandwidth |  |  | MHz | 20 |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  |  |  | SR.1.1 TDD | NA |
| RMSI CORESET Reference Channel |  |  |  | CR.1.1 TDD | NA |
| Dedicated CORESET Reference Channel |  |  |  | CCR.1.1 TDD | NA |
| OCNG Patterns |  |  |  | OP.1 | NA |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | NA |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -106 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -106 | -87 |
| in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | 2.5 |
| in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dB | 2.5 | -Infinity |
| SS-RSRP in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/SCS | -103.5 | -103.5 |
| SS-RSRP in slots corresponding to RSSI measurement time configuration (RMTC) |  |  |  | -103.5 | -Infinity |
| Io within measurement bandwidth in slots not corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -101.6 | -77.96 |
| Io within measurement bandwidth in slots corresponding to RSSI measurement time configuration (RMTC) |  |  | dBm/measBW | -101.6 | -58.96 |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: For UE supporting semi-static channel access and network configuring semi-static channel occupancy. For cells with CCA model, OCNG is transmitted only in the slots with downlink transmission burst and is not transmitted during the muted slots or during DBT window. Note 2: For UE supporting dynamic channel access and network configuring dynamic channel occupancy. Note 3: For a UE supporting both semi-static and dynamic channel access, the UE can be tested under dynamic channel occupancy only. |  |  |  |  |  |

Table 10.5.5.3.5-2: EN-DC FR1 RSSI RMTC parameters for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| measDurationSymbols-r16 | sym14or12 |
| --- | --- |
| rmtc-Periodicity-r16 | ms40 |
| rmtc-SubframeOffset-r16 | 20 |
| ref-SCS-CP-r16 | kHz15 |
| ReportInterval | ms120 |

Table 10.5.5.3.5-3: Absolute accuracy requirements for the reported values for EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA

| Normal Conditions | Test 1 All bands |
| --- | --- |
| Lowest RSSI reported value | 35 |
| Highest RSSI reported value | 49 |
| Extreme Conditions | Test 1 All bands |
| Lowest RSSI reported value | 32 |
| Highest RSSI reported value | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1 |  |

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

<< End of Changes >>