3GPP TSG-RAN5 Meeting #104	R5-245046r1

# Maastricht, The Netherlands,  19th August 2024 - 23th August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0388 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to MCPTT TC 6.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Element, FirstNet, Ericsson, Samsung, NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The core requirements for TC 6.1.2.2 are from Rel-13 but the TC applicability is Rel-14 so the core requirements needs to be updated to Rel-14 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Core requirements were updated to Rel-14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requiremens will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision (change to WIC) |  |  |  |  |  |  |  |  |

#### 6.1.2.2	On-network / Chat Group Call Using Pre-established Session Including Emergency and Imminent Peril Calls / Client Server originated Pre-established Session Release with associated MCPTT session / Client Origination (CO)

6.1.2.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { the MCPTT User requests the establishment of an MCPTT group session using an MCPTT group identity identifying a chat MCPTT group that is within a pre-established session}

then { UE (MCPTT Client) requests to join the Chat Group Call within a pre-established session by generating a SIP REFER message and, after indication from the MCPTT Server that the join request has been accepted, the user can participate in the group call }

}

(2)

with { UE (MCPTT Client) having established a Chat Group Call within a pre-established session }

ensure that {

when { the MCPTT User (MCPTT Client) requests the origination of an emergency group call }

then { UE (MCPTT Client) requests the set up and is able to join an emergency group call, or if unauthorised indicates to the user that they are not authorised to set up emergency calls and no call is set up }

}

(3)

with { UE (MCPTT Client) having established a Chat Group Call within a pre-established session }

ensure that {

when { the MCPTT User (MCPTT Client) requests the origination of an imminent peril group call }

then { UE (MCPTT Client) requests the set up and is able to join an imminent peril group call, or if unauthorised indicates to the user that they are not authorised to set up imminent peril calls and no call is set up }

}

(4)

with { UE (MCPTT Client) having established a Chat Group Call within a pre-established session }

ensure that {

when { the UE (MCPTT Client) wishes to leave the MCPTT session within a pre-established session  }

then { UE (MCPTT Client) initiates leaving by sending a SIP REFER and leaves the call  }

}

6.1.2.2.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 4.9, 6.2.4.2, 10.1.2.2.2, 10.1.2.2.3.2. Unless otherwise stated these are Rel-14 requirements.

[TS 24.379, clause 4.9]

When establishing a pre-established session, the MCPTT client negotiates the media parameters, including establishing IP addresses and ports using interactive connectivity establishment (ICE) as specified in IETF RFC 5245 with the participating MCPTT function, prior to using the pre-established session for establishing MCPTT sessions with other MCPTT users.

The pre-established session can later be used in MCPTT calls. This avoids the need to negotiate media parameters (including evaluating ICE candidates) and reserving bearer resources during the MCPTT call establishment that results in delayed MCPTT call establishment.

The use of pre-established session on the origination side is compatible with the use of on demand session on the termination side. The use of pre-established session on the termination side is compatible with the use of on demand session on the origination side.

[TS 24.379, clause 10.1.2.2.2]

Upon receiving a request from an MCPTT user to initiate or joinan MCPTT group session using an MCPTT group identity identifying a chat MCPTT group within the pre-established session, the MCPTT client shall generate a SIP REFER request as specified in IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], and in accordance with the UE procedures specified in 3GPP TS 24.229 [4], with the clarifications given below.

The MCPTT client:

1)	shall set the Request URI of the SIP REFER request to the session identity of the pre-established session;

2)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [25] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20], and with the Content-ID header field set to this "cid" URL;

3)	shall include in the application/resource-lists MIME body a single <entry> element containing a "uri" attribute set to the chat group identity, extended with the following URI header fields:

NOTE:	Characters that are not formatted as ASCII characters are escaped in the following URI header fields;

a)	the Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

b)	an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6]; and

c)	an hname "body" URI header field populated with:

i)	an application/sdp MIME body containing an SDP offer, if the session parameters of the pre-established session require modification or if implicit floor control is required, according to the conditions specified in clause 6.4; and

ii)	an application/vnd.3gpp.mcptt-info MIME body with:

A)	the <session-type> element set to a value of "chat"; and

B)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client;

4)	if the MCPTT user has requested the origination of an MCPTT emergency group call or is originating an MCPTT group call and the MCPTT emergency state is already set:

a)	if this is an authorised request for an MCPTT emergency group call as determined by the procedures of clause 6.2.8.8.1.8, shall comply with the procedures in clause 6.2.8.1.1; and

b)	if this is an unauthorised request for an MCPTT emergency group call as determined in step a) above, should indicate to the MCPTT user that they are not authorised to initiate an MCPTT emergency group call;

...

6)	if the MCPTT user has requested the origination of an MCPTT imminent peril group call:

a)	if this is an authorised request for an MCPTT imminent peril group call as determined by the procedures of clause 6.2.8.8.1.8, shall comply with the procedures in clause 6.2.8.1.9;

b)	if this is an unauthorised request for an MCPTT imminent peril group call as determined in step a) above, should indicate to the MCPTT user that they are not authorised to initiate an MCPTT imminent peril group call;

...

8)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), according to IETF RFC 6050 [9];

9)	shall include the following according to IETF RFC 4488 [22]:

a)	the option tag "norefersub" in the Supported header field; and

b)	the value "false" in the Refer-Sub header field.

10)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session;

11)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP REFER request according to IETF RFC 3840 [16]; and

12)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

On receiving a final SIP 2xx response to the SIP REFER request, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

On receiving a SIP 4xx response, SIP 5xx response or a SIP 6xx response to the SIP REFER request:

1)	if the MCPTT emergency group call state is set to "MEGC 2: emergency-call-requested" or "MEGC 3: emergency-call-granted"; or

2)	if the MCPTT imminent peril group call state is set to "MIGC 2: imminent-peril-call-requested" or "MIGC 3: imminent-peril-call-granted";

the MCPTT client shall perform the actions specified in clause 6.2.8.1.5 and shall skip the remaining steps.

On receiving a SIP re-INVITE request within the pre-established session targeted by the sent SIP REFER request, and if the sent SIP REFER request was a request for an MCPTT emergency group call or an MCPTT imminent peril group call, the MCPTT client:

1)	shall perform the actions specified in clause 6.2.8.1.16;

2)	shall check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of the present document to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

3)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

4)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

5)	shall send the SIP 200 (OK) response towards the participating MCPTT function according to rules and procedures of 3GPP TS 24.229 [4].

On call release by interaction with the media plane as specified in clause 9.2.2 of 3GPP TS 24.380 [5] if the sent SIP REFER request was a request for an MCPTT emergency group call or an MCPTT imminent peril group call, the MCPTT client shall perform the procedures specified in clause 6.2.8.1.17.

[TS 24.379, clause 6.2.4.2]

Upon receiving a request from an MCPTT user to leave an MCPTT session within a pre-established session, the MCPTT client:

...

2)	shall generate an initial SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27];

3)	shall set the Request-URI of the SIP REFER request to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

4)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

5)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

6)	shall set the Refer-To header field of the SIP REFER request to the MCPTT session identity to leave;

7)	shall include the "method" SIP URI parameter with the value "BYE" in the URI in the Refer-To header field;

8)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session; and

9)	shall send the SIP REFER request according to 3GPP TS 24.229 [4].

[TS 24.379, clause 10.1.2.2.3.2]

When an MCPTT client wants to leave the MCPTT session within a pre-established session, the MCPTT client shall follow the procedures as specified in clause 6.2.4.2.

6.1.2.2.3	Test description

6.1.2.2.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.579-1 [2] clause 5.4.2.

-	The UE has performed procedure 'MCX pre-established session establishment' as described in TS 36.579-1 [2] clause 5.3.3.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

-	A pre-established session is established.

6.1.2.2.3.2	Test procedure sequence

Table 6.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT client) request the establishment of a chat group call using the pre-established session with implicit floor request. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to establish a chat group call with implicit floor control? | - | - | 1 | P |
| 3-5 | Void | - | - | - | - |
| 6 | The SS sends Floor Granted. | <-- | Floor Granted | - | - |
| 7 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 1 | P |
| 8 | The procedure 'MCPTT CT call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.5.3-1 is performed. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | Make the UE (MCPTT client) request the establishment of an emergency chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 10A | Procedure 'MCX CO communication' as described in TS 36.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) send a SIP REFER request with <emergency_ind> set true? | --> | SIP REFER | 2 | P |
| 12 | The SS sends a SIP 403 (Forbidden). | <-- | SIP 403 (Forbidden) | - | - |
| 12A | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 13 | Check: Does the UE (MCPTT client) notify the user that the emergency group call is not allowed? (NOTE 1) | - | - | 2 | P |
| 14 | Make the UE (MCPTT client) request the establishment of an emergency chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 14A-15A | Check: Does the UE (MCPTT client) correctly perform steps 1a1 to 3 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to initiate establishment of an emergeny chat group call? | - | - | 2 | P |
| 16 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CT session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3.4.3-1 starting with step 2 to establish the emergency chat group call? | - | - | 2 | P |
| 17-17A | Void | - | - | - | - |
| 18 | Check: Does the UE (MCPTT client) notify the user that the emergency group call has been established? (NOTE 1) | - | - | 2 | P |
| 19 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 20 | The UE (MCPTT client) performs procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1. | - | - | - | - |
| 21 | Void | - | - | - | - |
| 22 | Make the UE (MCPTT client) request the establishment of an imminent peril chat group call using the pre-established session. (NOTE 1) | - | - | - | - |
| 22A | Procedure 'MCX CO communication' as described in TS 36.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 23 | Check: Does the UE (MCPTT client) send a SIP REFER request with <imminentperil_ind> set to true? | --> | SIP REFER | 3 | P |
| 24 | The SS sends a SIP 403 (Forbidden). | <-- | SIP 403 (Forbidden) | - | - |
| 25 | Check: Does the UE (MCPTT client) notify the user that the imminent peril group call is not allowed? (NOTE 1) | - | - | 3 | P |
| 25A | The procedure 'MCX communication release' as described in TS 36.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 26 | Make the UE (MCPTT client) request the establishment of an imminent peril chat group call using the pre-established session. (NOTE 1). | - | - | - | - |
| 26A-27A | Check: Does the UE (MCPTT client) correctly perform steps 1a1 to 3 of procedure 'MCPTT CO call establishment using a pre-established session' as described in TS 36.579-1 [2] Table 5.3A.3.3-1 to initiate establishment of an imminent peril chat group call? | - | - | 3 | P |
| 28 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CT session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3.4.3-1 starting with step 2 to establish the imminent peril chat group call? | - | - | 3 | P |
| 29- 29A | Void | - | - | - | - |
| 30 | Check: Does the UE (MCPTT client) notify the user that the imminent peril group call has been established? (NOTE 1) | - | - | 3 | P |
| 31 | Make the UE (MCPTT client) release the call keeping the pre-established session. (NOTE 1) | - | - | - | - |
| 32 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO call release keeping the pre-established session' as described in TS 36.579-1 [2] Table 5.3A.4.3-1? | - | - | 4 | P |
| 33-34 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.2.2.3.3	Specific message contents

Table 6.1.2.2.3.3-1: SIP REFER from the UE (Step 2, Table 6.1.2.2.3.2-1;
Step 2, TS 36.579-1 [2] Table 5.3A.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource list |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.2.2.3.3-2A |  |  |  |

Table 6.1.2.2.3.3-2: Void

Table 6.1.2.2.3.3-2A: Resource-lists in SIP REFER (Table 6.1.2.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1, condition PRE-ESTABLISH, CHAT-GROUP-CALL with the uri attribute of the entry extended with the SIP URI header fields as specified in Table 6.1.2.2.3.3-2B

Table 6.1.2.2.3.3-2B: SIP header fields extending the uri attribute of the resource-lists' single entry
(Table 6.1.2.2.3.3-2A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-2, condition CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | “application/sdp” |  |  |  |
| MIME-part-body | SDP Message as described in Table 6.1.2.2.3.3-2C |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.2.2.3.3-2D |  |  |  |

Table 6.1.2.2.3.3-2C: SDP Message in SIP header fields (Table 6.1.2.2.3.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition SDP_OFFER, PRE_ESTABLISHED_SESSION, IMPLICIT_GRANT_REQUESTED

Table 6.1.2.2.3.3-2D: MCPTT-Info in SIP header fields (Table 6.1.2.2.3.3-2B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition CHAT-GROUP-CALL, INVITE_REFER

Table 6.1.2.2.3.3-2E: SIP 200 (OK) from the SS (Step 2, Table 6.1.2.2.3.2-1;
Step 3, TS 36.579-1 [2] Table 5.3A.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition REFER-RSP

Table 6.1.2.2.3.3-2F: Void

Table 6.1.2.2.3.3-2G: Connect (step 2, Table 6.1.2.2.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3A.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.6.12-1, condition CHAT-GROUP-CALL, ACK

Table 6.1.2.2.3.3-3: SIP REFER from the UE (Steps 11, 15, Table 6.1.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3A.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1, condition EMERGENCY-CALL, GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource list |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.2.2.3.3-3A |  |  |  |

Table 6.1.2.2.3.3-3A: Resource-lists in SIP REFER (Steps 11, 15, Table 6.1.2.2.3.2-3)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1, condition PRE_ESTABLISHED_SESSION, CHAT-GROUP-CALL with the uri attribute of the entry extended with the SIP URI header fields as specified in Table 6.1.2.2.3.3-3B

Table 6.1.2.2.3.3-3B: SIP header fields extending the uri attribute of the resource-lists' single entry
(Table 6.1.2.2.3.3-3A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-2, condition CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.2.2.3.3-3C |  |  |  |

Table 6.1.2.2.3.3-3C: MCPTT-Info in SIP header fields (Table 6.1.2.2.3.3-3B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition EMERGENCY-CALL, CHAT-GROUP-CALL, INVITE_REFER

Table 6.1.2.2.3.3-4: Void

Table 6.1.2.2.3.3-4A: SIP 200 (OK) from the SS (Steps 15A, 27A, Table 6.1.2.2.3.2-1;
step 3, TS 36.579-1 [2] Table 5.3A.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition REFER_RSP

Table 6.1.2.2.3.3-4B: SIP INVITE from the SS (Step 16, Table 6.1.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition re_INVITE, MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME part body | SDP Message as described in Table 6.1.2.2.3.3-4C |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.2.2.3.3-4D |  |  |  |

Table 6.1.2.2.3.3-4C: SDP in SIP INVITE (Table 6.1.2.2.3.3-4B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition SDP_OFFER, PRE_ESTABLISHED_SESSION

Table 6.1.2.2.3.3-4D: MCPTT-Info in SIP INVITE (Table 6.1.2.2.3.3-4B)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1, condition CHAT-GROUP-CALL, EMERGENCY-CALL

Table 6.1.2.2.3.3-4E: SIP 200 (OK) from the UE (Steps 16, 28, Table 6.1.2.2.3.2-1;
step 4, TS 36.579-1 [2], Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.1-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME part body | SDP Message as described in Table 66.1.2.2.3.3-4F |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME part body | MCPTT-Info as described in Table 6.1.2.2.3.3-4G |  |  |  |

Table 6.1.2.2.3.3-4F: SDP in SIP 200 (OK) (Table 6.1.2.2.3.3-4E)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition SDP_ANSWER, PRE_ESTABLISHED_SESSION

Table 6.1.2.2.3.3-4G: MCPTT-Info in SIP 200 (OK) (Table 6.1.2.2.3.3-4E)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition INVITE-RSP

Table 6.1.2.2.3.3-5: SIP REFER from the UE (Steps 23, 27, Table 6.1.2.2.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3A.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1, condition IMMPERIL-CALL, CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | Resource-lists |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.2.2.3.3-5A |  |  |  |

Table 6.1.2.2.3.3-5A: Resource-lists in SIP REFER (Table 6.1.2.2.3.3-5)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1, condition PRE_ESTABLISHED_SESSION, CHAT-GROUP-CALL with the uri attribute of the entry extended with the SIP URI header fields as specified in Table 6.1.2.2.3.3-6A

Table 6.1.2.2.3.3-6: Void

Table 6.1.2.2.3.3-6A: SIP header fields extending the uri attribute of the resource-lists' single entry
(Table 6.1.2.2.3.3-5A)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-2, condition CHAT-GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.2.2.3.3-6B |  |  |  |

Table 6.1.2.2.3.3-6B: MCPTT-Info in SIP header fields (Table 6.1.2.2.3.3-6A)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition IMMPERIL-CALL, CHAT-GROUP-CALL, INVITE_REFER

Table 6.1.2.2.3.3-7: SIP INVITE from the SS (Step 28, Table 6.1.2.2.3.2-1;
Step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1, condition re_INVITE, MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP Message |  |  |
| MIME part body | SDP Message as described in Table 6.1.2.2.3.3-7B |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.2.2.3.3-7C |  |  |  |

Table 6.1.2.2.3.3-7A: Void

Table 6.1.2.2.3.3-7B: SDP in SIP INVITE (Table 6.1.2.2.3.3-7)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition SDP_OFFER, PRE_ESTABLISHED_SESSION

Table 6.1.2.2.3.3-7C: MCPTT-Info in SIP INVITE (Table 6.1.2.2.3.3-7)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1, condition CHAT-GROUP-CALL, IMMPERIL-CALL

Table 6.1.2.2.3.3-8..9: Void

Table 6.1.2.2.3.3-10: SIP 403 (Forbidden) (Steps 12, 24, Table 6.1.2.2.3.2-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.19.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Warning |  |  |  |  |
| mcptt-warn[1] | "function not allowed due to user authorisation” |  |  |  |