3GPP TSG-RAN5 Meeting #104	R5-245148r1

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0695 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FDD 4Rx antenna ports capabilities for handheld UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-18 RAN4 WI enabled several FDD bands to support 4Rx operation for handheld UE and defined the corresponding reference sensitivity requirements. RAN5 should added those bands into 4Rx antenna ports capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the FDD bands for handheld UE into Table A.4.3.9-4a. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 4Rx testing for mentioned bands can’t be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Correct the specification number to align with 3GU database. |  |  |  |  |  |  |  |  |

<Start of Changes>

Table A.4.3.9-4a: FDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 | Rel-15 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 | Rel-15 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 5 | FDD Band n5 | 38.101-1, 7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 7 | FDD Band n7 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 8 | FDD Band n8 | 38.101-1.7.3.2 | Rel-17 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 20 | FDD Band n20 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 26 | FDD Band n26 | 38.101-1.7.3.2 | Rel-18 | 4 Rx operation is supported by handheld UE |
| … |  |  |  |  |
| 28 | FDD Band n28 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| ... |  |  |  |  |
| 30 | FDD Band n30 | 38.101-1, 7.3.2 | Rel-16 |  |
| ... |  |  |  |  |
| 66 | FDD Band n66 | 38.101-1, 7.3.2 | Rel-15 |  |
| … |  |  |  |  |
| 70 | FDD Band n70 | 38.101-1, 7.3.2 | Rel-15 |  |
| 71 | FDD Band n71 | 38.101-1, 7.3.2 | Rel-16 | 4 Rx operation is supported by FWA form factor |
|  |  |  | Rel-18 | 4 Rx operation is supported by handheld UE |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/2. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4b: TDD 4 Rx antenna ports Capabilities

| Item | Band | Ref. | Release | Comments |
| --- | --- | --- | --- | --- |
| 34 | TDD Band n34 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 38 | TDD Band n38 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 39 | TDD Band n39 | 38.101-1, 7.3.2 | Rel-15 |  |
| ... |  |  |  |  |
| 40 | TDD Band n40 | 38.101-1, 7.3.2 | Rel-15 |  |
| 41 | TDD Band n41 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| ... |  |  |  |  |
| 48 | TDD Band n48 | 38.101-1, 7.3.2 | Rel-16 | NOTE 2 |
| … |  |  |  |  |
| 77 | TDD Band n77 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 78 | TDD Band n78 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| 79 | TDD Band n79 | 38.101-1, 7.3.2 | Rel-15 | NOTE 2 |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/3. NOTE 2: Support of 4 Rx for this band is mandatory for non-vehicular UEs i.e. if support has NOT been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |  |

Table A.4.3.9-4c: 2 Rx antenna ports Capabilities

| Item | Band | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | FDD Band n1 | 38.101-1, 7.3.2 |  |
| 2 | FDD Band n2 | 38.101-1, 7.3.2 |  |
| 3 | FDD Band n3 | 38.101-1, 7.3.2 |  |
| 4 | FDD Band n5 | 38.101-1, 7.3.2 |  |
| 5 | FDD Band n7 | 38.101-1, 7.3.2 | NOTE 2 |
| 6 | FDD Band n8 | 38.101-1, 7.3.2 |  |
| 7 | FDD Band n12 | 38.101-1, 7.3.2 |  |
| 7b | FDD Band n14 | 38.101-1, 7.3.2 |  |
| 8 | FDD Band n20 | 38.101-1, 7.3.2 |  |
| 8d | FDD Band n24 | 38.101-1, 7.3.2 |  |
| 9 | FDD Band n25 | 38.101-1, 7.3.2 |  |
| 9a | FDD Band n26 | 38.101-1, 7.3.2 |  |
| 10 | FDD Band n28 | 38.101-1, 7.3.2 |  |
| 10a | SDL Band n29 | 38.101-1, 7.3.2 |  |
| 10b | FDD Band n30 | 38.101-1, 7.3.2 |  |
| 11 | TDD Band n34 | 38.101-1, 7.3.2 |  |
| 12 | TDD Band n38 | 38.101-1, 7.3.2 | NOTE 2 |
| 13 | TDD Band n39 | 38.101-1, 7.3.2 |  |
| 14 | TDD Band n40 | 38.101-1, 7.3.2 |  |
| 15 | TDD Band n41 | 38.101-1, 7.3.2 | NOTE 2 |
| 16 | TDD Band n48 | 38.101-1, 7.3.2 |  |
| 17 | TDD Band n50 | 38.101-1, 7.3.2 |  |
| 18 | TDD Band n51 | 38.101-1, 7.3.2 |  |
| 18a | Reserved |  |  |
| 18b | TDD Band n53 | 38.101-1, 7.3.2 |  |
| 19 | FDD Band n65 | 38.101-1, 7.3.2 |  |
| 20 | FDD Band n66 | 38.101-1, 7.3.2 |  |
| 21 | FDD Band n70 | 38.101-1, 7.3.2 |  |
| 22 | FDD Band n71 | 38.101-1, 7.3.2 |  |
| 23 | FDD Band n74 | 38.101-1, 7.3.2 |  |
| 24 | TDD Band n77 | 38.101-1, 7.3.2 | NOTE 2 |
| 25 | TDD Band n78 | 38.101-1, 7.3.2 | NOTE 2 |
| 26 | TDD Band n79 | 38.101-1, 7.3.2 | NOTE 2 |
| 26a | FDD Band n85 | 38.101-1, 7.3.2 |  |
| 27 | FDD Band n91 | 38.101-1, 7.3.2 |  |
| 28 | FDD Band n92 | 38.101-1, 7.3.2 |  |
| 29 | FDD Band n93 | 38.101-1, 7.3.2 |  |
| 30 | FDD Band n94 | 38.101-1, 7.3.2 |  |
| 31 | FDD Band n100 | 38.101-1, 7.3.2 |  |
| 32 | TDD Band n101 | 38.101-1, 7.3.2 |  |
| 33 | FDD Band n106 | 38.101-1, 7.3.2 |  |
| NOTE 1: At least one band from those listed in the present table needs to be supported if UE has indicated support of the capability defined in Table A.4.3.1-7a/1. NOTE 2: Support of 2 Rx for this band is allowed only for vehicular UEs i.e. if support has been indicated to the capability specified in Table A.4.3.9-1/2. |  |  |  |

<End of Changes>