3GPP TSG-RAN5 Meeting #104	R5-245150r1

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2976 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5H.3.3.1 for inter-band CA with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.5H.3.3.1 general spurious emissions for inter-band CA with MIMO is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding minimum requirements of spurious emission for inter-band CA with UL MIMO. - Adding the new test case 6.5H.3.3.1 general spurious emissions for inter-band CA with MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5H.3.3.0 (new), 6.5H.3.3.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0942 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Adding the test tolerance table. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5H.3.1.5	Test requirements

Same test requirement as 6.5D.1 for the band supporting UL MIMO and 6.5.1 for the other band.

6.5H.3.2

6.5H.3.3	Spurious emission for inter-band UL CA with UL MIMO

6.5H.3.3.0	Minimum conformance requirements

For inter-band UL CA with UL MIMO in one of the two frequency bands, the general spurious emission requirements in Table 6.5.3.1.3-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.5.3.1.3-1 away from edges of the assigned channel bandwidth on a component carrier. The spurious emission requirements for co-existence in subclause 6.5A.3.2.0.3 apply with all component carriers are active.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5H.3.3.

6.5H.3.3.1	General spurious emissions for inter-band UL CA with UL MIMO

6.5H.3.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5H.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band UL CA with UL MIMO.

6.5H.3.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5H.3.3.0.

6.5H.3.3.1.4	Test description

Same test description as specified in clause 6.5A.3.1.1.4.1 with following exceptions:

Step 1 of Initial conditions is replaced by:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.7 for TE diagram and section A.3.2 for UE diagram.

Step 6 of Test procedure is replaced by:

6.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5H.3.3.1.5-1. For the carrier configured with UL MIMO, the power is measured as the sum from all UE transmit antenna connectors. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5H.3.3.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 ensuring Table 4.6.3-182 with the condition 2TX_UL_MIMO for PCell.

6.5H.3.3.1.5	Test requirement

The measured average power of spurious emission, derived in step 6, shall not exceed the described value in Table 6.5H.3.3.1.5-1.

Table 6.5H.3.3.1.5-1: General spurious emissions test requirements

| Frequency Range |  |  | Maximum Level | Measurement Bandwidth |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test requirements for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |
| 500 MHz ≤ f ≤ 520 MHz |  |  | -36 dBm +TT | 100 kHz |  |  |  |
| 1390 MHz ≤ f ≤ 2350 MHz 4690 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8020 MHz 8450 MHz ≤ f ≤ 10310 MHz |  |  | -30 dBm +TT | 1 MHz |  |  |  |
| Test requirements for CA_n5A-n77A Configuration |  |  |  |  |  |  |  |
| 1602 MHz ≤ f ≤ 3376 MHz 4124 MHz ≤ f ≤ 5898 MHz 5751 MHz ≤ f ≤ 9249 MHz |  |  | -30 dBm +TT | 1 MHz |  |  |  |
| Test requirements for CA_n8A-n78A Configuration |  |  |  |  |  |  |
| 1470 MHz ≤ f ≤ 2040 MHz 2385 MHz ≤ f ≤ 2920 MHz 4180 MHz ≤ f ≤4715 MHz 5060 MHz ≤ f ≤5630 MHz 5685 MHz ≤ f ≤6720 MHz 7480 MHz ≤ f ≤8515 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |
| Test requirements for CA_n25A-n77A Configuration |  |  |  |  |  |  |
| 500 MHz ≤ f ≤ 530 MHz |  | -36 dBm+TT |  | 100 kHz |  |  |
| 1385 MHz ≤ f ≤ 2350 MHz 4685 MHz ≤ f ≤ 6550 MHz 7000 MHz ≤ f ≤ 8030 MHz 8450 MHz ≤ f ≤ 10315 MHz |  | -30 dBm+TT |  | 1 MHz |  |  |
| Test requirements for CA_n28A-n41A Configuration |  |  |  |  |  |
| 1000MHz ≤ f ≤ 1284 MHz 1748MHz ≤ f ≤ 1987 MHz 3199MHz ≤ f ≤ 3438 MHz 3902MHz ≤ f ≤ 4186 MHz 4244MHz ≤ f ≤ 4677 MHz 5695MHz ≤ f ≤ 6128MHz |  | -25 dBm+TT | 1MHz |  |  |
| Test requirements for CA_n28A-n78A Configuration |  |  |  |  |  |
| 1804 MHz ≤ f ≤ 2394 MHz 2552 MHz ≤ f ≤ 3097 MHz 4003 MHz ≤ f ≤ 4548 MHz 4706 MHz ≤ f ≤ 5296 MHz 5852 MHz ≤ f ≤ 6897 MHz 7303 MHz ≤ f ≤ 8348 MHz |  | -30 dBm+TT | 1 MHz |  |  |
| Test requirements for CA_n66A-n71A Configuration |  |  |  |  |  |
| 314 MHz ≤ f < 454 MHz |  | -36 dBm+TT | 100 kHz |  |  |
| 1012 MHz ≤ f ≤ 1117 MHz 2373 MHz ≤ f ≤ 2478 MHz 2722 MHz ≤ f ≤ 2897 MHz 3036 MHz ≤ f ≤ 3176 MHz 4083 MHz ≤ f ≤ 4258 MHz |  | -30 dBm+TT | 1 MHz |  |  |
| Test requirements for CA_n66A-n77A Configuration |  |  |  |  |  |
| 260 MHz ≤ f ≤ 780 MHz |  | -36 dBm +TT | 100 kHz |  |  |
| 1520 MHz ≤ f ≤ 2490 MHz 4820 MHz ≤ f ≤ 6690 MHz 6720 MHz ≤ f ≤ 7760 MHz 8310 MHz ≤ f ≤ 10180 MHz |  | -30 dBm +TT | 1 MHz |  |  |
| Test requirements for CA_n71A-n77A Configuration |  |  |  |  |  |
| 1904 MHz ≤ f ≤ 3537 MHz 3963 MHz ≤ f ≤ 5596 MHz 5902 MHz ≤ f ≤ 9098 MHz |  | -30 dBm+TT | 1 MHz |  |  |

Table 6.5H.3.3.1.5-2: Test Tolerance for General spurious emissions

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 0 | 0 |
| 40MHz < BW ≤ 100MHz | 0 | 0 |

<End of Changes>