3GPP TSG-RAN5 Meeting #104	R5-245161r1

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1830 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex F for spurious emissions test cases for inter-band EN-DC with 3Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In affected CRs, the spurious emissions test cases for inter-band EN-DC with 3Tx are added. Correspondingly, the MU and TT analysis also need to be added into Annex F. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MU and TT analysis the spurious emissions test cases for inter-band EN-DC with 3Tx. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 CR 1826 - 1829 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Specify that MU and TT are referred to 6.xD or 6.xG in TS 38.5 -1 for the carrier with UL MIMO or TxD. |  |  |  |  |  |  |  |  |

<Start of Changes>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Clause |  | Maximum Test System Uncertainty |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (MULTE, MUSA) MULTE ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz MUSA f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  | MULTE is MU of LTE specified in clause 6.2.2 in TS 36.521-1 [10]. MUSA is MU of FR1 SA specified in clause 6.2.1 in TS 38.521-1 [8]. |
| <<Unchanged Text Skipped>> |  |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as clause 6.5.4.3 in TS 38.521-1 [8] |  |  |  |
| 6.5H.3.3.1 General spurious emissions for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5D.3.1 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5H.3.3.2 Spurious emissions for UE co-existence for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.6.3.2 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5D.3.2 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5L.3.3.1 General spurious emissions for inter-band EN-DC with Tx Diversity within FR1 | Same as clause 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5G.3.1 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5L.3.3.2 Spurious emissions for UE co-existence for inter-band EN-DC with Tx Diversity within FR1 | Same as clause 6.6.3.2 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5G.3.2 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |
| <<Unchanged Text Skipped>> |  |  |  |  |

<Unchanged Skipped>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause |  | Test Tolerance (TT) |  | Formula for test requirement |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2B.1.1 UE Maximum Output Power for Intra-Band Contiguous EN-DC |  | Same as 6.2.1 in TS 38.521-1 [8] |  |  |  |  |
| 6.2B.1.2 UE Maximum Output Power for Intra-Band Non-Contiguous EN-DC |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.2 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.1 in TS 38.521-1 [8]. |  |  |
| 6.2B.1.3 UE Maximum Output Power for Inter-Band EN-DC within FR1 (1 E-UTRA CC, 1 NR CC) |  | MAX (TTLTE, TTSA) TTLTE 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 4.2GHz TTSA f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz |  | TTLTE is TT of LTE specified in 6.2.3 in TS 36.521-1 [10]. TTSA is TT of FR1 SA specified in 6.2.2 in TS 38.521-1 [8]. |  |  |
| 6.2B.1.3_1 UE Maximum Output Power for Inter-Band EN-DC within FR1 (2 E-UTRA CCs, 1 NR CC) |  | Same as 6.2B.1.3 |  |  |  |  |
| 6.2B.1.4 UE Maximum Output Power for Inter-Band EN-DC including FR2 |  | Same as 6.2.1 in TS 38.521-2 [9] |  |  |  |  |
| 6.2B.1.4.1 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - EIRP and TR |  | Same as 6.2.1.1 in TS 38.521-2 [9] |  |  |  |  |
| 6.2B.1.4.2 UE Maximum Output Power for Inter-Band EN-DC including FR2 (1 NR CC) - Spherical Coverage |  | Same as 6.2.1.2 in TS 38.521-2 [9] |  |  |  |  |
| <<Unchanged Text Skipped>> |  |  |  |  |  |
| 6.5B.4.4_1.2 Additional Spurious Emissions for Inter-band including FR2 (3 NR CC) | Same as clause 6.5A.3.3.2 in TS 38.521-2 [9] |  |  |  |
| 6.5B.4.4_1.3 Additional Spurious Emissions for Inter-band including FR2 (4 NR CC) | Same as clause 6.5A.3.3.3 in TS 38.521-2 [9] |  |  |  |
| 6.5B.5.3 Transmit intermodulation for Inter-band EN-DC within FR1 | Same as 6.5.4 in TS 38.521-1 [8] |  |  |  |
| 6.5H.3.3.1 General spurious emissions for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5D.3.1 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5H.3.3.2 Spurious emissions for UE co-existence for inter-band EN-DC with UL MIMO within FR1 | Same as clause 6.6.3.2 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5D.3.2 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5L.3.3.1 General spurious emissions for inter-band EN-DC with Tx Diversity within FR1 | Same as clause 6.6.3.1 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5G.3.1 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.5L.3.3.2 Spurious emissions for UE co-existence for inter-band EN-DC with Tx Diversity within FR1 | Same as clause 6.6.3.2 in TS 36.521-1 [10] for E-UTRA CC Same as clause 6.5G.3.2 in TS 38.521-1 [8] for NR CC |  |  |  |
| 6.6B.4 Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.1 in TS 38.521-2 [9] |  |  |  |
| 6.6B.5 Enhanced Beam Correspondence for inter-band EN-DC including FR2 (1 NR CC) - EIRP | Same as clause 6.6.2 in TS 38.521-2 [9] |  |  |  |
| <<Unchanged Text Skipped>> |  |  |  |  |

<End of Changes>