3GPP TSG-RAN5 Meeting #104	R5-245255r3

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5342 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case 22.1.4 NB-IoT / User Plane CIoT Optimization / EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the LS exchange between GSMA and RAN5 in R5-243449 and R5-243972 test coverage for Rel-15 NB-IoT/ NTN / EDT was missing although there is industry requirement for it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test coverage for NB-IoT/EDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for NB-IoT/EDT will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR1463 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-245255->R5-245255r1 - corrected message name - clarified in which order the messages arrivein steps 3 & 6 - completed the UL ESM data transfer procedure R5-245255r1->R5-245255r2 - adding NTN to title - adding reference from 36.321 - adding new TP - update pre-test conditions - adding step to check RACH resource selection MAC PDU - some editorial changes R5-245255r2->R5-245255r3 - correcting typos |  |  |  |  |  |  |  |  |

### 22.1.4	NB-IoT / NTN / User Plane CIoT Optimization / EDT

#### 22.1.4.1	Test Purpose (TP)

(1)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported, with RRC Connection suspended }

ensure that {

when { EDT data transfer is triggered }

then { the UE selects a resource associated with EDT to initiate resumption of a suspended RRC connection with EDT}

}

(2)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported, with RRC Connection suspended }

ensure that {

when { EDT data transfer is triggered }

then { performs resumption of a suspended RRC connection by transmitting RRCConnectionResumeRequest-NB and transfers the EDT user data on DTCH multiplexed with the RRCConnectionResumeRequest-NB message on CCCH }

}

(3)

with { NB-IoT UE using the User Plane CIoT Optimization and with UP-EDT supported initiated by sending RRCConnectionResumeRequest-NB }

ensure that {

when { UE receives the RRCConnectionResume-NB message }

then { performs UP-EDT fallback by sending the RRCConnectionResumeComplete-NB message }

}

#### 22.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 7.3b.3, TS 36.321 Clause 5.1.2, TS 36.331, clause 5.3.1.1, 5.3.3.1b and 5.3.3.3a. Unless otherwise stated these are Rel-15 requirements.

[TS 36.300, clause 7.3b.3]

EDT for User Plane CIoT EPS optimizations, as defined in TS 24.301 [20], is characterized as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

-	Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	There is no transition to RRC CONNECTED.

The EDT procedure for User Plane CIoT EPS optimizations is illustrated in Figure 7.3b-2.

Figure 7.3b-2: EDT for User Plane CIoT EPS Optimizations

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected from the S-GW, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH.

NOTE 1:	If the MME or eNB decides the UE to move in RRC_CONNECTED mode, RRCConnectionResume message is sent in step 7 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 7 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest for EDT, the UE considers the UL data transmission not successful.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	for BL UEs or UEs in enhanced coverage or NB-IoT UEs, if EDT is initiated by the upper layers:

-	if the message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is larger than the TB size signalled in edt-TBS for the selected enhanced coverage level for EDT; or

-	if the PRACH resource associated with EDT for the selected enhanced coverage level is not available:

-	indicate to upper layers that EDT is cancelled;

-	for BL UEs or UEs in enhanced coverage, select the PRACH resource set corresponding to the selected enhanced coverage level. For EDT, the PRACH resource set shall correspond to the set associated with EDT for the selected enhanced coverage level.

-	if, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else if, for NB-IoT, ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-CFRA-Config is configured:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	if the UE is a BL UE or UE in enhanced coverage and EDT is initiated:

-	select the Random Access Preambles group corresponding to PRACH resource for EDT for the selected enhanced coverage level.

-	else if the UE is a BL UE or UE in enhanced coverage and Random Access Preamble group B does not exist:

-	select the Random Access Preambles group corresponding to the selected enhanced coverage level.

-	else if the UE is an NB-IoT UE:

-	randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group. For EDT, the PRACH resource shall correspond to resource associated with EDT for the selected enhanced coverage level.

[TS 36.331, clause 5.3.1.1]

In case of CP-EDT, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT, E-UTRAN may also choose to establish or resume the RRC connection.

[TS 36.331, clause 5.3.3.1b]

A BL UE, UE in CE or NB-IoT UE can initiate EDT when all of the following conditions are fulfilled:

1>	for CP-EDT, the upper layers request establishment of an RRC connection, the UE supports CP-EDT, and SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-EDT; or

1>	for UP-EDT, the upper layers request resumption of an RRC connection, the UE supports UP-EDT, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-EDT, and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	the establishment or resumption request is suitable for EDT as specified in TS 36.300 [9], clause 7.3b.1;

1>	SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes edt-Parameters;

1>	the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS as specified in TS 36.321 [6], clause 5.1.1;

1>	EDT fallback indication has not been received from lower layers for this establishment or resumption procedure;

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the size of UL data is suitable for EDT.

[TS 36.331, clause 5.3.3.3a]

1>	if the UE is initiating UP-EDT in accordance with conditions in 5.3.3.1b:

2>	restore the PDCP state and re-establish PDCP entities for all SRBs and all DRBs;

2>	if drb-ContinueROHC has been provided in immediately preceding RRC connection release message, and the UE is requesting to resume RRC connection in the same cell:

3>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

3>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

2>	else:

3>	indicate to lower layers that stored UE AS context is used;

3>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

2>	resume all SRBs and all DRBs;

2>	derive the KeNB key based on the KASME key to which the current KeNB is associated, using the stored value of nextHopChainingCount received in the RRCConnectionRelease message in the preceding connection, as specified in TS 33.401 [32];

2>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KRRCenc key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KUPenc key derived in this clause immediately to the user data sent and received by the UE;

2>	configure the lower layers to use EDT;

#### 22.1.4.3	Test description

##### 22.1.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 as specified in TS 36.508 clause 8.1.4.1.1 is configured.

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	px_DoAttachWithoutPDN set to false

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE shall be in state 2-NB.

##### 22.1.4.3.2	Test procedure sequence

Table 22.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionRelease-NB message including resumeIdentity and rrc-Suspend as releaseCause. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 2 | Trigger the UE to send Early Data. NOTE: It is FFS how this action is performed. | - | - | - | - |
| 3 | The UE transmits preamble on PRACH resources associated with EDT | --> | PRACH Preamble | 1 | P |
| 4 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 3, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 1 and the EDT data on DTCH? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | 2 | P |
| 6 | The SS transmits an RRCConnectionRelease-NB message including a new resumeIdentity and rrc-Suspend as releaseCause. | <-- | RRCConnectionRelease-NB | - | - |
| 7 | Trigger the UE to send Early Data. NOTE: It is FFS how this action is performed. | - | - | - | - |
| 8 | The UE transmits preamble on PRACH resources associated with EDT. | --> | PRACH Preamble | 1 | P |
| 9 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 8, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 10 | Check: Does the UE transmit a MAC PDU containing the RRCConnectionResumeRequest-NB message including the resumeIdentity and AS security context stored at Step 6 and the EDT data on DTCH? | --> | MAC PDU (RRC: RRCConnectionResumeRequest-NB, EDT data on DTCH) | - | - |
| 11 | The SS transmits an RRCConnectionResume-NB message. | <-- | RRC: RRCConnectionResume-NB | - | - |
| 12 | Check: Does the UE transmit an RRCConnectionResumeComplete-NB message? | --> | RRC: RRCConnectionResumeComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 3 | P |
| 13 | The SS transmits a SERVICE ACCEPT message. |  | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |

22.1.4.3.3	Specific message contents

Table 22.1.4.3.3-1: SystemInformationBlockType2-NB (preamble and all steps of Table 22.1.4.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 ::= SEQUENCE { |  |  |  |
| nprach-Config-v1530 ::= SEQUENCE { |  |  |  |
| edt-Parameters-r15 ::= SEQUENCE { |  |  |  |
| edt-SmallTBS-Subset-r15 | Not Present |  |  |
| edt-TBS-InfoList-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| edt-SmallTBS-Enabled-r15[1] | False |  |  |
| edt-TBS-r15 | b504 |  |  |
| } |  |  |  |
| nprach-ParametersListEDT-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { |  |  |  |
| nprach-Parameters-r14 ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms5120 |  |  |
| nprach-StartTime-r14 | ms10 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-EDT-r15 | False |  |  |
| up-EDT-r15 | True |  |  |
| } |  |  |  |

Table 22.1.4.3.3-2: RRCConnectionRelease-NB (steps 1 and 6, table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | rrc-Suspend |  |  |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) | Any value; Different value in steps 1 and 6 |  |
| extendedWaitTime-r13 | Not present |  |  |
| redirectedCarrierInfo-r13 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| redirectedCarrierInfo-v1430 | Not present |  |  |
| extendedWaitTime-CPdata-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | NCC | As defined in TS 33.401 |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.4.3.3-3: RRCConnectionResumeRequest-NB (step 5, Table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | BIT STRING (SIZE(40)) | Same value as Step 1 |  |
| shortResumeMAC-I-r13 | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| resumeCause-r13 | mo-Data |  |  |
| earlyContentionResolution-r14 | Not checked |  |  |
| cqi-NPDCCH-r14 | Not checked |  |  |
| spare | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.4.3.3-4: RRCConnectionResumeRequest-NB (step 10, Table 22.1.4.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | BIT STRING (SIZE(40)) | Same value as Step 6 |  |
| shortResumeMAC-I-r13 | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| resumeCause-r13 | mo-Data |  |  |
| earlyContentionResolution-r14 | Not checked |  |  |
| cqi-NPDCCH-r14 | Not checked |  |  |
| spare | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |