3GPP TSG-RAN5 Meeting #104	R5-245256r2

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5343 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding new test case 22.1.3 NB-IoT / Control Plane CIoT Optimization / EDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the LS exchange between GSMA and RAN5 in R5-243449 and R5-243972 test coverage for Rel-15 NB-IoT/EDT was missing although there is industry requirement for it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test coverage for NB-IoT/EDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for NB-IoT/EDT will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.3 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.523-2 CR1463 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-245256->R5-245256r1 - aligned TP2 and test procedurewith figure 5.3.3.1-8 from TS 36.331. R5-245256r1->R5-245256r2 - adding NTN to title - adding reference from 36.321 - adding new TP - update pre-test conditions - adding step to check RACH resource selection MAC PDU - some editorial changes |  |  |  |  |  |  |  |  |

### 22.1.3	NB-IoT / NTN / Control Plane CIoT Optimization / EDT

#### 22.1.3.1	Test Purpose (TP)

(1)

with { NB-IoT UE using the Control Plane CIoT Optimization in RRC_IDLE state }

ensure that {

when { EDT data transfer is triggered }

then { the UE selects a resource associated with EDT to initiate the RRC connection establishment

}

(2)

with { NB-IoT UE using the Control Plane CIoT Optimization in RRC_IDLE state }

ensure that {

when { EDT data transfer is triggered }

then { initiate the RRC connection establishment by transmitting RRCEarlyDataRequest-NB }

}

(3)

with { NB-IoT UE using the Control Plane CIoT Optimization and having transmitted RRCEarlyDataRequest-NB }

ensure that {

when { SS sends RRCConnectionSetup-NB}

then { the UE replies with RRRCConnectionSetupComplete-NB and successfully completest the data transfer procedure }

}

#### 22.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.321 Clause 5.1.2,TS 36.331, clauses 5.3.3.1, 5.3.3.3b and TS 24.301 clause 5.6.1.2.2. Unless otherwise stated these are Rel-15 requirements.

[TS 36.321, clause 5.1.2]

The Random Access Resource selection procedure shall be performed as follows:

-	for BL UEs or UEs in enhanced coverage or NB-IoT UEs, if EDT is initiated by the upper layers:

-	if the message size (UL data available for transmission plus MAC header and, where required, MAC control elements) is larger than the TB size signalled in edt-TBS for the selected enhanced coverage level for EDT; or

-	if the PRACH resource associated with EDT for the selected enhanced coverage level is not available:

-	indicate to upper layers that EDT is cancelled;

-	for BL UEs or UEs in enhanced coverage, select the PRACH resource set corresponding to the selected enhanced coverage level. For EDT, the PRACH resource set shall correspond to the set associated with EDT for the selected enhanced coverage level.

-	if, except for NB-IoT, ra-PreambleIndex (Random Access Preamble) and ra-PRACH-MaskIndex (PRACH Mask Index) have been explicitly signalled and ra-PreambleIndex is not 000000:

-	the Random Access Preamble and the PRACH Mask Index are those explicitly signalled;

-	else if, for NB-IoT, ra-PreambleIndex (Random Access Preamble) and PRACH resource have been explicitly signalled:

-	the PRACH resource is that explicitly signalled;

-	if the ra-PreambleIndex signalled is not 000000:

-	if ra-CFRA-Config is configured:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + nprach-NumCBRA-StartSubcarriers + (ra-PreambleIndex modulo (nprach-NumSubcarriers - nprach-NumCBRA-StartSubcarriers)), where nprach-SubcarrierOffset, nprach-NumCBRA-StartSubcarriers and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	the Random Access Preamble is set to nprach-SubcarrierOffset + (ra-PreambleIndex modulo nprach-NumSubcarriers), where nprach-SubcarrierOffset and nprach-NumSubcarriers are parameters in the currently used PRACH resource.

-	else:

-	select the Random Access Preamble group according to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group.

-	randomly select a Random Access Preamble within the selected group.

-	else the Random Access Preamble shall be selected by the MAC entity as follows:

-	if the UE is a BL UE or UE in enhanced coverage and EDT is initiated:

-	select the Random Access Preambles group corresponding to PRACH resource for EDT for the selected enhanced coverage level.

-	else if the UE is a BL UE or UE in enhanced coverage and Random Access Preamble group B does not exist:

-	select the Random Access Preambles group corresponding to the selected enhanced coverage level.

-	else if the UE is an NB-IoT UE:

-	randomly select one of the PRACH resources corresponding to the selected enhanced coverage level according to the configured probability distribution, and select the Random Access Preambles group corresponding to the PRACH resource and the support for multi-tone Msg3 transmission. A UE supporting multi-tone Msg3 shall only select the single-tone Msg3 Random Access Preambles group if there is no multi-tone Msg3 Random Access Preambles group. For EDT, the PRACH resource shall correspond to resource associated with EDT for the selected enhanced coverage level.

[TS 36.331, clause 5.3.3.1]

Figure 5.3.3.1-7: CP-EDT, successful

Figure 5.3.3.1-8: CP-EDT fallback to RRC connection establishment, successful

[TS 36.331, clause 5.3.3.3b]

The UE shall set the contents of RRCEarlyDataRequest message as follows:

1>	set the s-TMSI to the value received from upper layers;

1>	set the establishmentCause in accordance with the information received from upper layers;

1>	if the UE is a NB-IoT UE:

2>	if the UE supports DL channel quality reporting and cqi-Reporting is present in SystemInformationBlockType2-NB:

3>	set the cqi-NPDCCH to include the latest results of the downlink channel quality measurements of the serving cell as specified in TS 36.133 [16];

NOTE:	The downlink channel quality measurements may use measurement period T1 or T2, as defined in TS 36.133 [16]. In case period T2 is used the RRC-MAC interactions are left to UE implementation.

1>	set the dedicatedInfoNAS to include the information received from upper layers;

The UE shall configure the lower layers to use EDT and submit the RRCEarlyDataRequest message to the lower layers for transmission.

[TS 24.301, clause 5.6.1.2.2]

UE has pending IP or non-IP user data that is to be sent via the control plane radio bearers, the Control plane service type of the CONTROL PLANE SERVICE REQUEST message shall indicate "mobile originating request". The UE shall include an ESM DATA TRANSPORT message in the ESM message container IE. If the UE supports the CP-EDT (see 3GPP TS 36.300 [20]), the UE shall provide the CONTROL PLANE SERVICE REQUEST message in the NAS request to the lower layer to establish a RRC connection as specified in subclause 5.3.1.1.

#### 22.1.3.3	Test description

22.1.3.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 as specified in TS 36.508 clause 8.1.4.1.1 is configured.

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

-	px_DoAttachWithoutPDN set to false

Preamble:

-	The UE shall be in state 3-NB according to TS 36.508 [18].

22.1.3.3.2	Test procedure sequence

Table 22.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to send Early Data. NOTE: It is FFS how this action is performed. | - | - | - | - |
| 2 | The UE transmits preamble on PRACH resources associated with EDT | --> | PRACH Preamble | 1 | P |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit an RRCEarlyDataRequest-NB message including a CONTROL PLANE SERVICE REQUEST with ESM DATA TRANSPORT? | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 2 | P |
| 5 | The SS transmits an RRCEarlyDataComplete-NB message including a CONTROL PLAIN SERVICE ACCEPT message. | <-- | RRC : RRCEarlyDataComplete-NB NAS: CONTROL PLANE SERVICE ACCEPT | - | - |
| 6 | Trigger the UE to send Early Data. NOTE : It is FFS how this action is performed. | - | - | - | - |
| 7 | The UE transmits preamble on PRACH resources associated with EDT | --> | PRACH Preamble | 1 | P |
| 8 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 7, including TC-RNTI and not including Back off Indicator sub header. | <-- | Random Access Response | - | - |
| 9 | Check: Does the UE transmit an RRCEarlyDataRequest-NB message including a CONTROL PLANE SERVICE REQUEST with ESM DATA TRANSPORT? | --> | RRC : RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 10 | Execute steps 2 to 4 of test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport in 8.1.5A.3 in TS 36.508. | - | - | 3 | P |

22.1.3.3.3	Specific message contents

Table 22.1.3.3.3-1: SystemInformationBlockType2-NB (preamble and all steps of Table 22.1.3.3.2-1)

| Derivation Path: 36.508 clause 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 ::= SEQUENCE { |  |  |  |
| nprach-Config-v1530 ::= SEQUENCE { |  |  |  |
| edt-Parameters-r15 ::= SEQUENCE { |  |  |  |
| edt-SmallTBS-Subset-r15 | Not Present |  |  |
| edt-TBS-InfoList-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| edt-SmallTBS-Enabled-r15[1] | False |  |  |
| edt-TBS-r15 | b504 |  |  |
| } |  |  |  |
| nprach-ParametersListEDT-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { |  |  |  |
| nprach-Parameters-r14 ::= SEQUENCE { |  |  |  |
| nprach-Periodicity-r14 | ms5120 |  |  |
| nprach-StartTime-r14 | ms10 |  |  |
| nprach-SubcarrierOffset-r14 | n12 |  |  |
| nprach-NumSubcarriers-r14 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r14 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r14 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r14 | v4 |  |  |
| npdcch-Offset-RA-r14 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r14 | n8 |  |  |
| npdcch-CarrierIndex-r14 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-EDT-r15 | True |  |  |
| up-EDT-r15 | False |  |  |
| } |  |  |  |

Table 22.1.3.3.3-2: RRCEarlyDataRequest-NB (steps 4 and 9, table 22.1.3.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCEarlyDataRequest-NB-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcEarlyDataRequest-r15 SEQUENCE { |  |  |  |
| s-TMSI-r15 | Set to the value of the S-TMSI of the UE |  |  |
| establishmentCause-r15 | mo-Data |  |  |
| cqi-NPDCCH-r15 | Not checked |  |  |
| dedicatedInfoNAS-r15 | CONTROL PLANE SERVICE REQUEST | Table 22.1.3.3.3-3 |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.3.3.3-3: CONTROL PLANE SERVICE REQUEST (Table 22.1.3.3.3-2)

| Derivation Path: 36.508 table 4.7.2-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Data service type |  |  |  |
| Data service type value | Any allowed value |  |  |
| ESM message container | ESM DATA TRANSPORT |  |  |
| NAS message container | Not present |  |  |

Table 22.1.3.3.3-4: RRCEarlyDataComplete -NB (step 5, Table 22.1.3.3.2-1)

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCEarlyDataComplete-NB-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcEarlyDataComplete-r15 SEQUENCE { |  |  |  |
| dedicatedInfoNAS-r15 | CONTROL PLANE SERVICE ACCEPT message |  |  |
| extendedWaitTime-r15 | Not present |  |  |
| redirectedCarrierInfo-r15 | Not present |  |  |
| redirectedCarrierInfoExt-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |