3GPP TSG-RAN5 Meeting #104	R5-245292r2

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0047 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | min test time update for nb-ntn demod test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc, Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current minimum test time specified for NPDSCH and NPDCCH Demodulation test case is very large (> 1 hour) which leads to long test time. The minimum test time was not determined based on actual simulation results. Discussion paper R5-245291 contains the simulation results to come up with the min test time for these tests. Hence the Annex G min test time table needs to be updated. Below table with MNAS based on QC and MTK simulation results (R5-245291, R5-246051) for the 4 NB-NTN scenarios |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the minimum test time Annex G tables for the NPDSCH test cases based on simulation results provided in the discussion paper R5-245291, R5-246051 - Removed editor notes regarding minimum test time for the 3 NPDSCH test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will need to be run for a much longer time than necessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.1.1, 8.3.1.1.2, 8.3.1.1.3, Annex G Table G.3.5-1, Table G.3.5-2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on discussion paper R5-245291 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: swapped the WIC order in the coversheet. Removed the editor’s note for 8.3.1.1.1 since the test case is now complete Updated Annex G table for min test time based on QC simulation results. R2: - Added calculation for converting MNAS to MNSF for HD-FDD based on the provided simulation results by QC and MTK. - Added the “Min Gap between consecutive NPDCCH taking into account RTT” to indicate the minimum gap needed between consecutive NPDCCH. The actual gap between consecutive NPDCCH to be scheduled by the TE needs to align to the closest multiple of Rmax*G. - Removed TDD calculations from the Table G.3.5-1 and G.3.5-2 since the RAN4 requirements are only for HD-FDD configuration. |  |  |  |  |  |  |  |  |

##### 8.3.1.1.1	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions

8.3.1.1.1.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on one antenna ports.

8.3.1.1.1.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that supports satellite access operation and only NGSO or both GSO and NGSO. Test 2 requires support of non-anchor mode of operation.

8.3.1.1.1.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.1.3-2, with the addition of the parameters in Table 8.3.1.1.1.3-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the performance.

Table 8.3.1.1.1.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 128 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 256 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.1.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

The normative reference for this requirement is TS 36.102[11] clause 8.3.1.1.1.1.

<Several sections skipped>

##### 8.3.1.1.2	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2

8.3.1.1.2.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on one antenna ports.

8.3.1.1.2.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that supports satellite access operation.

8.3.1.1.2.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.2.3-2, with the addition of the parameters in Table 8.3.1.1.2.3-1 and the downlink physical channel setup according to Annex C.3.2. Using this configuration the fraction of maximum throughput percentage shall meet or exceed the minimum requirements specified in Table 8.3.1.1.2.3-2 for the specified SNR.

Table 8.3.1.1.2.3-1: Test Parameters for NPDSCH under standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -99 (Note 2) |
| NPDCCH repetition number |  | subframe | 32 for Test 1; 256 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 64 for Test 1; 512 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.2.3-2: Minimum performance under standalone

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.6 FDD (Note 2) | 32 | EPA5 | 1 | 70% | -3.4 | NB1, NB2 |
| 2 (Note 1) | 200kHz | Non-anchor | R.NB.6-1 FDD (Note 2) | 256 | ETU1 | 1 | 70% | -10.2 | NB1, NB2 |
| Note 1: Applicable to UE supporting Non-Anchor mode of operation. Note 2: R.NB.6 FDD has the same parameters with R.NB.1 FDD in Table A.3.12.1.1-1. R.NB.6-1 FDD has the same parameters with R.NB.2 FDD in Table A.3.12.1.1-1. |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.1.1.2.

<Several sections skipped>

##### 8.3.1.1.3	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2

8.3.1.1.3.1	Test purpose

To verify the UE's ability to receive a predefined test signal, representing a multi-path fading channel that is determined by the SNR with a percentage of the information bit throughput for a specified downlink Reference Measurement Channel (RMC) not falling below a specified value for transmission on one antenna port.

8.3.1.1.3.2	Test applicability

This test applies to all types of NB-IoT FDD UE release 17 and forward of category NB2 that supports satellite access operation.

8.3.1.1.3.3	Minimum conformance requirements

The requirements are specified in Table 8.3.1.1.3.3-2, with the addition of the parameters in Table 8.3.1.1.3.3-1 and the downlink physical channel setup according to Annex C.3.2. Using this configuration the fraction of maximum throughput percentage shall meet or exceed the minimum requirements specified in Table 8.3.1.1.3.3-2 for the specified SNR.

Table 8.3.1.1.3.3-1: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | -93 (Note 1) |
|  |  | dBm/15kHz | -96 (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1 |
| (npdcch-NumRepetitions-r13) |  | subframe | 8 for Test 1 |
| (nPDCCH-startSF-USS-r13) |  |  | 1.5 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission; Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 8.3.1.1.3.3-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Non-anchor | R.NB.7 FDD | 1 | EPA5 | 1 | 70% | 9.4 | NB2 |

The normative reference for this requirement is TS 36.101 [2] clause 8.12.1.1.3.

<Several sections skipped>

Table G.3.5-1: Minimum Test time for PDSCH for UE Category NB1

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | Min Gap between consecutive NPDCCH taking into account RTT (ms) | Actual Gap between consecutive NPDCCH taking into account RTT (ms) | MNSF (Min No Sub frames) HD-FDD (Note2) |
| --- | --- | --- | --- | --- | --- |
| 8.3.1.1.1 Test 1 | R.NB.1 FDD (200kHz, QPSK,1/2) (1x1) NTN TDLC5-200 | 845 (Note 1) | 239 | 288 | 243360 |
| 8.3.1.1.1 Test 2 | R.NB.2 FDD (200kHz, QPSK,1/3) (1x1) NTN TDLA100-10 | 523 (Note 1) | 879 | 1152 | 602496 |
| 8.3.1.1.2 Test 1 | R.NB.6 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 683 (Note 1) | 487 | 576 | 393408 |
| 8.3.1.1.2 Test 2 | R.NB.6-1 FDD (200kHz, QPSK,1/2) (1x1) ETU1 | 440 (Note 1) | 1980 | 2304 | 1013760 |
| Note1: Minimum test time has been derived by simulating the test scenario taking into account the minimum scheduling gap needed between successive NPDCCH/NPDSCH Note2: For MNSF calculations, with the current assumption of SIB2/SIB31 scheduling every 640ms with max 8 subframes and max 2 repetition per SI window, 16 additional subframes are needed every 640ms to avoid collisions with NPDSCH. |  |  |  |  |  |

Table G.3.5-2: Minimum Test time for PDSCH for UE Category NB2

| Test No | Demodulation scenario (info only) | MNAS (Simulation) | MNSF (Min No Sub Frames, mandatory) HD-FDD |  |
| --- | --- | --- | --- | --- |
| 8.3.1.1.3 | R.NB.7 FDD (200kHz, QPSK,1/2) (1x1) EPA5 | 3609 | 44000 |  |