3GPP TSG-RAN5 Meeting #104	R5-245293r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1485 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SIB31 update for RF and DEMOD test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc, Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Ul-SyncValidityDuration for RF NB-NTN test cases are currently set to s240. The intention of this IE is to let the UE know the validity duration of the satellite ephemeris data. In RF, DEMOD and RRM test cases, the satellite ephemeris data never changes during the entire duration of the test. Hence this IE value can be set to the max allowed value TS 36.331 ul-SyncValidityDuration-r17 ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900}, ul-SyncValidityDuration Validity duration of the satellite ephemeris data and common TA parameters, i.e. maximum time duration (from epochTime) during which the UE can apply the satellite ephemeris without acquiring new satellite ephemeris, see TS 36.213 [23]. Unit in second. Value s5 corresponds to 5 seconds, value s10 corresponds to 10 seconds and so on. The ul-SyncValidityDuration is only updated when at least one of epochTime, nta-CommonParameters, ephemerisInfo is updated. - To reduce the current long test time for Demodulation test cases, one way is to choose the largest SIB periodicity to reduce collisions with NPDCCH/NPDSCH scheduling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Setting ul-SyncValidityDuration-r17 IE value to s900 for RF,Demod and RRM test cases - Updated SIB periodicity to 4096 frames just during measurement period. No change to SIB periodicity during initial attach procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unnecessarily lowering the validity duration of the satellite ephemeris data. - Unnecessary long test time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: swapped the order of the WIC in the coversheet. Updated SIB2/31 periodicity during measurement period for RF test cases |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

##### 8.2.2.1.3	NB-IoT Common contents of system information messages

MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1.

SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

SystemInformationBlockType31-NB

Table 8.2.2.1.3-1: SystemInformationBlockType31-NB

| Derivation Path: TS 36.508 clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | See cl. 8.2.6.2 |  |  |
| positionY-r17 | See cl. 8.2.6.2 |  |  |
| positionZ-r17 | See cl. 8.2.6.2 |  |  |
| velocityVX-r17 | See cl. 8.2.6.2 |  |  |
| velocityVY-r17 | See cl. 8.2.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.2.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s900 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

<<< End OF CHANGES 1>>>

<<< START OF CHANGES 2>>>

#### 8.2.2.1	NB-IoT Common parameters for simulated E-UTRA cells

##### 8.2.2.1.1		NB-IoT Combinations of system information blocks

The NB-IoT combination of system information blocks required by a test case depends on the test case scenario. In clause 8.1.4 of this document, the NB-IoT combinations of system information blocks are defined.

Combination 1, defined in clause 8.1.4, is used by default in test cases defined in TS 36.521-1 [21].

##### 8.2.2.1.2	NB-IoT Scheduling of system information blocks

Same NB-IoT scheduling of system information blocks as defined in clause 8.1.4.3.1.2 with the following exception.

Table 8.1.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB | During attach procedure |
|  | 4096 | every8thRF | 552 | SIB2-NB | During measurement period |
| 2 | 64 | every16thRF | 256 | SIB31-NB | During attach procedure |
|  | 4096 | every8thRF | 256 | SIB31-NB | During measurement period |

##### 8.2.2.1.3	NB-IoT Common contents of system information messages

MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1.

SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

SystemInformationBlockType31-NB

Table 8.2.2.1.3-1: SystemInformationBlockType31-NB

| Derivation Path: TS 36.508 clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | See cl. 8.2.6.2 |  |  |
| positionY-r17 | See cl. 8.2.6.2 |  |  |
| positionZ-r17 | See cl. 8.2.6.2 |  |  |
| velocityVX-r17 | See cl. 8.2.6.2 |  |  |
| velocityVY-r17 | See cl. 8.2.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.2.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |