3GPP TSG-RAN5 Meeting #104	R5-245303r2

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0878 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to PDSCH CRS interference mitigation under NR-LTE coexistence test cases 5.2.2.1.18, 5.2.2.2.19, 5.2.3.1.17, 5.2.3.2.18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PDSCH CRS interference mitigation under NR-LTE coexistence test cases 5.2.2.1.18, 5.2.2.2.19, 5.2.3.1.17, 5.2.3.2.18 are incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor’s Note removed for Clauses 5.2.2.1.18, 5.2.2.2.19, 5.2.3.1.17, 5.2.3.2.18 - Annex F updated for 5.2.2.1.18, 5.2.2.2.19, 5.2.3.1.17, 5.2.3.2.18 - Annex G updated for R.PDSCH.1-7.3 FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.3, Annex F, G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated TT analysis is in R5-245304 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated Message exceptions Removed editor’s note since test case is now complete R2: Added the associated TT analysis Tdoc# |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 5.2.2.1.18	2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.2.1.18.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.1.18.0-4, with the addition of test parameters in Tables 5.2.2.1.18.0-2 and 5.2.2.1.18.0-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.18.0-1.

Table 5.2.2.1.18.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell | 1-1 |

Table 5.2.2.1.18.0-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 2 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier. |  |  |  |

Table 5.2.2.1.18.0-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.2.1.18.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.9 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.18.

###### 5.2.2.1.18_1	2Rx	FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both NSA and SA

5.2.2.1.18_1.1	Test purpose

To verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell.

5.2.2.1.18_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support CRS-IM-DSS-15kHzSCS-r17.

5.2.2.1.18_1.3	Test description

5.2.2.1.18_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.2.1.18.0-2 and 5.2.2.1.18.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.18_1.3.3.

5.2.2.1.18_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.2.1.18_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.2.1.18_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.2.2.1.18_1.4-2 as appropriate.

5.2.2.1.18_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.1.18_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.1.18_1.3.3_1-1: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.1.18_1.3.3_1-2: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n2 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

Table 5.2.2.1.18_1.3.3_1-3: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| frequencyShift7p5khz | true |  |  |
| } |  |  |  |

5.2.2.1.18_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.18_1.3.3_1

5.2.2.1.18_1.4	Test requirement

Tables 5.2.2.1.18_1.4-1 and 5.2.2.1.18_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.18_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.18_1.4-1: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 11.25 | 5.4 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.2.1.18_1.4-2: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 13.7 |

##### 5.2.2.1.19	2Rx FDD FR1 for PDSCH with inter cell CRS interference

5.2.2.1.19.0	Minimum conformance requirements

The performance requirements are specified in Tables 5.2.2.1.19.0-4 and 5.2.2.1.19.0-6, with the addition of test parameters in Tables 5.2.2.1.19.0-2 and 5.2.2.1.19.0-3 and the downlink physical channel setup according to Annex C.3.1.

The requirements for UE capable of performing CRS-IM with the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.2.1.19.0-4.

The requirements for UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.2.1.19.0-6.

The test purposes are specified in Table 5.2.2.1.19.0-1.

Table 5.2.2.1.19.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1, 2-1 |

Table 5.2.2.1.19.0-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.19.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD | FDD |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth (Note 5) |  | MHz | 20 | 20 |
| Carrier centre subcarrier location (Note 6) |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame Note 5: This parameter is informed to UE via network assistance signalling for Test 1-1 in Table 5.2.2.1.19.0-4. Note 6: Single entry is included in IE LTE-NeighCellsCRS-AssistInfoList-r17 that applies for both cells for cases |  |  |  |  |

Table 5.2.2.1.19.0-4: Minimum performance for Rank 1 with the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-18.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x2, ULA Low | 70 | 11.9 |

Table 5.2.2.1.19.0-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | 7 |
| Measurement gap timing advance | ms | 0 |

Table 5.2.2.1.19.0-6: Minimum performance for Rank 1 without the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-17.2 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x2, ULA Low | 70 | 11.9 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.19.

###### 5.2.2.1.19_1	2Rx FDD FR1 for PDSCH with inter cell CRS interference scenario – 4x2 MIMO for both NSA and SA

5.2.2.1.19_1.1	Test purpose

To verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal.

5.2.2.1.19_1.2	Test applicability

Test 2-1 applies to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-15kHzSCS-r17.

Test 1-1 applies to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-NWA-15kHzSCS-r17.

5.2.2.1.19_1.3	Test description

5.2.2.1.19_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.8 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.2.1.19.0-2 and 5.2.2.1.19_1.4-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.19_1.3.3.

6.  For UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth, ensure the network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations according to Table 5.2.2.1.19.0-5. PDSCH is not scheduled, and throughput is not counted during 4.64s after the beginning of test. PDSCH is not scheduled in the measurement gaps.

5.2.2.1.19_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.19_1.4-2 and 5.2.2.1.19_1.4-3 as appropriate. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.19_1.4-2 and 5.2.2.1.19_1.4-3 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.19_1.4-2 and 5.2.2.1.19_1.4-3 as appropriate.

5.2.2.1.19_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.1.19_1.3.3_1	Message exceptions for SA

As defined in clause 4.6.3 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.1.19_1.3.3_1-1: LTE-NeighCellsCRS-AssistInfoList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-67A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17 { | 1 entry |  | Test 1-1 |
| LTE-NeighCellsCRS-AssistInfo-r17[1] SEQUENCE { |  |  |  |
| neighCarrierBandwidthDL-r17 | n100 |  |  |
| neighNrofCRS-Ports-r17 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.19_1.3.3_1-2: MeasGapConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  | Test 2-1 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 7 |  |  |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.2.2.1.19_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.19_1.3.3_1

<< Skipped sections >>

##### 5.2.2.2.19	2Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.2.2.19.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.19.0-4, with the addition of test parameters in Tables 5.2.2.2.19.0-2 and 5.2.2.2.19.0-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.19.0-1.

Table 5.2.2.2.19.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell | 1-1 |

Table 5.2.2.2.19.0-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate Matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | Hz | 20 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier |  |  |  |

Table 5.2.2.2.19.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 1) |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Defined in B.6.1 Note 2: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

Table 5.2.2.2.19.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.5 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.2.19.

###### 5.2.2.2.19_1	2Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x2 MIMO for both NSA and SA

5.2.2.2.19_1.1	Test purpose

To verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell.

5.2.2.2.19_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support CRS-IM-DSS-15kHzSCS-r17.

5.2.2.2.19_1.3	Test description

5.2.2.2.19_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.2.2.19.0-2 and 5.2.2.2.19.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.2.19_1.3.3.

5.2.2.2.19_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.2.2.19_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.2.2.19_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.2.2.2.19_1.4-2 as appropriate.

5.2.2.2.19_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.2.19_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.2.19_1.3.3_1-1: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.2.19_1.3.3_1-2: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n100 | 20MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

5.2.2.2.19_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.2.19_1.3.3_1

5.2.2.2.19_1.4	Test requirement

Tables 5.2.2.2.19_1.4-1 and 5.2.2.2.19_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.19_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.19_1.4-1: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 1) |  | dB | 11.25 | 5.4 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Defined in B.6.1 Note 2: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

Table 5.2.2.2.19_1.4-2: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 14.3 |

##### 5.2.2.2.20	2Rx TDD FR1 for PDSCH with inter cell CRS interference

5.2.2.2.20.0	Minimum conformance requirements

The performance requirements are specified in Tables 5.2.2.2.20.0-4 and 5.2.2.2.20.0-6, with the addition of test parameters in Tables 5.2.2.2.20.0-2 and 5.2.2.2.20.0-3 and the downlink physical channel setup according to Annex C.3.1.

The requirements for UE capable of performing CRS-IM with the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.2.2.20.0-4.

The requirements for UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.2.2.20.0-6.

The test purposes are specified in Table 5.2.2.2.20.0-1.

Table 5.2.2.2.20.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1, 1-2, 2-1, 2-2 |

Table 5.2.2.2.20.0-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.20.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 5) |  | dB | 10.45 | 4.6 |
| LTE Bandwidth (Note 6) |  | MHz | 20 | 20 |
| Carrier centre subcarrier location (Note 7) |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-2. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-2. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 for test 1-1 1.5 for test 1-2 | -1 for test 1-1 -0.5 for test 1-2 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame Note 5: Defined in B.6.1 Note 6: This parameter is informed to UE via network assistance signalling for Test 1-1 and 1-2 in Table 5.2.2.2.20.0-4. Note 7 Single entry is included in IE LTE-NeighCellsCRS-AssistInfoList-r17 that applies for both cells for cases with network signalling assistance |  |  |  |  |

Table 5.2.2.2.20.0-4: Minimum performance for Rank 1 with the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-4.1 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.3 |
| 1-2 | R.PDSCH.2-28.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | 11.7 |

Table 5.2.2.2.20.0-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length (mgl) | ms | 6 |
| Measurement Gap Repetition Period (mgrp) | ms | 40 |
| Gap offset (gapoffset) | ms | 1 |
| Measurement gap timing advance (mgta) | ms | 0 |

Table 5.2.2.2.20.0-6: Minimum performance for Rank 1 without the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-4.2 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.3 |
| 2-2 | R.PDSCH.2-29.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | 11.7 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.2.20.

###### 5.2.2.2.20_1	2Rx TDD FR1 for PDSCH with inter cell CRS interference scenario – 4x2 MIMO for both NSA and SA

5.2.2.2.20_1.1	Test purpose

To verify PDSCH performance under 2 receive antenna conditions when PDSCH is interfered by inter cell CRS signal.

5.2.2.2.20_1.2	Test applicability

Tests 2-1 and 2-2 apply to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-15kHzSCS-r17 and/or crs-IM-nonDSS-30kHzSCS-r17.

Tests 1-1 and 1-2 apply to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-NWA-15kHzSCS-r17 and/or crs-IM-nonDSS-NWA-30kHzSCS-r17.

5.2.2.2.20_1.3	Test description

5.2.2.2.20_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.8 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.2.2.20.0-2 and 5.2.2.2.20_1.4-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.2.20_1.3.3.

6.	For UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth, ensure the network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations according to Table 5.2.2.2.20.0-5. PDSCH is not scheduled, and throughput is not counted during 4.64s after the beginning of test. PDSCH is not scheduled in the measurement gaps.

5.2.2.2.20_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.2.20_1.4-2 and 5.2.2.2.20_1.4-3 as appropriate. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.2.20_1.4-2 and 5.2.2.2.20_1.4-3 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.2.20_1.4-2 and 5.2.2.2.20_1.4-3 as appropriate.

5.2.2.2.20_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.2.20_1.3.3_1	Message exceptions for SA

As defined in clause 4.6.3 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.2.20_1.3.3_1-1: LTE-NeighCellsCRS-AssistInfoList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-67A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17 { | 1 entry |  | Tests 1-1 and 1-2 |
| LTE-NeighCellsCRS-AssistInfo-r17[1] SEQUENCE { |  |  |  |
| neighCarrierBandwidthDL-r17 | n100 |  |  |
| neighNrofCRS-Ports-r17 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.20_1.3.3_1-2: MeasGapConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  | Tests 2-1 and 2-2 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 1 |  |  |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.2.2.2.20_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.2.20_1.3.3_1

5.2.2.2.20_1.4	Test requirement

<< Skipped sections >>

##### 5.2.3.1.17	4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.3.1.17.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.1.17.0-4, with the addition of test parameters in Table 5.2.3.1.17.0-2 for the serving cell and Table 5.2.3.1.17.0-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.17.0-1.

Table 5.2.3.1.17.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 4 receive antenna conditions with CRS rate matching configured for the serving cell. | 1-1 |

Table 5.2.3.1.17.0-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 2 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier. |  |  |  |

Table 5.2.3.1.17.0-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.3.1.17.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 8.0 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.3.1.17.

###### 5.2.3.1.17_1	4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x4 MIMO for both NSA and SA

5.2.3.1.17_1.1	Test purpose

To verify PDSCH CRS interference mitigation performance under 4 receive antenna conditions with CRS rate matching configured for the serving cell.

5.2.3.1.17_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support CRS-IM-DSS-15kHzSCS-r17.

5.2.3.1.17_1.3	Test description

5.2.3.1.17_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.3.1.17.0-2 and 5.2.3.1.17.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.1.17_1.3.3.

5.2.3.1.17_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.3.1.17_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.3.1.17_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.2.3.1.17_1.4-2 as appropriate.

5.2.3.1.17_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.3.1.17_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.3.1.17_1.3.3_1-1: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.3.1.17_1.3.3_1-2: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n2 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

Table 5.2.3.1.17_1.3.3_1-3: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| frequencyShift7p5khz | true |  |  |
| } |  |  |  |

5.2.3.1.17_1.3.3_2	Message exceptions for NSA

Same as 5.2.3.1.17_1.3.3_1

5.2.3.1.17_1.4	Test requirement

Tables 5.2.3.1.17_1.4-1 and 5.2.3.1.17_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.3.1.17_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.3.1.17_1.4-1: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 11.25 | 5.4 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.3.1.17_1.4-2: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 9.8 |

##### 5.2.3.1.18	4Rx FDD FR1 for PDSCH with inter cell CRS interference

5.2.3.1.18.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.1.18.0-4 and Table 5.2.3.1.18.0-6, with the addition of test parameters in Table 5.2.3.1.18.0-2 and 5.2.3.1.18.0-3 and the downlink physical channel setup according to Annex C.3.1.

The requirements for UE capable of performing CRS-IM with the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.3.1.18.0-4.

The requirements for UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.3.1.18.0-6.

The test purposes are specified in Table 5.2.3.1.18.0-1.

Table 5.2.3.1.18.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1 and 2-1 |

Table 5.2.3.1.18.0-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.3.1.18.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD | FDD |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth (Note 5) |  | MHz | 20 | 20 |
| Carrier centre subcarrier location (Note 6) |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame Note 5: This parameter is informed to UE via network assistance signalling for Test 1-1 in Table 5.2.3.1.18.0-4. Note 6: Single entry is included in IE LTE-NeighCellsCRS-AssistInfoList-r17 that applies for both cells for cases with network signalling assistance |  |  |  |  |

Table 5.2.3.1.18.0-4: Minimum performance for Rank 1 with the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-18.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 8.0 |

Table 5.2.3.1.18.0-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length | ms | 6 |
| Measurement Gap Repetition Period | ms | 40 |
| Gap offset | ms | 7 |
| Measurement gap timing advance | ms | 0 |

Table 5.2.3.1.18.0-6: Minimum performance for Rank 1 without the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-17.2 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 4x4, ULA Low | 70 | 8.0 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.3.1.18.

###### 5.2.3.1.18_1	4Rx FDD FR1 for PDSCH with inter cell CRS interference scenario – 4x4 MIMO for both NSA and SA

5.2.3.1.18_1.1	Test purpose

To verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal.

5.2.3.1.18_1.2	Test applicability

Test 2-1 applies to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-15kHzSCS-r17.

Test 1-1 applies to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-NWA-15kHzSCS-r17.

5.2.3.1.18_1.3	Test description

5.2.3.1.18_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.5 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.3.1.18.0-2 and 5.2.3.1.18_1.4-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.1.18_1.3.3.

6.	For UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth, ensure the network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations according to Table 5.2.3.1.18.0-5. PDSCH is not scheduled, and throughput is not counted during 4.64s after the beginning of test. PDSCH is not scheduled in the measurement gaps.

5.2.3.1.18_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.3.1.18_1.4-2 and 5.2.3.1.18_1.4-3 as appropriate. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.3.1.18_1.4-2 and 5.2.3.1.18_1.4-3 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.3.1.18_1.4-2 and 5.2.3.1.18_1.4-3 as appropriate.

5.2.3.1.18_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.3.1.18_1.3.3_1	Message exceptions for SA

As defined in clause 4.6.3 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.3.1.18_1.3.3_1-1: LTE-NeighCellsCRS-AssistInfoList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-67A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17 { | 1 entry |  | Test 1-1 |
| LTE-NeighCellsCRS-AssistInfo-r17[1] SEQUENCE { |  |  |  |
| neighCarrierBandwidthDL-r17 | n100 |  |  |
| neighNrofCRS-Ports-r17 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.1.18_1.3.3_1-2: MeasGapConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  | Test 2-1 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 7 |  |  |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.2.3.1.18_1.3.3_2	Message exceptions for NSA

Same as 5.2.3.1.18_1.3.3_1

5.2.3.1.18_1.4	Test requirement

<< Skipped sections >>

##### 5.2.3.2.18	4Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.3.2.18.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.2.18.0-4, with the addition of test parameters in Table 5.2.3.2.18.0-2 for the serving cell and Table 5.2.3.2.18.0-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.18.0-1.

Table 5.2.3.2.18.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 4 receive antenna conditions with CRS rate matching configured for the serving cell. | 1-1 |

Table 5.2.3.2.18.0-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate Matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | Hz | 20 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier |  |  |  |

Table 5.2.3.2.18.0-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 1) |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Defined in B.6.1 Note 2: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

Table 5.2.3.2.18.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | 8.8 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.3.2.18.

###### 5.2.3.2.18_1	4Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x4 MIMO for both NSA and SA

5.2.3.2.18_1.1	Test purpose

To verify PDSCH CRS interference mitigation performance under 4 receive antenna conditions with CRS rate matching configured for the serving cell.

5.2.3.2.18_1.2	Test applicability

This test applies to all types of NR UE release 17 and forward that support CRS-IM-DSS-15kHzSCS-r17.

5.2.3.2.18_1.3	Test description

5.2.3.2.18_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.3.2.18.0-2 and 5.2.3.2.18.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.2.18_1.3.3.

5.2.3.2.18_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.3.2.18_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.3.2.18_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Table 5.2.3.2.18_1.4-2 as appropriate.

5.2.3.2.18_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.3.2.18_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.2.19_1.3.3_1-1: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.2.19_1.3.3_1-2: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n100 | 20MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

5.2.3.2.18_1.3.3_2	Message exceptions for NSA

Same as 5.2.3.2.18_1.3.3_1

5.2.3.2.18_1.4	Test requirement

Tables 5.2.3.2.18_1.4-1 and 5.2.3.2.18_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.3.2.18_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.3.2.18_1.4-1: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 1) |  | dB | 11.25 | 5.4 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Defined in B.6.1 Note 2: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

Table 5.2.3.2.18_1.4-2: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | 10.6 |

##### 5.2.3.2.19	4Rx TDD FR1 for PDSCH with inter cell CRS interference

5.2.3.2.19.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.2.19.0-4 and Table 5.2.3.2.19.0-6, with the addition of test parameters in Table 5.2.3.2.19.0-2 for the serving cell and Table 5.2.3.2.19.0-3 for the LTE interference cells and the downlink physical channel setup according to Annex C.3.1.

The requirements for UE capable of performing CRS-IM with the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.3.2.19.0-4.

The requirements for UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth are specified in Table 5.2.3.2.19.0-6.

The test purposes are specified in Table 5.2.3.2.19.0-1.

Table 5.2.3.2.19.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal | 1-1, 1-2, 2-1 and 2-2 |

Table 5.2.3.2.19.0-2: Tests parameter for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD UL-DL pattern |  |  | FR1.15-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.3.2.19.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR |  | dB | 10.45 | 4.6 |
| LTE Bandwidth (Note 5) |  | MHz | 20 | 20 |
| Carrier centre subcarrier location (Note 6) |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-2. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-1. 10% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain for test 1-2. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 for test 1-1 1.5 for test 1-2 | -1 for test 1-1 -0.5 for test 1-2 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: No MBSFN is configured on LTE carrier. Note 3: Network-based CRS interference mitigation is disabled on LTE carrier. Note 4: The start of transmission of LTE frame is delayed by 2 LTE subframes with respect to the start of transmission of NR frame Note 5: This parameter is informed to UE via network assistance signalling for Test 1-1 and 1-2 in Table 5.2.3.2.19.0-4. Note 6: Single entry is included in IE LTE-NeighCellsCRS-AssistInfoList-r17 that applies for both cells for cases with network signalling assistance |  |  |  |  |

Table 5.2.3.2.19.0-4 Minimum performance for Rank 1 with the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-4.1 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | 8.6 |
| 1-2 | R.PDSCH.2-28.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 8.2 |

Table 5.2.3.2.19.0-5: Measurement Gap configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| Measurement Gap Length (mgl) | ms | 6 |
| Measurement Gap Repetition Period (mgrp) | ms | 40 |
| Gap offset (gapoffset) | ms | 1 |
| Measurement gap timing advance (mgta) | ms | 0 |

Table 5.2.3.2.19.0-6: Minimum performance for Rank 1 without the assistance of network signalling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-4.2 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x4, ULA Low | 70 | 8.6 |
| 2-2 | R.PDSCH.2-29.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x4, ULA Low | 70 | 8.2 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.3.2.19.

###### 5.2.3.2.19_1	4Rx TDD FR1 for PDSCH with inter cell CRS interference scenario – 4x4 MIMO for both NSA and SA

5.2.3.2.19_1.1	Test purpose

To verify PDSCH performance under 4 receive antenna conditions when PDSCH is interfered by inter cell CRS signal.

5.2.3.2.19_1.2	Test applicability

Tests 2-1 and 2-2 apply to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-15kHzSCS-r17 and/or crs-IM-nonDSS-30kHzSCS-r17.

Tests 1-1 and 1-2 apply to all types of NR UE release 17 and forward that support CRS-IM-nonDSS-NWA-15kHzSCS-r17 and/or crs-IM-nonDSS-NWA-30kHzSCS-r17.

5.2.3.2.19_1.3	Test description

5.2.3.2.19_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.5 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Tables 5.2.3.2.19.0-2 and 5.2.3.2.19_1.4-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.2.19_1.3.3.

6.  For UE capable of performing CRS-IM without the assistance of network signalling on LTE channel bandwidth, ensure the network configures an inter-RAT LTE measurement object of the interfering cells to the tested UE. Inter-RAT measurement is configured at the beginning of the test and applied throughout the test with gap pattern configurations according to Table 5.2.3.2.19.0-5. PDSCH is not scheduled, and throughput is not counted during 4.64s after the beginning of test. PDSCH is not scheduled in the measurement gaps.

5.2.3.2.19_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.3.2.19_1.4-2 and 5.2.3.2.19_1.4-3 as appropriate. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.3.2.19_1.4-2 and 5.2.3.2.19_1.4-3 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.3.2.19_1.4-2 and 5.2.3.2.19_1.4-3 as appropriate.

5.2.3.2.19_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.3.2.19_1.3.3_1	Message exceptions for SA

As defined in clause 4.6.3 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.3.2.19_1.3.3_1-1: LTE-NeighCellsCRS-AssistInfoList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-67A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTE-NeighCellsCRS-AssistInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofCRS-IM-InterfCell-r17)) OF LTE-NeighCellsCRS-AssistInfo-r17 { | 1 entry |  | Tests 1-1 and 1-2 |
| LTE-NeighCellsCRS-AssistInfo-r17[1] SEQUENCE { |  |  |  |
| neighCarrierBandwidthDL-r17 | n100 |  |  |
| neighNrofCRS-Ports-r17 | n4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.3.2.19_1.3.3_1-2: MeasGapConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  | Tests 2-1 and 2-2 |
| gapUE CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset | 1 |  |  |
| mgl | ms6 |  |  |
| mgrp | ms40 |  |  |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.2.3.2.19_1.3.3_2	Message exceptions for NSA

Same as 5.2.3.2.19_1.3.3_1

5.2.3.2.19_1.4	Test requirement

<< Skipped sections >>

# F.1	Measurement uncertainties and test tolerances for FR1

## F.1.1	Acceptable uncertainty of test system (normative)

The maximum acceptable uncertainty of the Test System is specified below for each test, where appropriate. The Test System shall enable the stimulus signals in the test case to be adjusted to within the specified range, and the equipment under test to be measured with an uncertainty not exceeding the specified values. All ranges and uncertainties are absolute values, and are valid for a confidence level of 95 %, unless otherwise stated.

A confidence level of 95 % is the measurement uncertainty tolerance interval for a specific measurement that contains 95 % of the performance of a population of test equipment.

For RF tests it should be noted that the uncertainties in clause F.1 apply to the Test System operating into a nominal 50 ohm load and do not include system effects due to mismatch between the DUT and the Test System.

The downlink signal uncertainties apply at each receiver antenna connector.

### F.1.1.1	Measurement of test environments

The measurement accuracy of the UE test environments defined in TS 38.508-1 [5] subclause 4.1, Test environments shall be

-	Pressure				5 kPa.

-	Temperature			2 degrees.

-	Relative Humidity	5 %.

-	DC Voltage			1,0 %.

-	AC Voltage			1,5 %.

-	Vibration				10 %.

-	Vibration frequency	0,1 Hz.

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

### F.1.1.2	Measurement of Demod Performance requirements

This clause defines the maximum test system uncertainty for Demod Performance requirements. The maximum test system uncertainty allowed for the measurement uncertainty contributors are defined in Table F.1.1.2-1.

Table F.1.1.2-1: Maximum measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Channel BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWconfig | dB | ±2.0 | Same as in LTE |
| Signal to noise ratio uncertainty | dB | ±0.3 | Same as in LTE |
| Signal to noise ratio variation | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 1Tx | dB | ±0.5 | Same as in LTE |
| Fading profile power uncertainty for 2Tx | dB | ±0.7 | Same as in LTE |
| Fading profile power uncertainty for 4Tx | dB | ±0.7 | Same as in LTE |

The maximum test system uncertainty for test cases defined in section 5 is defined in Table F.1.1.2-2.

Table F.1.1.2-2: Maximum test system uncertainty for FR1 demodulation performance test cases

| Subclause |  |  |  |  | Maximum Test System Uncertainty |  |  |  |  | Derivation of Test System Uncertainty |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler ± 0.6 dB for test 1-6 ± 0.9 dB for test 1-7 |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) + SNR uncertainty due to finite test time2 Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB For test point 1-6, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For test point 1-7, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |  |  |  |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.1_3 2Rx FDD FR1 PDSCH Mapping Type A performance – 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-8 Downlink EVM ≤ 2.5% for f ≤ 4.2 GHz Downlink EVM ≤ [2.5%] for f > 4.2 GHz |  |  |  |  | For test point 1-8, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 ) = 0.9 dB 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is not limited by the Test system EVM. |  |  |  |  |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.25 dB for 1% residual BLER For test point 1-1, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB |  |  |  |  |
| 5.2.2.1.7_1 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |  |  |  |  |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty, ±0.3 dB 2. Effect of AWGN flatness and signal flatness, ±2.0 dB 3. SNR uncertainty due to finite test time, ±0.0 dB for >10Hz Doppler. Items 1, 2, 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB |  |  |  |  |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.1.15_1 2Rx FDD FR1 PDSCH with inter-cell interference – 2x2 MIMO for both SA and NSA |  |  |  |  | Es/Noc ± 0.8 dB for interfering cells |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity |  |  |  |  |
| 5.2.2.1.16_1 2Rx FDD FR1 PDSCH with intra-cell inter-user interference |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.2.1.17 2Rx FDD FR1 PDSCH performance for RedCap |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.1.18_1 2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.2.1.15_1 |  |  |  |  | Same as 5.2.2.1.15_1 |  |  |  |  |
| 5.2.2.1.19_1 2Rx FDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x2 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.2.1.15_1 |  |  |  |  | Same as 5.2.2.1.15_1 |  |  |  |  |
| 5.2.2.1.20 2Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.1.21 2Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-10 ± 0.6 dB for test 1-11 For other TPs, same as 5.2.2.1.1_1 |  |  |  |  | For test point 1-10, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.9 dB For test point 1-11, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) = 0.6 dB For other TPs, same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.1_3 2Rx TDD FR1 PDSCH Mapping Type A performance – 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-12 Downlink EVM ≤ 2.5% for f ≤ 4.2 GHz Downlink EVM ≤ [2.5%] for f > 4.2 GHz |  |  |  |  | For test point 1-12, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 ) = 0.9 dB 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is not limited by the Test system EVM. |  |  |  |  |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.5_1 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.2.2.15 2Rx TDD FR1 PDSCH mapping Type A performance on band with shared spectrum access |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.16_1 2Rx TDD FR1 for PDSCH with inter-cell interference – 2x2 MIMO for both NSA and SA |  |  |  |  | Es/Noc ± 0.8 dB for interfering cells |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity |  |  |  |  |
| 5.2.2.2.17_1 2Rx TDD FR1 PDSCH with intra-cell inter-user interference |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.2.2.18 2Rx TDD FR1 PDSCH performance for RedCap |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1 dB for 10Hz doppler |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2.2.2.19_1 2Rx TDD FR1 PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x2 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.2.2.16_1 |  |  |  |  | Same as 5.2.2.2.16_1 |  |  |  |  |
| 5.2.2.2.20_1 2Rx TDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x2 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.2.2.16_1 |  |  |  |  | Same as 5.2.2.2.16_1 |  |  |  |  |
| 5.2.2.2.21 2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.2.2.22 2Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | ± 0.9 dB for > 10Hz doppler ± 1.0 dB for 10Hz doppler |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB |  |  |  |  |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.1_5 4Rx FDD FR1 PDSCH Mapping Type A performance – 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-8 Downlink EVM ≤ 2.5% for f ≤ 4.2 GHz Downlink EVM ≤ [2.5%] for f > 4.2 GHz |  |  |  |  | For test point 1-8, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 ) = 0.9 dB 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is not limited by the Test system EVM. |  |  |  |  |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  |
| 5.2.3.1.15_1 4Rx FDD FR1 PDSCH with inter-cell interference – 2x4 MIMO for both SA and NSA |  |  |  |  | Es/Noc ± 0.8 dB for interfering cells |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity |  |  |  |  |
| 5.2.3.1.16_1 4Rx FDD FR1 PDSCH with intra-cell inter-user interference – 2x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.3.1.16_2 4Rx FDD FR1 PDSCH with intra-cell inter-user interference – 4x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.3.1.17_1 4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.15_1 |  |  |  |  | Same as 5.2.3.1.15_1 |  |  |  |  |
| 5.2.3.1.18_1 4Rx FDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.15_1 |  |  |  |  | Same as 5.2.3.1.15_1 |  |  |  |  |
| 5.2.3.1.19 4Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.1.20 4Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.2.1_1 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.1_2 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.1_4 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.1_5 4Rx TDD FR1 PDSCH Mapping Type A performance – 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA |  |  |  |  | ± 0.9 dB for test 1-12 Downlink EVM ≤ 2.5% for f ≤ 4.2 GHz Downlink EVM ≤ [2.5%] for f > 4.2 GHz |  |  |  |  | For test point 1-12, Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 ) = 0.9 dB 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is not limited by the Test system EVM. |  |  |  |  |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  | Same as 5.2.2.1.5_1 |  |  |  |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  | Same as 5.2.2.1.6_1 |  |  |  |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  | Same as 5.2.3.1.9_1 |  |  |  |  |
| 5.2.3.2.10_1 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA |  |  |  |  | Same as 5.2.3.1.10_1 |  |  |  |  | 5.2.3.1.10_1 |  |  |  |  |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  | Same as 5.2.3.1.6_1 |  |  |  |  |
| 5.2.3.2.15 4Rx TDD FR1 PDSCH mapping Type A performance on band with shared spectrum access |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2.3.2.16_1 4Rx TDD FR1 for PDSCH with inter-cell interference – 2x4 MIMO for both NSA and SA |  |  |  |  | Es/Noc ± 0.8 dB for interfering cells |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity |  |  |  |  |
| 5.2.3.2.17_1 4Rx TDD FR1 PDSCH with intra-cell inter-user interference – 2x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.3.2.17_2 4Rx TDD FR1 PDSCH with intra-cell inter-user interference – 4x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 There is no additional measurement uncertainty introduced due to the co-scheduled UE |  |  |  |  |
| 5.2.3.2.18_1 4Rx TDD FR1 PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.2.16_1 |  |  |  |  | Same as 5.2.3.2.16_1 |  |  |  |  |
| 5.2.3.2.19_1 4Rx TDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x4 MIMO for both NSA and SA |  |  |  |  | Same as 5.2.3.2.16_1 |  |  |  |  | Same as 5.2.3.2.16_1 |  |  |  |  |
| 5.2.3.2.20 4Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  |
| 5.2.3.2.21 4Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  | Same as 5.2.3.2.9_1 |  |  |  |  |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 6% |  |  |  |  | 6% EVM is equivalent to a Test system downlink SNR of 24.4dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 5.2A.2.3 2Rx TDD FR1 PDSCH mapping Type A performance of Scell on band with shared spectrum access |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  | Same as 5.2.2.1.1_1 |  |  |  |  |
| 5.2A.2.4.1 2Rx PDSCH Demodulation Performance for HST-SFN CA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2A.2.5.1 2RX PDSCH Demodulation Performance for HST-DPS CA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.2.1 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.3 4Rx TDD FR1 PDSCH mapping Type A performance of Scell on band with shared spectrum access |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  | Same as 5.2.3.1.1_1 |  |  |  |  |
| 5.2A.3.4.1 4Rx PDSCH Demodulation Performance for HST-SFN CA |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  | Same as 5.2.2.1.9_1 |  |  |  |  |
| 5.2A.3.5.1 4Rx PDSCH Demodulation Performance for HST-DPS CA |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  | Same as 5.2.2.1.10_1 |  |  |  |  |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  | Same as 5.2.2.1.1_1 for 2Rx CC and 5.2.3.1.1_1 for 4Rx CC |  |  |  |  |
| 5.3.1.1.1 1Rx FDD FR1 PDCCH performance for RedCap |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.1.2.1 1Rx TDD FR1 PDCCH performance for RedCap |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | ± 0.9 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |  |  |  |  |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | ± 1.0 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2+ SNR uncertainty due to finite test time2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |  |  |  |  |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.1.4 2Rx FDD FR1 PDCCH performance for RedCap |  |  |  |  | ± 0.9 dB for Test 1 ± 1.0 dB for Test 2 |  |  |  |  | Same as 5.2.3.1.1_1 For Test 1, Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB For Test 2, Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.4 dB |  |  |  |  |
| 5.3.2.1.5 2RX FDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.2.2.4 2Rx TDD FR1 PDCCH performance for RedCap |  |  |  |  | ± 0.8 dB for Test 1 ± 0.9 dB for Test 2 |  |  |  |  | Same as 5.2.3.1.1_1 For Test 1, Signal-to-noise ratio uncertainty 0.3 dB Fading profile power uncertainty ±0.5 dB for 1 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time 0 dB For Test 2, Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time 0 dB |  |  |  |  |
| 5.3.2.2.5 2RX TDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.1.4 4RX FDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 with Dl 1024QAM |  |  |  |  | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 9.4B.1.1 |  |  |  |  | 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.3.3.2.3 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  | Same as 5.3.2.1.1 |  |  |  |  |
| 5.3.3.2.4 4RX TDD Minimum requirements for PDCCH with intra-slot repetition |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  | Same as 5.3.2.1.2 |  |  |  |  |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) |  |  |  |  | Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% |  |  |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 5.5.2 FR1 Sustained downlink data rate performance for single carrier with DL1024QAM |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 2.5% |  |  |  |  | 2.5% EVM is equivalent to a Test system downlink SNR of 32.04dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 5.5.3 FR1 Sustained downlink data rate performance for RedCap |  |  |  |  | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Downlink EVM ≤ 3% |  |  |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 |  |  |  |  | E-UTRA CC: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz NR CC: Same as 5.5.1 |  |  |  |  | Same as 5.5.1 |  |  |  |  |
| 9.4B.1.1_1 Sustained downlink data rate performance for EN-DC within FR1 (2 NR CCs) |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  |
| 9.4B.1.1_2 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  |
| 9.4B.1.1_3 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  | Same as 9.4B.1.1 |  |  |  |  |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link |  |  |  |  | ± 0.8 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link |  |  |  |  | ± 0.6 dB |  |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1, and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |  |  |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  | Same as 11.1.2.1.1_1 |  |  |  |  |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs |  |  |  |  | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% |  |  |  |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |  |  |  |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs |  |  |  |  | Same as 11.1.8.1.1_1 |  |  |  |  | Same as 11.1.8.1.1_1 |  |  |  |  |

<< Skipped sections >>

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

The Test Requirements in the present document have been calculated by relaxing the Minimum Requirements of the core specification using the Test Tolerances defined in this clause. When the Test Tolerance is zero, the Test Requirement will be the same as the Minimum Requirement. When the Test Tolerance is non-zero, the Test Requirements will differ from the Minimum Requirements, and the formula used for the relaxation is given in this clause.

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. As a result, the Test Tolerances may sometimes be set to zero.

The test tolerances should not be modified for any reason e.g. to take account of commonly known test system errors (such as mismatch, cable loss, etc.).

The downlink Test Tolerances apply at each receiver antenna connector.

### F.1.3.1	Measurement of test environments

The UE test environments are set to the values defined in TS 36.508 subclause 4.1, without any relaxation. The applied Test Tolerance is therefore zero.

### F.1.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 5 is defined in Table F.1.3.2-1.

Table F.1.3.2-1: Derivation of Test Requirements (FR1 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |  |
| --- | --- | --- | --- | --- |
| 5.2.1.1.1 1 Rx FDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.1.2.1 1Rx TDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_1 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.6 dB for test 1-6 0.9 dB for test 1-7 | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_2 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.1_3 2Rx FDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.2_1 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.3_1 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.4_1 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 + 0.5 = 1.1 dB | Additional relaxation of 0.5 dB added to the SNR to ensure early pass for a compliant device within a reasonable test duration Formula: SNR + TT + additional relaxation T-put limit unchanged |  |
| 5.2.2.1.6_1 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT |
| 5.2.2.1.7 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.8_1 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.9_1 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.10_1 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.11_1 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.12_1 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.13_1 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.14_1 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7] | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.15_1 2Rx FDD FR1 PDSCH with inter-cell interference – 2x2 MIMO for both SA and NSA | SNRs as specified | 1.7 dB for Test 1-1 1.8 dB for Test 1-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.1.16_1 2Rx FDD FR1 PDSCH intra-cell inter-user interference | SNRs as specified | 0.9 dB for > 10 Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.17 2Rx FDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.18_1 2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Test 1-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.1.19_1 2Rx FDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x2 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Tests 1-1 and 2-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.1.20 2Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.1.21 2Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_1 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler 0.9 dB for test 1-10 0.6 dB for test 1-11 | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_2 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with enhanced receiver type X for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.1_3 2Rx TDD FR1 PDSCH Mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.2_1 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.3_1 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.4_1 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.5_1 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 + 0.5 = 1.1 dB | Additional relaxation of 0.5 dB added to the SNR to ensure early pass for a compliant device within a reasonable test duration Formula: SNR + TT + additional relaxation T-put limit unchanged |  |
| 5.2.2.2.6_1 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT |  |
| 5.2.2.2.7_1 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.8_1 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.9_1 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.10_1 2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.11_1 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.12_1 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.13_1 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.14_1 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.16_1 2Rx TDD FR1 for PDSCH with inter-cell interference – 2x2 MIMO for both NSA and SA | SNRs as specified | 1.7 dB for Test 1-1 1.8 dB for Test 1-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.2.17_1 2Rx TDD FR1 PDSCH intra-cell inter-user interference | SNRs as specified | 0.9 dB for > 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.15 2Rx TDD FR1 PDSCH mapping Type A performance on band with shared spectrum access | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.18 2Rx TDD FR1 PDSCH performance for RedCap | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.19_1 2Rx TDD FR1 PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x2 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Test 1-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.2.20_1 2Rx TDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x2 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Tests 1-1, 1-2, 2-1, 2-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.2.2.21 2Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.2.2.22 2Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_2 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_4 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | SNRs as specified | 0.9 dB for > 10Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.1_5 4Rx FDD FR1 PDSCH Mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.2_1 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.3_1 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.4_1 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.5_1 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 + 0.5 = 1.1 dB | Additional relaxation of 0.5 dB added to the SNR to ensure early pass for a compliant device within a reasonable test duration Formula: SNR + TT + additional relaxation T-put limit unchanged |  |
| 5.2.3.1.6_1 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9dB | Formula: SNR + TT |  |
| 5.2.3.1.7_1 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.8_1 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.9_1 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.10_1 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.11_1 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.12_1 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.13_1 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.15_1 4Rx FDD FR1 PDSCH with inter-cell interference – 2x4 MIMO for both SA and NSA | SNRs as specified | 1.7 dB for Test 1-1 1.8 dB for Test 1-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.1.16_1 4Rx FDD FR1 PDSCH intra-cell inter-user interference – 2x4 MIMO for both NSA and SA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.16_2 4Rx FDD FR1 PDSCH intra-cell inter-user interference – 4x4 MIMO for both NSA and SA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.17_1 4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x4 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Test 1-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.1.18_1 4Rx FDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x4 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Tests 1-1 and 2-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.1.19 4Rx FDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.20 4Rx FDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.1.14_1 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.1_5 4Rx TDD FR1 PDSCH Mapping Type A performance – 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.2_1 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.3_1 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.4_1 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.5_1 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 + 0.5 = 1.1 dB | Additional relaxation of 0.5 dB added to the SNR to ensure early pass for a compliant device within a reasonable test duration Formula: SNR + TT + additional relaxation T-put limit unchanged |  |
| 5.2.3.2.6_1 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.9dB | Formula: SNR + TT |  |
| 5.2.3.2.7_1 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.8_1 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.9_1 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.10_1, 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.11_1 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.12_1 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.13_1 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.14_1 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | SNRs as specified | [0.7dB] | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.16_1 4Rx TDD FR1 for PDSCH with inter-cell interference – 2x4 MIMO for both NSA and SA | SNRs as specified | 1.7 dB for Test 1-1 1.8 dB for Test 1-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.2.17_1 4Rx TDD FR1 PDSCH intra-cell inter-user interference – 2x4 MIMO for both NSA and SA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.17_2 4Rx TDD FR1 PDSCH intra-cell inter-user interference – 4x4 MIMO for both NSA and SA | SNRs as specified | 0.9 dB for > 10 Hz doppler 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.18_1 4Rx TDD FR1 PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 4x4 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Test 1-1 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.2.15 4Rx TDD FR1 PDSCH mapping Type A performance on band with shared spectrum access | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.19_1 4Rx TDD FR1 for PDSCH with inter-cell CRS interference scenario – 4x4 MIMO for both NSA and SA | SNRs as specified | 1.8 dB for Tests 1-1, 1-2, 2-1, 2-2 | Formula: SNR + TT T-put limit unchanged The analysis is recorded in 3GPP TR 38.903 [26] |  |
| 5.2.3.2.20 4Rx TDD FR1 PDSCH HST-SFN Scheme A performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2.3.2.21 4Rx TDD FR1 PDSCH HST-SFN Scheme B performance - 2x4 MIMO for both SA and NSA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.1 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.2 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.1.3 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.2.1 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Power level as specified | No TT added | T-put limit unchanged |  |
| 5.2A.2.3 2Rx TDD FR1 PDSCH mapping Type A performance of Scell on band with shared spectrum access | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.4.1 2RX PDSCH Demodulation Performance for HST-SFN CA | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.2.5.1 2RX PDSCH Demodulation Performance for HST-DPS CA | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.1 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.2 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.1.3 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.2.1 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.3 4Rx TDD FR1 PDSCH mapping Type A performance of Scell on band with shared spectrum access | SNRs as specified | 1.0 dB for 10Hz doppler | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.4.1 4Rx PDSCH Demodulation Performance for HST-SFN CA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3.5.1 4Rx PDSCH Demodulation Performance for HST-DPS CA | SNRs as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.1 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.2 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.2A.3A.1.3 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | SNRs as specified | 2Rx CC: 1.0 dB for 10Hz doppler 4Rx CC: 0.9dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.1.1.1 1Rx FDD FR1 PDCCH performance for RedCap | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.1.2.1 1Rx TDD FR1 PDCCH performance for RedCap | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.1 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.2 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.3 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.4 2Rx FDD FR1 PDCCH performance for RedCap | SNRs as specified | 0.9 dB for Test 1 1.0 dB for Test 2 | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.1.5 2RX FDD Minimum requirements for PDCCH with intra-slot repetition | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.1 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.2 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.3 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.4 2Rx TDD FR1 PDCCH performance for RedCap | SNRs as specified | 0.8 dB for Test 1 0.9 dB for Test 2 | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.2.2.5 2RX TDD Minimum requirements for PDCCH with intra-slot repetition | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.1 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.2 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.3 4Rx FDD FR1 PDCCH 1 Tx antenna performance power saving | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.1.4 4RX FDD Minimum requirements for PDCCH with intra-slot repetition | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.1 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.2 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |  |
| 5.3.3.2.4 4RX TDD Minimum requirements for PDCCH with intra-slot repetition | SNRs as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.2.1.1_1 2Rx FR1 PSSCH performance - single active PSSCH link | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.3.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link | SNR as specified | 0.8 dB | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.5.1.1_1 2Rx FR1 PSCCH performance - single active PSSCH link | SNR as specified | 0.8 dB | Formula: SNR + TT missing detection probability limit unchanged |  |
| 11.1.6.1.1_1 2Rx FR1 Power imbalance performance - two active PSSCH link | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.7.1.1_1 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes | SNR as specified | 0.6 dB | Formula: SNR + TT T-put limit unchanged |  |
| 11.1.8.1.1_1 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs | sidelink power | 0 dB | sidelink power unchanged missing detection probability limit unchanged |  |
| 11.1.9.1.1_1 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs | sidelink power | 0 dB | sidelink power unchanged missing detection probability limit unchanged |  |
| 5.5.1 FR1 Sustained downlink data rate performance for single carrier | Downlink power -112 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |  |
| 5.5.2 FR1 Sustained downlink data rate performance for single carrier with DL1024QAM | Downlink power -110.46 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged Downlink power calculated by replacing EVM value to 2.5% instead of 3% in the formula mentioned in clause 4.4.4.2.1 |  |
| 5.5.3 FR1 Sustained downlink data rate performance for RedCap | Downlink power -112 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |  |
| 5.5A.1.1 FR1 SDR performance for CA (2DL CA) | Same as 5.5.1 | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.2 FR1 SDR performance for CA (3DL CA) | Same as 5.5.1 | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.3 FR1 SDR performance for CA (4DL CA) | Same as 5.5.1 | Same as 5.5.1 | Same as 5.5.1 |
| 5.5A.1.4 FR1 SDR performance for CA (5DL CA) | Same as 5.5.1 | Same as 5.5.1 | Same as 5.5.1 |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 | Downlink power -112 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 9.4B.1.1 Sustained downlink data rate performance for EN-DC within FR1 with DL 1024QAM | Downlink power -110.5 dBm/Hz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 9.4B.1.1_1 Sustained downlink data rate performance for EN-DC within FR1 (2 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_2 Sustained downlink data rate performance for EN-DC within FR1 (3 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |
| 9.4B.1.1_3 Sustained downlink data rate performance for EN-DC within FR1 (4 NR CCs) | Same as 9.4B.1.1 | Same as 9.4B.1.1 | Same as 9.4B.1.1 |

### F.1.3.3	Measurement of Channel State Information reporting

<< Sections skipped >>

## G.1.5	Minimum Test time

<< Sections skipped >>

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern |  |  | Reference Channel |  |  | Propagation condition |  |  | Demodulation scenario (doppler speed) |  |  | Minimum number of active subframes (MNAS) |  |  | MNAS to MNS Scaling factor (Note 3) |  |  | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | HST-750 |  |  | 750 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.2 FDD |  |  | HST-972 |  |  | 972 Hz |  |  | 6000 (Note 1) |  |  | 1.0526 |  |  | 6400 |  |  |
| NA |  |  | R.PDSCH.1-8.1 FDD |  |  | TDLC300-600 |  |  | 600 Hz |  |  | 8000 (Note 1) |  |  | 1.0526 |  |  | 9000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.0526 |  |  | 11000 |  |  |
| NA |  |  | R.PDSCH.1-1.1 HD-FDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.3333 |  |  | 14000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, R.PDSCH.5-1.1 FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.0526 |  |  | 22000 |  |  |
| NA |  |  | R.PDSCH.1-1.2 HD-FDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.3333 |  |  | 27000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 FDD, R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-2.5 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-3.2 FDD, R.PDSCH.1-3.3 FDD, R.PDSCH.1-3.4 FDD, R.PDSCH.1-3.5 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-4.2 FDD, R.PDSCH.1-12.1 FDD, R.PDSCH.2-1.1 FDD, R.PDSCH.5-1.2 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| NA |  |  | R.PDSCH.1-1.3 HD-FDD, R.PDSCH.1-1.4 HD-FDD, R.PDSCH.1-1.5 HD-FDD, R.PDSCH.1-2.1 HD-FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.3333 |  |  | 100000 |  |  |
| NA |  |  | R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD, R.PDSCH.1-7.3 FDD, R.PDSCH.1-18.1 FDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.25 |  |  | 94000 |  |  |
| NA |  |  | R.PDSCH.1-17.1 FDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.0526 |  |  | 74000 |  |  |
| NA |  |  | R.PDSCH.1-8.3 FDD, R.PDSCH.1-13.1 FDD, R.PDSCH.1-13.2 FDD, R.PDSCH.1-13.3 FDD, R.PDSCH.1-13.4 FDD, R.PDSCH.1-13.5 FDD, R.PDSCH.1-14.1 FDD, R.PDSCH.1-14.2 FDD, R.PDSCH.1-14.3 FDD, R.PDSCH.1-14.4 FDD |  |  | HST-SFN |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-8.5 FDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-8.4 FDD, R.PDSCH.1-15.1 FDD, R.PDSCH.1-15.2 FDD, R.PDSCH.1-15.3 FDD, R.PDSCH.1-15.4 FDD, R.PDSCH.1-15.5 FDD, R.PDSCH.1-16.1 FDD, R.PDSCH.1-16.2 FDD, R.PDSCH.1-16.3 FDD, R.PDSCH.1-16.4 FDD |  |  | HST-DPS |  |  | 870 Hz |  |  | 30000 (Note 1) |  |  | 1.0526 |  |  | 32000 |  |  |
| NA |  |  | R.PDSCH.1-9.1 FDD, R.PDSCH.1-9.2 FDD, R.PDSCH.1-9.3 FDD, R.PDSCH.1-9.4 FDD, R.PDSCH.1-9.5 FDD, R.PDSCH.1-10.1 FDD, R.PDSCH.1-10.2 FDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.0526 |  |  | 79000 |  |  |
| NA |  |  | R.PDSCH.1-17.2 FDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.5385 |  |  | 116000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-1.1 TDD, R.PDSCH.1-1.2 TDD, R.PDSCH.1-1.3 TDD, R.PDSCH.1-4.1 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 2.8571 |  |  | 215000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-2.1 TDD, R.PDSCH.1-2.2 TDD, R.PDSCH.1-2.3 TDD, R.PDSCH.1-2.4 TDD, R.PDSCH.1-2.5 TDD, R.PDSCH.1-3.1 TDD, R.PDSCH.1-3.2 TDD, R.PDSCH.1-3.3 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.3333 |  |  | 100000 |  |  |
| FR1.15-1 |  |  | R.PDSCH.1-4.2 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 2.5 |  |  | 188000 |  |  |
| FR1.30-1A |  |  | R.PDSCH.2-1.1 TDD, R.PDSCH.2-1.5 TDD |  |  | TDLB100-400 |  |  | 400 Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD, R.PDSCH.2-26.1 TDD, R.PDSCH.7-1.1 TDD |  |  | TDLC300-100 |  |  | 100 Hz |  |  | 20000 (Note 1) |  |  | 1.2903 |  |  | 26000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.2 TDD |  |  | TDLC300-1200 |  |  | 12000 Hz |  |  | 20000 (Note 1) |  |  | 1.2903 |  |  | 26000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-2.2 TDD, R.PDSCH.2-2.3 TDD, R.PDSCH.2-2.4 TDD, R.PDSCH.2-2.5 TDD, R.PDSCH.2-27.1 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-3.2 TDD, R.PDSCH.2-3.3 TDD, R.PDSCH.2-3.4 TDD, R.PDSCH.2-3.5 TDD, R.PDSCH.2-4.1 TDD, R.PDSCH.2-4.2 TDD, R.PDSCH.2-4.3 TDD, R.PDSCH.7-1.2 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-29.1 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.9048 |  |  | 143000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-28.1 TDD |  |  | TDLA30-10 |  |  | 10Hz |  |  | 75000 (Note 1) |  |  | 1.7391 |  |  | 131000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-1.3 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-25.1 TDD |  |  | TDLD30-5 |  |  | 5 Hz |  |  | 70000 (Note 1) |  |  | 1.2903 |  |  | 91000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-5.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-2 |  |  | R.PDSCH.2-17.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 5 |  |  | 375000 |  |  |
| FR1.30-3 |  |  | R.PDSCH.2-6.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.4815 |  |  | 112000 |  |  |
| FR1.30-4 |  |  | R.PDSCH.2-9.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-5 |  |  | R.PDSCH.2-11.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-6 |  |  | R.PDSCH.2-12.1 TDD |  |  | TDLB100-400 |  |  | 400Hz |  |  | 10000 (Note 1) |  |  | 1.2903 |  |  | 13000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD |  |  | HST-1000 |  |  | 1000 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.1 TDD, R.PDSCH.2-10.3 TDD |  |  | HST-1667 |  |  | 1667 Hz |  |  | 15000 (Note 1) |  |  | 1.4815 |  |  | 23000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.4 TDD, R.PDSCH.2-19.1 TDD, R.PDSCH.2-19.2 TDD, R.PDSCH.2-19.3 TDD, R.PDSCH.2-19.4 TDD, R.PDSCH.2-19.5 TDD, R.PDSCH.2-20.1 TDD, R.PDSCH.2-20.2 TDD, R.PDSCH.2-20.3 TDD, R.PDSCH.2-20.4 TDD, R.PDSCH.2-20.5 TDD, R.PDSCH.2-21.1 TDD |  |  | HST-SFN |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-10.5 TDD, R.PDSCH.2-22.1 TDD, R.PDSCH.2-22.2 TDD, R.PDSCH.2-22.3 TDD, R.PDSCH.2-22.4 TDD, R.PDSCH.2-22.5 TDD, R.PDSCH.2-23.1 TDD, R.PDSCH.2-23.2 TDD, R.PDSCH.2-23.3 TDD, R.PDSCH.2-23.4 TDD, R.PDSCH.2-23.5 TDD, R.PDSCH.2-24.1 TDD |  |  | HST-DPS |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-30.1 TDD |  |  | HST-SFN Scheme A HST-SFN Scheme B |  |  | 1667 Hz |  |  | 30000 (Note 1) |  |  | 1.4815 |  |  | 45000 |  |  |
| FR1.30-1 |  |  | R.PDSCH.2-13.1 TDD, R.PDSCH.2-13.2 TDD, R.PDSCH.2-13.3 TDD, R.PDSCH.2-13.4 TDD, R.PDSCH.2-13.5 TDD, R.PDSCH.2-14.1 TDD, R.PDSCH.2-14.2 TDD, R.PDSCH.2-14.3 TDD, R.PDSCH.2-14.4 TDD, R.PDSCH.2-14.5 TDD, R.PDSCH.2-15.1 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 1) |  |  | 1.2903 |  |  | 97000 |  |  |
| FR1.30-7 |  |  | R.PDSCH.2-18.1 TDD, R.PDSCH.2-18.2 TDD, R.PDSCH.2-18.3 TDD, R.PDSCH.2-18.4 TDD |  |  | TDLA30-10 |  |  | 10 Hz |  |  | 75000 (Note 2) |  |  | 1.6129 (Note 4) |  |  | [121000] |  |  |
| FR2.60-1 |  |  | R.PDSCH.4-1.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1A |  |  | R.PDSCH.5-1.1 TDD |  |  | TDLC60-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-2.1 TDD, R.PDSCH.5-2.2 TDD, R.PDSCH.5-2.3 TDD, R.PDSCH.5-3.1 TDD |  |  | TDLA30-300 |  |  | 300 Hz |  |  | 10000 (Note 2) |  |  | 1.25 |  |  | 13000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-1.2 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.25 |  |  | 25000 |  |  |
| FR2.120-2 |  |  | R.PDSCH.5-4.1 TDD, R.PDSCH.5-5.1 TDD, R.PDSCH.5-5.2 TDD, R.PDSCH.5-6.1 TDD |  |  | TDLA30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.33 |  |  | 27000 |  |  |
| FR2.120-1 |  |  | R.PDSCH.5-10.1 TDD |  |  | TDLD30-75 |  |  | 75 Hz |  |  | 20000 (Note 2) |  |  | 1.26 |  |  | 26000 |  |  |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNAS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots. Note 4: To account for the LBT failure probability of 0.25, MNS scaled up by 25% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>