3GPP TSG-RAN5 Meeting #104	R5-245327r1

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0373 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to HTTP messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - As per RFC 2616, cl. 13.1.3, cache-control header in Http Request is optional and a client may not include cache-control header in HTTP requests - As per RFC 2616, cl. 7.2.1, the “content type” header is optional, and may not be included in HTTP GET |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Table 5.5.4.2/3/4/5-1 to make cache-control as optional in HTTP Request messages - Updated Table 5.5.4.2-1 to make content type header optional in GET |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.2, 5.5.4.3, 5.5.4.4, 5.5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1-> Re-instated the AUTH condition for the Content-Type in Table 5.5.4.2-1 and reverted changes in NOTE 3 |  |  |  |  |  |  |  |  |

### 5.5.4	Default HTTP message and other information elements

#### 5.5.4.1	General

The HTTP Messages are specified in RFC 2616 [26]. Wherever another reference applies to their content it is explicitly indicated.

The following conditions apply throughout clause 5.5:

Table 5.5.4.1-1: Conditions

| Condition | Explanation |
| --- | --- |
| AUTH | Message/IE sent only as part of an MCX UE authentication |
| UEINITIALCONFIG | Message/IE sent only as part of an MCX UE initial configuration |
| USERAUTH | Message/IE sent only as part of an MCX UE user authentication |
| UECONFIG | Message/IE sent only as part of an MCX UE configuration |
| UEUSERPROF | Message/IE sent only as part of an MCX UE User profile configuration |
| UESERVCONFIG | Message/IE sent only as part of an MCX UE service configuration |
| GROUPCONFIG | Message/IE sent only as part of an MCX group configuration |
| TEMPGROUP | Message/IE sent only in temporary group creation scenario |
| TOKEN | Message/IE sent only as part of an MCX token exchange |
| KMSINIT | Message/IE sent only as part of an MCX KMS initialisation |
| KMSKEY | Message/IE sent only as part of an MCX KMS key exchange |
| FD_HTTP | Message/IE sent only as part of MCData signalling for FD using HTTP |

#### 5.5.4.2	GET

Table 5.5.4.2-1: HTTP GET

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Method | "GET" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCX_IdMS_auth_UriPath | points to the Authorisation endpoint of the IdM Server | TS 33.180 [94] | AUTH |
|  | px_MCX_InitialConfigServer_UriPath | points to initial UE Configuration document | TS 24.484 [14] | UEINITIALCONFIG |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID1 & "/users/" & XUI & ue-config-docname | points to UE Configuration document (NOTE 1a, 2, 3, 5) | TS 24.484 [14] | UECONFIG |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID2 & "/users/" & XUID & ""/" & user-profile-docname | points to UE User Profile document (NOTE 1b, 2, 4) | TS 24.484 [14] | UEUSERPROF |
|  | tsc_MCX_CMSXCAPRootURI & "/" & AUID3 & "/global/service-config.xml" | points to UE Service Configuration document (NOTE 1c, 2) | TS 24.484 [14] | UESERVCONFIG |
|  | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/global/byGroupID/" & group-id | points to group configuration document (NOTE 6) | TS 24.481 [11] | GROUPCONFIG |
|  | URI as contained in the payload of the FD SIGNALLING PAYLOAD indication the file upload |  |  | FD_HTTP |
| query | As described in Table 5.5.4.10.1-1 |  | TS 33.180 [94] | AUTH |
| HTTP-Version | "HTTP/1.1" |  |  |  |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  |  | RFC 2617 [72] | UECONFIG UEUSERPROF UESERVCONFIG GROUPCONFIG FD_HTTP |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Authorization | not present |  |  |  |
| Content-Type | if present |  |  | AUTH |
| media-type | "application/x-www-form-urlencoded" |  |  |  |
| Content-Type | Not present |  |  |  |
| Message-body | Not present |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcptt.ue-config" for Condition MCPTT AUID1 = "org.3gpp.mcvideo.ue-config" for Condition MCVIDEO AUID1 = "org.3gpp.mcdata.ue-config" for Condition MCDATA NOTE 1b: AUID2 = "org.3gpp.mcptt.user-profile" for Condition MCPTT AUID2 = "org.3gpp.mcvideo.user-profile" for Condition MCVIDEO AUID2 = "org.3gpp.mcdata.user-profile" for Condition MCDATA NOTE 1c: AUID3 = "org.3gpp.mcptt.service-config" for Condition MCPTT AUID3 = "org.3gpp.mcvideo.service-config" for Condition MCVIDEO AUID3 = "org.3gpp.mcdata.service-config" for Condition MCDATA NOTE 2: XUID = "sip:" & px_MCPTT_ID_User_A for Condition MCPTT XUID = "sip:" & px_MCVideo_ID_User_A for Condition MCVIDEO XUID = "sip:" & px_MCData_ID_User_A for Condition MCDATA NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: user-profile-docname = "mcptt-user-profile-" & profile-index & ".xml" for Condition MCPTT user-profile-docname = "mcvideo-user-profile-" & profile-index & ".xml" for Condition MCVIDEO user-profile-docname = "mcdata-user-profile-" & profile-index & ".xml" for Condition MCDATA with profile-index being the same as in the <user-profile-index> attribute of the corresponding document NOTE 5: ue-config-docname = "mcptt-ue-configuration.xml" for Condition MCPTT ue-config-docname = "mcvideo-ue-configuration.xml" for Condition MCVIDEO ue-config-docname = "mcdata-ue-configuration.xml" for Condition MCDATA NOTE 6: group-id = px_MCPTT_Group_A_ID for Condition MCPTT group-id = px_MCVideo_Group_A_ID for Condition MCVIDEO group-id = px_MCData_Group_A_ID for Condition MCDATA |  |  |  |  |

#### 5.5.4.3	POST

Table 5.5.4.3-1: HTTP POST

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Status-Line |  |  |  |  |
| Method | "POST" |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCX_IdMS_auth_UriPath | points to the Authorisation endpoint of the IdM Server | TS 33.180 [94] | AUTH |
|  | tsc_MCX_IdMS_userauth_UriPath | points to the endpoint verifying the user authentication; same URI as provided to the UE in the action attribute of the HTML login form | TS 33.180 [94] HTML 4.01 Specification [105] | USERAUTH |
|  | tsc_MCX_IdMS_token_UriPath | points to the Token endpoint of the IdM Server | TS 33.180 [94] | TOKEN |
|  | UriScheme & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath | "KMS Initialize" request according to TS 33.180 [94] D.2.3 (NOTE 2) | TS 33.180 [94] | KMSINIT |
|  | UriScheme & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath | "KMS KeyProvision" request according to TS 33.180 [94] D.2.4 (NOTE 2) | TS 33.180 [94] | KMSKEY |
|  | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & temporary-group-id | Points to the temporary group configuration document to be created (NOTE 1) | TS 24.481[11] clause 6.3.14.2 | TEMPGROUP |
|  | tsc_MCData_MSF_URI | The absolute URI identifying the resource on a media storage function | TS 24.282 [87], clause 10.2.2.1 | FD_HTTP |
| HTTP-Version | "HTTP/1.1" |  |  |  |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  |  | RFC 2617 [72] | KMSINIT, KMSKEY, TEMPGROUP, FD_HTTP |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Host |  |  |  | FD_HTTP |
| host | tsc_MCData_MSF_Hostname | hostname identifying the media storage function | TS 24.282 [87], clause 10.2.2.1 |  |
| port | not present |  |  |  |
| Content-Type |  |  |  | AUTH, USERAUTH, TOKEN |
| media-type | "application/x-www-form-urlencoded" |  |  |  |
| Content-Type |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| media-type | "application/xml" |  | RFC 7303 [112] |  |
| Content-Type |  |  |  | TEMPGROUP |
| media-type | "application/vnd.3gpp.GMOP+xml" |  |  |  |
| Content-Type |  |  |  | FD_HTTP |
| media-type | "multipart/mixed" |  | TS 24.282 [87], clause 10.2.2.1 |  |
| Message-body |  |  |  | AUTH |
| Authentication Request | As described in Table 5.5.4.10.1-1 |  |  |  |
| Message-body |  |  | HTML 4.01 Specification [105] | USERAUTH |
| user | px_MCX_User_A_username |  |  |  |
| password | px_MCX_User_A_password |  |  |  |
| Message-body |  |  |  | TOKEN |
| Token request | As described in Table 5.5.4.10.3-1 |  |  |  |
| Message-body |  | present in case of KMS request security |  | (KMSINIT OR KMSKEY) AND pc_MCX_KMS_RequestSecurity |
| Signed KMS Request | As described in Table 5.5.4.10.9-1 |  |  |  |
| Message-body |  |  |  | TEMPGROUP |
| Temporary Group Creation Document" | As described in Table 5.5.7.4-2 |  |  |  |
| Message-body |  |  |  | FD_HTTP |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/vnd.3gpp.mcdata-info+xml" |  |  |  |
| MIME-part-body | MCData-Info described in Table 5.5.3.2.1-3 |  |  |  |
| MIME body part |  | File content | TS 24.282 [87] clause 10.2.2.1 |  |
| MIME-part-headers |  |  |  |  |
| MIME-Content-Type | "application/octet-stream" |  |  |  |
| MIME-part-body | binary data representing the file |  |  |  |
| NOTE 1: temporary-group-id = px_MCPTT_Group_T_ID for Condition MCPTT temporary-group-id = px_MCVideo_Group_T_ID for Condition MCVIDEO temporary-group-id = px_MCData_Group_T_ID for Condition MCDATA NOTE 2: UriScheme is either "http" or "https" |  |  |  |  |

#### 5.5.4.4	PUT

Table 5.5.4.4-1: HTTP PUT

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "PUT" |  |  |  |
| Request-URI | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & document name (NOTE 1) | XCAP URI in users tree where the XUI is set to a group creation XUI configuration parameter | TS 24.481 [11] clause 6.3.2.2.1 | GROUPCREATE |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  | TS 24.482 [12] A.2.3: Expected by the server to validate and identify the client | RFC 2617 [72] |  |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| Content-Type |  |  |  | GROUPCREATE |
| media-type | application/vnd.oma.poc.groups+xml |  |  |  |
| Message-body |  |  |  | GROUPCREATE |
| Group Creation Document | As described in Table 5.5.7.4-1 |  |  |  |
| NOTE 1: document name is the name of the group document contained in the message body |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GROUPCREATE | Message/IE sent only in group creation scenario |
| NOTE: For further conditions see table 5.5.1-1 |  |

#### 5.5.4.5	DELETE

Table 5.5.4.5-1: HTTP DELETE

| Derivation Path: RFC 2616 [26] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-line |  |  |  |  |
| Method | "DELETE" |  |  |  |
| Request-URI | tsc_MCX_GMSXCAPRootURI & "/" & "org.openmobilealliance.groups/users/" & px_MCX_GroupCreationXUI & "/" & temporary-group-id | Points to the group configuration document (NOTE 1) | TS 24.481 [11] | TEMPGROUP |
| Cache-Control | if present |  | RFC 2616 [26] |  |
| cache-directive | "no-cache" |  |  |  |
| Authorization |  | TS 24.482 [12] A.2.3: Expected by the server to validate and identify the client | RFC 2617 [72] |  |
| authentication-scheme | “Bearer” |  | RFC 6750 [104] |  |
| b64token | Access token as assigned to the UE by Token Response |  | RFC 6750 [104] |  |
| NOTE 1: temporary-group-id = px_MCPTT_Group_T_ID for Condition MCPTT temporary-group-id = px_MCVideo_Group_T_ID for Condition MCVIDEO temporary-group-id = px_MCData_Group_T_ID for Condition MCDATA |  |  |  |  |