3GPP TSG-RAN5 Meeting #105	R5-246175r2

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5348 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 13.4.3.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Solutions, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Prose, in parallel to Steps 24–31, the Steps specified in Table 13.4.3.14.3.2-4 or Step 37Aa1 in table 13.3.3.14.3.2-2 take place. These are IMS signalling steps. If a UE does not support IMS over UTRAN, these steps will never get executed. This can cause a conformant UE to fail the test case. So, it needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to check whether the UE supports IMS over UTRAN or not, and based on this the Steps in Table 13.4.3.14.3.2-4 or Step 37Aa1 in table 13.4.3.14.3.2-2 will be called. For the IMS over UTRAN dependency, a new PICS pc_IMS_UTRAN is introduced in CR R5-246312. The EXCEPTION before Step 1 in table 13.4.3.14.3.2-3 and before Step 37Aa1 in table 13.4.3.14.3.2-2 have been modified with pc_IMS_UTRAN dependency. A similar EXCEPTION has been added before Step 5Aa1 in table 13.4.3.14.3.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.3.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2. CR R5-246312. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-246175r1: Removed the newly added EXCEPTION before Step 37Aa1 and modified the already existing EXCEPTION before Step 37Aa1. Removed the EXCEPTION introduced before Step1 in table 13.4.3.14.3.2-4. Modified the Exception before Step 1 in table 13.4.3.14.3.2-3 with pc_IMS_UTRAN PICS dependency. Introduced a new EXCEPTION before Step 5A of table 13.4.3.14.3.2-3 and changed Step 5A to Step 5Aa1 as the relevant behavior is optional. Updated the Summary of change. R5-246175r2: Updated the EXCEPTION before Step 5Aa1 in table 13.4.3.14.3.2-3. |  |  |  |  |  |  |  |  |

#### <Start of change>

#### 13.4.3.14	Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / aSRVCC / MO call

13.4.3.14.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives a MobilityFromEUTRACommand message and an MO IMS voice call is in alerting state and an UTRA PS RB + Speech combination is configured for an UTRA cell}

then { UE transmits a HANDOVER TO UTRAN COMPLETE message on the utra cell}

}

(2)

with { UE having transmitted a HANDOVER TO UTRAN COMPLETE message }

ensure that {

when { the voice call is accepted }

then { UE transmits a CONNECT ACKNOWLEDGE message on the utra cell}

}

(3)

with { UE having sent CONNECT ACKLOWDGEMENT}

ensure that {

when { the voice call is accepted }

then { UE deletes early IMS dialog }

13.4.3.14.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.4.3.3, TS 24.237 clause 12.2.3B.1 and clause 12.2.3B.3.2.

[TS 36.331, clause 5.4.3.3]

The UE shall be able to receive a MobilityFromEUTRACommand message and perform a cell change order to GERAN, even if no prior UE measurements have been performed on the target cell.

The UE shall:

1>	stop timer T310, if running;

1>	if the MobilityFromEUTRACommand message includes the purpose set to 'handover':

2>	if the targetRAT-Type is set to 'utra' or 'geran':

3>	consider inter-RAT mobility as initiated towards the RAT indicated by the targetRAT-Type included in the MobilityFromEUTRACommand message;

3>	forward the nas-SecurityParamFromEUTRA to the upper layers;

3>	access the target cell indicated in the inter-RAT message in accordance with the specifications of the target RAT;

[TS 24.237 clause 12.2.3B.1]

The SC UE shall apply the procedures in subclauses 12.2.3B.3 for access transfer for calls in alerting state if:

1)	the SC UE supports single radio PS to CS access transfer for calls in alerting state; and

2)	there are one or more early dialogs created by the same SIP INVITE request with at least one dialog that is an early dialog supporting a session with active speech media component where the SC UE:

-	has sent a Contact header field in a SIP INVITE request or 180 (Ringing) response containing the g.3gpp.srvcc-alerting media feature tag (as described in annex C); and

-	has received a Feature-Caps header field in a SIP INVITE request or 180 (Ringing) response containing the g.3gpp.srvcc-alerting feature capability indicator (as described in annex C).

[TS 24.237 clause 12.2.3B.3.2]

If the SC UE has initiated an outgoing call which is in the early dialog state according to the conditions in subclauses 12.1 and 12.2.3B.1 and the SC UE successfully performs access transfer to the CS domain, then the UE continues in Ringing state in CS, i.e. UE moves to Call Delivered (U4) state as described in 3GPP TS 24.008 [8].

[TS 24.237 clause 12.3.8]

Upon receiving the SIP ACK request from target access leg, and after an operator specific timer has expired, the SCC AS shall:

1)	for each session where no in-dialog request has been received in the source access leg of the session with transferred media component(s) within the operator defined time:

a)	if the session is a session with an active or inactive media component, send a SIP BYE request toward the S-CSCF for sending to the served SC UE;

b)	if the session is an early dialog on originating side send a SIP 404 (Not Found) response; and

c)	if the session is an early dialog on terminating side send a SIP CANCEL request; and

NOTE 1:	The SC UE will receive the SIP request or response only if the SC UE is using Gm after the PS-CS access transfer is completed.

NOTE 2:	Delaying the SIP request or response as described above allows an ICS UE to add Gm control if needed and an SC UE to reuse the PS dialog in case of SRVCC cancellation.

2)	for each session in the transferable session set for which the speech media component, or the speech media and video media component in case of vSRVCC, was not transferred:

a)	if the speech media component or the speech media and video media components is the only media component(s) of the session, release remote leg and source access leg; and

b)	if the speech media component or the speech media and video media components are not the only media components of the session, modify the remote leg and source access leg and remove the media component(s).

NOTE 3:	In case of a SIP INVITE request due to STN-SR, video media components are not removed or causing release of the remote leg.

[TS 24.237 clause A.17.3]

In the example flow at the figure A.17.3-1, SC UE A has invited for an originating session with speech media component which is anchored at SCC AS. The session is in alerting phase. Based upon measurement reports sent from the UE to E-UTRAN, the source E-UTRAN decides to trigger a PS to CS SRVCC handover to CS access.

Figure A.17.3-1: PS-CS SRVCC, incoming call in alerting phase

NOTE 1:	For clarity, the SIP 100 (Trying) responses are not shown in the signalling flow.

1.	SC UE A has setup an outgoing call

The outgoing call has been anchored at the SCC AS of SC UE A. Both ends have reserved the resources and SC UE A has received a SIP 180 (Ringing) response.

1a.	The ringing tone is played to the originating user

The ringing tone is played by the originating UE as the locally generated ringing tone.

2.	SC UE A attaches to the CS domain

UE A sends the measurement reports to E-UTRAN, and the source E-UTRAN decides to trigger an PS to CS SRVCC handover to CS access. The MSC server initiates the session transfer with the STN-SR, refer to 3GPP TS 23.237 [9]. The ringing tone is kept playing to the originating user.

3.	SIP INVITE request transferring the session (MSC server to intermediate IM CN subsystem entities) - see example in table A.17.3-1

The MSC server sends an initial SIP INVITE request with STN-SR.

Table A.17.3-1: SIP INVITE request (MSC server to intermediate IM CN subsystem entities)

INVITE tel: +1-237-555-3333 SIP/2.0

Via: SIP/2.0/UDP msc1.visit1.net;branch=z9hG4bk731b87

Max-Forwards: 70

Route: <sip:icscf1.visit1.net;lr>

P-Asserted-Identity: <tel:+1-237-555-1111>

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi=visit1.net

Privacy: none

From: <tel:+1-237-555-1111>;tag=171828

To: <tel:+1-237-555-3333>

Call-ID: cb03a0s09a2sdfglkj490334

Cseq: 127 INVITE

Supported: 100rel, precondition, gruu

Accept-Contact: *;+g.3gpp.icsi-ref="urn%3Aurn-7%3gpp-service.ims.icsi.mmtel"

P-Asserted-Service: urn:urn-7:3gpp-service.ims.icsi.mmtel

Contact: <sip: msc1.visit1.net:1357>;+g.3gpp.icsi-ref="urn%3Aurn-7%3gpp-service.ims.icsi.mmtel"; +g.3gpp.srvcc-alerting

Allow: INVITE, ACK, CANCEL, BYE, PRACK, UPDATE, REFER

Recv-Info: g.3gpp.state-and-event

Content-Type: application/sdp

Content-Length: (…)

P-Early-Media: supported

v=0

o=- 2987933615 2987933615 IN IP6 5555::aaa:bbb:ccc:eee

s=

c=IN IP6 5555::aaa:bbb:ccc:eee

t=0 0

m=audio 3456 RTP/AVP 97 96

a=tcap:1 RTP/AVPF

a=pcfg:1 t=1

b=AS:25.4

a=curr:qos local sendrecv

a=curr:qos remote none

a=des:qos mandatory local sendrecv

a=des:qos none remote sendrecv

a=rtpmap:97 AMR

a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2

a=rtpmap:96 telephone-event

a=maxptime:20

Request-URI:	contains the STN-SR.

SDP:	The SDP contains set of codecs supported by the MGW.

Contact:	contains the +g.3gpp.srvcc-alerting feature tag.

4.	SIP INVITE request transferring the session (intermediate IM CN subsystem entities to SCC AS)

The SIP INVITE is routed towards the SCC AS, based on filter criteria in S-CSCF.

4a.	Remote Leg Update

The SCC AS correlates SIP INVITE request to the local and remote call legs of the existing session between the UE A and the remote end. The SCC AS performs the Remote Leg update by sending SIP UPDATE request towards the remote UE B.

5.	SIP UPDATE request (SCC AS to intermediate IM CN subsystem entities)

The SCC AS acting as a B2BUA generates a SIP UPDATE request based upon the received SIP INVITE request and the information previously stored against this session.

6.	SIP UPDATE request (Intermediate IM CN subsystem entities to UE B)

The intermediate IM CN subsystem entities forward the SIP UPDATE request to remote UE B.

7.	SIP 200 (OK) response (UE B to Intermediate IM CN subsystem entities)

Upon receiving the SIP UPDATE request containing the SDP offer for the leg to the MSC, the far end sends a SIP 200 (OK) response.

8.	SIP 200 (OK) response (Intermediate IM CN subsystem entities to SCC AS)

The intermediate IM CN subsystem entities forward the SIP 200 (OK) response to the SCC AS.

9.	SIP 183 (Session Progress) response (SCC AS to Intermediate IM CN subsystem entities)

The SCC AS sends a SIP 183 (Session Progress) response containing the SDP answer as received from the far end UE. The SDP answer indicates that resources are available

10.	SIP 183 (Session Progress) response (Intermediate IM CN subsystem entities to MSC server)

The intermediate IM CN subsystem entities forward the 183 (Session Progress) response to the MSC server.

11.	SIP PRACK request (MSC server to Intermediate IM CN subsystem entities)

The MSC acknowledges the receipt of the SIP 183 (Session Progress) response.

12.	SIP PRACK request (Intermediate IM CN subsystem entities to SCC AS)

The intermediate IM CN subsystem entities forward the SIP PRACK request to the SCC AS.

13.	SIP 200 (OK) response (SCC AS to Intermediate IM CN subsystem entities)

The SCC AS acknowledges the PRACK request.

14.	SIP 200 (OK) response (Intermediate IM CN subsystem entities to MSC server)

The intermediate IM CN subsystem entities forward the SIP 200 (OK) response to the MSC server.

15.	SIP INFO request (SCC AS to intermediate IM CN subsystem entities) - see example in table A.17.3-2

Table A.17.3-2: INFO request (SCC AS to intermediate IM CN subsystem entities)

INFO sip: msc1.visit1.net:1357 SIP/2.0

Via SIP/2.0/UDP sip:sccas1.home1.net;branch=z9hG4bK332b23.1

Max-Forwards: 68

Route: <sip:scscf1.home1.net;lr>

From: <tel: +1-237-555-3333>;tag=314159

To: <tel:+1-237-555-1111>;tag=171828

Call-ID: cb03a0s09a2sdfglkj490334

Cseq: 129 INFO

Info-Package: g.3gpp.state-and-event

Content-Disposition: Info-Package

Content-Type: application/vnd.3gpp.state-and-event-info+xml

Content-Length:

<?xml version="1.0" encoding="UTF-8"?>

<state-and-event-info>

<state-info>early</state-info>

<direction>initiator</direction>

</state-and-event-info>

16.	SIP INFO request (Intermediate IM CN subsystem entities to MSC server)

The intermediate IM CN subsystem entities forward the SIP INFO request to the MSC server. The MSC server is aware that the call that is transferred is in originating alerting state.

17.	SIP 200 (OK) response (MSC server to Intermediate IM CN subsystem entities)

The MSC Server acknowledges the receipt of the SIP INFO request.

18.	SIP 200 (OK) response (Intermediate IM CN subsystem entities to SCC AS)

The intermediate IM CN subsystem entities forward the SIP 200 (OK) response to the SCC AS.

19.	MSC goes in Call delivered state

The MSC enters Call delivered state due to the information received in the SIP INFO request.

20.	SIP 200 (OK) response (UE B to intermediate IM CN subsystem entities)

The UE B accepts the call and sends a SIP 200 (OK) response.

21.	SIP 200 (OK) response (Intermediate IM CN subsystem entities to SCC AS)

The SIP 200 (OK) response is forwarded to SCC AS.

22	SIP 200 (OK) response (SCC AS to intermediate IM CN subsystem entities)

The SCC AS sends the SIP 200 (OK) response to indicate that the terminating UE B has accepted the call.

23	200 (OK) response (Intermediate IM CN subsystem entities to MSC server)

The SIP 200 (OK) response is forwarded to the MSC server.

24	CC CONNECT message (MSC server to SC UE A)

The MSC server indicates to the SC UA A that the far end has accepted the call.

24a	Stop the ringing tone

The UE stops playing the locally generated ringing tone.

25	CC CONNECTACKNOWLEDGE (MSC server to SC UE A)

SC UE A acknowledges the CS CONNECT message.

26	SIP ACK request (MSC server to intermediate IM CN subsystem entities)

The MSC server acknowledges the SIP 200 (OK) response received from SCC AS

27.	SIP ACK request (Intermediate IM CN subsystem entities to SCC AS)

The SIP ACK request is forwarded to the SCC AS.

28	SIP ACK request (SCC AS to intermediate IM CN subsystem entities)

The SCC AS acknowledges the SIP 200 (OK) response received towards far end.

29	SIP ACK request (Intermediate IM CN subsystem entities to far end)

The SIP ACK request is forwarded towards the far end.

30 – 33 The SCC AS releases the original source leg towards the SC UE A

The SCC AS sends a SIP 404 (Not Found) response in order to release to original source dialog towards the SC UE A

NOTE:	Steps 31-32 are performed only if the SC UE A uses Gm the PS-CS access transfer in alerting phase is completed; otherwise, the SC UE A and the network release the source access leg locally, without any signalling between the SC UE A and the network.

13.4.3.14.3	Test description

13.4.3.14.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

13.4.3.14.3.2	Test procedure sequence

Table 13.4.3.14.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 13.4.3.14.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -60 | - | The power level values are such that entering conditions for event B2 are not satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -88 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -88 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -84 | - | The power level values are such that entering conditions for event B2 are satisfied. |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | Non-suitable “Off” | - |  |
|  | CPICH_Ec (UTRA FDD) | dBm/3.84 MHz | - | -64 |  |
|  | PCCPCH_Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -64 |  |

Table 13.4.3.14.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS configures UTRA cell 5 to reference configuration according to TS 36.508 Table 4.8.3-1, condition UTRA PS RB + Speech. | - | - | - | - |
| 2-15 | Steps 1 to 14 of the generic test procedure for IMS MO speech call and aSRVCC (TS 36.508 4.5A.10.3-1). | - | - | - | - |
| 16 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to setup inter RAT measurement and reporting for event B2. | <-- | RRCConnectionReconfiguration | - | - |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 18 | The SS changes the power level for Cell 1 and Cell 5 according to the row "T1" in Table 13.4.3.14.3.2-1 | - | - | - | - |
| 19 | The UE transmits a MeasurementReport message on Cell 1 to report event B2 for Cell 5. | --> | MeasurementReport | - | - |
| 20 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E-UTRA and UTRA. | <-- | UECapabilityEnquiry | - | - |
| 21 | The UE transmits a UECapabilityInformation message on Cell 1. NOTE: The start-PS and start-CS values received, should be used to configure ciphering on Cell 5. | --> | UECapabilityInformation | - | - |
| 22 | The SS transmits a MobilityFromEUTRACommand message on Cell 1. | <-- | MobilityFromEUTRACommand | - | - |
| 23 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message on Cell 5? | --> | HANDOVER TO UTRAN COMPLETE | 1 | P |
| - | EXCEPTION: In parallel to the events described in step 24 to 31 the steps specified in Table 13.4.3.14.3.2-3 take place. | - | - | - | - |
| 24 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 25 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 26 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 27 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 28 | The SS transmits a TMSI REALLOCATION COMMAND message. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 29 | The UE transmits a TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 30 | The SS transmits a CONNECT message on Cell 5. | <-- | CONNECT | - | - |
| 31 | Check: Does the UE transmit a CONNECT ACKNOWLEDGE message on Cell 5? | --> | CONNECT ACKNOWLEDGE | 2 | P |
| 32-37 | Void | - | - | - | - |
| - | EXCEPTION: Step 37Aa1; the “lower case letter” identifies a step sequence that may take place if pc_IMS_UTRAN = TRUE AND IMS SIP de-registration defined in Table 13.4.3.14.3.2-4 has not occurred. | - | - | - | - |
| 37Aa1 | Generic test procedure to remove IMS early dialog of outgoing call definedin Annex C.34 of TS 34.229-1 [35]. | - | - | - | - |
| 37A-37B | Void | - | - | - | - |
| 38 | SS adjusts cell levels according to row T2 of Table 13.4.3.14.3.2-1. | - | - | - | - |
| - | The UE is in end state UTRA CS call (U5). | - | - | - | - |

Table 13.4.3.14.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: If pc_IMS_UTRAN = TRUE THEN In parallel to the events described in step 1 to 5, the steps defined in Table 13.4.3.14.3.2-4 may take place. | - | - | - | - |
| 1 | Check: Does the UE transmit a ROUTING AREA UPDATE REQUEST message? | --> | ROUTING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a SECURITY MODE COMMAND message for the PS domain on Cell 5. | <-- | SECURITY MODE COMMAND | - | - |
| 3 | The UE transmits a SECURITY MODE COMPLETE message on Cell 5. | --> | SECURITY MODE COMPLETE | - | - |
| 4 | The SS transmits a ROUTING AREA UPDATE ACCEPT message on Cell 5. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 5 | The UE transmits a ROUTING AREA UPDATE COMPLETE message on Cell 5. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: If pc_IMS_UTRAN = TRUE THEN Step 5Aa1 will take place. | - | - | - | - |
| 5Aa1 | Wait for 2 seconds to provide enough time to finish up optional IMS deregistration. | - | - | - | - |
| - | EXCEPTION: Step 6a1-6a2 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a certain action. | - | - | - | - |
| 6a1 | The UE transimts PDP CONTEXT Deactivaiton message | <-- | DEACTIVATE PDP CONTEXT REQUEST | - | - |
| 6a2 | The UE transimts PDP CONTEXT Deactivaiton message | <-- | DEACTIVATE PDP CONTEXT ACCEPT | - | - |

Table 13.4.3.14.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Generic test procedure for mobile initiated IMS SIP re-registration or de-registration defined in Annex C.46 or C.30, respectively, of TS 34.229-1 [35]. | - | - | - | - |

13.4.3.14.3.3	Specific message contents

Table 13.4.3.14.3.3-0: Conditions for specific message contents
in Table 13.4.3.14.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 13.4.3.14.3.3-1: ATTACH REQUEST (preamble)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MS network capability | SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported |  |  |
| Mobile station classmark 2 | Any allowed value |  |  |
| Supported Codecs | Any allowed value |  |  |

Table 13.4.3.14.3.3-2: RRCConnectionReconfiguration (step 16, Table 13.4.3.14.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 13.4.3.14.3.3-3: MeasConfig (step 16, Table 13.4.3.14.3.2-2)

| Derivation path: 36.508 clause 4.6.6 table 4.6.6-1 with condition UTRAN |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measurementConfiguration ::= SEQUENCE { |  |  |  |
| measObjectToAddModifyList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| measObject[1] | MeasObjectUTRA-GENERIC(f8) |  |  |
| measObjectId[2] | IdMeasObject-f1 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModifyList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-UTRA |  |  |
| reportConfig[1] | ReportConfigInterRAT-B2-UTRA (-72, -76) |  |  |
| } |  |  |  |
| measIdToAddModifyList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f8 |  |  |
| reportConfigId[1] | IdReportConfigInterRAT-B2-UTRA |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { |  |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE {} |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.14.3.3-4: MeasurementReport (step 19, Table 13.4.3.14.3.2-2)

| Derivation Path: 36.508, table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 5 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| utra-RSCP | (-5..91) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.14.3.3-5: MobilityFromEUTRACommand (step 20, Table 13.4.3.14.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| mobilityFromEUTRACommand-r8 SEQUENCE { |  |  |  |
| cs-FallbackIndicator | False |  |  |
| purpose CHOICE{ |  |  |  |
| handover SEQUENCE { |  |  |  |
| targetRAT-Type | Utra |  |  |
| targetRAT-MessageContainer | HANDOVER TO UTRAN COMMAND(UTRA RRC message) |  |  |
| nas-SecurityParamFromEUTRA | The 4 least significant bits of the NAS downlink COUNT value |  |  |
| systemInformation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.14.3.3-6: HANDOVER TO UTRAN COMMAND (step 20, Table 13.4.3.14.3.3-5)

Derivation Path: 36.508, Table 4.7B.1-1, condition UTRA PS RB + Speech

Table 13.4.3.14.3.3-7: UECapabilityEnquiry (step 21, Table 13.4.3.14.3.2-2)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 2 entry |  |  |
| RAT-Type[1] | eutra |  |  |
| RAT-Type[2] | utra |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.4.3.14.3.3-8: SECURITY MODE COMMAND (step 26, Table 13.4.3.14.3.2-2)

| Derivation Path: 36.508, Table 4.7B.1-n |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

Table 13.4.3.14.3.3-9: ROUTING AREA UPDATE ACCEPT (step 4, Table 13.4.3.2.3.2-3)

| Derivation path: 36.508, Table 4.7B.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Update result | 0 'follow-on proceed' |  |  |
| PDP context status | ‘0010000000000000’B | NSAPI 5 |  |

#### <End of change>