3GPP TSG-RAN5 Meeting #105	R5-247480r1

# Orlando, USA, November 18 - 22 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0532 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Applicability of RACH-based LTM PSCell switch test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Applicability is missing for LTM PSCell switch and |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability has been corrected in Table 4.2-3 and 4.2-4 for few test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases and the work plan will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: revised to add 2Rx/4Rx/8Rx branch for |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause |  | TC Title |  | Release |  | Applicability |  |  |  | Additional Information |  | Branch |  | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |
| 6.1 |  | RRC_IDLE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1 |  | NR cell re-selection |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.1.1 |  | NR SA FR1 cell re-selection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.2 |  | NR SA FR1-FR1 cell re-selection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.3 |  | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.4 |  | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.5 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.6 |  | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C093 |  | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.7 |  | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 |  | Rel-16 |  | C052 |  | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.8 |  | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 |  | Rel-17 |  | C052a |  | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.1.9 |  | NR SA FR1 cell reselection to intra-frequency NR case for UE operating on a cell with less than 5MHz BW |  | Rel-18 |  | C342 |  | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.1.2 |  | NR – E-UTRA cell re-selection |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1.2.1 |  | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.2.2 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.2.3 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C094 |  | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.2.4 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C094 |  | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.1.2.5 |  | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Rel-16 |  | C025b |  | UEs supporting 5GS NR SA FR1 and E-UTRA and E-UTRA inter-RAT measurement enhancements in HST |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.2 |  | RRC_INACTIVE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.1 |  | NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) |  | Rel-17 |  | C001m |  | UEs supporting 5GS NR SA FR1, CG-SDT and long DRX cycle |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3 |  | RRC_CONNECTED state mobility |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.1 |  | Handover |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.1.1 |  | NR SA FR1 handover with known target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.2 |  | NR SA FR1 handover with unknown target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.3 |  | NR SA FR1-FR1 handover with unknown target cell |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.4 |  | NR SA FR1 – E-UTRA handover with known target cell |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.5 |  | NR SA FR1 – E-UTRA handover with unknown target cell |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.6 |  | NR SA FR1 – UTRAN FDD handover with known target cell |  | Rel-16 |  | C096 |  | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.7 |  | NR SA FR1 synchronous DAPS handover |  | Rel-16 |  | C101 |  | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.8 |  | NR SA FR1 asynchronous DAPS handover |  | Rel-16 |  | C102 |  | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.9 |  | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | Rel-16 |  | C107 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.10 |  | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | Rel-16 |  | C108 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.11 |  | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover |  | Rel-16 |  | C107 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | For test configuration 1, 2, 4, 5, 9 |  | 2Rx 4Rx 8Rx |  |  |
|  |  |  |  |  |  | C109 |  | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell |  | For test configuration 3, 6, 7, 8 |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.12 |  | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover |  | Rel-16 |  | C108 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | For test configuration 1, 2, 4, 5, 9 |  | 2Rx 4Rx 8Rx |  |  |
|  |  |  |  |  |  | C110 |  | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell |  | For test configuration 3, 6, 7, 8 |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.1.18 |  | NR SA FR1 Intra-frequency handover for unknown target cell operating with 12 PRB SSB bandwidth |  | Rel-18 |  | C343 |  | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.2 |  | RRC connection mobility control |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.1 |  | RRC re-establishment |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.1.1 |  | NR SA FR1 RRC re-establishment |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.1.2 |  | NR SA FR1 - FR1 RRC re-establishment |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.1.3 |  | NR SA FR1 RRC re-establishment without serving cell timing |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.2 |  | Random access |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.2.1 |  | NR SA FR1 contention based random access |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.3.2.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.2.2 |  | NR SA FR1 non-contention based random access |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.3.2.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  | Subtest 2: F005 |
| 6.3.2.2.3 |  | NR SA FR1 2-step contention based random access |  | Rel-16 |  | C159 |  | UEs supporting 5GS NR SA FR1and 2-step RACH |  | Test execution not necessary if test 4.3.2.2.3 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.2.4 |  | NR SA FR1 2-step non-contention based random access |  | Rel-16 |  | C159 |  | UEs supporting 5GS NR SA FR1and 2-step RACH |  | Test execution not necessary if test 4.3.2.2.4 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.3 |  | RRC connection release with redirection |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.3.1 |  | NR SA FR1 RRC connection release with redirection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.3.2 |  | NR SA FR1 - E-UTRA RRC connection release with redirection |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.2.4 |  | LTM PDCCH-order Random Access |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.2.4.1 |  | PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.2.4.2 |  | PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.2.4.3 |  | PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.3 |  | Conditional handover |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.3.1 |  | NR SA FR1 conditional handover |  | Rel-16 |  | C105 |  | UEs supporting 5GS NR SA FR1 and Conditional handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.3.2 |  | NR SA FR1-FR1 conditional handover |  | Rel-16 |  | C105 |  | UEs supporting 5GS NR SA FR1 and Conditional handover |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.3.4 |  | LTM PCell Switch |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.4.1 |  | RACH-based Intra-frequency PCell switch from FR1 to FR1 |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.4.2 |  | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.4.3 |  | RACH-less Intra-frequency PCell switch from FR1 to FR1 |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.3.5 |  | LTM PSCell Switch |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.3.5.1 |  | NR SA FR1 RACH-based Intra-frequency LTM PSCell switch |  | Rel-18 |  | FFS |  |  |  | NOTE 1 |  | 2Rx 4Rx 8Rx |  |  |
| 6.4 |  | Timing |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.1 |  | UE transmit timing |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.1.1 |  | NR SA FR1 UE transmit timing accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.4.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  | Subtest 2: F006 |
| 6.4.2 |  | UE timer accuracy |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.3 |  | Timing advance |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.4.3.1 |  | NR SA FR1 timing advance adjustment accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.4.3.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5 |  | Signalling characteristics |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.1 |  | Radio Link Monitoring |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.1.1 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.5.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.2 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.5.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.3 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.5.1.3 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.4 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.5.1.4 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.5 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C037 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | Test execution not necessary if test 4.5.1.5 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.6 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C037 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM |  | Test execution not necessary if test 4.5.1.6 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.7 |  | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C037a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | Test execution not necessary if test 4.5.1.7 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.8 |  | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C037a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle |  | Test execution not necessary if test 4.5.1.8 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.9 |  | SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion |  | Rel-17 |  | C037b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and RLM relaxation criteria rlm-Relaxation-r17 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.1.13 |  | NR SA FR1 radio Link Monitoring out-of-sync Test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW |  | Rel-18 |  | C344 |  | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz and long DRX cycle |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.5.1.14 |  | NR SA FR1 radio Link Monitoring out-of-sync Test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW |  | Rel-18 |  | C343 |  | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.5.2 |  | Interruption |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.2.1 |  | NR SA FR1 interruptions during measurements on deactivated NR SCC |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.2.2 |  | SA FR1 interruptions at NR SRS carrier based switching |  | Rel-16 |  | C031d |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SRS carrier based switching |  | Note 1 |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3 |  | Scell activation and deactivation delay |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.3.1 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.2 |  | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.3 |  | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Rel-15 |  | C031 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.4 |  | NR SA FR1 direct SCell activation at SCell addition of known SCell |  | Rel-16 |  | C244 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.5 |  | NR SA FR1 direct SCell activation at handover with known SCell |  | Rel-16 |  | C244 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.10 |  | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle |  | Rel-17 |  | C270 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.3.11 |  | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle |  | Rel-17 |  | C270 |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.4 |  | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.4.1 |  | NR SA FR1 UE UL carrier RRC reconfiguration delay |  | Rel-15 |  | C002 |  | UEs supporting 5GS NR SA FR1 and SUL |  | Test execution not necessary if test 4.5.4.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5 |  | Link recovery procedures |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.5.1 |  | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C084 |  | UEs supporting 5GS NR SA FR1 and link recovery |  | Test execution not necessary if test 4.5.5.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.2 |  | NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C084a |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery |  | Test execution not necessary if test 4.5.5.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.3 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C085 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery |  | Test execution not necessary if test 4.5.5.3 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.4 |  | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C085a |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  | Test execution not necessary if test 4.5.5.4 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.5 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX |  | Rel-16 |  | C173 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell |  | Test execution not necessary if test 4.5.5.5 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.6 |  | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX |  | Rel-16 |  | C174 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell and long DRX cycle |  | Test execution not necessary if test 4.5.5.6 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.5.7 |  | NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX |  | Rel-17 |  | C085b |  | UEs supporting TRP specific 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.6 |  | Active BWP switch delay |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.1 |  | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.1.1 |  | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066a |  | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.6.1.2 |  | NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066 |  | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.6.2 |  | RRC-based active BWP switch |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.2.1 |  | NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | Rel-15 |  | C066b |  | UEs supporting 5GS NR SA FR1 and (Support of BWP adaptation upto2 or upto4) |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.6.3 |  | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.3.1 |  | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | Rel-16 |  | C066e |  | UEs supporting 5GS NR SA FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs and 3DL CA |  | NOTE 1 |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.6.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.5 |  | Simultaneous RRC-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.6.5.1 |  | RRC based BWP switch on multiple CCs |  | Rel-16 |  | C066d |  | UEs supporting 5GS NR SA FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs and 2DL CA |  | NOTE 1 |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7 |  | DL interruptions at switching between two uplink carriers |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.7.1 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA |  | Rel-16 |  | C051 |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7.2 |  | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA |  | Rel-16 |  | C051a |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7A |  | NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.7A.1 |  | NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA |  | Rel-17 |  | C051b |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7A.2 |  | NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA |  | Rel-17 |  | C051c |  | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7B |  | DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.7B.1 |  | NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA |  | Rel-17 |  | C051d |  | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7B.2 |  | NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA |  | Rel-17 |  | C051e |  | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7C |  | DL interruptions at switching between two uplink bands with two transmit antenna connectors |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.7C.1 |  | NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA |  | Rel-17 |  | C051f |  | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7C.2 |  | NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA |  | Rel-17 |  | C051g |  | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7D |  | DL interruptions at UE switching across three or four uplink bands |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.7D.1 |  | NR SA FR1 DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG |  | Rel-18 |  | C338 |  | UEs supporting 5GS NR SA FR1, inter-band UL CA (three bands), dynamic UL Tx switching across up to 4 bands and simultaneous transmission and reception in TDD-TDD inter-band NR CA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7D.2 |  | NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG |  | Rel-18 |  | C339 |  | UEs supporting 5GS NR SA FR1, inter-band UL CA (four bands), dynamic UL Tx switching across up to 4 bands and simultaneous transmission and reception in FDD-TDD inter-band NR CA |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.7D.3 |  | NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs |  | Rel-18 |  | C340 |  | UEs supporting 5GS NR SA FR1, inter-band UL CA (three bands), dynamic UL Tx switching across up to 4 bands, simultaneous transmission and reception in FDD-TDD inter-band NR CA and at least two TAGs |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.5.8 |  | UE specific CBW change |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.5.8.1 |  | UE specific CBW change on PCell in FR1 in non-DRX |  | Rel-16 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6 |  | Measurement procedures |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.1.1 |  | NR SA FR1 event-triggered reporting without gap in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.6.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.2 |  | NR SA FR1 event-triggered reporting without gap in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.6.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.3 |  | NR SA FR1 event-triggered reporting with gap in non-DRX |  | Rel-15 |  | C041 |  | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | Test execution not necessary if test 4.6.1.3 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.4 |  | NR SA FR1 event-triggered reporting with gap in DRX |  | Rel-15 |  | C041a |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle |  | Test execution not necessary if test 4.6.1.4 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.5 |  | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading |  | Rel-15 |  | C024 |  | UEs supporting 5GS NR FDD SA FR1 |  | Test execution not necessary if test 4.6.1.5 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.6 |  | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading |  | Rel-15 |  | C041b |  | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction |  | Test execution not necessary if test 4.6.1.6 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.7 |  | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-16 |  | C052 |  | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | Test execution not necessary if test 4.6.1.7 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.1.8 |  | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 |  | Rel-17 |  | C052b |  | UEs supporting 5GS NR SA FR1 and CA measurement enhancements in HST |  | Test execution not necessary if test 4.6.1.8 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.2.1 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.6.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.2 |  | NR SA FR1-FR1 event-triggered reporting in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.6.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.5 |  | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.6.2.5 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.6 |  | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.6.2.6 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.9 |  | NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern |  | Rel-16 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.10 |  | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used |  | Rel-16 |  | C287 |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and inter-frequency SSB based measurements without measurement gaps |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.11 |  | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used |  | Rel-16 |  | C286 |  | UEs supporting 5GS NR SA FR1 and inter-frequency SSB based measurements without measurement gaps |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.2.12 |  | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 |  | Rel-17 |  | C052c |  | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  | Test execution not necessary if test 4.6.2.9 has been executed. |  | 2Rx 4Rx 8Rx |  | Subtest 2: F017 |
| 6.6.3 |  | Inter-RAT measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.3.1 |  | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | Rel-15 |  | C025 |  | UEs supporting 5GS NR SA FR1 and E-UTRAN |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.3.2 |  | NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Rel-15 |  | C025a |  | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.3.3 |  | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-16 |  | C025c |  | UEs supporting 5GS NR SA FR1 and E-UTRAN, long DRX cycle and E-UTRA inter-RAT measurement enhancements in HST |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4 |  | L1-RSRP measurement for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.4.1 |  | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.6.4.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4.2 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.6.4.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4.3 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.6.4.3 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4.4 |  | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Rel-15 |  | C001b |  | UEs supporting 5GS NR SA FR1 and long DRX cycle |  | Test execution not necessary if test 4.6.4.4 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4.5 |  | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Rel-16 |  | C001f |  | UEs supporting 5GS NR SA FR1, long DRX cycle and intra-NR measurement enhancements in HST |  | Test execution not necessary if test 4.6.4.5 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.4.6 |  | NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX |  | Rel-17 |  | C278 |  | UEs supporting 5GS NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.5 |  | UTRAN inter-RAT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.5.1 |  | NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  | Rel-16 |  | C096 |  | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.8 |  | L1-SINR measurement for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.8.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C144 |  | UEs supporting 5GS NR SA FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.8.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C145 |  | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.8.3 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C146 |  | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.9 |  | Idle Mode CA/DC Measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.9.1 |  | NR SA FR1 Idle mode CA/DC measurement for FR1 |  | Rel-16 |  | C031a |  | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.15 |  | Idle Mode inter-RAT CA/DC Measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.15.1 |  | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting |  | Rel-16 |  | C031b |  | UEs supporting NE-DC FR1 and E-UTRA measurements in RRC_IDLE/RRC_INACTIVE |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.17 |  | SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.17.1 |  | NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG |  | Rel-17 |  | C259 |  | UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.17.2 |  | NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | Rel-17 |  | C260 |  | UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredNW-ControlledMeasGap-r17 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.18 |  | SA event triggered reporting tests with concurrent gaps |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.18.1 |  | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers |  | Rel-17 |  | C264 |  | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.18.2 |  | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers |  | Rel-17 |  | C264 |  | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.18.3 |  | NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 |  | Rel-17 |  | C265 |  | UEs supporting 5GS NR SA FR1 and E-UTRA and more than 1 per-UE measurement gap configurations and the configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.18.4 |  | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used |  | Rel-17 |  | C266 |  | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations and two independent measurement gap configurations for FR1 and FR2 for PRS measurement |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.19 |  | SA event triggered reporting tests with NCSG |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.19.1 |  | NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 |  | Rel-17 |  | C253 |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction, NR only NCSG patterns and reporting of NCSG requirement information but don’t support per-FR NCSG |  | For sub-test 1 |  | 2Rx 4Rx 8Rx |  |  |  |
|  |  |  |  |  |  | C254 |  | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns |  | For sub-test 2 |  | 2Rx 4Rx 8Rx |  |  |  |
| 6.6.19.2 |  | NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement |  |  |  | C255 |  | UEs supporting 5GS NR SA FR1, NR only NCSG patterns and reporting of NCSG requirement information but don’t support per-FR NCSG |  | For sub-test 1 |  | 2Rx 4Rx 8Rx |  |  |  |
|  |  |  |  |  |  | C256 |  | UEs supporting 5GS NR SA FR1, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns |  | For sub-test 2 |  | 2Rx 4Rx 8Rx |  |  |  |
| 6.6.19.3 |  | NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG |  | Rel-17 |  | C257 |  | UEs supporting 5GS NR SA FR1, E-UTRAN, reporting of NCSG requirement information for E-UTRA and NCSG patterns |  |  |  | 2Rx 4Rx 8Rx |  |  |  |
| 6.6.19.4 |  | NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX |  | Rel-17 |  | C258 |  | UEs supporting 5GS NR SA FR1, CA (2DL CA), reporting of NCSG requirement information and NR only NCSG patterns |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.20 |  | UE Rx-Tx time difference measurement for propagation delay compensation |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.20.1 |  | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA |  | Rel-17 |  | C212 |  | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with PRS |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.21 |  | UE Rx-Tx time difference measurement for propagation delay compensation with TRS |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.21.1 |  | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA |  | Rel-17 |  | C213 |  | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with TRS |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.6.26 |  | LTM Intra-frequency L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.26.1 |  | Intra-frequency SSB based L1-RSRP measurement |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.6.27 |  | LTM Inter-frequency L1-RSRP measurement with measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.27.1 |  | Inter-frequency SSB based L1-RSRP measurement with measurement gap |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.6.28 |  | LTM Inter-frequency L1-RSRP measurement without measurement gap |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.6.28.1 |  | Inter-frequency SSB based L1-RSRP measurement without measurement gap |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.7 |  | Measurement performance requirements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1 |  | SS-RSRP |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1.1 |  | NR SA FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.1.1.2 |  | NR SA FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.1.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 |  | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.1.2.2 |  | NR SA FR1-FR1 SS-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.1.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.2 |  | SS-RSRQ |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.2.1 |  | NR SA FR1 SS-RSRQ measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.2.2.1 |  | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.2.2.2 |  | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.2.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.3 |  | SS-SINR |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.3.1 |  | NR SA FR1 SS-SINR measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.3.2.1 |  | NR SA FR1-FR1 SS-SINR absolute measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.3.2.2 |  | NR SA FR1-FR1 SS-SINR relative measurement accuracy |  | Rel-15 |  | C034 |  | UEs supporting 5GS NR SA FR1 and SS-SINR-meas |  | Test execution not necessary if test 4.7.3.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.4 |  | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 |  | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.4.1.2 |  | NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.4.2.1 |  | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.2.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.4.2.2 |  | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  | Test execution not necessary if test 4.7.4.2.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.5 |  | E-UTRAN RSRP |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.5.1 |  | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.6 |  | E-UTRAN RSRQ |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.6.1 |  | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy |  | Rel-15 |  | C001 |  | UEs supporting 5GS NR SA FR1 |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.7 |  | E-UTRAN RS-SINR |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.7.1 |  | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy |  | Rel-15 |  | C168 |  | UEs supporting 5GS NR SA FR1 and E-UTRA RS-SINR measurements |  |  |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.9 |  | L1-SINR |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.9.1.1 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy |  | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | Test execution not necessary if test 4.7.7.1.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.9.1.2 |  | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy |  | Rel-16 |  | C132 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  | Test execution not necessary if test 4.7.7.1.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.9.2 |  | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C133 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.9.3.1 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy |  | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.3.1 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.9.3.2 |  | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy |  | Rel-16 |  | C134 |  | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | Test execution not necessary if test 4.7.7.3.2 has been executed. |  | 2Rx 4Rx 8Rx |  |  |
| 6.7.17 |  | LTM L1-RSRP measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.17.1.1 |  | Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| 6.7.17.1.2 |  | Inter-frequency L1-RSRP relative accuracy requirements for neighbour cell in FR1 |  | Rel-18 |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx 4Rx |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. NOTE 3: Test cases in TS 38.533 [5] clause 6 only apply to FR1 non-RedCap UEs. For FR1 RedCap UEs, Test cases in TS 38.533 [5] clause 16 apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.2-3a: Void

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 7.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 7.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 7.1.1.1 | NR SA FR2 cell re-selection | FFS | FFS | FFS |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | FFS | FFS | FFS |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C095 | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  |
| 7.1.1.7 | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-17 | C240 | UEs supporting 5GS NR SA FR2 and intra-frequency RRC_IDLE measurements in HST | NOTE 1 |  |  |
| 7.1.1.8 | NR SA FR2 inter-frequency cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-18 | C345 | UEs supporting 5GS NR SA FR2 and inter-frequency RRC_IDLE measurements in HST | NOTE 1 |  |  |
| 7.2 | RRC_INACTIVE state mobility |  |  |  |  |  |  |
| 7.2.1 | Small Data Transmission |  |  |  |  |  |  |
| 7.2.1.1 | TA Validation for CG-SDT in FR2 | Rel-17 | C095a | UEs supporting 5GS NR SA FR2 and TA Validation for CG-SDT in FR2 | NOTE 1 |  |  |
| 7.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 7.3.1 | Handover |  |  |  |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | Rel-16 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 7.3.2.1 | RRC re-establishment |  |  |  |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.3.2.1.2 | NR SA FR2 - FR2 RRC re-establishment | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  |  |
| 7.3.2.2 | Random access |  |  |  |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  | Subtest 2: F007 |
| 7.3.2.2.4 | NR SA FR2 2-step non-contention based random access | Rel-16 | C160 | UEs supporting 5GS NR SA FR2 and 2-step RACH | NOTE 1 |  |  |
| 7.3.2.3 | RRC connection release with redirection |  |  |  |  |  |  |
| 7.3.2.3.1 | NR SA FR2-FR2 RRC connection release with redirection | FFS | FFS | FFS |  |  |  |
| 7.3.3 | Conditional Handover |  |  |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | Rel-16 | C106 | UEs supporting 5GS NR SA FR2 and conditional handover | NOTE 1 |  |  |
| 7.3.5 | LTM PSCell Switch |  |  |  |  |  |  |
| 7.3.5.1 | NR SA FR2 RACH-based Intra-frequency LTM PSCell switch | Rel-18 | FFS |  | NOTE 1 |  |  |
| 7.4 | Timing |  |  |  |  |  |  |
| 7.4.1 | UE transmit timing |  |  |  |  |  |  |
| 7.4.1.1 | NR SA FR2 UE transmit timing accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 | Subtest 2: F008 |
| 7.4.1.3 | NR SA FR2 UE Transmit Timing Test with 2-TA for UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Rel-18 | C349 | UEs supporting 5GS NR SA FR2 and 2 TAGs for multi-DCI multi-TRP operation | NOTE 1 |  |  |
| 7.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 7.4.3 | Timing advance |  |  |  |  |  |  |
| 7.4.3.1 | NR SA FR2 timing advance adjustment accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.5 | Signalling characteristics |  |  |  |  |  |  |
| 7.5.1 | Radio Link Monitoring |  |  |  |  |  |  |
| 7.5.1.1 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.5.1.2 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.5.1.3 | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.5.1.4 | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.5.1.5 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C164 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM | NOTE 1 |  |  |
| 7.5.1.6 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode | Rel-15 | C164 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM | NOTE 1 |  |  |
| 7.5.1.7 | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C165 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM and log DRX cycle |  |  |  |
| 7.5.1.8 | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Rel-15 | C165 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM and log DRX cycle |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | Rel-15 | C006n | UEs supporting 5GS NR SA FR2 and PDCCH monitoring in any symbol of the slot (with or without span gap) | NOTE 1 |  |  |
| 7.5.2 | Interruption |  |  |  |  |  |  |
| 7.5.2.1 | NR SA FR2 interruptions during measurements on deactivated NR SCC | Rel-15 | C006b | UEs supporting 5GS NR SA FR2 and 2DL CA in NR |  |  |  |
| 7.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 7.5.3.1 | NR SA FR2-FR2 intra-band SCell activation and deactivation delay | FFS | FFS | FFS | NOTE 1 |  |  |
| 7.5.3.2 | NR SA FR1-FR2 inter-band SCell activation and deactivation delay | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.5.3.3 | SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX | Rel-17 | C031c | UEs supporting 5GS NR SA FR2 and 2DL CA in NR |  |  |  |
| 7.5.3.4 | NR SA FR2 direct SCell activation at SCell addition of known SCell | Rel-16 | C241 | UEs supporting 5GS NR SA FR2 and 2DL CA in NR and direct SCell activation |  |  |  |
| 7.5.3.5 | NR SA FR2 direct SCell activation at handover with known SCell | Rel-16 | C241 | UEs supporting 5GS NR SA FR2 and 2DL CA in NR and direct SCell activation |  |  |  |
| 7.5.3.13 | NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX | Rel-17 | C271 | UEs supporting 5GS NR SA intra-band FR2 and 2DL CA in NR and fast SCell activation |  |  |  |
| 7.5.3.14 | NR SA FR2 SCell Activation for known SCell in FR2 inter-band | Rel-17 | C271a | UEs supporting 5GS NR SA inter-band FR2 and 2DL CA in NR and fast SCell activation | NOTE 1 |  |  |
| 7.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C164 | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C165 | UEs supporting 5GS NR SA FR2, long DRX cycle and CSI-RS based RLM |  |  |  |
| 7.5.5.5 | NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.5.5.6 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-16 | C147 | UEs supporting 5GS NR SA FR2 and CSI-RS based BFR on Scell |  |  |  |
| 7.5.5.7 | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX | Rel-16 | C148 | UEs supporting 5GS NR SA FR2 and long DRX cycle and CSI-RS based BFR on Scell |  |  |  |
| 7.5.5.9 | NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | Rel-17 | C148a | UEs supporting 5GS NR TRP specific SA FR2 and long DRX cycle and CSI-RS based BFR on Scell |  |  |  |
| 7.5.5.10 | NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX | Rel-17 | C006x | UEs supporting 5GS NR TRP specific SA FR2 SSB based BFR |  |  |  |
| 7.5.6 | Active BWP switch delay |  |  |  |  |  |  |
| 7.5.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 7.5.6.1.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 |  |  |
| 7.5.6.1.2 | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.5.6.1.3 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 | NOTE 1 |  |  |
| 7.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |  |
| 7.5.6.2.1 | NR SA FR2 RRC-based DL active BWP switch in non-DRX | FFS | FFS | FFS | NOTE 1 |  |  |
| 7.5.7 | PSCell addition and release delay |  |  |  |  |  |  |
| 7.5.7.1 | NR SA FR2 addition and release delay of known PSCell | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.5.7.2 | NR SA FR2 addition and release delay of unknown PSCell | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.5.8 | Active TCI state switch delay |  |  |  |  |  |  |
| 7.5.8.1 | NR SA FR2 MAC-CE based active TCI state switch |  |  |  |  |  |  |
| 7.5.8.1.1 | NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | Rel-15 | C006m | UEs supporting 5GS NR SA FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 |  |  |  |
| 7.5.8.2 | NR SA FR2 RRC based active TCI state switch |  |  |  |  |  |  |
| 7.5.8.2.1 | NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state | Rel-15 | C006m | UEs supporting 5GS NR SA FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 |  |  |  |
| 7.5.8.3 | MAC-CE based active TCI state switch for HST FR2 scenario |  |  |  |  |  |  |
| 7.5.8.3.1 | NR PCell FR2 HST active TCI state switch for a known TCI state | Rel-17 | C240a | UEs supporting 5GS NR SA FR2 and one-shot large UL timing adjustment.in HST | NOTE 1 |  |  |
| 7.5.8.3.2 | NR SA FR2 HST active TCI state switch for PC6 UE supporting tci‑StateSwitchInd‑r18 a known TCI state | Rel-18 | C345a | UEs supporting 5GS NR SA FR2 and enhanced one-shot large UL timing adjustment and enhanced TCI state switching delay requirements in HST | NOTE 1 |  |  |
| 7.5.11 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |
| 7.5.11.1 | UE UL carrier RRC reconfiguration delay | Rel-17 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.5.12 | Conditional PSCell addition and release delay (FR2 SA) |  |  |  |  |  |  |
| 7.5.12.1 | NR SA FR2 Addition and Release Delay of PSCell | Rel-17 | C268b | UEs supporting Inter-band NR-DC between FR1 and FR2 and conditional PSCell addition in NR-DC |  |  |  |
| 7.5.13 | Unified TCI state switch delay |  |  |  |  |  |  |
| 7.5.13.1 | NR SA FR2 MAC-CE based active joint TCI state switch | Rel-17 | C281 | UEs supporting 5GS NR SA FR2, and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |  |  |  |
| 7.5.13.2 | NR SA FR2 MAC-CE based active uplink TCI state switch | Rel-17 | C282 | UEs supporting 5GS NR SA FR2, and unified TCI state operation with separate DL/UL TCI update for intra-cell beam management |  |  |  |
| 7.5.13.3 | NR SA FR2 MAC-CE based active downlink TCI state switch | Rel-17 | C283 | UEs supporting 5GS NR SA FR2, and supporting RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info, and unified TCI with separate DL/UL TCI update for inter-cell beam management |  |  |  |
| 7.5.13.6 | NR SA FR2 MAC-CE based active uplink TCI state switch for two known TCI states | Rel-18 | C348 | UEs supporting 5GS NR SA FR2, and unified TCI state operation with separate DL/UL TCI update for intra-cell beam management with multi-TRP | NOTE 1 |  |  |
| 7.5.14 | PSCell activation and deactivation delay |  |  |  |  |  |  |
| 7.5.14.1 | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | Rel-17 | C284 | UEs supporting Inter-band NR-DC between FR1 and FR2 and activation and deactivation on SCG in NR-DC |  |  |  |
| 7.6 | Measurement procedures |  |  |  |  |  |  |
| 7.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  | PC1 PC3 |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C166 | UEs supporting 5GS NR SA FR2, CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | Rel-15 | C167 | UEs supporting 5GS NR SA FR2 long DRX cycle, CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  |  |
| 7.6.1.5 | NR SA FR2 event-triggered reporting without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17 | Rel-17 | C240 | UEs supporting 5GS NR SA FR2 and intra-frequency measurements in HST | NOTE 1 |  |  |
| 7.6.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.6.2.10 | SA FR2-FR2 event triggered reporting test without gap under non-DRX | Rel-16 | C334 | UEs supporting 5GS NR SA FR2 and inter-frequency SSB based measurements without measurement gaps |  |  |  |
| 7.6.2.11 | SA FR2-FR2 event triggered reporting test without gap under DRX | Rel-16 | C335 | UEs supporting 5GS NR SA FR2 and long DRX cycle and inter-frequency SSB based measurements without measurement gaps |  |  |  |
| 7.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C006a | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  |
| 7.6.3.5 | NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | Rel-17 | C240 | power class 6 UEs supporting 5GS NR SA FR2 configured and HST intra-frequency measurements | NOTE 1 |  |  |
| 7.6.3.6 | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | Rel-17 | C006p | UEs supporting 5GS NR SA FR2 and Inter-band CA (2UL CA) |  |  |  |
| 7.6.4.1 | NR SA FR2 SRS-RSRP measurement in non-DRX | Rel-16 | C006l | UEs supporting 5GS NR SA FR2 and SRS-RSRP measurements |  |  |  |
| 7.6.6 | L1-SINR measurement for beam reporting |  |  |  |  |  |  |
| 7.6.6.1 | NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C144a | UEs supporting 5GS NR SA FR2 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |  |
| 7.6.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C145a | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |  |
| 7.6.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C146a | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |  |
| 7.6.13 | UE Rx-Tx time difference measurements for PDC |  |  |  |  |  |  |
| 7.6.13.1 | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS | Rel-17 | C212 | UEs supporting 5GS NR SA FR2 and RTT-based PDC for Rx-Tx measurement with PRS |  |  |  |
| 7.6.13.2 | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS | Rel-17 | C213 | UEs supporting 5GS NR SA FR2 and RTT-based PDC for Rx-Tx measurement with TRS | NOTE 1 |  |  |
| 7.6.14 | SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |
| 7.6.14.1 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation | Rel-17 | C290 | UEs supporting 5GS NR SA FR2, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 | NOTE 1 |  |  |
| 7.6.14.2 | NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | Rel-17 | C291 | UEs supporting 5GS NR SA FR2, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredNW-ControlledMeasGap-r17 | NOTE 1 |  |  |
| 7.6.15 | SA event triggered reporting tests with concurrent gaps |  |  |  |  |  |  |
| 7.6.15.1 | NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements | Rel-17 | C292 | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations | NOTE 1 |  |  |
| 7.6.15.2 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) | Rel-17 | C292 | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations | NOTE 1 |  |  |
| 7.6.15.3 | NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement | Rel-17 | C293 | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations and two independent measurement gap configurations for FR1 and FR2 for PRS measurement | NOTE 1 |  |  |
| 7.6.16 | SA event triggered reporting tests with NCSG |  |  |  |  |  |  |
| 7.6.16.1 | NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | Rel-17 | C294 | UEs supporting 5GS NR SA FR2, CSI-RS-based RLM, BWP operation without bandwidth restriction, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns | NOTE 1 |  |  |
| 7.6.16.2 | NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used (PCell in FR2) | Rel-17 | C295 | UEs supporting 5GS NR SA FR2, reporting of NCSG requirement information and NR only NCSG patterns but not supporting ncsg-MeasGapPerFR-r17 | For sub-test 1 NOTE 1 |  |  |
|  |  |  | C296 | UEs supporting 5GS NR SA FR2, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns | For sub-test 2 NOTE 1 |  |  |
| 7.6.16.3 | NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX | Rel-17 | C297 | UEs supporting 5GS NR SA FR2, CA (2DL CA), reporting of NCSG requirement information and NR only NCSG patterns | NOTE 1 |  |  |
| 7.6.22 | LTM Inter-frequency L1-RSRP measurement without measurement gap |  |  |  |  |  |  |
| 7.6.22.1 | NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | Rel-18 | FFS |  | NOTE 1 |  |  |
| 7.7 | Measurement performance requirements |  |  |  |  |  |  |
| 7.7.1 | SS-RSRP |  |  |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  | PC1 PC3 |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.1.3 | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP measurement accuracy | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 |  |  |
| 7.7.2 | SS-RSRQ |  |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.3 | SS-SINR |  |  |  |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | Rel-16 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 7.7.4.1 | NR SA FR2 SSB based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.4.2 | NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Rel-15 | C006 | UEs supporting 5GS NR SA FR2 |  |  |  |
| 7.7.5 |  |  |  |  |  |  |  |
| 7.7.5.1 | NR SA FR2 SRS-RSRP measurement accuracy | Rel-16 | C006l | UEs supporting 5GS NR SA FR2 and SRS-RSRP measurements |  |  |  |
| 7.7.6 | L1-SINR |  |  |  |  |  |  |

/// End of changes