3GPP TSG-RAN5 Meeting #106	R5-250026

# Athens, , 17th Feb 2025 - 21st Feb 2025

Title:	Discussion on parameters of NB-IoT NTN RRM enhanced coverage TCs

Source:	MediaTek Inc.

Agenda item:	5.5.1.13

Document for:	Discussion and endorsement

# 1.	Introduction

There are 2 kinds of coverage type in 36.521-3 clause 13. Enhanced coverage is introduced to handle low SNR radio condition. Theoretically, the demodulation related parameters of enhanced coverage like NPDSCH/NPDCCH repetition should be larger than that of normal coverage. However, there is only one set of parameters for both normal and enhanced coverage TCs in 36.508.

This document proposes a way forward to handle enhanced coverage TCs for NB-IoT NTN.

Discussion in section 2.1 and 2.2 are trying to provide a set of default parameters for S-criteria and other demodulation procedure not defined in RAN4 respectively.

# 2. 	Discussion

## 2.1 S-criteria: q-QualMin-r13

Currently, default q-QualMin-r13 = -20dB in 36.508 is used for NB IoT NTN enhanced coverage TCs.

UE should pass the S criteria in 36.304 to camp on target cell.

Srxlev > 0 AND Squal > 0 where: Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset) – Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) – Qoffsettemp

Considering test settings in 36.521-3 clause 13,

- Qoffsettemp = 0 based on the definition in 36.304.

- Qqualminoffset = 0 according to 36.331.

- Qqualmeas = NRSRQ = N*(NRSRP – RSSI) = -10log10(12) + SNR, where NRSRP (linear) can be ignored compared to Noc (linear) in enh TCs.

Take TC 13.6.2.2 in 36.521-3 as an example,

NRSRQ = -10.8 -12 = -22.8

Squal = NRSRQ +20 = -2.8 < 0. Then UE will waive this cell.

Observation 1: A good UE may fail camping on cell in enh TCs with current setting in 36.508.

It’s reasonable to set lower value of q-QualMin-r13. Considering S criteria is not the check point of those TCs, minimum value is selected.

Proposal 1: Set q-QualMin-r13 = -34 for all RRM enh TCs in 36.521-3 clause 13.

## 2.2 Demodulation related

Repetition is introduced to increase the received SNR in UE side to handle low SNR scenario. For stable test, high repetition should be offered to the UE to camp on cell smoothly especially in enhanced coverage TCs (Should not impact the parameters defined by RAN4).

Discussion in section 2.2.1 and 2.2.2 provide the perspective from simulation and commercial field data to find the proper values.

### 2.2.1 Simulation perspective

Simulation is run and the SNR vs BLER curve is shown, blue, green and black lines denote SIB2 performance with 8, 16 and 32 repetition respectively. The enhanced coverage SNR condition in 36.521-3 is approximately -12dB. Extremely high Bler is observed for low-cost UEs. Demodulation performance is not the check point in those RRM TCs. To test smoothly, relaxed parameters should be used. Hence, 32 repetition is an acceptable choice.

Observation 2.1: Conformant UEs might fail RRM NB-IoT NTN test cases in enhanced coverage.

### 2.2.2 Commercial data perspective

To get rational values, the author also collected the statistics in commercial environment and listed them in section 2.3.

## 2.3 Suggestion

Based on above discussions, Table 2.3-1 was finalized.

Table 2.3-1 Recommended values in enhanced coverage TCs in 36.521-3 Clause 13

| Parameters (if not defined in RAN4) | Current 36.508 | Commercial Field @ -8dB | Deduced | Enh recommend @-12dB |
| --- | --- | --- | --- | --- |
| q-QualMin-r13 | -20 | -34 | Section 2.1 | -34 |
| SIB2/31 pattern (repetition num) | 2/1 | 8/8 | 8 in -8dB >=16 in -11dB 32 in -12dB | 32/32 |
| NPDSCH Repetition (Repetition number in DCI N1) | 1 | 8 (Mcs=7) |  | 32 (Mcs =low mcs in QPSK) |
| USS NPDCCH Rmax (npdcch-NumRepetitions-r13) | 1 | 8 |  | 32 |
| RAR NPDCCH Rmax (npdcch-NumRepetitions-RA-r13) | 16 | 8 |  | 32 |
| NPDCCH-NumRepetitionPaging-R13 | 8 | 8 |  | 32 |

Different requirements are requested in different test group (RA, DCQR, RLM, …). Above parameters will be updated in 36.508 as default settings and limited to RRM enhanced coverage TCs in 36.521-3 Clause 13. Hence RAN4 defined parameters will not be impacted by this update (RAN4 configuration is explicitly introduced in 36.521-3).

Proposal 2: Add recommended parameters in Table 2.3-1 to 36.508 as the default parameters for enhanced RRM TCs in 36.521-3 Clause 13.

# 3. 	Conclusion

This document proposes a way forward to handle enhanced coverage TCs for NB-IoT NTN.

The following observations and proposals are made:

Observation 1: A good UE may fail camping on cell in enh TCs with current setting in 36.508.

Proposal 1: Set q-QualMin-r13 = -34 for all RRM enh TCs in 36.521-3 clause 13.

Observation 2.1: Conformant UEs might fail RRM NB-IoT NTN test cases in enhanced coverage.

Proposal 2: Add recommended parameters in Table 2.3-1 to 36.508 as the default parameters for enhanced RRM TCs in 36.521-3 Clause 13 and update the Cell specific Test Parameters in TS 36.521-3.

Summary of all associated CRs are shown below,

R5-250025 Addition of SIB33 to NB-IoT NTN TCs

R5-250030 Update to NB-IoT NTN RRM enhanced coverage TCs

R5-250035 Update of q-QualMin-r13 to NB-IoT NTN RRM enhanced coverage TCs

R5-250036 Update of test configuration to NB-IoT NTN RRM enhanced coverage TCs

# 4. 	References