3GPP TSG-RAN5 Meeting #106	R5-250050

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3656 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 7.6.21.1 for FR2 LTM inter L1-RSRP measurement with gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LTM inter-frequency L1-RSRP measurement with gap, new test case 7.6.21.1 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case 7.6.21.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.21.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0548 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes>>

#### 7.6.21.1	NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM

Editor's Note: This test case is incomplete in following aspects:

-    LTM specific configurations are missing.

-    Test applicability is FFS.

-    TT analysis is missing.

-    Test parameters contain [ ].

7.6.21.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of inter-frequency SSB based L1-RSRP measurement with measurement gap for candidate neighbour cell in FR2 within L1-RSRP measurement requirements specified in TS 38.133 [6] clause 9.15.

7.6.21.1.2	Test applicability

This test applies to all types of NR UE supporting SSB based inter-frequency L1-RSRP measurements with measurement gaps and reports for LTM from release 18 and forward.

7.6.21.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.21.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.21.1.

7.6.21.1.4	Test description

7.6.21.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.21.1.4.1-1.

Table 7.6.21.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.21.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.6.21.1-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 7.6.21.1.4.1-2.

Table 7.6.21.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clauses 7.2.3 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.21.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

1.	The general test parameter settings are set up according to Table 7.6.21.1.4.1-3.

2.	Message contents are defined in clause 7.6.21.1.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

4.	The AoA setup for this test is Setup 1 as defined in clause A.9. The UE RX beam peak direction has been obtained previously using one of the search procedures as described in Annex I.

Table 7.6.21.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| A3-Offset |  | dB | [-15] |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | true |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| Gap Pattern Id |  |  | 13 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 320 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | true | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 2 |  |
| T2 |  | s | 2 |  |

7.6.21.1.4.2	Test procedure

UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. Measurement gap pattern configuration defined in Table 7.6.21.1.4.1-3 is provided.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.6.21.1.5-1.

3.	SS shall transmit RRCReconfiguration message to configure Event A3 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	The UE shall transmit MeasurementReport message triggered by Event A3 including Cell 2 measurement report with SSB index.

6.	SS shall transmit RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

8.	The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 320 slots.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.21.1.5-1. T2 starts.

10.	The UE shall continue to send valid L1-RSRP reports. The SS shall check the following requirements:

- R1:	the UE shall start to transmit valid reports no later than 1320 ms from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement (Table 7.6.21.1.5-2 for test configuration 1 and Table 7.6.21.1.5-3 for test configuration 2). For candidate Cell 2, the reported L1-RSRP value in T2 shall be larger than the value reported in T1. If the first valid report is received earlier than the specified time and includes a larger L1-RSRP value of Cell 2, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 320 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	the L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. In all consecutive reports after the first valid value is received, if the reported value is outside the limits in Table 7.6.21.1.5-2 for test configuration 1 and in Table 7.6.21.1.5-3 for test configuration 2, or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

11.	The SS waits until T2 expires.

12.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

OR

- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.21.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 7.6.21.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 with condition SSB Index Table H.3.1-6 with conditions gapUE and Pattern #13 Table H.3.1-7 with conditions INTER-FREQ and SSB Index |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.6.21.1.4.3-2: ReportConfigNR (a3-Offset) (step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | [-30] | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.6.21.1.4.3-3: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12-1 with condition L1-RSRP Table H.3.12-2 with condition L1-RSRP Table H.3.12-3 Table H.3.12-4 Table H.3.12-5 with condition [TBD] Table H.3.12-6 Table H.3.12-8 Table H.3.12-9 with conditions FR2 and SSB#0 |

7.6.21.1.5	Test requirement

Table 7.6.21.1.5-1 and Table 7.6.21.1.5-2 define the primary level settings including test tolerances for all tests.

Table 7.6.21.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 | freq 2 |
| Duplex mode | 1~2 |  | TDD |  |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |  |
| BWchannel | 1~2 | MHz | 100: NPRB,c = 66 |  |
| Data RBs allocated | 1~2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PUCCH/PUSCH subcarrier spacing | 1~2 | KHz | 120 |  |
| PDSCH Reference measurement channel | 1 |  | SR.3.2 TDD | N/A |
|  | 2 |  | SR.3.3 TDD | N/A |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | N/A |
|  | 2 |  | CR.3.2 TDD | N/A |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | N/A |
|  | 2 |  | CCR.3.7 TDD | N/A |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
|  | 2 |  | SSB.4 FR2 | SSB.4 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |  |
| SMTC configuration | 1~2 |  | SMTC.1 | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD | N/A |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 | N/A |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSNote 1 |  |  |  |  |
| Propagation condition | 1~2 |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table 7.6.21.1.5-2: NR OTA cell specific test parameters

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~2 |  | Setup 1 according to A.9.1 |  |  |  |
| Beam AssumptionNote 4 | 1~2 |  | Rough |  | Rough |  |
| Ês/IotBBNote 5 | 1~2 | dB | 16.5+TT | 16.5+TT | -1.5+TT | 16.5+TT |
| SSB_RPNote 3 | 1 | dBm/SSB SCS | -85+TT | -85+TT | -103+TT | -85+TT |
|  | 2 |  | -82+TT | -82+TT | -100+TT | -82+TT |
| IoNote 3 | 1 | dBm/95.04MHz | -56+TT | -56+TT | -74+TT | -56+TT |
|  | 2 |  | -56+TT | -56+TT | -74+TT | -56+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 320 slots in T2. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 960 ms plus 320 slots from the beginning of time period T2. During T2, each L1-RSRP measurement report of Cell 2 shall meet the corresponding absolute accuracy requirements in Table 7.6.20.1.5-3 for test configuration 1, and the corresponding absolute accuracy requirements in Table 7.6.20.1.5-4 for test configuration 2.

Table 7.6.20.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 1

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | TBD |
| Highest reported value (Cell 2) | TBD |

Table 7.6.20.1.5-4: L1-RSRP absolute accuracy requirements for the reported values for test configuration 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | TBD |
| Highest reported value (Cell 2) | TBD |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

<<End of changes>>