3GPP TSG-RAN WG5 Meeting #106	R5-250109

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5404 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to IoT NTN discontinuous coverage TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SIB combination is missing in the discontinuous coverage TC and needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIB combination 14 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IoT NTN discontinuous coverage TCs can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.31 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.508 CR 1503 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to Tdoc R5-250108 (CR number 1503). The new SIB combination 14 defined in 36.508 is introduced by Tdoc R5-250108. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

22.4.31	NB-IoT / NTN / discontinuous coverage

22.4.31.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state supports discontinuous coverage and SIB33-NB broadcasts neighbour cell information }

ensure that {

when { t-Service expires and T310 is still running }

then { the UE enters RRC_IDLE state and reselects to the neighbour cell after UE is in its coverage }

}

22.4.31.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.3.23. Unless and otherwise stated these are Rel-18 requirements.

[TS 36.331, clause 5.3.3.23]

In discontinuous coverage scenario, upon expiry of t-Service or being out of the current serving cell coverage, the UE shall:

1>	if timer T310 is running:

2>	stop timer T310, and perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.331, clause 7.3.1]

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T310 | Upon detecting physical layer problems for the PCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure, upon initiating the connection re-establishment procedure, and upon initiating the MCG failure information procedure, upon expiry of t-Service or being out of the current serving cell coverage in discontinuous coverage scenario. | If security is not activated and the UE is not a NB-IoT UE that supports RRC connection re-establishment for the Control Plane CIoT EPS/5GS optimisation: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.6.26 or the connection re-establishment procedure as specified in 5.3.7. |

22.4.31.3	Test description

22.4.31.3.1	Pre-test conditions

System Simulator:

-	Ncells 1, 2 in different tracking areas according to TS 36.508 [18] clause 8.1.4.2.

-	System information combination 14 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used in Ncell 1 and System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used in Ncell 2.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE shall be in State 2A-NB, Connected Mode on Ncell 1 according to TS 36.508 [18] and power level of Ncell 1 is described in Table 22.4.31.3.2-1.

22.4.31.3.2	Test procedure sequence

Table 22.4.31.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration T0 indicates the initial conditions for preamble. The configuration marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 22.4.31.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 22.4.31.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -115 | -115 |  |
| T2 | NRS EPRE | dBm/15kHz | “Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 22.4.31.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts Ncell levels according to row T1 of table 22.4.31.3.2-1 | - | - | - | - |
| 2 | Wait for 2 seconds to let t-Service-r17 expire but T310 still running, | - | - | - | - |
| 3 | After t-Service-r17 expires, SS locally releases the RRC connection. | - | - | - | - |
| 4 | SS adjusts Ncell levels according to row T2 of table 22.4.31.3.2-1 | - | - | - | - |
| 5 | Check: Does the test result of generic test procedure in TS 36.508 [18] subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | P |

22.4.31.3.3	Specific message contents

Table 22.4.31.3.3-1: SystemInformationBlockType3-NB for Ncell 1 (preamble and all steps, table 22.4.31.3.2-2)

Derivation path: TS 36.508 [18] Table 8.1.4.3.3-2 with condition DiscontinousCoverage

Table 22.4.31.3.3-2: SystemInformationBlockType2-NB for Ncell 1 (preamble and all steps, table 22.4.31.3.2-2)

| Derivation Path: 36.508 [18], clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t310-r13 | ms8000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.31.3.3-3: SystemInformationBlockType33-NB for Ncell 1 (preamble and all steps, table 22.4.31.3.2-2)

| Derivation Path: TS 36.508 [18], clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| t-ServiceStartNeigh-r18 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.31.3.3-4: SystemInformationBlockType31-NB for Ncell 1 (preamble and all steps, table 22.4.31.3.2-2)

| Derivation Path: TS 36.508 [18], clause 8.1.4.3.3 with condition NTN enh |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  |  |
| satelliteId-r18 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

<< END OF CHANGES >>