3GPP TSG-RAN WG5 Meeting #106	R5-250121

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3413 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test CBW for band n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test CBWs need to be specified for band n104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Low, Mid and High test channel bandwidths in clause 4.3.1.0 for band 104 have been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0A, 4.3.1.0B, 4.3.1.0C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.1	Test frequencies

Editor's Note: n65 (High: 50MHz), n66 (High: 45MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8], TS 38.101-3 [9] and TS 38.101-5 [56].

4.3.1.0A	Mid test channel bandwidth

The Mid test channel bandwidth definition for RF is given in Table 4.3.1.0A-1, Table 4.3.1.0A-2 and Table 4.3.1.0A-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Mid Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 25 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 20 | 15 |
| n12 | 10 | 10 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 2511 | 15 |
| n26 | 15 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 25 | 15 |
| n40 | 50 | 15 |
| n41 | 50 | 15 |
| n46 | 604, 405 | N/A |
| n48 | 2011 | 15 |
| n50 | 404, 205,11 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 15 | 15 |
| n66 | 20 | 15 |
| n70 | 152, 103,12 | 152, 103 |
| n71 | 153,12, 202 | 15 |
| n72 | 5 | 5 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 50 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n85 | 10 | 10 |
| n86 | 203 | [15] |
| n91 | 52,12, 103 | [52,103] |
| n92 | 15 | [15] |
| n93 | 52,12, 103 | [52,103] |
| n94 | 15 | [15] |
| n95 | 103 | [10] |
| n96 | 60 | N/A |
| n97 | 503 | [15] |
| n98 | 253 | [153] |
| n99 | 103 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n104 | 60 | 20 |
| n105 | 153, 202 | 15 |
| n106 | 3 | N/A |
| n109 | 25 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 (most updated version of latest release available) [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: This Mid test channel bandwidth is chosen since it is more commonly used. Note 11: This is an exception to Note 1 in this table to avoid not allowed asymmetric channel bandwidths. Note 12: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

Table 4.3.1.0A-2: Mid Test Channel bandwidths for each NR band, FR2

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 200 | 100 |
| n258 | 200 | 100 |
| n259 | 200 | N/A |
| n260 | 200 | N/A |
| n261 | 200 | 100 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-2 [15] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the closest bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. |  |  |

Table 4.3.1.0A-3: Mid Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Mid Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 15 |
| n255 | 15 |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-5 [57] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closest bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in TS 38.306 clause 4.2.7.2 [55] for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

4.3.1.0B	Low test channel bandwidth

The low test channel bandwidth definition for RF is given in Table 4.3.1.0B-1, Table 4.3.1.0B-2 and Table 4.3.1.0B-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0B-1: Low Test Channel bandwidths for each NR band, FR1

| NR Band | UE Low Test Channel bandwidth [MHz]1, 1a, 1b | RedCap and eRedCap UE Low Test Channel bandwidth [MHz] 1, 1a, 1b |
| --- | --- | --- |
| n1 | 5 | 5 |
| n2 | 5 | 5 |
| n3 | 5 | 5 |
| n5 | 5 | 5 |
| n7 | 5 | 5 |
| n8 | 5 | 5 |
| n12 | 5 | 5 |
| n13 | 5 | 5 |
| n14 | 5 | 5 |
| n20 | 5 | 5 |
| n24 | 5 | 5 |
| n25 | 5 | 5 |
| n26 | 36,5 | 5 |
| n28 | 36,5 | 5 |
| n29 | 52 | N/A |
| n30 | 5 | 5 |
| n31 | 3 | 5 |
| n34 | 5 | 5 |
| n38 | 5 | 5 |
| n39 | 5 | 5 |
| n40 | 54,105 | 10 |
| n41 | 57,8,10 | 10 |
| n46 | 104,205 | N/A |
| n48 | 54, 105 | 10 |
| n50 | 54,105 | 10 |
| n51 | 5 | 5 |
| n53 | 5 | 5 |
| n54 | 5 | 5 |
| n65 | 5 | 5 |
| n66 | 5 | 5 |
| n70 | 5 | 5 |
| n71 | 5 | 5 |
| n72 | 3 | 5 |
| n74 | 5 | 5 |
| n75 | 52 | N/A |
| n76 | 52 | N/A |
| n77 | 10 | 10 |
| n78 | 10 | 10 |
| n79 | 10 | 10 |
| n80 | 53 | [5] |
| n81 | 53 | [5] |
| n82 | 53 | [5] |
| n83 | 53 | [5] |
| n84 | 53 | [5] |
| n85 | 36,5 | 5 |
| n86 | 53 | [5] |
| n91 | 5 | [5] |
| n92 | 5 | [5] |
| n93 | 5 | [5] |
| n94 | 5 | [5] |
| n95 | 53 | [5] |
| n96 | 20 | N/A |
| n97 | 53 | [5] |
| n98 | 53 | [53] |
| n99 | 53 | [5] |
| n100 | 36,5 | 5 |
| n101 | 5 | 5 |
| n104 | 20 | 20 |
| n105 | 5 | 5 |
| n106 | 3 | N/A |
| n109 | 5 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 1a: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable for use as SCell in CA or SCell in DC configuration. Note 5: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 6: This UE channel bandwidth is optional in Release 18. Note 7: This UE channel bandwidth is optional in Release 17. Note 8: Not all frequency positions of 5 MHz carriers are possible due limitations of the SSB position relative to the 5 MHz channels. 5 MHz channels with Fc such that 2499+N*1.2 ≤Fc<2499.3+N*1.2MHz for 0≤N<157 are not compatible with SSB positions and cannot be used for 5 MHz n41. |  |  |

Table 4.3.1.0B-2: Low Test Channel bandwidths for each NR band, FR2

| NR Band | UE Low Test Channel bandwidth [MHz]1, 2, 3 | RedCap UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- | --- |
| n257 | 50 | 50 |
| n258 | 50 | 50 |
| n259 | 50 | N/A |
| n260 | 50 | N/A |
| n261 | 50 | 50 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the minimum bandwidth defined for that band in the UE release specification shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-2 [8] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 3: For CA and DC, the low-test channel bandwidth per component carrier is chosen to allow minimum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve minimum aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the aggregated bandwidth. |  |  |

Table 4.3.1.0B-3: Low Test Channel bandwidths for each NTN satellite band, FR1-NTN

| NTN satellite Band | UE Low Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n256 | 5 |
| n255 | 5 |
| Note 1: Minimum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the minimum bandwidth applicable for such subcarrier spacing shall be tested. Note 2: Values listed in this table assume that the non-optional channel bandwidths specified in Table 5.3.5-1 of TS 38.101-5 [56] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. |  |

4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1, Table 4.3.1.0C-2 and Table 4.3.1.0C-3 for FR1, FR2 and FR1-NTN respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap and eRedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 401,14, 452 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n31 | 5 | 5 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 100 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n54 | 5 | 5 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 201,14, 352 | 20 |
| n72 | 5 | 5 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n85 | 15 | 15 |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n98 | 401 | [201] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| n104 | 100 | 20 |
| n105 | 201,14, 352 | 20 |
| n106 | 3 | N/A |
| n109 | 30, 502 | 20 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum channel BW value among all the possible channel BW-SCS combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7] is supported. However, these channel bandwidths (except for 100MHz of bands n41, n48, n77, n78, n79) are mandatory with capability signalling as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL and channelBWs-DL-v1590/channelBWs-UL-v1590 parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Channel bandwidth 100MHz of bands n41, n48, n77, n78, and n79 is mandatory without capability signalling (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.7.2 for channelBWs-DL/channelBWs-UL parameters. Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, testing could be performed with one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth instead of the values described in the Table. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

<< END OF CHANGES >>