3GPP TSG-RAN WG5 Meeting #106	R5-250122

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0975 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update REFSENSE TP analysis for CA_n41C-n79C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Ref Sense TP analysis for CA configuration CA_n41C-n79C needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ref Sense TP analysis for CA configuration CA_n41C-n79C has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.1.3	Test point analysis per NR CA configuration

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1 through Table 4.1.3.1-5.

This clause contains information on test point analysis, test frequency selection and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1 through Table 4.1.3.1 4. The principles for selection of reference sensitivity test points for NR CA is given in Annex B including a test point analysis check list in B.9.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band)

| CA config | Single band refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n2(2A) |  | NE | RAN5#100 |
| CA_n5B |  | NE | RAN5#104 |
| CA_n41C |  | NE | RAN5#89-e |
| CA_n48(2A) | - | NE | RAN5#102 |
| CA_n48B |  | NE | RAN5#94-e |
| CA_n48C |  | NE | RAN5#103 |
| CA_n66B |  | NE | RAN5#86-e |
| CA_n66(2A) |  | NE | RAN5#86-e |
| CA_n66(3A) |  | NE | RAN5#100 |
| CA_n71(2A) |  | NE | RAN5#94-e |
| CA_n77(2A) |  | NE | RAN5#100 |
| CA_n77C |  | NE | RAN5#96e |
| CA_n78C |  | NE | RAN5#87-e |
| CA_n78(2A) |  | NE | RAN5#101 |
| CA_n79C |  | NE | RAN5#103 |
| NOTE 1: Void. NOTE 2: Notations used: NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config | Two band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n1A-n3A | n1 (IMD3) n3(CBI) | IMD,CBI | RAN5#95-e |
| CA_n1A-n5A | - | NE | RAN5#105 |
| CA_n1A-n8A | n1 (IMD4) | IMD | RAN5#95-e |
| CA_n1A-n28A | n1 (HD3) | HD | RAN5#101 |
| CA_n1A-n41A | n1(CBI), n41(CBI) | CBI | RAN5#102 |
| CA_n1A-n77A | n77 (HD2) | HD | RAN5#89-e |
| CA_n1A-n78A | n1 (IMD4) | IMD | RAN5#94 |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#96-e |
| CA_n2A-n5A | - | NE | RAN5#97 |
| CA_n2A-n14A | - | NE | RAN5#100 |
| CA_n2A-n48A | n48 (HD2), n2(IMD4) | HD, IMD | RAN5#100 |
| CA_n2A-n48(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n2A-n48B | (NOTE 2) | NE | RAN5#101 |
| CA_n2A-n66A | n2(IMD3), n66(IMD5) | IMD | RAN5#96-e |
| CA_n2A-n77A PC3 | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) | HD, HM, IMD, | RAN5#99 |
| CA_n2A-n77A PC2 | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4) n2(CBI) | HD, HM, IMD, CBI | RAN5#99 |
| CA_n2A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n2A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n3A-n5A | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) | IMD | RAN5#95-e |
| CA_n3A-n8A | n8(IMD4), n3(IMD5) | IMD | RAN5#100 |
| CA_n3A-n28A | - | NE | RAN5#101 |
| CA_n3A-n40A | - | NE | RAN5#105 |
| CA_n3A-n41A PC2 | n3 (IMD4), n3 (CBI) | IMD, CBI | RAN5#100 |
| CA_n3A-n41A PC3 | n3 (IMD4), n3 (CBI), n41 (CBI) | IMD, CBI | RAN5#100 |
| CA_n3A-n77A PC3 | n77 (HD2), n3 (IMD2), n3 (IMD4) | HD, IMD | RAN5#99 |
| CA_n3A-n77A PC2 | n3 (HM), n3 (IMD2), n3 (IMD4) | HM, IMD | RAN5#104 |
| CA_n3A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n3A-n78A PC3 | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) | HD, IMD | RAN5#101 |
| CA_n3A-n78A PC2 | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2), n3 (HM) | HD, HM, IMD | RAN5#101 |
| CA_n5A-n48A | - | NE | RAN5#102 |
| CA_n5A-n48(2A) | - | NE | RAN5#102 |
| CA_n5A-n48B | - | NE | RAN5#102 |
| CA_n5A-n66A | n5(IMD2) | IMD | RAN5#96-e |
| CA_n5A-n77A PC3 | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) | HD, HM, IMD | RAN5#99 |
| CA_n5A-n77A PC2 | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4) | HD, HM, IMD | RAN5#99 |
| CA_n5A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n5A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n5A-n78A | n78 (HD4), n5 (IMD4) | HD, IMD | RAN5#105 |
| CA_n7A-n78A | n7 and n78 (CBI) | CBI | RAN5#94-e |
| CA_n8A-n78A | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) | HD, IMD | RAN5#87-e |
| CA_n14A-n30A | - | NE | RAN5#100 |
| CA_n14A-n66A | - | NE | RAN5#100 |
| CA_n14A-n77A | n77 (HD5), n14 (HM), n14 (IMD5) | HD, HM, IMD | RAN5#100 |
| CA_n20A-n78A | n78 (HD4), n20 (IMD4) | HD, IMD | RAN5#100 |
| CA_n25A-n66A | n25 (IMD3), n66 (IMD3), n66 (IMD5) | IMD | RAN5#101 |
| CA_n25A-n77A | n77 (HD2), n25 (HM), n25 (IMD2), n25 (IMD4), n25 (IMD5) | HD, HM, IMD | RAN5#101 |
| CA_n25A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n25A-n78A | n78 (HD2), 25 (IMD2) | HD, IMD | RAN5#101 |
| CA_n25A-n78(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n26A-n66A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n26A-n70A | n26 (IMD2) | IMD | RAN5#94-e |
| CA_n28A-n40A | n28 (HM) | HM | RAN5#103 |
| CA_n28A-n41A |  | NE | RAN5#91-e |
| CA_n28A-n41C | (NOTE 2) | NE | RAN5#104 |
| CA_n28A-n77A | n77 (HD5), n28 (IMD5) | HD, IMD | RAN5#99 |
| CA_n28A-n77(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n28A-n78A | n78 (HD5, HM) | HD, HM | RAN5#102 |
| CA_n28A-n79A |  | NE | RAN5#91-e |
| CA_n29A-n66A |  | NE | RAN5#94-e |
| CA_n29A-n70A |  | NE | RAN5#94-e |
| CA_n29A-n71A | n29 (CBI) | CBI | RAN5#95-e |
| CA_n39A-n41A | - | NE | RAN5#100 |
| CA_n39A-n41A4 | n39(HM), n41(HM), n39(CBI), n41(CBI) | HM, CBI | RAN5#103 |
| CA_n40A-n77A | n40(HM), n40(CBI) | HM, CBI | RAN5#105 |
| CA_n41A-n66A3 | n66 (CBI), n66 (IMD5) | CBI | RAN#97 |
| CA_n41A-n71A3 | n41(HD4), n71 (IMD4) | HD | RAN5#98 |
| CA_n41A-n77A PC3 | n41(HM), n77(IMD9), n41(CBI), n77(CBI) | HM, IMD, CBI | RAN5#102 |
| CA_n41A-n77A PC2 | n41(HM), n77(IMD5), n41(CBI), n77(CBI) | HM, IMD, CBI | RAN5#104 |
| CA_n41A-n79A PC3 |  | NE | RAN5#83 |
| CA_n41A-n79A PC2 | n41 (CBI), n79 (CBI) | CBI | RAN5#100 |
| CA_n41A-n79C | (NOTE 2) | NE | RAN5#104 |
| CA_n41C-n79A | (NOTE 2) | NE | RAN5#104 |
| CA_n41C-n79C | (NOTE 2) | NE | RAN5#106 |
| CA_n48A-n66A | n48 (HD2), n66 (IMD5) | HD, IMD | RAN5#94-e |
| CA_n48A-n66(2A) | (NOTE 2) | NE | RAN5#95-e |
| CA_n48A-n70A | n70 (IMD2) | IMD | RAN5#94-e |
| CA_n48A-n71A |  | NE | RAN5#94-e |
| CA_n48A-n71(2A) | - | NE | RAN5#95-e |
| CA_n48A-n77A PC3 | - | NE | RAN5#97-e |
| CA_n48A-n77A PC2 | - | NE | RAN5#102 |
| CA_n48A-n77C | - | NE | RAN5#102 |
| CA_n48B-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48B-n71A | - | NE | RAN5#95-e |
| CA_n48B-n77A | - | NE | RAN5#102 |
| CA_n48(2A)-n66A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n66(2A) | (NOTE 2) | NE | RAN5#96-e |
| CA_n48(2A)-n70A | (NOTE 2) | NE | RAN5#95-e |
| CA_n48(2A)-n71A | - | NE | RAN5#95-e |
| CA_n48(2A)-n71(2A) | - | NE | RAN5#96-e |
| CA_n48(2A)-n77A | - | NE | RAN5#102 |
| CA_n66A-n70A |  | NE | RAN5#94-e |
| CA_n66A-n71A | n66 (IMD4) | IMD | RAN5#87-e |
| CA_n66A-n77A PC3 | n77 (HD2), n66(IMD2), n66(IMD5), n66(IMD7) | HD, IMD | RAN5#102 |
| CA_n66A-n77A PC2 | n77 (HD2), n66(IMD2), n66(IMD5), n66(CBI) | HD, IMD, CBI | RAN5#99 |
| CA_n66A-n77(2A) PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n66A-n77C PC3 | (NOTE 2) | NE | RAN5#101 |
| CA_n66A-n77C PC2 | (NOTE 2) | NE | RAN5#102 |
| CA_n66A-n78A | n78 (HD2), n66(IMD5) | HD, IMD | RAN5#101 |
| CA_n66A-n78(2A) | (NOTE 2) | NE | RAN5#101 |
| CA_n70A-n71A | n70 (HD3), n70 (IMD4) | HD, IMD | RAN5#85 |
| CA_n71A-n77A | n71 (IMD5) | IMD | RAN5#97-e |
| CA_n71A-n77(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n71A-n78A | n78 (HD5), n71 (IMD5) | HD, IMD | RAN5#102 |
| CA_n71A-n78(2A) | (NOTE 2) | NE | RAN5#102 |
| CA_n78A-n79A PC2 | n78 (CBI), n79(CBI) | CBI | RAN5#103 |
| CA_n78A-n79A PC3 | n78 (CBI), n79 (CBI) | CBI | RAN5#103 |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case. NOTE 3: Only single uplink analysed. IMD exception not yet covered. NOTE 4: Exception for this configuration is tested only when the UE reports supporting Simultaneous Rx/Tx capability. |  |  |  |

<< END OF CHANGES >>