3GPP TSG-RAN WG5 Meeting #106	R5-250124

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3153 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Ref Sense power level for CA_n41C-n79C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Ref Sense power level for CA configuration CA_n41C-n79C needs to be added as per WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Ref Sense power level for CA configuration CA_n41C-n79C has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The configuration can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

7.3A.3	Reference sensitivity power level for 4DL CA

NOTE: Intraband contiguous  4DL CA is FFS

7.3A.3.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 4DL CA.

7.3A.3.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.3.4	Test description

7.3A.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.3.4.1-1: Test Configuration Table for 4DL CA

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For Inter-band CA: CA_nXA-nYA-nZA-nRA: Mid range for PCC and SCC with exceptions CA_nX(2A)-nYA-nZA: For nY and nZ bands, Mid range for PCC and SCC with exceptions. CA_nX(3A)-nYA: For nY band, Mid range for PCC and SCC with exceptions. CA_nXC-nYA-nZA and CA_nXB-nYA-nZA : Low range, High Range for nXC and nXB, mid range for nYA for PCC and SCC with exceptions. Exceptions for CA configurations containing the following band combinations: CA_n1-n77: Mid in band n1 and Low in band n77 CA_n2-n77: UL and DL Mid in band n2 and band n77 at 3850MHz CA_n3-n77: TBD in band n3 and TBD in band n77. CA_n3-n78: Mid in band n3 and High in band n78. CA_n8-nX: Low range for PCC in Band n8 CA_n70-n71: High range for PCC in band n71 In the case of CA_nX(2A)-nYA-nZA, these exceptions apply only for nY and nZ bands. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in Tables 5.5A.1-1, 5.5A.2-1, or tables in clauses 5.5A.3.x for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB”and “SCC NRB ” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0) |  |  |
|  | CA configuration |  |  |  |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | SCC3 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  | Wgap2 | SCC2 |  | Wgap3 | SCC3 |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band | Range |  | Band | Range |  | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nXC-nYA-nZA and CA_nXB-nYA-nZA Configurations (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nX | default | N/A | nY | default | N/A | nZ | default | Highest NRB_agg | Highest NRB_agg | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | default | N/A | nX | default | N/A | nZ | default | Highest (NOTE 11) | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX | default | N/A | nX | default | N/A | nY | default | Highest (NOTE 11) | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DL Allocation |  |  | UL allocation (NOTE2.0) |  |  |
|  | CA configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PCC NRB | SCC1 NRB | SCC2 NRB | SCC3 NRB | CC Mod | PCC & SCC RB allocation |  | CC Mod | PCC & SCC RB allocation |  |
|  | PCC |  | Wgap1 | SCC1 |  |  | Wgap2 | SCC2 |  | Wgap3 |  | SCC3 |  |  |  |  |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range |  | Band |  | Range |  | Band | Range |  |  | Band |  | Range |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a CA_nX(2A)-nYA-nZA Configuration (Intra-band non-contiguous 2CC + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX |  | CC2 | N/A | nY | N/A | N/A |  | nZ |  | default |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX |  | CC1 | Max (NOTE 7) | nX | CC2 | N/A |  | nZ |  | default |  | Highest (NOTE 11) | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX |  | CC1 | Max (NOTE 7) | nX | CC2 | N/A |  | Ny |  | default |  | Highest (NOTE 11) | Highest NRB_agg | Highest NRB_agg | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(2A)-nY(2A) Configuration (Intra-band non-contiguous + Intra-band non-contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX |  | CC2 | N/A | nY | CC1 | Max (NOTE 7) |  | nY |  | CC2 |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | CC1 | Max (NOTE 7) | nY |  | CC2 | N/A | nX | CC1 | Max (NOTE 7) |  | nX |  | CC2 |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nX(3A)-nYA Configuration (Intra-band non-contiguous 3CC + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | Max (NOTE 7) | nX | CC2 |  | Max (NOTE 7) | nX | CC3 | N/A | nY |  | default |  | Highest NRB_agg (NOTE 6) |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX | CC1 |  | Max (NOTE 7) | nX | CC2 | Max (NOTE 7) | nX |  | CC3 |  | Highest (NOTE 11) |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for a CA_nXB-nYC and CA_nXC-nYC Configuration (Intra-band contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | CC1 | NA | nX |  | CC2 | N/A | nY | CC1 | NA |  | nY |  | CC2 |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | CC1 | NA | nY |  | CC2 | N/A | nX | CC1 | NA |  | nX |  | CC2 |  | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | Highest NRB_agg (NOTE 6) | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Default Test Settings for CA_nXA-nYA-nZA-nRA Configurations (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | nX | default | N/A | nY |  | default | N/A | nZ | default | N/A |  | nR |  | default |  | Highest (NOTE 11) | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 2 | nY | default | N/A | nX |  | default | N/A | nZ | default | N/A |  | nR |  | default |  | Highest (NOTE 11) | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 3 | nZ | default | N/A | nX |  | default | N/A | nY | default | N/A |  | nR |  | default |  | Highest (NOTE 11) | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| 4 | nR | default | N/A | nX |  | default | N/A | nY | default | N/A |  | nZ |  | default |  | Highest (NOTE 11) | Highest | Highest | Highest | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2.0: REFSENS refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. Note 2: Use CA Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: Inter-band: nX,nY,nZ, nR correspond to the different bands in the CA Configuration. E.g. for CA_n1A-n3A-n28A-n78A, nX=n1, nY=n3, nZ=n28, nR=n78. Note 4: Intra-band contiguous + Inter-band: nX,nY,nZ correspond to the different bands in the CA Configuration, e.g. for CA_n1C-n3A-n8A, nX=n1, nY=n3, nZ = n8 Note 5: Intra-band non-contiguous 2CC + Inter-band: nX, nY and nZ correspond to the different bands in the CA Configuration, e.g., for CA_n48(2A)-n66A-n70A, nX=n48, nY =n66, nZ = n70. Intra-band non-contiguous 3CC + Inter-band: nX and nY correspond to the different bands in the CA Configuration, e.g., for CA_n66(3A)-n77A, nX=n66, nY=n77. Note 6: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested Note 7: The Wgap is defined to be widest possible on band based on the PCC and SCC configuration for Intra-band non-contiguous Note 8: For band combinations including operating bands without uplink band (as noted in Table 5.2-1), only the CA configurations where PCC band has uplink band shall be tested Note 9: The fallback configurations including CA_nXA-nYA for 4CA configurations nXC-nYA-nZA and nXB-nYA-nZA do not need to be tested even if the test frequency differs. 7.3A.1_1 shall be tested for all nXA-nYA combinations including exceptions. Note 10: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 11: Each of Highest UL and Highest DL shall be selected according to clause 5.5A.3.3. DL channel bandwidth shall be selected first. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Table 7.3A.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.2.1.4.3.

7.3A.3.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 38.508-1 [5] clause 5.5.1.  Message contents are defined in clause 7.3A.3.1.4.3.

3.	SS activates SCCs by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1-1 for C_RNTI to transmit the DL RMC according to Table 7.3A.3.4.1-1 on both PCC and SCC.  The SS sends downlink MAC padding bits on the DL RMC

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3A.3.4.1-1 on PCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3.2.5-1 and 7.3.2.5-2 as appropriate. Send continuously uplink power control "up" commands in every uplink scheduling information to the UE to ensure the UE transmits PUMAX level for at least the duration of the throughput measurement.  Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

7.	Measure the average throughput for each component carrier for a duration sufficient to achieve statistical significance according to Annex H.2A.

7.3A.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

7.3A.3.5	Test requirement

For 4DL carrier aggregation, test parameters are specified in table 7.3A.3.4.1-1. For the CA configurations listed in table 7.3A.3.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference sensitivity power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC  given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.3.5-1: Reference sensitivity requirement for 4DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Inter-band 4DL CA | CA_n1A-n3A-n28A-n78A | - |
|  | CA_n2A-n5A-n48(2A) |  |
|  | CA_n2A-n5A-n48B |  |
|  | CA_n2(2A)-n5A-n66A | - |
|  | CA_n2A-n5A-n66(2A) | - |
|  | CA_n2A-n5A-n77C |  |
|  | CA_n2(2A)-n5A-n77A | - |
|  | CA_n2A-n5A-n77(2A) | - |
|  | CA_n2A-n48A-n77C |  |
|  | CA_n2A-n48(2A)-n66A |  |
|  | CA_n2A-n48(2A)-n77A |  |
|  | CA_n2A-n48B-n66A |  |
|  | CA_n2A-n48B-n77A |  |
|  | CA_n2A-n66A-n77C |  |
|  | CA_n2(2A)-n66A-n77A | - |
|  | CA_n2A-n66(2A)-n77A | - |
|  | CA_n2A-n66A-n77(2A) | - |
|  | CA_n2A-n66(3A) | - |
|  | CA_n2(2A)-n77(2A) |  |
|  | CA_n3A-n28A-n41A-n77A | - |
|  | CA_n5A-n48A-n77C |  |
|  | CA_n5A-n48(2A)-n66A |  |
|  | CA_n5A-n48(2A)-n77A |  |
|  | CA_n5A-n48B-n66A |  |
|  | CA_n5A-n48B-n77A |  |
|  | CA_n5A-n66A-n77C |  |
|  | CA_n5A-n66(2A)-n77A | - |
|  | CA_n5A-n66A-n77(2A) | - |
|  | CA_n5A-n66(3A) | - |
|  | CA_n5B-n77C |  |
|  | CA_n14A-n66(3A) | - |
|  | CA_n26A-n66(2A)-n70A | - |
|  | CA_n30A-n66(3A) | - |
|  | CA_n41C-n79C | - |
|  | CA_n48A-n66A-n71(2A) | - |
|  | CA_n48A-n66A-n77C |  |
|  | CA_n48A-n66(2A)-n70A | - |
|  | CA_n48A-n66(2A)-n71A | - |
|  | CA_n48A-n70A-n71(2A) | - |
|  | CA_n48B-n66A-n70A | - |
|  | CA_n48B-n66A-n71A | - |
|  | CA_n48B-n66A-n77A |  |
|  | CA_n48B-n70A-n71A | - |
|  | CA_n48(2A)-n66A-n70A | - |
|  | CA_n48(2A)-n66A-n71A | - |
|  | CA_n48(2A)-n66(2A) | - |
|  | CA_n48(2A)-n66A-n77A |  |
|  | CA_n48(2A)-n70A-n71A | - |
|  | CA_n48(2A)-n71(2A) | - |
|  | CA_n66A-n70A-n71(2A) | - |
|  | CA_n66B-n70A-n71A | - |
|  | CA_n66(2A)-n70A-n71A | - |
|  | CA_n66(2A)-n71(2A) | - |
|  | CA_n66(2A)-n77(2A) | - |
|  | CA_n66(3A)-n77A | - |
|  | CA_n2A-n5A-n48A-n66A | - |
|  | CA_n2A-n5A-n48A-n77A | - |
|  | CA_n2A-n5A-n66A-n77A | - |
|  | CA_n2A-n48A-n66A-n77A | - |
|  | CA_n5A-n48A-n66A-n77A | - |
|  | CA_n66A-n71A-n77(2A) | - |
|  | CA_n66A-n71A-n78(2A) | - |
| SDL configuration | CA_n29A-n66B-n70A | - |
|  | CA_n29A-n66(2A)-n70 | - |

<< END OF CHANGES >>