3GPP TSG-RAN5 Meeting #106	R5-250146

# Athens, Greece, Feb 17th - Feb 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.918 | CR | 0004 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of A.3.3.2 in TR 38.918 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2025-1-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - After URSP update, the updated network slices in step 12 is incorrectly used in step 13 and 14. In the current test procedure, step 12 configures APP A with SST: '00000001'B and SD: 0x000002, and APP B with SST: '00000001'B and SD: 0x000001. But step 13 and 14 use APP A with SST: '00000001'B and SD: 0x000001, and APP B with SST: '00000001'B and SD: 0x000002 (as shown in Table A.3.3.2.4.2-5 and Table A.3.3.2.4.2-6), it’s incorrect. 2. Update the typo of slice service performance testing in A.3.3.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revise Table A.3.3.2.4.2-5 and Table A.3.3.2.4.2-6 to enable the APP A use SST: '00000001'B and SD: 0x000002, and APP B use SST: '00000001'B and SD: 0x000001 correctly. - Update the typo of slcie service performance testing procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test procedure is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

A.3.3.2	5G NR / Service Performance / Multiple Network Slicing / URSP update

A.3.3.2.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with the change of network slicing.

A.3.3.2.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 for UL.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.2.4	Test Description

A.3.3.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  REGISTRATION ACCEPT message is defined in Table A.3.2.1.4.2-1 and MANAGE UE POLICY COMMAND message is defined in Table A.3.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP B, generate traffic matching OS App Id value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1.

6.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value are selected.

7.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

8.	Using the simulated APP A and APP B, begin uplink data transfer from UE to the Application Server Simulator.

9.	Wait for 15 seconds and then start to measure the throughput result of simulated APP A and latency result of simulated APP B in application layer (This is iteration 1). Continue data transfer for 1 minute.

10.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

11.	Calculate and record the average application layer data throughput of simulated APP A and latency  of simulated APP B across three iterations.

12.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

13.	Repeat step 2-4 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

14.	Repeat step 5-7 with the message exception defined in Table A.3.2.2.4.2-5 and A.3.2.2.4.2-6

15.	Repeat step 8-11 to get the average application layer data throughput of simulated APP A and latency of simulated APP B in parallel.

16.	Observe that the throughput in step 15 is not lower than the throughput benchmark in step 11 and the latency in step 15 is not higher than the latency benchmark in step 11.

Table A.3.3.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '00000001'B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.3.2.4.2-2: UL NAS Transport (step 3, step 6)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  | step 3 |  |
|  | '00000010'B |  | step 6 |  |
| SD | 0x000001 |  |  |  |

Table A.3.3.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 7)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  | step 4 |
|  |  | '00000010'B |  | step 7 |
| SD |  | 0x000001 |  |  |

Table A.3.3.2.4.2-4: MANAGE UE POLICY COMMAND (step 12)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10100000'B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  |  |

Table A.3.3.2.4.2-5: UL NAS Transport (step 13, step 14)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  |  |  |
| SD | 0x000002 |  | step 13 |  |
|  | 0x000001 |  | step 14 |  |

Table A.3.3.2.4.2-6: PDU SESSION ESTABLISHMENT ACCEPT (step 13, step 14)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000002 |  | step 13 |
|  |  | 0x000001 |  | step 14 |

<End of modified section>