3GPP TSG-RAN5 Meeting #106	R5-250148

# Athens, Greece, Feb 17th - Feb 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.918 | CR | 0006 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC A.3.3.1A in TR 38.918 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2025-1-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the service performance testing of network slicing, when different resources are allocated to different slices, the service performance will also be different accordingly. However, the existing TR 38.918 does not test this case, so the service performance test is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new TC A.3.3.1A in TR 38.918 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The service performance testing is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.1A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

A.3.3.1A	5G NR / Service Performance / Single Network Slicing / URSP update / Different Resource Allocation

A.3.3.1A.1	Definition

The UE procedure for receiving and utilizing updated URSP rules is one of the fundamental functional requirements for network slicing.

A.3.3.1A.2	Test Purpose

To measure the performance of single services in application layer of the 5G NR UE with the different resource allocation of the same slice.

A.3.3.1A.3	Test Parameters

The common test parameters are defined in TS 38.521-4 [12] Table 5.5.1.3-1. The parameters specified in TS 38.521-4 [12] Table 5.5.1.3-2 are applicable for tests on FDD bands and parameters specified in TS 38.521-4 [12] Table 5.5.1.3-3 are applicable for tests on TDD bands.

Configurations of PUSCH and PUCCH are specified in TS 38.521-1 [13] Annex G. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 with the exception of using half of the allocated resource block at step 5-7. Configure the TBsize, UL RMC, PDCP size from TS 38.521-4 [12] Annex A.2 at step 9-13.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

A.3.3.1A.4	Test Description

A.3.3.1A.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [6].

UE:

-	Empty URSP Configuration.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.3.1A.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed, except the MANAGE UE POLICY COMMAND message is defined in Table A.2.2.2.4.2-1.

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching OS App Id in Traffic descriptor in URSP rule provisioned to the UE in step 1.

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI value is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

6.	Wait for 15 seconds and then start to measure the throughput result in transport layer (This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 6 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer.

8.	Calculate and record the average application layer data throughput across three iterations.

9.	The SS transmits a DL NAS TRANSPORT message including UE Policy Container and MANAGE UE POLICY COMMAND message to update URSP rules.

10.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

11.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

12.	Using the Application Client Simulator to simulate APP A, begin uplink data transfer from UE to the Application Server Simulator.

13.	Repeat steps 6-8 to get the average application layer data throughput of the data traffic of simulated APP A.

14.	Observe that the throughput in step 13 is two times of the throughput benchmark in step 8.

Table A.3.3.1A.4.2-1: UL NAS Transport (step 3, 10)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| Payload container type | '0001'B | N1 SM information |  |  |
| PDU session ID | PSI-1 |  |  |  |
| Request type | '001'B | Initial request |  |  |
| S-NSSAI | Not Present |  |  |  |
| Length of S-NSSAI contents | '00000100'B | SST and SD |  |  |
| SST | '00000001'B |  |  |  |
| SD | 0x000001 |  | step 3 |  |
|  | 0x000003 |  | step 10 |  |

Table A.3.3.1A.4.2-2: PDU SESSION ESTABLISHMENT ACCEPT (step 4, step 11)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000001'B |  |  |
| SD |  | 0x000001 |  | step 4 |
|  |  | 0x000003 |  | step 11 |

Table A.3.3.1A.4.2-3: MANAGE UE POLICY COMMAND (step 9)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | '10001000'B |  | DNN Type |
| Traffic descriptor component |  |  |  |  |
| DNN length |  | Set to the actual length of 'DNN value' in bytes |  |  |
| DNN value |  | pc_APN_ID_Specific |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | '00000010'B |  | S-NSSAI type |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | '00000100'B | SST and SD |  |
| SST |  | '00000010'B |  |  |
| SD |  | 0x000003 |  |  |

<End of modified section>