3GPP TSG-RAN5 Meeting #106	R5-250199

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	FR2 MU – On MU and TT definition for EVM measurement on PUCCH

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to define MU and TT for FR2 EVM test case 6.4.2.1 defined in [4].

# Discussion

## Background

#### MU and TT definition PUSCH

- Current definition of MU and TT for EVM measurement on PUSCH can be well summarized in [1]. Regarding MU, essentially it is considered EVM due to available SNR and EVM that is intrinsic to the TE, with both EVM combined via RSS in order to calculate the resulting MU.

- Regarding EVM due to available SNR, it can be checked that it has been calculated taking into account the resulting minimum PUSCH power considering the applicable MPR and the whole cell bandwidth, even if in some test points the RB allocation is partial.

Observation 1. In some EVM PUSCH test points the RF allocation is full while in other test points the RB allocation is partial.

Observation 2. EVM MU factor due to available SNR in PUSCH was calculated considering the noise of the whole channel BW and not only the noise in the particular RB allocation.

#### MU and TT definition PUCCH

- First attempt to define MU and TT for PUCCH EVM measurement was made in [3]. For the sake of clarity, let’s reproduce the discussion and proposal here:

The measurement uncertainty for PUCCH has yet not been specified. The test procedures for both PUSCH and PUCCH test the EVM at the UE's maximum output power level PUMAX. For PUSCH an Outer_Full allocation is applied. For PUCCH Format 1 and Format 3 are applied which use up to 16 RBs. Thus, the power spectral density (PSD) for PUCCH is higher than that for PUSCH. Therefore, the MU for EVM of PUSCH is an upper bound for the MU for EVM of PUCCH and we propose to reuse the MU and TT of EVM PUSCH. - Proposal 3: For EVM PUCCH reuse the MU and TT of EVM PUSCH.

- As it can be seen, previous proposal is not studying in detail the specific configuration of PUCCH in terms of RB allocations and applicable MPR not a proper comparison on how MU and TT were derived for PUSCH depending on that. In addition, it is proposing to reuse the MU and TT from defined for PUSCH but not setting clearly which values to apply. As a consequence, in this document a more detailed analysis of the PUCCH configuration will be made in order to make more specific proposals on the MU and TT to apply.

## Proposals

There are 2 test points to EVM on PUCCH, one is using PUCCH format 1 and the other is using PUCCH format 3. In case of format 1, 1PRB applies and QPSK modulation. In case of format 3, up to 16 PRB can apply and pi/2-BPSK could be configured as well. However, according to default configuration in TS 38.508-1[5], also in case of PUCCH format 3 1PRB will be used with QPSK modulation.

Another aspect determined by the default configuration in TS 38.508-1[5] is the location of the PUCCH in frequency domain. Configuration indicates that there is no frequency hopping nor offset configured, so PUCCH PRB is located in PRB#0. Therefore, in terms of determining the MPR to apply for PUCCH, it must be considered an outer or edge allocation.

Observation 3. Default PUCCH configuration implies that PUCCH will only use QPSK modulation on single PRB#0, what corresponds with an outer or edge allocation.

Despite effectively PSD will be higher in case of PUCCH compared to PUSCH if compared with outer full allocation, as captured in Observation 1, the specific PSD based on the actual PRB allocation was not considered for the PUSCH analysis. Therefore, it is proposed to follow same approach in case of PUCCH.

Proposal 1. To define EVM MU factor due to available SNR for QPSK in the same way it was defined for PUSCH, i.e., considering the noise of the whole channel BW and not only the noise in the particular RB allocation.

In the same way, regarding EVM MU factor due to intrinsic TE EVM, the same values assumed for PUSCH should be applicable for PUCCH while there is no explicit argument to consider it differently.

As a result, considering the type of PUCCH MPR region, the modulation, it is proposed to reuse MU and TT definition from PUSCH Test ID 10 in case of CP-OFDM and PUSCH Test ID 4 in case of DFT-s-OFDM.

Proposal 2. For PUCCH, reuse MU and TT definition from PUSCH Test ID 10 in case of CP-OFDM and PUSCH Test ID 4 in case of DFT-s-OFDM.

Despite the reutilization proposal for MU and TT from PUSCH values to PUCCH, given the applicable cases are so specific, it is proposed to define PUCCH explicitly in independent tables rather than complicating existing ones with additions and clarifications.

Proposal 3. Define final MU and TT values in independent tables for PUCCH.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. In some EVM PUSCH test points the RF allocation is full while in other test points the RB allocation is partial.

Observation 2. EVM MU factor due to available SNR in PUSCH was calculated considering the noise of the whole channel BW and not only the noise in the particular RB allocation.

Observation 3. Default PUCCH configuration implies that PUCCH will only use QPSK modulation on single PRB#0, what corresponds with an outer or edge allocation.

Proposal 1. To define EVM MU factor due to available SNR for QPSK in the same way it was defined for PUSCH, i.e., considering the noise of the whole channel BW and not only the noise in the particular RB allocation.

Proposal 2. For PUCCH, reuse MU and TT definition from PUSCH Test ID 10 in case of CP-OFDM and PUSCH Test ID 4 in case of DFT-s-OFDM.

Proposal 3. Define final MU and TT values in independent tables for PUCCH.

# References

- R5-221615, “Discussion on MU and TT proposal for FR2 EVM”, Keysight, RAN5#94-e

- R5-223620, “FR2 SA EVM test case update based on TT analysis”, Keysight, RAN5#96-e

- R5-247337, “On the MU and TT for SISO and MIMO test cases”, R&S, RAN5#105

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v18.5.2 (2024-12)

- TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment”, v18.5.0 (2024-12)

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.5.0 (2024-12)