3GPP TSG-RAN5 Meeting #106	R5-250202

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	5.3.7.9

Source:	Keysight Technologies

Title:	FR2 MU - Discussion on pending MU and TT definition for PC6

Document for:	Discussion and Endorsement

# Introduction

The aim of this contribution is to continue the analysis started in [1] and progress in the MU and TT definition for PC6 in the following test cases:

- OBW

- EVM

- Spurious tests

# Discussion

## OBW

In OBW test case 6.5.1 in TS 38.521-2 [3], only QPSK is tested doing an EIRP type measurement while UE power is pushed to its maximum. The test requirements are independent of the power class.

For modulations other than 256QAM, the MPR that applies to PC6 is the same as for PC5 as defined in section 6.2.2 in TS 38.521-2[3]. Regarding the maximum output power, in case of EIRP, PC6 and PC5 have the same requirements as defined in section 6.2.1 in TS 38.521-2[3]. Therefore, in OBW test case, the UE expected power in worst case and therefore the minimum SNR will be the same for PC6 as for PC5.

In OBW test case, the measurement uncertainty depends mainly on the available SNR and the measurement BW vs the channel BW. If the measurement BW is defined independently of the power class then it can be concluded and proposed to define OBW MU and TT for PC6 same as already defined for PC5.

Observation 1. In OBW test case, requirements are independent of the power class and are only verified using QPSK via EIRP measurements while UE power is pushed to its maximum.

Observation 2. In OBW test case, in worst case analysis, expected minimum UE power for PC6 will be the same expected for PC5.

Proposal 1. For OBW test case, for PC6, define MU and TT same as defined for PC5.

## EVM

In EVM test case 6.4.2.1 in TS 38.521-2 [3], also UE power is pushed to its maximum, at least when PUSCH or PUCCH are tested. As per analysis made in [2], EVM MU depends on the available SNR and the intrinsic TE EVM. From those 2 MU factors, only the one depending on the available SNR will have a dependency with the power class due to the maximum output power and applicable MPR for a given test point will depend on the power class.

In EVM test case in current definition, 64QAM is the highest modulation tested. Therefore, as per explanations made for OBW test above, for PC6 UEs, for MU analysis same SNR as for PC5 must be assumed.

Therefore, it can be concluded and proposed to define EVM MU and TT for PC6, PUSCH and PUCCH, same as already defined for PC5.

Proposal 2. For EVM test case, for PUSCH and PUCCH, define MU and TT same as defined for PC5.

## Spurious tests

As per analysis made in [1], influence of noise for any kind of spurious test case, General Spurious, Spurious co-existence, RX Spurious… is the same for PC6 as for PC5.  If in addition we consider that in the mentioned test case there are defined no particular exception applicable per power class, it can be concluded and proposed to reuse for PC6 the MU and TT already defined for PC5.

Proposal 3. For all Spurious test cases where MU and TT are defined for PC5, reuse the values for PC6.

It must be remarked in general that in case of PC6, MU and TT must be defined for band n261 but such a band is not applicable for PC5. Therefore, when reusing MU and TT from PC5 to PC6, the addition of band n261 must only been made for PC6. All power requirements for band n261 for PC6 are the same as for band n257 and this band is applicable to PC5.

Proposal 4. For PC6, define MU and TT for band n261 same as for n257 clarifying, in case values are reused from PC5, that band n261 is exclusive to PC6.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. In OBW test case, requirements are independent of the power class and are only verified using QPSK via EIRP measurements while UE power is pushed to its maximum.

Observation 2. In OBW test case, in worst case analysis, expected minimum UE power for PC6 will be the same expected for PC5.

Proposal 1. For OBW test case, for PC6, define MU and TT same as defined for PC5.

Proposal 2. For EVM test case, for PUSCH and PUCCH, define MU and TT same as defined for PC5.

Proposal 3. For all Spurious test cases where MU and TT are defined for PC5, reuse the values for PC6.

Proposal 4. For PC6, define MU and TT for band n261 same as for n257 clarifying, in case values are reused from PC5, that band n261 is exclusive to PC6.

# References

- R5-244172, “Discussion on Influence of noise for PC6 in FR2 test cases”, Keysight, RAN5#104

- R5-221615, “Discussion on MU and TT proposal for FR2 EVM”, Keysight, RAN5#94-e

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v18.5.2(2024-12)

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v18.5.0 (2024-12)