3GPP TSG-RAN5 Meeting #106	R5-250256

# Athens, Greece, 17 - 21 Feb 2025

Title:							Discussion on FR2 UE Beamlock test function

Source:			Nokia

Agenda Item:			5.3.29.9

Document for:			Discussion

# 1.	Background

In the LS [1] RAN4 asked how to extend the current FR2 UE Beamlock test function to lock UE two Tx beams from two panels simultaneously. Topic was discussed in RAN5#104 [2] and response LS was provided in [3]. RAN5 decided to finalize the design of UE Beamlock test function for two Tx beams from two panels simultaneously once the related WI is started. RAN4 provided response LS in [4]

This document provides our proposal on the question based on information in RAN4 LS [4].

# 2	Discussion

# 2.1	Related Work and Study Items

In the LS [4] RAN4 respond that requirements for STxMP have been specified under Rel-18 WI on MIMO Evolution for Downlink and Uplink (Acronym: NR_MIMO_evo_DL_U). Hence the FR2 UE Beamlock test function that RAN4 asked RAN5 to evaluate is for the testability of requirements defined in Clause 6.2K of TS 38.101-2 in Rel-18.

RAN5 has corresponding Rel-18 WI (Acronym: NR_MIMO_evo_DL_UL-UEConTest) ongoing but currently it does not include specification TS 38.509 [5].

# 2.2	UE special conformance testing functions

The utilization of any UE special conformance test functions shall be considered as putting the UE in a test mode. TS 38.509 [5] defines the UE special conformance test functions including any relevant procedure and the Test Mode Control (TMC) message contents used for information exchange.

The TMC messages for UE Beamlock Test Function (UBF) are defined in TS 38.509 [5].

The default TMC messages for UBF and test procedures to activate / deactivate UBF are defined in TS 38.508-1 [6].

# 2.3	Proposed solution

The current UBF only support locking ONE antenna pattern once it has formed ONE beam. Therefore, the UBF need to be extended to support locking two beams to fulfil the request in RAN4 LS [1].

Note that only bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE are currently used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE.

Our proposal is to set independently the number of Tx antenna patterns forming the Tx beams to be locked by the UE. Below we propose two alternative solutions.

Alternative A:

We propose an alternative to independently set the number of Tx antenna patterns forming the Tx beams to be locked by the UE as following:

- Use bit 3 in the octet of ‘UE Beamlock test Function’ IE to denote a binary number 0 or 1 (2 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

Alternative B:

As more than one beams can be formed by the same antenna patterns, we propose another alternative to independently set the number of Tx beams formed by the Tx antenna pattern(s) to be locked by the UE as following:

- Use bit 3 in the octet of ‘UE Beamlock test Function’ IE to denote a binary number 0 or 1 (2 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

Bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE shall continue to be used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE, to ensure no backward compatibility issue for legacy UE.

Note that proposed solution can readily be further extended to cover the cases with more than 2 Tx beams in future when testing with more than two MIMO layers are required.

# 2.4	Proposed changes to TS 38.508-1

Proposal can be implemented as new default TMC messages and test procedures into TS 38.508-1 as following. Modifications are shown by track changes for the current text. Below we propose two alternative solutions.

Alternative A:

### 4.7A.3 Beamlock messages #### - ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Tx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 0; Rx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 1; Tx and Rx. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Best with Tx. UE Beamlock test Function; 0 0 0 0 0 1 0 1; Best 2 with Tx. Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best UE antenna pattern with Tx. Best 2 with Tx; Lock the best two UE antenna pattern with Tx. #### - ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1.

#### - DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-3: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. #### - DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-4: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

### 4.9.2 Test procedure to activate UE Beamlock Test Function (UBF) 4.9.2.1 Initiation UE is operating in FR2 in RRC_CONNECTED state. 4.9.2.2 Procedure Table 4.9.2.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to activate UE beamlock function.; <--; ACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; ACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.2.3 Specific Message contents Table 4.9.2.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Tx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 0; Rx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 1; Tx and Rx. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Best with Tx. UE Beamlock test Function; 0 0 0 0 0 1 0 1; Best 2 with Tx.

Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best UE antenna pattern with Tx. Best 2 with Tx; Lock the best two UE antenna pattern with Tx. Table 4.9.2.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1. ### 4.9.3 Test procedure to deactivate UE Beamlock Test Function (UBF) 4.9.3.1 Initiation UE is operating in FR2 in RRC_CONNECTED state with UE beamlock test function activated. 4.9.3.2 Procedure Table 4.9.3.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to deactivate UE beamlock function.; <--; DEACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; DEACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.3.3 Specific Message contents Table 4.9.3.3-1: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. Table 4.9.3.3-2: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

Alternative B:

### 4.7A.3 Beamlock messages #### - ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Tx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 0; Rx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 1; Tx and Rx. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Best with Tx. UE Beamlock test Function; 0 0 0 0 0 1 0 1; Best 2 with Tx. Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best beam formed by the UE antenna pattern with Tx. Best 2 with Tx; Lock the best two beams formed by the UE antenna pattern(s) with Tx. #### - ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1.

#### - DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-3: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. #### - DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-4: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

### 4.9.2 Test procedure to activate UE Beamlock Test Function (UBF) 4.9.2.1 Initiation UE is operating in FR2 in RRC_CONNECTED state. 4.9.2.2 Procedure Table 4.9.2.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to activate UE beamlock function.; <--; ACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; ACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.2.3 Specific Message contents Table 4.9.2.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Tx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 0; Rx Only. UE Beamlock test Function; 0 0 0 0 0 0 1 1; Tx and Rx. UE Beamlock test Function; 0 0 0 0 0 0 0 1; Best with Tx. UE Beamlock test Function; 0 0 0 0 0 1 0 1; Best 2 with Tx.

Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best beam formed by the UE antenna pattern with Tx. Best 2 with Tx; Lock the best two beams formed by the UE antenna pattern(s) Tx. Table 4.9.2.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1. ### 4.9.3 Test procedure to deactivate UE Beamlock Test Function (UBF) 4.9.3.1 Initiation UE is operating in FR2 in RRC_CONNECTED state with UE beamlock test function activated. 4.9.3.2 Procedure Table 4.9.3.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to deactivate UE beamlock function.; <--; DEACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; DEACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.3.3 Specific Message contents Table 4.9.3.3-1: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. Table 4.9.3.3-2: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

# 2.5	Proposed changes to TS 38.509

Proposal can be implemented as new test procedures and TMC messages into TS 38.509 as following. Modifications are shown by track changes for the current text. Below we propose two alternative solutions.

Alternative A:

## 5.4 UE Beamlock test Function (UBF) ### 5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern(s) once it has formed beam(s) towards the base station (SS) direction(s) following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern(s) of all active intra-band component carriers and all MIMO layers affected by the test function. The Beamlock test function is mandatory for applicable UEs operating in Frequency Range 2 (FR2). The SS uses the UE Beamlock test mode activation procedure to command the UE to lock the UE antenna pattern(s). The Beamlock activation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-1: UE Beamlock test mode activation procedure The SS uses the UE Beamlock test mode deactivation procedure to command the UE to re-tracking the beam(s) towards the base station direction(s). The Beamlock deactivation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-2: UE Beamlock test mode deactivation procedure ### 5.4.2 Activate Beamlock procedure #### 5.4.2.1 Initiation The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message. #### 5.4.2.2 Reception of ACTIVATE BEAMLOCK message by UE When UE receives ACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state: 2> if bits 2 and 1 in UE Beamlock test Function = 01 3> Lock the UE antenna pattern with Tx only 2> else if bits 2 and 1 in UE Beamlock test Function = 10 3> Lock the UE antenna pattern with Rx only 2> else if bits 2 and 1 in UE Beamlock test Function = 11 3> Lock the UE antenna pattern with both TxRx 2> if bit 3 in UE Beamlock test Function = 0 3> Lock the best UE antenna pattern with Tx 2> else if bit 3 in UE Beamlock test Function = 1 3> Lock the best two UE antenna patterns with Tx 2> Transmit ACTIVATE BEAMLOCK COMPLETE message 1> else: 2> the UE behaviour is unspecified. ### 5.4.3 Deactivate Beamlock procedure #### 5.4.3.1 Initiation The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state. #### 5.4.3.2 Reception of DEACTIVATE BEAMLOCK message by UE When UE receives DEACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active: 2> unlock all active UE antenna pattern(s) and transmit DEACTIVATE BEAMLOCK COMPLETE message; 1> else: 2> the UE behaviour is unspecified. #### 5.4.3.3 Release of antenna beamlock by UE When the UE leaves the RRC_CONNECTED state, the UE shall: 1> if the UE is operating in FR2 AND the UE Beamlock test Function is active 2> unlock all active UE antenna pattern(s);

## 6.4 Beamlock messages ### 6.4.1 ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. UE Beamlock test Function; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 0; octet 1. where UE Beamlock test Function is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. T1; T2; X1; X2; octet 1. where X1,X2 = 01 for activate beamlock of Tx only, 10 for activate beamlock of Rx only and 11 for activate beamlock of both TxRx, where T1,T2 = 00 for activate beamlock of best Tx antenna pattern, 01 for activate beamlock of best two Tx antenna patterns. NOTE: X1,X2 = 00 is not used ### 6.4.2 ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 1; octet 1. ### 6.4.3 DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 0; octet 1. ### 6.4.4 DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 1; octet 1.

Alternative B:

## 5.4 UE Beamlock test Function (UBF) ### 5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern(s) once it has formed beam(s) towards the base station (SS) direction(s) following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern(s) of all active intra-band component carriers and all MIMO layers affected by the test function. The Beamlock test function is mandatory for applicable UEs operating in Frequency Range 2 (FR2). The SS uses the UE Beamlock test mode activation procedure to command the UE to lock the UE antenna pattern(s). The Beamlock activation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-1: UE Beamlock test mode activation procedure The SS uses the UE Beamlock test mode deactivation procedure to command the UE to re-tracking the beam(s) towards the base station direction(s). The Beamlock deactivation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-2: UE Beamlock test mode deactivation procedure ### 5.4.2 Activate Beamlock procedure #### 5.4.2.1 Initiation The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message. #### 5.4.2.2 Reception of ACTIVATE BEAMLOCK message by UE When UE receives ACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state: 2> if bits 2 and 1 in UE Beamlock test Function = 01 3> Lock the UE antenna pattern with Tx only 2> else if bits 2 and 1 in UE Beamlock test Function = 10 3> Lock the UE antenna pattern with Rx only 2> else if bits 2 and 1 in UE Beamlock test Function = 11 3> Lock the UE antenna pattern with both TxRx 2> if bit 3 in UE Beamlock test Function = 0 3> Lock the best beam formed by the UE antenna pattern with Tx 2> else if bit 3 in UE Beamlock test Function = 1 3> Lock the best two beams formed by the UE antenna pattern(s) with Tx 2> Transmit ACTIVATE BEAMLOCK COMPLETE message 1> else: 2> the UE behaviour is unspecified. ### 5.4.3 Deactivate Beamlock procedure #### 5.4.3.1 Initiation The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state. #### 5.4.3.2 Reception of DEACTIVATE BEAMLOCK message by UE When UE receives DEACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active: 2> unlock all active beam(s) formed by the UE antenna pattern(s) and transmit DEACTIVATE BEAMLOCK COMPLETE message; 1> else: 2> the UE behaviour is unspecified. #### 5.4.3.3 Release of antenna beamlock by UE When the UE leaves the RRC_CONNECTED state, the UE shall: 1> if the UE is operating in FR2 AND the UE Beamlock test Function is active 2> unlock all active beam(s) formed by the UE antenna pattern(s);

## 6.4 Beamlock messages ### 6.4.1 ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. UE Beamlock test Function; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 0; octet 1. where UE Beamlock test Function is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. T1; T2; X1; X2; octet 1. where X1,X2 = 01 for activate beamlock of Tx only, 10 for activate beamlock of Rx only and 11 for activate beamlock of both TxRx, where T1,T2 = 00 for activate beamlock of best beam formed by the Tx antenna pattern, 01 for activate beamlock of best two beams formed by the Tx antenna pattern(s). NOTE: X1,X2 = 00 is not used ### 6.4.2 ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 1; octet 1. ### 6.4.3 DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 0; octet 1. ### 6.4.4 DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 1; octet 1.

# 3.	Conclusion

To fulfil the request in RAN4 LS [1] our proposal is to set independently the number of Tx antenna patterns forming the Tx beams to be locked by the UE. Below we propose two alternative solutions.

Alternative A:

We propose an alternative to independently set the number of Tx antenna patterns forming the Tx beams to be locked by the UE as following:

- Use bit 3 in the octet of ‘UE Beamlock test Function’ IE to denote a binary number 0 or 1 (2 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

Alternative B:

As more than one beams can be formed by the same antenna patterns, we propose another alternative to independently set the number of Tx beams formed by the Tx antenna pattern(s) to be locked by the UE as following:

- Use bit 3 in the octet of ‘UE Beamlock test Function’ IE to denote a binary number 0 or 1 (2 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

Bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE shall continue to be used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE, to ensure no backward compatibility issue for legacy UE.

Note that our solution can readily be further extended to cover the cases with more than 2 Tx beams in future when testing with more than more than two MIMO layers are required.

We propose RAN5 to consider this solution. If the solution is agreeable, we propose RAN5 to:

- provide the response LS to RAN4 [9],

- revise corresponding Rel-18 WI (Acronym: NR_MIMO_evo_DL_UL-UEConTest) to include also TS 38.509 [5] as impacted specification,

- consider agreeing CR in [7] implementing proposed solution to TS 38.508-1 [6]. CR is implementing Alternative A but can also be changed to Alternative B based on RAN5 feedback

Corresponding draft CR to TS 38.509 [5] is shown as information in [8]. Formal CR for agreements can be provided for the next meeting after revising the “NR_MIMO_evo_DL_UL-UEConTest” WID in RAN#107.

# 4.	Reference

[1]	R4-2409999, “LS on FR2 UE Beamlock test function” RAN4

[2] R5-244394, “Discussion on FR2 UE Beamlock test function”, Nokia

[3] R5-24530, “Reply LS on FR2 UE Beamlock test function”, RAN5

[4] R4-2418847, “Reply LS on FR2 UE Beamlock test function”, RAN4

[5] TS 38.509, “SGS; Special conformance testing functions for User Equipment (UE) (Release 17)”

[6] TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment (Release 18)”

[7] R5-250257, “FR2 UE Beamlock test function modification of TMC messages and test procedures”, Nokia

[8] R5-250258, “Modification of Beamlock test function to support locking two TX beams from two panels simultaneously”, Nokia

[9] R5-250259, “Reply LS on FR2 UE Beamlock test function”, RAN5