3GPP TSG-RAN5 Meeting #106 	R5-250260

# Athens, Greece, 17 - 21 Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5531 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A-MPR for NS_06 PC1 in band 12 for 64QAM and 256QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NR_HPUE_FWVM_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A-MPR requirements for NS_06 for PC1 in Band 12 for 64QAM and 256QAM are tested in TC 6.2.4_1 A-MPR for HPUE, but generally 64QAM and 256QAM are tested in TC 6.2.4_2 A-MPR for UL 64QAM and TC 6.2.4_4 A-MPR for UL 256QAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test points for 64QAM were moved from 6.2.4_1 to 6.2.4_2. - Test points for 256QAM were moved from 6.2.4_1 to 6.2.4_4. - Editorial correction for Table 6.2.4_1.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration of test cases will make a confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4_1, 6.2.4_2, 6.2.4_4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Informal note for the implementer: For Table 6.2.4_1.3-1, please change the setting of Text wrapping from ‘Around’ to ‘None’ at Table Properties. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.4_1	Additional Maximum Power Reduction (A-MPR) for HPUE

#### 6.2.4_1.1	Test purpose

Same test purpose as in clause 6.2.4.1 with the follow exception:

-	Instead of Table 6.2.2.3-1  use Table 6.2.2_1.3-1

#### 6.2.4_1.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 Additional Spectrum Emission Mask and 6.6.3.3 Additional spurious emissions for network signalled values NS_04, NS_06 and NS_44 to all types of E-UTRA Power Class 1 or Power Class 2 UE release 10 and forward.

NOTE:	As a result TC 6.2.4_1 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4_1.3	Minimum conformance requirements

For UE Power Class 1 or Power Class 2 the specific requirements and identified sub-clauses are specified in Table 6.2.4_1.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified in Table 6.2.4_1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3_1. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2.5_1 apply.

The normative reference for this requirement is TS 36.101 clause 6.2.4.

Table 6.2.4_1.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_04 | 6.6.2.2.3.2, 6.6.3.3.3.1 | 41 | 5,10,15,20 | Table 6.2.4_1.3-2 |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | Table 6.2.4_1.3-4 (NOTE 2) |
| NS_44 | 6.6.3.3.33 | 38 (Note 1) | 5, 10, 15, 20 | Table 6.2.4_1.3-3 |  |
| NOTE 1: Applicable for power class 2 UE in E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz. NOTE 2: Applicable only for power class 1 UE in Band 12. Not applicable for all other cases. |  |  |  |  |  |

Table 6.2.4_1.3-2: A-MPR requirements for "NS_04" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  | 9 - 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 3 |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  | > 2504 |
|  | RBstart | 0 – 8 |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 3 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  | > 2510.8 |
|  | RBstart | 0 – 13 |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 36 | ≥ 36 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 40 | ≥ 40 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |  |  |  |

For a power class 2 capable UE operating in Band 41, A-MPR according to Table 6.2.4.3-2, Table 6.2.4.3-3, or Table 6.2.4.3-4 for power class 3 is allowed when an IE P-max as defined in [5] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

Table 6.2.4_1.3-3: A-MPR requirements for "NS_44" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Fc [MHz] | ≥2605 |  |  |  | <2605 |
|  | RBstart | 0 – 5 |  | 38-49 |  | 0 - 49 |
|  | LCRB [RBs] | ≥ 45 |  | < 13 |  | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | NA |  | N/A |
|  | A-MPR [dB] | ≤ 1 |  | ≤ 1 |  | 0 |
| 15 | Fc [MHz] | ≥2597.5 |  |  |  | <2597.5 |
|  | RBstart | 0 – 18 | 19-30 |  | 54-74 | 0 - 74 |
|  | LCRB [RBs] | N/A | ≥ 45& <56 |  | < 17 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 63 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 1 | ≤ 1 |  | ≤ 1 | 0 |
| 20 | Fc [MHz] | ≥2590 |  |  |  | <2590 |
|  | RBstart | 0 – 31 | 32-55 |  | 70-99 | 0 - 99 |
|  | LCRB [RBs] | N/A | ≥ 45& <68 |  | < 19 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 76 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 1 |  | ≤ 1 | 0 |

Table 6.2.4_1.3-4: A-MPR requirements for "NS_06" for Power Class 1 UE in Band 12

| Channel bandwidth [MHz] | LCRB | Allocation positions | Modulation / A-MPR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK | 16-QAM | 64-QAM | 256-QAM |
| 1.4 | all | all | 0 | 0 | 0 | 0 |
| 3 | = 15 | RBstart = 0 | 1 | 0.5 | 0 | 0 |
| 5 | 15 | RBstart 2 or RBstart + LCRB 23 | 1 | 0.5 | 0 | 0 |
| 10 | 18 | RBstart 2 or RBstart + LCRB 48 | 1 | 0.5 | 0 | 0 |

#### 6.2.4_1.4	Test description

Same test description as in clause 6.2.4.4 with the following exceptions:

Instead of Table 6.2.4.4.1-1 to 6.2.4.4.1-26  use Table 6.2.4_1.4-1 or Table 6.2.4_1.4-2.

-------- Skipped unchanged clauses --------

Table 6.2.4_1.4-4: Test Configuration Table (network signalled value "NS_06") for Power Class 1 UE in Band 12

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz |  |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low | 1.4MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 | High | 1.4MHz |  |  | QPSK | 1 | 5 |
| 3 | Low | 1.4MHz |  |  | QPSK | 6 | 0 |
| 4 | Low | 1.4MHz |  |  | 16QAM | 1 | 0 |
| 5 | High | 1.4MHz |  |  | 16QAM | 1 | 5 |
| 6 | Low | 1.4MHz |  |  | 16QAM | 6 | 0 |
| 7 | Low | 3MHz |  |  | QPSK | 15 | 0 |
| 8 | Low | 3MHz |  |  | 16QAM | 15 | 0 |
| 9 | Low | 5MHz |  |  | QPSK | 25 | 0 |
| 10 | Low | 5MHz |  |  | 16QAM | 25 | 0 |
| 11 | Low | 10MHz |  |  | QPSK | 50 | 0 |
| 12 | Low | 10MHz |  |  | 16QAM | 50 | 0 |

#### 6.2.4_1.5	Test requirements

The maximum output power, derived in step 2 of clause 6.2.4_1.4.2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.4_1.5-1 and 6.2.4_1.5-2. The allowed A-MPR values specified in Table 6.2.4_1.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3_1. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5_1.3-1 apply.

-------- Skipped unchanged clauses --------

Table 6.2.4_1.5-4: UE Power Class 1 test requirements (network signalled value "NS_06") in Band 12

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 2 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 3 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 4 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 5 | 1 | 0 | 0 | 30 | 2.0 | 33.7 | 26.3 |
| 6 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 7 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 8 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |
| 9 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 10 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |
| 11 | 1 | 1 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 12 | 2 | 0.5 | 0 | 28.5 | 2.0 | 33.7 | 24.8 |

### 6.2.4_2	Additional Maximum Power Reduction (A-MPR) for UL 64QAM

Editor's note:

- Test requirements values in this section need to be reviewed in order to take into account notes in Table 6.2.5.3-2: ΔTIB,c regarding ΔTIB,c and asymmetric lower tolerances in table 6.2.2.3-1.

#### 6.2.4_2.1	Test purpose

Same test purpose as in clause 6.2.4.1

#### 6.2.4_2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_1 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

The requirements of this test apply in test case 6.6.3.3_1 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_38, NS_39, NS_56, NS_62 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

NOTE:	As a result TC 6.2.4_2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4_2.3	Minimum conformance requirements

Same minimum conformance requirements as in clauses 6.2.4.3 and 6.2.4_1.3.

#### 6.2.4_2.4	Test description

##### 6.2.4_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_2.4.1-1 through table 6.2.4_2.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged sections skipped>>

Table 6.2.4_2.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | 64QAM | 5 |
| 2 | 5MHz |  |  | 64QAM | 8 |
| 3 | 10MHz |  |  | 64QAM | 12 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_2.5 as not all combinations are necessarily required based on the applicability of the UE. Note 3: The RBstart of partial RB allocation shall be RB# 0 and RB# (max +1 - RB allocation) of the channel bandwidth. Note 4: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_06. |  |  |  |  |  |

Table 6.2.4_2.4.1-4a: Test Configuration Table (network signalled value "NS_06") for Power Class 1 UE in Band 12

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz |  |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low | 1.4MHz | N/A for A-MPR testing |  | 64QAM | 1 | 0 |
| 2 | High | 1.4MHz |  |  | 64QAM | 1 | 5 |
| 3 | Low | 1.4MHz |  |  | 64QAM | 6 | 0 |
| 4 | Low | 3MHz |  |  | 64QAM | 15 | 0 |
| 5 | Low | 5MHz |  |  | 64QAM | 25 | 0 |
| 6 | Low | 10MHz |  |  | 64QAM | 50 | 0 |

-------- Skipped unchanged clauses --------

#### 6.2.4_2.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4_2.5-1 to Table 6.2.4_2.5-29. The allowed A-MPR values specified in Table 6.2.4_2.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

-------- Skipped unchanged clauses --------

Table 6.2.4_2.5-6: UE Power Class other than 1 test requirements (network signalled value "NS_06")
(for Band 12)

| Configuration ID |  | EUTRA band | Test Freq. | Class 2 (dBm) | Tol. (dB) | Class 3 (dBm) | Tol. (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 12 | Mid |  |  | 23 | +2.7 / -4.7 |  |
| 1 |  | 12 | Low, High |  |  | 23 | +2.7 / -5.2 |  |
| 2 |  | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  | 23 | +2.7 / -4.7 |  |
| 2 |  | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  | 23 | +2.7 / -7.7 |  |
| 3 |  | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  | 23 | +2.7 / -4.7 |  |
| 3 |  | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  | 23 | +2.7 / -7.7 |  |

Table 6.2.4_2.5-6a: UE Power Class 1 test requirements (network signalled value "NS_06") in Band 12

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 2 | 2 | 0 | 0 | 29 | 2.0 | 33.7 | 25.3 |
| 3 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 4 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 5 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |
| 6 | 3 | 0 | 0 | 28 | 2.0 | 33.7 | 24.3 |

-------- Skipped unchanged clauses --------

### 6.2.4_4	Additional Maximum Power Reduction (A-MPR) for UL 256QAM

Editor's note: Some test requirements for NS_24 and NS_25 - for E-UTRA band 65 - need further investigation, due to unclear conformance requirements.

#### 6.2.4_4.1	Test purpose

Same test purpose as in clause 6.2.4.1

#### 6.2.4_4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

The requirements of this test apply in test case 6.6.3.3_2 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_56, NS_62 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

NOTE:	As a result, TC 6.2.4_4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4_4.3	Minimum conformance requirements

Same minimum conformance requirements as in clauses 6.2.4.3 and 6.2.4_1.3.

#### 6.2.4_4.4	Test description

##### 6.2.4_4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_4.4.1-1 through table 6.2.4_4.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

-------- Skipped unchanged clauses --------

Table 6.2.4_4.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  | Lowest, 5MHz, 10MHz, Highest |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |
|  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configura-tion ID | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | 1.4MHz | N/A for A-MPR testing |  | 256QAM | 6 |
| 2 | 5MHz |  |  | 256QAM | 25 |
| 3 | 10MHz |  |  | 256QAM | 50 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_4.5 as not all combinations are necessarily required based on the applicability of the UE. |  |  |  |  |  |

Table 6.2.4_4.4.1-4a: Test Configuration Table (network signalled value "NS_06") for Power Class 1 UE in Band 12

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | 1.4MHz, 3MHz, 5MHz, 10MHz |  |  |
| Test Parameters for NS_06 A-MPR |  |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID | Frequency | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 | Low | 1.4MHz | N/A for A-MPR testing |  | 256QAM | 1 | 0 |
| 2 | High | 1.4MHz |  |  | 256QAM | 1 | 5 |
| 3 | Low | 1.4MHz |  |  | 256QAM | 6 | 0 |
| 4 | Low | 3MHz |  |  | 256QAM | 15 | 0 |
| 5 | Low | 5MHz |  |  | 256QAM | 25 | 0 |
| 6 | Low | 10MHz |  |  | 256QAM | 50 | 0 |

-------- Skipped unchanged clauses --------

#### 6.2.4_4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4_4.5-1 to Table 6.2.4_4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

-------- Skipped unchanged clauses --------

Table 6.2.4_4.5-6: UE Power Class other than 1 test requirements (network signalled value "NS_06")
(for Band 12)

| Configuration ID |  | EUTRA band | Test Freq. | Class 2 (dBm) |  | Tol. (dB) | Class 3 (dBm) |  | Tol. (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 12 | Mid |  |  |  | 23 |  | +2.7 / -9.7 |  |
| 1 |  | 12 | Low, High |  |  |  | 23 |  | +2.7 / -12.2 |  |
| 2 |  | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  | 23 |  | +2.7 / -9.7 |  |
| 2 |  | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  | 23 |  | +2.7 / -12.2 |  |
| 3 |  | 12 | Low @ RB#(max+1-RB allocation), Mid, High @ RB#0 |  |  |  | 23 |  | +2.7 / -9.7 |  |
| 3 |  | 12 | Low @ RB#0, High @ RB#(max+1-RB allocation) |  |  |  | 23 |  | +2.7 / -12.2 |  |
| NOTE: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4_4.5-6a: UE Power Class 1 test requirements (network signalled value "NS_06") in Band 12

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 2 | 0 | 0 | 0 | 31 | 2.0 | 33.7 | 27.3 |
| 3 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 4 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 5 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |
| 6 | 5 | 0 | 0 | 26 | 2.0 | 33.7 | 22.3 |

-------- End of changes --------