Title:	Discussion on test point selection for CA_NS_202 A-MPR in FR2

Source:	Rohde & Schwarz

# Introduction

The purpose of this paper is to provide test point selection proposal for 38.101-2 A-MPR requirements when CA_NS_202 is signalled (band n257 and n258). Additional RF emission requirements apply in this scenario in the form of Spurious emissions, meaning this proposal applies for test cases 6.2A.3 (A-MPR for CA) and 6.5A.3.3 (Additional Spurious emission for CA).

Test point selection is considered for PC1, PC2, PC3 and PC4.

# Discussion

Since A-MPR for CA_NS_202 in RAN4 is defined together with A-Spurious requirements, a combined analysis is required. In general, the following non-compliant UE behaviors need to be checked:

- UE apply too much A-MPR (more than RAN4 allow)

- UE apply to little A-MPR (causing too much RF emissions)

Case A can be verified in A-MPR test case.

Case B can be verified in A-Spurious test case and does not require check of UE output power if it is ensured that the same test point is tested inside A-MPR test.

A-MPR for CA_NS_202 is defined in 38.101-2 [1] for intra-band contiguous CA as:

For power class 1, A-MPR for CA_NS_202 shall be 11.0 dB

For power class 2, A-MPR for CA_NS_202 shall be 2.0 dB.

For power class 3, A-MPR for CA_NS_202 shall be 2.0 dB.

For power class 4, A-MPR for CA_NS_202 shall be 2.0 dB.

For power class 5, A-MPR for CA_NS_202 shall be 2.0 dB.

A-Spurious for CA_NS_202 is defined in 38.101-2 [2] as:

When "CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in

Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |  |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz | 1 |
| NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |

## Power classes

For power classes PC2 to PC5, and for all UL modulations and cumulative aggregated bandwidths, A-MPR is smaller than MPRC_CA and hence does not need to be considered in the test point selection. There is another conformance requirement valid in the case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration. Despite this does not fulfil the purpose of UL CA test where more than one UL CC shall be allocated, same situation happens for NS_203 and there it is agreed to test only with UL allocation on the PCC. Therefore, this test point selection analysis focuses on UL CA for PC1 and single UL in PCC only otherwise.

## Test Environment

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested in Normal conditions, and the same selection is appropriate also for Additional Spurious emission.

## Test Frequencies

Protected range for CA_NS_202 is both below and above band n257 and n258. It is judged that the worst case is when the carrier frequency is as close as possible to the protected range, which implies Low and High frequency range.

## Test Channel Bandwidth

The A-MPR requirements are the same for a particular power class for any channel bandwidth. Therefore, test bandwidth can be decided based on what is most demanding for Spurious emission. Selection can therefore be reused from normal spurious test case which means Highest aggregated bandwidth.

## Test Subcarrier Spacing

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with 120 kHz. Selection can therefore be reused from normal spurious test case which means 120 kHz.

## UL Modulation

RF emission requirements for CA_NS_202 are defined as Spurious emissions. Regular spurious emissions for FR2 is tested with QPSK DFT-s-OFDM modulation to maximize UE output power (lowest power backoff). Testing at a high output power is considered more important than testing a more demanding modulation scheme for Spurious testing.

For PC1 where A-MPR is higher than MPR regardless of modulation for a cumulative aggregated channel bandwidth < 800 MHz also 64QAM is tested. The reason is that in this case 64QAM does not give the UE more allowed MPR and should therefore be added as a worst case. The same UL modulation is applied on all CCs.

## Uplink RB allocation

RF emission requirements for CA_NS_202 are defined as Spurious emissions.

For PC1, A-MPR is always 11 dB meaning the UE output power may be the same regardless of test parameters. A worst case to test will then be an outer allocation closest to the band edge with high order modulation.

For power classes PC2 to PC6, as mentioned in section 2.1, the single carrier MPR will apply. The Uplink RB allocation needs to be selected so that MPR is less than A-MPR. For indicated power classes, Inner RB Region 1 will lead to a 0dB MPR and could be selected as test configuration.

## Check of alignment of test parameters between power measurement test case (A-MPR) and RF emissions test case (A-Spur)

Now that test point proposal is completed, it is required to check that all test points selected in RF emissions test case (Spurious in this case) is covered in A-MPR test case.

Result:

The test points in this paper matches the test points in A-Spurious and A-MPR test cases in TS38.521-2 for all power classes.

# Proposal

Following test points are proposed for CA_NS_202 A-MPR and A-Spur test cases (PC1)

- Test environment: Normal Conditions

- Test frequencies: Low and High range

- Test Channel Bandwidth: Highest aggregated bandwidth

- Test SCS: 120 kHz

- RB allocation and modulation:

  - PC1: Outer_Full (DFT-s-OFDM QPSK ), Outer_Full (64QAM) on all CCs.

  - PC2 to PC6: Inner RB Region 1 (DFT-s-OFDM QPSK ) on PCC only

# Number of Test Points

The number of test points for NS_202 A-MPR is 4 for PC1, 2 for PC2 to PC6.

The number of test points for NS_202 A-Spurious is 4 for PC1, 2 for PC2 to PC6.

# References

- TS 38.101-2 v17.8.0 User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (Release 17)