3GPP TSG-RAN5 Meeting #106	R5-250288

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1133 | rev | - | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for CA NS 202 and NS 203 in 38.521-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TP analysis update R5-250287 (CR 0984), it is proposed to extend CA_NS_202 testing to power classes other than PC1 in the same way CA_NS_203 is tested for them. In addition, test requirements for CA_NS_203 are incorrect for power classes other than PC1 where A-MPR for PC1 has been incorrectly used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated A-MPR and A-SE CA test cases according to updated TP analysis for CA_NS_202. Corrected test requirements for CA_NS_202 for power classes other than PC1. General update of 6.5A.3.3.x to get it fully aligned with 6.2A.3.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of CA_NS_202 will remain limited to PC1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1, 6.2A.3.2, 6.2A.3.3, 6.5A.3.3.1, 6.5A.3.3.2, 6.5A.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.2A.3.1	UE maximum output power with additional requirements for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The UPLF test mode is applicable to UEs Release 16 and forward.

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

- For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

-	Whether additional check is needed in the test procedure to ensure UE continues transmissions on the SCell is FFS

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and intra-band non-contiguous CA are TBD.

-	The test points for higher bandwidth classes with testability problem need an update to decrease the UL bandwidth until they become testable.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.

6.2A.3.1.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward supporting 2UL CA.

6.2A.3.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.3.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in Table 6.2A.3.1.4.1-1 and Table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.3.1.4.1-1: Test Configuration Table for CA_NS_202 (Power Class 1)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM 64QAM | Outer_Full |
|  | SCCs |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.1.4.1-1b: Test Configuration Table for CA_NS_202 (Power Class 2, 3, 4, 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table for CA_NS_203 (Power Class 1, 2, 3, 4 and 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 Inner_Full_Region1 for PC1 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5 or Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2A.3.1.4.1-1 to Table 6.2A.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.3.1.4.3.

6.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in subclause 6.2A.3.1.4.3.

3.	Apply the test step based on the 5G NR UE Release:

3a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF).  The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using TOTAL NR AGGREGATED BANDWIDTH and PCELL NR bandwidth as per Test CC Combination setting. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 4.

3b.	For Release 15 5G NR UEs: No action.

3c.	For testing single CC A-MPR requirement: No action.

4.  SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.3.1.4.1-1 to Table 6.2A.3.1.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	Apply the test step based on the 5G NR UE Release:

7a.	For Release 16 and forward 5G NR UEs: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7b.	For Release 15 5G NR UEs: Send uplink power control commands in uplink scheduling information to the UE per UL CC until the Power Headroom Report (PHR) from the UE for each UL CC is at the target value according to Table 6.2A.3.1.4.2-1; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

Table 6.2A.3.1.4.2-1: Power target values per UL CC for test procedure using PHR

| BW ratio (Note 1) | Xmax [dB] (Note 2) | Target PHR | ΔPHR [dB] (Note 3) | BW combination examples |
| --- | --- | --- | --- | --- |
| 1/2 | 3.0 | POWER_HEADROOM_36 (3  PH  4) | 1 | 2CC equal BW |
| 1/3 | 4.8 | POWER_HEADROOM_38 (5  PH  6) | 1.2 | 2CC 50+100 MHz CC1 |
| 2/3 | 1.8 | POWER_HEADROOM_35 (2  PH  3) | 1.2 | 2CC 50+100 MHz CC2 |
| 1/5 | 7.0 | POWER_HEADROOM_40 (7  PH  8) | 1.0 | 2CC 50+200 MHz CC1 |
| 4/5 | 1.0 | POWER_HEADROOM_34 (1  PH  2) | 1.0 | 2CC 50+200 MHz CC2 |
| 1/9 | 9.5 | POWER_HEADROOM_43 (10  PH  11) | 1.5 | 2CC 50+400 MHz CC1 |
| 8/9 | 0.5 | POWER_HEADROOM_34 (1  PH  2) | 1.5 | 2CC 50+400 MHz CC2 |
| Note 1: The BW ratio is the ratio of BW of the CC over the total Aggregated UL BW Note 2: Xmax = 10log(BW ratio) Note 3: ΔPHR is the worst case UE output power decrease due to Xmax and 1 dB reporting granularity of PHR according to TS38.133 [25]. |  |  |  |  |

7c.	For testing single CC A-MPR requirement: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

8.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

9.	Measure UE EIRP in the Tx beam peak direction in the accumulative aggregated channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2A.3.1.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

10.	Apply the test step based on the 5G NR UE Release:

10a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS deactivates the UE Power Limit Function (UPLF) by performing the DEACTIVATE POWER LIMIT REQUEST procedure as specified in TS 38.508-1 [10] clause 4.9.33.

10b.	For Release 15 5G NR UEs: No action.

10c.	For testing single CC A-MPR requirement: No action.

11.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2A.3.1.4.1-1 to Table 6.2A.3.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with the following exceptions for Release 15 5G NR UE.

Table 6.2A.3.1.4.3-1: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2A.3.1.4.3-2: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -4 |  | 50 MHz |
| p0-NominalWithGrant | -8 |  | 100 MHz |
| p0-NominalWithGrant | -10 |  | 200 MHz |
| p0-NominalWithGrant | -14 |  | 400 MHz |
| } |  |  |  |

Table 6.2A.3.1.4.3-3: BSR-Config (Rel-15 UE only)

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BSR-Config ::= SEQUENCE { |  |  |  |
| periodicBSR-Timer | infinity |  |  |
| retxBSR-Timer | sf80 |  |  |
| logicalChannelSR-DelayTimer | Not present |  |  |
| } |  |  |  |

6.2A.3.1.4.3.1	Message contents exceptions (network signalling value " CA_NS_202" on PCC and SCC)

Table 6.2A.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_202"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_202) |  | band n257 |
|  | 2 (CA_NS_202) |  | band 258 |

6.2A.3.1.4.3.2	Message contents exceptions (network signalling value " CA_NS_203" on PCC and SCC)

Table 6.2A.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_203"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (CA_NS_203) |  | band n258 |

6.2A.3.1.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.1.5-1 to Table 6.2A.3.1.5-6.

Table 6.2A.3.1.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.1.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: Cumulative aggregated BW = 800MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2.0 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

#### 6.2A.3.2	UE maximum output power with additional requirements for CA (3UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The UPLF test mode is applicable to UEs Release 16 and forward.

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

- For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

-	Whether additional check is needed in the test procedure to ensure UE continues transmissions on the SCell is FFS

-	Measurement Uncertainties and Test Tolerances are are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and intra-band non-contiguous CA are TBD.

-	The test points for higher bandwidth classes with testability problem need an update to decrease the UL bandwidth until they become testable.

6.2A.3.2.1	Test purpose

Same as test purpose in 6.2A.3.1.1.

6.2A.3.2.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward supporting 3UL CA.

6.2A.3.2.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.3.

6.2A.3.2.4	Test description

6.2A.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in Table 6.2A.3.2.4.1-1 and Table 6.2A.3.2.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.3.2.4.1-1: Test Configuration Table for CA_NS_202 (Power Class 1)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM 64QAM | Outer_Full |
|  | SCCs |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.2.4.1-1b: Test Configuration Table for CA_NS_202 (Power Class 2, 3, 4, 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.2.4.1-2: Test Configuration Table for CA_NS_203 (Power Class 1, 2, 3, 4 and 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3, PC4 and PC5 Inner_Full_Region1 for PC1 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4 or Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2A.3.2.4.1-1 to Table 6.2A.3.2.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.3.2.4.3.

6.2A.3.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in subclause 6.2A.3.2.4.3.

3.	Apply the test step based on the 5G NR UE Release:

3a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF).  The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using TOTAL NR AGGREGATED BANDWIDTH and PCELL NR bandwidth as per Test CC Combination setting. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 4.

3b.	For Release 15 5G NR UEs: No action.

3c.	For testing single CC A-MPR requirement: No action.

4.  SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.3.2.4.1-1 to Table 6.2A.3.2.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	Apply the test step based on the 5G NR UE Release:

7a.	For Release 16 and forward 5G NR UEs: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7b.	For Release 15 5G NR UEs: Send uplink power control commands in uplink scheduling information to the UE per UL CC until the Power Headroom Report (PHR) from the UE for each UL CC is at the target value according to Table 6.2A.3.2.4.2-1; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

Table 6.2A.3.2.4.2-1: Power target values per UL CC for test procedure using PHR

FFS

7c.	For testing single CC A-MPR requirement: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

8.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

9.	Measure UE EIRP in the Tx beam peak direction in the accumulative aggregated channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2A.3.2.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

10.	Apply the test step based on the 5G NR UE Release:

10a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS deactivates the UE Power Limit Function (UPLF) by performing the DEACTIVATE POWER LIMIT REQUEST procedure as specified in TS 38.508-1 [10] clause 4.9.33.

10b.	For Release 15 5G NR UEs: No action.

10c.	For testing single CC A-MPR requirement: No action.

11.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2A.3.2.4.1-1 to Table 6.2A.3.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with the following exceptions for Release 15 5G NR UE.

Table 6.2A.3.2.4.3-1: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2A.3.2.4.3-2: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -4 |  | 50 MHz |
| p0-NominalWithGrant | -8 |  | 100 MHz |
| p0-NominalWithGrant | -10 |  | 200 MHz |
| p0-NominalWithGrant | -14 |  | 400 MHz |
| } |  |  |  |

Table 6.2A.3.2.4.3-3: BSR-Config (Rel-15 UE only)

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BSR-Config ::= SEQUENCE { |  |  |  |
| periodicBSR-Timer | infinity |  |  |
| retxBSR-Timer | sf80 |  |  |
| logicalChannelSR-DelayTimer | Not present |  |  |
| } |  |  |  |

6.2A.3.2.4.3.1	Message contents exceptions (network signalling value " CA_NS_202" on PCC and SCC)

Table 6.2A.3.2.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_202"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_202) |  | band n257 |
|  | 2 (CA_NS_202) |  | band 258 |

6.2A.3.2.4.3.2	Message contents exceptions (network signalling value " CA_NS_203" on PCC and SCC)

Table 6.2A.3.2.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_203"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (CA_NS_203) |  | band n258 |

6.2A.3.2.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.2.5-1 to Table 6.2A.3.2.5-6.

Table 6.2A.3.2.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.2.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: 800MHz <= Cumulative aggregated BW < 1400MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

#### 6.2A.3.3	UE maximum output power with additional requirements for CA (4UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	The UPLF test mode is applicable to UEs Release 16 and forward.

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

- For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

-	Whether additional check is needed in the test procedure to ensure UE continues transmissions on the SCell is FFS

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and intra-band non-contiguous CA are TBD.

-	The test points for higher bandwidth classes with testability problem need an update to decrease the UL bandwidth until they become testable.

6.2A.3.3.1	Test purpose

Same as test purpose in 6.2A.3.1.1.

6.2A.3.3.2	Test applicability

The requirements of this test apply to all types of NR UE release 15 and forward supporting 4UL CA.

6.2A.3.3.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-2 [3] clause 6.2A.3.

6.2A.3.3.4	Test description

6.2A.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration and subcarrier spacing, are shown in Table 6.2A.3.3.4.1-1 and Table 6.2A.3.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2A.3.3.4.1-1: Test Configuration Table for CA_NS_202 (Power Class 1)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 | PCC |  | DFT-s-OFDM 64QAM | Outer_Full |
|  | SCCs |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.3.4.1-1b: Test Configuration Table for CA_NS_202 (Power Class 2, 3, 4, 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.2A.3.3.4.1-2: Test Configuration Table for CA_NS_203 (Power Class 1, 2, 3, 4, and 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3, PC4 and PC5 Inner_Full_Region1 for PC1 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4 and PC5 or Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.2A.3.3.4.1-1 to Table 6.2A.3.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2A.3.3.4.3.

6.2A.3.3.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 and Annex C.3.0 for all downlink physical channels

2.	The SS shall configure SCC as per TS 38.508-1 [10] subclause 5.5.1 Procedure to configure SCC(s) for NR RF CA testing. Message contents are defined in subclause 6.2A.3.3.4.3.

3.	Apply the test step based on the 5G NR UE Release:

3a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF).  The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using TOTAL NR AGGREGATED BANDWIDTH and PCELL NR bandwidth as per Test CC Combination setting. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 4.

3b.	For Release 15 5G NR UEs: No action.

3c.	For testing single CC A-MPR requirement: No action.

4.  SS activates SCC by sending the activation MAC CE (Refer TS 38.321, clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

5.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2A.3.3.4.1-1 to Table 6.2A.3.3.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

6.	Set the UE in the Tx beam peak direction found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7.	Apply the test step based on the 5G NR UE Release:

7a.	For Release 16 and forward 5G NR UEs: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

7b.	For Release 15 5G NR UEs: Send uplink power control commands in uplink scheduling information to the UE per UL CC until the Power Headroom Report (PHR) from the UE for each UL CC is at the target value according to Table 6.2A.3.3.4.2-1; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

Table 6.2A.3.3.4.2-1: Power target values per UL CC for test procedure using PHR

FFS

7c.	For testing single CC A-MPR requirement: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200 msec starting from the first TPC command in this step to ensure that the UE transmits at its maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

8.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

9.	Measure UE EIRP in the Tx beam peak direction in the accumulative aggregated channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2A.3.3.5. EIRP test procedure is defined in Annex K.1.3. The measuring duration is one active uplink subframe. EIRP is calculated considering both polarizations, theta and phi.

10.	Apply the test step based on the 5G NR UE Release:

10a.	For Release 16 and forward 5G NR UEs supporting the UPLF test mode: SS deactivates the UE Power Limit Function (UPLF) by performing the DEACTIVATE POWER LIMIT REQUEST procedure as specified in TS 38.508-1 [10] clause 4.9.33.

10b.	For Release 15 5G NR UEs: No action.

10c.	For testing single CC A-MPR requirement: No action.

11.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 2:	When switching to DFT-s-OFDM waveform, as specified in Table 6.2A.3.3.4.1-1 to Table 6.2A.3.3.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [10] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6 with the following exceptions for Release 15 5G NR UE.

Table 6.2A.3.3.4.3-1: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2A.3.3.4.3-2: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -4 |  | 50 MHz |
| p0-NominalWithGrant | -8 |  | 100 MHz |
| p0-NominalWithGrant | -10 |  | 200 MHz |
| p0-NominalWithGrant | -14 |  | 400 MHz |
| } |  |  |  |

Table 6.2A.3.3.4.3-3: BSR-Config (Rel-15 UE only)

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BSR-Config ::= SEQUENCE { |  |  |  |
| periodicBSR-Timer | infinity |  |  |
| retxBSR-Timer | sf80 |  |  |
| logicalChannelSR-DelayTimer | Not present |  |  |
| } |  |  |  |

6.2A.3.3.4.3.1	Message contents exceptions (network signalling value " CA_NS_202" on PCC and SCC)

Table 6.2A.3.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_202"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_202) |  | band n257 |
|  | 2 (CA_NS_202) |  | band 258 |

6.2A.3.3.4.3.2	Message contents exceptions (network signalling value " CA_NS_203" on PCC and SCC)

Table 6.2A.3.3.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_203"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (CA_NS_203) |  | band n258 |

6.2A.3.3.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.3.5-1 to Table 6.2A.3.3.5-6.

Table 6.2A.3.3.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.3.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: 800MHz <= Cumulative aggregated BW < 1400MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MPR) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

##### 6.5A.3.3.1	Additional spurious emissions for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258	.

-	Test procedure only includes the testing of smartphone and is FFS for laptop.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5A.3.3.1.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 2UL CA.

6.5A.3.3.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.1.4	Test description

6.5A.3.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.5A.3.3.1.4.1-1 and Table 6.5A.3.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.3.1.4.1-1: Test Configuration Table for CA_NS_202 (Power Class 1)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range (NOTE 2) |  |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | CC | Downlink Configuration |  | UL Modulation | UL RB allocation (NOTE 1) |
| 1 (NOTE 4) | PCC | - |  | DFT-s-OFDM QPSK | Outer_Full |
|  | SCCs |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 2 (NOTE 4) | PCC |  |  | DFT-s-OFDM 64QAM | Outer_Full |
|  | SCCs |  |  | DFT-s-OFDM 64QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-2 for PC1. NOTE 2: When testing Low range test only in Frequency Range lower than (FUL_low – ΔfOOB) and when testing High range test only in Frequency Range higher than (FUL_high + ΔfOOB). NOTE 3: Void NOTE 4: This Test ID applies only to PC1. NOTE 5: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |

Table 6.5A.3.3.1.4.1-1b: Test Configuration Table for CA_NS_202 (Power Class 2, 3, 4, 5)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range, High range |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |
| Test ID | CC | Downlink Configuration | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 |
|  | SCCs |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |

Table 6.5A.3.3.1.4.1-2: Test Configuration Table for CA_NS_203 (Power Class 1, 2, 3, 4, 5)

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for different CA bandwidth classes |  |  | Low range |  |  |
| Test CC combination setting as specified in TS 38.508-1 [10] subclause 4.3.1.2.3 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  | Maximum aggregated BW (contiguous CA) with cumulative aggregated BW <= 400MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | 120kHz |  |  |
| Test Parameters |  |  |  |  |  |
| Test ID | CC | Downlink Configuration |  | UL Modulation | UL RB allocation (NOTE 1) |
| 1 | PCC | - |  | DFT-s-OFDM QPSK | Inner_Full for PC2, PC3 PC4, PC5 Inner_Full_Region1 for PC1 |
|  | SCCs |  |  | - | - |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3, PC4, PC5 or Table 6.1-2 for PC1. NOTE 2: Number of DL CCs shall be configured the same as number of UL CCs. The requirements are appliable as per 5.3A.4: "The requirements are applicable only when Uplink CCs are configured within the frequency range between lower edge of lowest downlink component carrier and upper edge of highest downlink component carrier". |  |  |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.3 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.3.1.4.1-1 and Table 6.5A.3.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.5A.3.3.1.4.3.

6.5A.3.3.1.4.2	Test procedure

1.	Select any of the three Alignment Options (1, 2, or 3) from Tables N.2-1 through N.2-3 to mount the DUT inside the QZ.

2. 	If the re-positioning concept is applied, position the device in DUT Orientation 1 if the maximum beam peak direction is within zenith angular range 0o≤≤90o for the alignment option selected in step 1; position the device in DUT Orientation 2 (either Options 1 or 2) if the maximum beam peak direction is within zenith angular range 90o<≤180o for DUT Orientation 1 for the alignment option selected in step 1. If the re-positioning concept is not applied, position the device in DUT Orientation 1.

3.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

4.	The SS shall configure SCC as per TS 38.508-1 [10] clause 5.5.1. Message contents are defined in clause 6.5A.3.3.1.4.3.

5.	Apply the test step based on the 5G NR UE Release:

5a.	For Release 16 and forward 5G NR UEs: SS applies a backoff on the PCell power by activating the UE Power Limit Function (UPLF). The ACTIVATE POWER LIMIT REQUEST procedure is performed as specified in TS 38.508-1 [10] clause 4.9.32 using TOTAL NR AGGREGATED BANDWIDTH and PCELL NR bandwidth as per Test CC Combination setting. UE shall transmit ACTIVATE POWER LIMIT RESPONSE to SS. Go to step 6.

5b.	For Release 15 5G NR UEs: No action.

6.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [28], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[25], clause 9.3).

7.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5A.3.3.1.4.1-1 or Table 6.5A.3.3.1.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

8.	Set the UE in the Inband Tx beam peak direction [(same as that found for single carrier in clause 6.5.3)] found with a 3D EIRP scan as performed in Annex K.1.1. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 3) for the UE Tx beam selection to complete.

9.	Apply the test step based on the 5G NR UE Release:

9a.	For Release 16 and forward 5G NR UEs: Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200msec for the UE to reach PUMAX. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 3) for the UE Tx beam selection to complete.

9b For Release 15 5G NR UEs: Send uplink power control commands in uplink scheduling information to the UE per UL CC until the Power Headroom Report (PHR) from the UE for each UL CC is at the target value according to Table 6.2A.2.1.4.2-1; allow at least 200 ms for the UE to reach maximum output power. Allow at least BEAM_SELECT_WAIT_TIME (NOTE 1) for the UE Tx beam selection to complete.

10.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Tx only.

11.	Measure the spurious emissions as per steps outlined below with an exception to the procedure in Annex K if the re-positioning concept is applied (NOTE 4). During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

(a)	Perform coarse TRP measurements to identify spurious emission frequencies and corresponding power level according to the procedures in Annex L, using coarse TRP measurement grid selection criteria as per Tables 6.5.3.1.4.2-1 through 6.5.3.1.4.2-3. The measurement is completed in both polarizations  and  over frequency range and measurement bandwidth according to Table 6.5A.3.3.1.5-2. Optionally, a larger and non-constant measurement bandwidth than that of Table 6.5A.3.3.1.5-2 may be applied as long as the SNR (ratio of test limit to floor noise of test equipment) ≥ 10dB is guaranteed. The measurement period shall capture the [active time slots]. For each spurious emission frequency with coarse TRP identified to be less than the offset listed in Tables 6.5.3.1.4.2-1 through 6.5.3.1.4.2-3 from the TRP limit according to Table 6.5A.3.3.1.5-2, either continue with another coarse TRP procedure and corresponding offset according to step (a) or continue with fine TRP procedures according to step (b).

Different coarse TRP grids and corresponding offset values may be used for different frequencies. Multiple coarse TRP grids measurements with the corresponding offset values can be performed before the fine TRP measurement grid is applied. The coarse TRP grids and offset values used shall be recorded in the test report.

(b)	Measure fine TRP measurements according to procedures in Annex K, using fine TRP measurement grid selection criteria as per Table M.4.5-3 in Annex M, for each of the spurious emission frequency identified in step (a). Apply a measurement bandwidth according to Table 6.5A.3.3.1.5-2.

12.	Apply the test step based on the 5G NR UE Release:

12a.	For Release 16 and forward 5G NR UEs: SS deactivates the UE Power Limit Function (UPLF) by performing the DEACTIVATE POWER LIMIT REQUEST procedure as specified in TS 38.508-1 [10] clause 4.9.33.

12b.	For Release 15 5G NR UEs: No action.

13.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

NOTE 1: 	The frequency range defined in Table 6.5A.3.3.1.5-2 may be split into ranges. For each range a different test system, e.g. antenna and/or chamber, may be used. To pass the test case all verdicts of the frequency ranges must pass.

NOTE 2: 	Void.

NOTE 3:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.

NOTE 4:	If the (in-band) beam peak is within 0o≤≤90o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 1 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 2. If the (in-band) beam peak is within 90o<≤180o: perform first hemispherical TRP scan (0o≤≤90o) in DUT Orientation 2 and second hemispherical TRP scan (90o>≥0o) in DUT Orientation 1. The DUT with UBF activated needs to be re-positioned during the test.

6.5A.3.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.1 with the following exceptions for Release 15 5G NR UE.

Table 6.5A.3.3.1.4.3-1: PUSCH-PowerControl

| Derivation Path: TS 38.508-1 [10], Table 4.6.3-120 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-PowerControl ::= SEQUENCE { |  |  |  |
| p0-AlphaSets SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF SEQUENCE { | 1 entry |  |  |
| P0-PUSCH-AlphaSet[1] SEQUENCE { |  |  |  |
| alpha | alpha0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5A.3.3.1.4.3-2: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[10], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -4 |  | 50 MHz |
| p0-NominalWithGrant | -8 |  | 100 MHz |
| p0-NominalWithGrant | -10 |  | 200 MHz |
| p0-NominalWithGrant | -14 |  | 400 MHz |
| } |  |  |  |

6.5A.3.3.1.4.3.1	Message contents exceptions (network signalling value " CA_NS_202" on PCC and SCC)

Table 6.5A.3.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_202"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_202) |  | band n257 |
|  | 2 (CA_NS_202) |  | band n258 |

6.5A.3.3.1.4.3.2	Message contents exceptions (network signalling value " CA_NS_203" on PCC and SCC)

Table 6.5A.3.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_203"

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (CA_NS_203) |  | band n258 |

6.5A.3.3.1.5	Test requirement

This clause specifies the requirements for the specified NR band for Transmitter Spurious emissions for UE co-existence requirement with frequency range as indicated in Table 6.5A.3.3.1.5-2.

The maximum TRP power of spurious emission for UE co-existence, measured using RMS detector, shall not exceed the described value in the applicable table from Table 6.5A.3.3.1.5-2 to Table 6.5A.3.3.1.5-3.

The additional spurious emission for CA limits in Table 6.5A.3.3.1.5-2 to Table 6.5A.3.3.1.5-3 apply for all transmitter band configurations (NRB) and channel bandwidths.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5A.3.3.1.5-1: Void

Table 6.5A.3.3.1.5-2: Additional spurious emissions for CA (CA_NS_202) test requirements

| Frequency Range | Maximum Level (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 7.25 GHz ≤ f ≤ 12.75 GHz | -10 | 100 MHz |  |
| 12.75 GHz ≤ f ≤ 23.45 GHz | -10 + 13 | 100 MHz | NOTE 1 |
| 23.45 GHz ≤ f ≤ 40.8 GHz | -10 + 13 | 100 MHz | NOTE 1 |
| 40.8 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 + 13 | 100 MHz | NOTE 1 |
| 23.6 GHz ≤ f ≤ 24.0 GHz | +1 +0.3 | 200 MHz | NOTE 2 |
| NOTE 1: 13 dB relaxation due to testability limit. NOTE 2: 0.3 dB relaxation due to testability limit. |  |  |  |

Table 6.5A.3.3.1.5-3: Additional spurious emissions for CA (CA_NS_203) test limits

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 23.6 ≤ f ≤ 24.0 | +1 + 0.3 | 200 MHz | NOTE 1 |
| NOTE 1: 0.3 dB relaxation due to testability limit. |  |  |  |

##### 6.5A.3.3.2	Additional spurious emissions for CA (3UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5A.3.3.2.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 3UL CA and PC1.

6.5A.3.3.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.2.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.2.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5

##### 6.5A.3.3.3	Additional spurious emissions for CA (4UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	This test case is incomplete for Power classes other than 1, 3, 5 and CA other than intra-band contiguous.

-	Measurement Uncertainties and Test Tolerances are FFS for power class other than 1, 3, 5 and CA other than intra-band contiguous.

-	The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258.

-	Test procedure only includes the testing of smartphone and is FFS for laptop.

-	For a transition period until RAN#99, the stability and repeatability of test procedure with PHR (variant b) for Rel-15 UEs is under evaluation.

6.5A.3.3.3.1	Test purpose

Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

6.5A.3.3.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports FR2 4UL CA and PC1.

6.5A.3.3.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.5A.3.3.0.

6.5A.3.3.3.4	Test description

Same test description as in clause 6.5A.3.3.1.4.

6.5A.3.3.3.5	Test requirement

The test requirement is the same as in clause 6.5A.3.3.1.5.

## <<< END OF CHANGES >>>