3GPP TSG-RAN5 Meeting #106	R5-250309

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3619 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR FR1 less than 5MHz BW: Initial Test Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To implement NR band n106 with CBW = 3MHz, the NR test model needs to be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a clarification for CORESET#0 puncturing in clause 7.1.2.1.2. Added a clarification for the resource allocation for frequency domain in clause 7.1.2.2.3.1. Added a clarification for the UL grant size in clause 7.1.2.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to implement NR band n106. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.1.2, 7.1.2.2.3.1, 7.1.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.1.2.1.2	PDCCH search spaces

For each configured DL BWP, the TTCN may configure one or several search spaces.

For each search space, TTCN configures the SS with:

-	The configuration of this search space as given to the UE, enabling the SS to determine the PDCCH transmission occasions (same as the UE PDCCH monitoring occasions) and associated CORESET.
For SearchSpaceZero the configuration according to TS 38.213 [21] clauses 10.1 and 13 is mapped to ASN.1 type SearchSpace to configure the SS; for CORESET#0 the configuration according to TS 38.211 [19] clause 7.3.2.2 is mapped to ASN.1 type ControlResourceSet to configure the SS.

-	The CCE aggregation level L that the SS shall use for PDCCH transmission on this search space.
In general an aggregation level of 4 is used for SearchSpaceZero and common search space, aggregation level of 2 is used for UE-specific search space. When puncturing on CORESET#0 is applied, the CCEs are obtained prior to puncturing of corresponding RBs, according to TS 38.211 [19] clause 7.4.3.2.2.

-	The priority P of this search space that the SS shall consider in its PDCCH candidate selection algorithm.

-	a value of 0 represents the highest priority, a value of 1 the second highest priority and so on.

NOTE:	“Search space” is used in terms of TS 38.213 [21] clause 10.1 and a single search space configuration (ASN.1 type ‘SearchSpace’) may contain several search spaces (see NR_BWP_SearchSpaceConfig_Type in Annex D).

<TEXT SKIPPED>

###### 7.1.2.2.3.1	DL scheduling scheme: Frequency domain multiplexing, RA type1, non-interleaved

This scheduling scheme multiplexes the different kinds of PDSCH transmissions in the frequency domain of a single BWP by exclusively using resource allocation type 1 with non-interleaved VRB-to-PRB mapping. Assuming the resource blocks being numbered from 0 to NBWP-1 (with NBWP being the size of the BWP) the following allocation is done:

Table 7.1.2.2.3.1-1: Resource allocation for frequency domain multiplexing, RA type1, non-interleaved

| Kind of PDSCH transmission (Note 1) | CORESET# | SSB index of the cell (Note 4) | Resource block allocation (Note 2) |  |
| --- | --- | --- | --- | --- |
|  |  |  | RBstart (Note 2) | LRBs,max (Note 2) |
| System information | 0 | SSB#1 | 0 | NSI |
|  |  | SSB#0 | NBWP-NSI | 7NSI |
| Paging (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| Random access response (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| CCCH and DCCH transmission on CSS (Note 3) | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| DCCH/DTCH transmissions (Note 3) | 1 | SSB#1 | NSI+ Offset Carrier CORESET#0 [RBs] (Note 5) | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| MCCH/MTCH | 0 | SSB#1 | NSI | NBWP-NSI |
|  |  | SSB#0 | 0 | NBWP-NSI |
| NOTE 1: In context of a generic 5G test model it is not relevant whether or not there is SI and Paging for a given deployment option (e.g. EN-DC). NOTE 2: LRBs ≤ LRBs,max with LRBs: number of resource blocks being eventually used for a particular transmission. LRBs and RBstart are as specified in TS 38.214 [22] clause 5.1.2.2.2. NOTE 3: In general Paging, Random access response and CCCH/DCCH/DTCH transmissions are mutual exclusive and therefore share the same allocation. NOTE 4: The SSB index used by an NR Cell is specified in TS 38.508-1 [5] Table 4.4.2-2. NOTE 5: Offset Carrier CORESET#0 [RBs] values are specified in the frequency tables in TS 38.508-1 [5]. |  |  |  |  |

In order to achieve a test case behaviour being independent from the frequency channel bandwidth NBWP is limited to the minimum value of 24 RBs, except for channel bandwidth 3 MHz, NBWP is limited to the minimum value of 15 RBs in accordance with Table 5.3.2-1 of TS 38.101-1/2 [5, 6]. This implies that LRBs,max< 24 RBs for all configurations, except for channel bandwidth 3 MHz LRBs,max< 15 RBs.

NSI is the number of RBs reserved for system information, it is set to 7 RBs by default. NSI is set to 9 RBs in test cases requiring an extended SIB1 TBS size.

In general PDCCH and corresponding PDSCH transmissions are in the same slot (K0 = 0).

<TEXT SKIPPED>

##### 7.1.2.3.3	Default grants

In general test cases for layer 3 and above do not need test case specific grants but use default grants as listed in table 7.1.2.3.3-1.

Table 7.1.2.3.3-1: Default grants

| Purpose | Grant size (NOTE 1) or {LRBs IMCS} pair | Comment |
| --- | --- | --- |
| Scheduling of RA Msg3: for initial access | 256 bits (NOTE 2) | grant provided by RAR (RA Msg2) |
| Scheduling of RA Msg3: for handover, EN-DC or RRC re-establishment procedure | 48 bits (NOTE 2, 3) | grant provided by RAR (RA Msg2) |
| RA Msg4 for C-RNTI based CBRA: for synchronisation (e.g. EN-DC) | LRBs = 1, IMCS = 0 | minimum grant (24 or 32 bits) |
| RA Msg4 for C-RNTI based CBRA: for handover or RRC re-establishment procedure | 144 bits (NOTE 4) |  |
| Default USS grant | LRBs = 24, LRBs = 15 for CBW=3 MHz, IMCS = 9 (NOTE 5) | default grant: e.g. for test cases with main focus on control plane signalling |
| NOTE 1: In general for a given grant size TTCN uses the {LRBs IMCS} pair according to annex B.2. NOTE 2: According to TS 38.213 [21] clause 8.3, the RAR UL grant is always interpreted according to DCI format 0_0, so contents of Table 7.1.2.3.2.2-1 apply. NOTE 3: Minimum grant which can be assigned by the RAR: The grant is sufficient to convey C-RNTI (3 bytes) and short BSR (2 bytes) or long BSR (minimum of 3 bytes). Even with short BSR there is not enough space to convey any segment of the RRCReconfigurationComplete or RRCReestablishmentComplete (at least 5 bytes: 2 bytes MAC header + 2 bytes RLC header + 1 byte payload). NOTE 4: Initial grant of RA procedure big enough to completely convey the RRCReconfigurationComplete (10 bits) or RRCReestablishmentComplete (<10 bits). This requires a minimum of 12 bytes (2 bytes MAC header + 2 bytes RLC header + 6 bytes PDCP header + 2 bytes payload). Additionally, an optional PHR MAC element (3 bytes) needs to be considered since the PHR has higher priority than the MAC SDU. Any further user data would require a minimum of 6 additional bytes for a non-RedCap UE (2 bytes MAC header + 3 bytes RLC header + 1 byte payload, it is assumed that if UE has any data to transmit that will be on AM DRB) or 5 additional bytes for a RedCap UE (2 bytes MAC header + 2 bytes RLC header + 1 byte payload, it is assumed that if UE has any data to transmit that will be on AM DRB). NOTE 5: Applicable only when MCS index table 5.1.3.1-1 or 6.1.4.1-1 is configured to be used for UL grants (e.g. in case of mcs-Table = qam256, a different IMCS value needs to be defined). |  |  |