3GPP TSG-RAN5 Meeting #106	R5-250327

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4825 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 8.1.3.1.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_QoE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. From steps 1 to 15, nothing is tested, there is no TP. It is establishing a video call and releasing it. There is no requirement for TP1 to do so. It is proposed to delete these steps. 2. The UM DRBs for video are not configured until step 10 of the generic procedure for IMS video call establishment, specified in table 4.9.26.2-2-1 of TS 38.508-1. Step 22 state that 38.508-1 Table 4.8.1-1CAA specifies the procedure, but this table simply contains the definition of RRCReconfiguration-SRB2-SRB4-DRB(n, m). 3.The wording of TP1 is ambiguous because as per TS 38.331 clause 5.7.16.2 UE may initiate application layer measurement reporting “when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network”. But TP1 does not mention configuration of a corresponding SRB. Moreover, there are no means for the SS to check whether an application measurement report container is received by the UE. It is proposed to rephrase the conditioning “when { UE receives an application layer measurement report container from upper layers}” with when {video call is ongoing} and to mention configured SRB4 in preconditions to ensure unambiguous testing terms. 4. In step 10 in tables 4.9.26.2.2-1 of TS 38.508-1, when the IMS video DRBs are established, the logical channel identity is set as per default with DRBid+3. But in this scenario SRB4 is configured, hence the logical channel id shall be set to DRBid+4. 5. In step 30, the procedure from TS 38.508-1 shall be used instead of TS 34.229-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Voided steps 1 to 15. 2. Corrected text of step 22. 3. Updated wording for TP2. 4. Added RRCReconfiguration message content for step 25. 5. Updated step 30 to use the test procedure TS 38.508-1 cl.4.9.18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1, 2 Test does not implement real network behaviour. 3. TP1 definition will remain ambiguous. 4. Message content will remain incorrect. 5. Test case prose will remain inconsistent |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.3.1.25	Measurement configuration control and reporting for video / QoE

8.1.3.1.25.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state configured with application layer measurement and reporting and with SRB4 }

ensure that {

when { video call is ongoing}

then { UE sends a MeasurementReportAppLayer message }

}

8.1.3.1.25.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clause 5.7.16.2.

[TS 38.331 clause 5.7.16.2]

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement and reporting, i.e. when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network.

Upon initiating the procedure, the UE shall:

…

1>	for each measConfigAppLayerId received from upper layers:

2>	if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and

…

2>	set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received together with application layer measurement report information;

…

1>	for each encoded MeasurementReportAppLayer message generated above which contains either only encapsulated application layer measurement reports or both encapsulated application layer measurement reports and RAN visible application layer measurement reports:

2>	if reportingSRB is not configured for the measConfigAppLayerId:

…

3>	else:

4>	submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4.

8.1.3.1.25.3	Test description

8.1.3.1.25.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in state RRC_IDLE using generic procedure parameter Connectivity (NR) according to TS 38.508-1 [4], Table 4.5.1-1.

8.1.3.1.25.3.2	Test procedure sequence

Table 8.1.3.1.25.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-15 | Void | - | - | - | - |
| 16-21 | Steps 1-6 of test procedure specified in Table 4.9.26.2.2-1 of TS 38.508-1 [4] are performed. | - | - | - | - |
| 22 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2, SRB4 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-SRB4-DRB(n, m) where n and m are the number of DRB(s) configured during the preamble. The RRCReconfiguration message is specified in Table 4.8.1-1CAA of TS 38.508-1 [4]. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 23 | The UE transmits a RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 24-28 | Steps 9-13 of generic procedure specified in Table 4.9.26.2.2-1 of TS 38.508-1 [4] are performed. The RRCReconfiguration message in step 10 is configured as per Table 8.1.3.1.25.3.3-1. | - | - | - | - |
| 29 | Check: Does the UE transmit a MeasurementReportAppLayer message? | --> | NR RRC: MeasurementReportAppLayer | 1 | P |
| 30 | Test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. | - | - | - | - |
| 31 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

8.1.3.1.25.3.3	Specific message contents

Table 8.1.3.1.25.3.3-1: RRCReconfiguration (step 25, Table 8.1.3.1.25.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-Video | TS 38.508-1[4] Table 4.8.1-6 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.25.3.3-2: CellGroupConfig-DRB (Table 8.1.3.1.25.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfigWithSRB4 with conditions UM and DRBj (with j=BID+k) | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfigWithSRB4 with conditions UM and DRBj (with j=BID+k) | entry 2 |  |
| } |  |  |  |
| } |  |  |  |