3GPP TSG-RAN5 Meeting #106	R5-250342

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1565 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of annex A.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Conditions A5 and A6 mix up a PICS (pc_IMS_SRVCCAlert) with dynamic test case behaviour ("180 sent by the UE/SS for a voice or video call"). Nevertheless, A5 and A6 are never specified for any specific message content of a test procedure or a test case and in TTCN only the direction and the PICS are considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A.2.6 aligned with TTCN implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.2.6	180 Ringing for INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 180 |  |  |
| Reason-Phrase |  | Ringing |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route |  | as defined for the common 183 response, see A.2.3 |  |  |
|  | A7 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |
|  | A8 | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in INVITE message |  |  |
| Require |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag | A3 | 100rel |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | same value as received in INVITE message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in INVITE message |  |  |
| tag |  | as defined for the common 183 response, see A.2.3 |  |  |
| P-Asserted-Identity | A4 |  |  | RFC 3325 [89] |
| addr-spec |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |
| uri-parameter |  | lr |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec |  | as defined for the common 183 response, see A.2.3 |  |  |
| feature-param | A2 AND A5 | +g.3gpp.srvcc-alerting |  |  |
|  | A1 | audio |  |  |
|  | A2 AND A9 AND (A10 OR A11) | audio |  |  |
| RSeq |  |  |  | RFC 3262 [33] |
| response-num | A3 AND NOT A12 | previous RSeq number sent in the same direction incremented by one |  |  |
|  | A1 AND A12 | 122 |  |  |
|  | A2 AND A12 | any value |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in INVITE message |  |  |
| P-Access-Network-Info | A2 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A13 | access network information and, if applicable, the cell ID |  |  |
|  | A14 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| P-Access-Network-Info | A1 | not present |  |  |
| Feature-Caps |  |  |  |  |
| feature-param | A1 AND A5 | +g.3gpp.srvcc-alerting |  |  |
| Content-Length | A1 |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | 180 sent by the SS |
| A2 | 180 sent by the UE |
| A3 | Response sent reliably (e.g. always when it contains an SDP body) |
| A4 | 180 sent by the SS when setting up an emergency call or a non-UE detectable emergency call |
| A5 | UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |
| A6 | Void |
| A7 | Response sent by SS for emergency call without emergency registration |
| A8 | Response sent by SS for emergency call with emergency registration or a non-UE detectable emergency call |
| A9 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A10 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A. |
| A11 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23. |
| A12 | 180 Ringing is first provisional response sent reliably in this dialog |
| A13 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A14 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |