3GPP TSG-RAN5 Meeting #106	R5-250347

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1570 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of eCall test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - R5-250346 adds clarifications for conditions of messages used in C.47 of 34.229-1: For the 200 OK for INVITE per default condition A13 shall be used in addition to condition A12. - Test cases 21.1 and 21.2 refer to C.33 for call release, whereas test cases 21.4, 21.5 and 21.6 explicitly use BYE and 200 OK for BYE. - R5-250343 adds a new condition A9 to 34.229-1 A.2.8 to specify correctly the MT BYE in case emergency call (eCall) release. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specific message contents of eCall test cases aligned with C.47. - C.33 used in test cases 21.4, 21.5 and 21.6 too. - Condition A9 added to specific message contents for MT BYE. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.1.4, 21.2.4, 21.4.4, 21.5.4, 21.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1569, 1566 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 21.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application with eCall subscription on UICC. The UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate a manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

BYE (Step 9, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

<End of modification>

<Start of modification>

### 21.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. The UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends 200 OK with ACK.

5)	Void

6)-9)	MT Call release according to steps 1-4 of procedure C.33.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | UE is triggered to initiate an automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-8 |  |  | Steps 1-3 defined in C.47 | eCall setup |
| 9-12 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

BYE (Step 9, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

<End of modification>

<Start of modification>

### 21.4.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Manual eCall initiated at UE.

2)	Emergency registration according to C.20 is executed

3)	SS waits for UE to send an INVITE request.

4)	SS sends with 200 OK.

5)	SS waits for ACK.

6)	SS sends INFO request to transfer MSD.

7)	SS expects and receives 200 OK.

8)	SS expects and receives INFO request from the UE with MSD contents.

9)	SS responds with 200 OK for INFO request from UE.

10)	SS sends BYE to the UE.

11)	SS expects and receives 200 OK for BYE from the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt manual eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-12 |  |  | Steps 1-7 defined in C.47 | eCall setup |
| 13-16 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A20.

BYE (Step 13, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

<End of modification>

<Start of modification>

### 21.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Automatic eCall over IMS initiated at UE

2-5)	UE executes the procedures described in TS 36.508 [94] table 4.5A.26.3-1 steps 2 to 15 and parallel behaviour steps 1-4 for EPS emergency bearer context activation, and subsequent IMS emergency registration,

6-12)	eCall is established successfully and MSD transferred successfully using INFO procedure as defined in C.47

13)	SS sends BYE to the UE

14)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE to initiate automatic eCall |  |
| 2-5 |  |  | Steps 1-4 defined in C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-12 |  |  | Steps 1-7 defined in C.47 | eCall Setup and MSD transfer using INFO message |
| 13-16 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

BYE (Step 13, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

<End of modification>

<Start of modification>

### 21.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. UE has discovered P-CSCF, and registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1) 	Automatic eCall over IMS initiated at UE

2-5)	UE executes the procedures described in TS 36.508 [94] table 4.5A.26.3-1 steps 2 to 15 and parallel behaviour steps 1-4 for EPS emergency bearer context activation, and subsequent IMS emergency registration,

6-7)	eCall is established successfully

8-11)	Execute MSD transfer procedure as defined in Annex C.47 from steps 4-7

12)	SS sends BYE to the UE

13)	SS expects and receives 200 OK for BYE from the UE

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt Automatic eCall |  |
| 2-5 |  |  | Step 1-4 defined in C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-7A |  |  | Steps 1-3 defined in C.47 | eCall over IMS initiated automatically. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 8-11 |  |  | Steps 4-7 defined in C.47 | eCall over IMS initiated automatically. Referred from 36.508 [94] table 4.5A.26 for a UE with E-UTRA support. |
| 12-15 |  |  | Steps 1-4 defined in C.33 | The SS releases the call |

NOTE:	The default messages contents in annex A are used with condition "IMS security" or "GIBA" when applicable

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

INFO (Step 8, step 4 of C.47)

Use the default message “MT INFO” in annex A.2.20 with the datatype attribute set to ‘eCall.invalidMSD’ in the message body.

INFO (Step 10, step 6 of C.47)

Use the default message “MO INFO” in annex A.2.19 with condition A4 instead of A3.

BYE (Step 12, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

<End of modification>