3GPP TSG-RAN5 Meeting #106	R5-250348

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1571 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of emergency test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Procedure C.22 has not fully specified the conditions for emergency calls with and without registration and the conditions have not been consistently specified in the specific message contents of test cases. R5-250345 changes C.22 so that the case of emergency call with registration is fully specified. - Especially the specific message contents for the INVITE have been referring to A.2.1 instead of C.22 (Step 1) and therefore the SDP body has been left unspecified. - In case of emergency calls without registration the BYE at call release (C.32) needs condition A6. - In several emergency test cases C.32 has not been used for call release and the MMI has been missing. - In case of non-UE detectable emergency call the 200 OK for UPDATE needs condition A6. - Editorial issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Conditions and specific message contents aligned to C.22. - References in specific message contents for emergency call establishment changed to the respective steps of C.22. - Condition A6 added to specific message contents of BYE for emergency calls without registration. - C.32 used for call release (where not used yet). - Specific message content for 200 OK for UPDATE with condition A6 added to test case 19.3.1. - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.1.1.4, 19.1.2.4, 19.1.5.4, 19.3.3.4, 19.3.4.4, 19.4.1.4, 19.4.2.4, 19.4.5.4, 19.4.5.4, 19.4.7.4, 19.5.7.4, 19.5.9.4, G.19.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-1 CR 1568 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

#### 19.1.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE uses Geolocation header for providing its geographical location. This shall be done by use of the test function Update UE Location Information defined in TS 34.109 [117] or in TS 36.509 [118] depending on the RAT being used in the test case, if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-15)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation, IMS emergency registration and subsequent IMS emergency speech call.

16)	Call is released on the UE using C.32 procedure.

17)	 Void

18)	Emergency Bearer context is deactivated

Expected sequence:

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-15 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 16-16A4 |  |  | Steps defined in annex C.32 | The UE releases the call |
| 17 |  |  | Void |  |
| 18 |  |  | EPS emergency bearer context deactivation by the SS. | EPS Bearer Deactivation procedure according TS 36.508 [94] subclause 4.5A.15, applied to the identity of the Default EPS Bearer of the emergency PDN. |

Specific Message Contents

INVITE (step 1 of Annex C.22)

Use INVITE as specified for step 1 in annex C.22 with condition A8 in addition.

<End of modification>

<Start of modification>

#### 19.1.2.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test environment shall ensure that UE can not access any information (such as GPS signal) from which the UE would be able to derive its geographical location. The UE shall only be able to read the global cell ID as provided by the SS.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-15)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation, IMS emergency registration and subsequent IMS emergency speech call.

16)	Call is released on the UE using Annex C.32 procedure

17)	Void

18)	Emergency Bearer context is deactivated

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 16-16A4 |  |  | Steps defined in Annex C.32 | The UE releases the call |
| 17 |  |  | Void |  |
| 18 |  |  | EPS emergency bearer context deactivation by the SS. | EPS Bearer Deactivation procedure according TS 36.508 [94] subclause 4.5A.15, applied to the identity of the Default EPS Bearer of the emergency PDN. |

Specific Message Contents

INVITE (Step 1 of Annex C.22)

Use INVITE as specified for C.22 step 1; condition A8 shall not be applied.

<End of modification>

<Start of modification>

#### 19.1.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step and thereafter executing the generic test procedure in Annex C.21 up to its last step for a multimedia non-emergency call.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Ongoing multimedia call is put on hold.

2)	IMS emergency call is initiated on the UE.

3)-15)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation, IMS emergency registration and subsequent IMS emergency speech call establishment with PSAP.

15A	User initiates resumption of Multimedia call.

16-17)	UE releases the emergency call.

18)	Void.

19-24)	Multimedia call is resumed and released.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | User initiates hold of ongoing call |  |
| 2-5 |  |  | Steps defined in annex C.8 | Ongoing call is put on hold by UE |
| 6 |  |  | User initiates an emergency call |  |
| 7-15 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 15A |  |  | User resumes the ongoing call which is on hold | Triggers release of the emergency call. |
| 16-17 |  |  | The UE releases the emergency call using steps 2-5 of Annex C.32 |  |
| 18 |  |  | Void |  |
| 19-22 |  |  | Steps defined in Annex C.8 | Ongoing call is resumed. |
| 23-27 |  |  | Steps defined in Annex C.32 | The UE releases the call |

Specific Message Contents

None.

<End of modification>

<Start of modification>

#### 19.1.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the NW is VPLMN and the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)-12) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 and parallel behaviour steps 1 for EPS emergency bearer context activation,

13) SS waits for the UE to send an initial REGISTER request containing “sos” SIP URI parameter in the Contact header field.

14) The SS responds to the REGISTER request with a 420 Bad Extension response,

15) The UE initiates IMS registration indicating support of GIBA. SS waits for the UE to send an initial REGISTER request.

16) The SS responds to the REGISTER request with valid 200 OK response,

17) The SS waits for the UE to send an INVITE request.

18)-20A) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 and parallel behaviour Steps 2-5 defined in annex C.22 of TS 34.229-1 for IMS Emergency call for EPS is established,

21)-24A) The UE releases the call.

25)-26) The SS deactivates EPS emergency bearer context.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-12 |  |  | Steps defined in TS 36.508 [94] table 4.5A.4.3-1 | EPS Bearer Activation procedure and IP address allocation according TS 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 13 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 14 |  |  | 420 Bad Extension | The SS responds with a failure. |
| 15 |  |  | REGISTER | The UE sends initial registration for IMS services indicating support for GIBA procedure by not including an Authorization header field. |
| 16 |  |  | 200 OK | The SS responds with 200 OK. |
| 17 |  |  | INVITE | UE sends INVITE request. |
| 18-20A |  |  | Steps 2 to 5 defined in annex C.22 | IMS emergency call setup with PSAP |
| 21-24A |  |  | Steps defined in annex C.32 | The UE releases the call |
| 25-26 |  |  | EPS emergency bearer context deactivation by the SS. | EPS Bearer Deactivation procedure according TS 36.508 [94] subclause 4.5A.15, applied to the identity of the Default EPS Bearer of the emergency PDN. |

Specific Message Contents

REGISTER (Step 13)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”

420 Bad Extension for REGISTER (Step 14)

Use the default message “420 Bad Extension for REGISTER” in annex A.1.8 with condition A1 “IMS emergency registration failure for an anonymous emergency call”.

REGISTER (Step 15)

Use the default message “REGISTER” in annex A.1.1 with condition A3 "GIBA" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”.

200 OK for REGISTER (Step 16)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition A3 "Response for an emergency registration"

INVITE (Step 17)

Use INVITE as specified for step 1 in annex C.22 with condition A19 "INVITE for creating an emergency session within an emergency registration using GIBA" instead of A1 (IMS security).

SIP messages from step 18 onward

Use conditions for GIBA instead of IMS security.

<End of modification>

<Start of modification>

#### 19.3.1.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC.

Test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE is capable of obtaining location information. This shall be done by use of the test function Update UE Location Information defined in TS 34.109 [117] or in TS 36.509 [118] depending on the RAT being used in the test case, if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

- A non-emergency MO call is initiated up following the generic procedure in Annex C.21.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  |  | Steps 1-13 of Annex C.21. The UE initiates a non-emergency call. |

Specific Message Contents

183 Session Progress (Step 4)

Use the “183 Session Progress (Step 4)” of annex C.21 applying condition A5 of A.2.3 in addition.

UPDATE (Step 7)

Use the "UPDATE (Step 7)" of annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Geolocation | A1 |  | Rel-9 | RFC 6442 [98] |
| locationURI |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |
| Geolocation-Routing | A1 | “yes” | Rel-9 | RFC 6442 |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec | A2 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  |  |
| addr-spec | A3 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
| addr-spec | A4 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
| Content-Type |  |  |  |  |
| media-type | A1 not A1 | multipart/mixed application/sdp |  | TS 24.229 [10] RFC 3261 [15] |
| Message-body | A1 | The "application/pidf+xml" multipart-mime body shall contain a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more ‘geopriv’ elements (Note), each containing: - At least one ‘location-info’ element describing the location of the UE. Note: PIDF-LO shall contain at least one <geopriv> element in a <tuple> element according to RFC 4119 or in a <device> or <person> element according to RFC 4119 and RFC 5491. The "application/sdp" multipart-mime body shall contain the SDP message as specified for the "UPDATE (Step 7)" of annex C.21. |  | RFC 6442 [98] |
|  | not A1 | SDP message as specified for the "UPDATE (Step 7)" of annex C.21. |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | UE is capable of obtaining location information |
| A2 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A3 | Not A2 and (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A4 | Not A2 and (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) |

200 OK for UPDATE (Step 8)

Use the default message "200 OK" in annex A.3.1 with condition A6.

180 Ringing (Step 9)

Use the default message “180 Ringing” in annex A.2.6 with conditions A4 and A8.

200 OK for INVITE (Step 12)

Use the default message "200 OK" in annex A.3.1 with condition A6.

<End of modification>

<Start of modification>

#### 19.3.3.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE has activated EPS bearers, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

1-2)	MO call is initiated on the UE by dialling a non emergency number.

3)	SS responds to the INVITE request with a 380 Alternative Service.

4)	SS waits for the UE to send an ACK to acknowledge receipt of the 380 Alternative Service.

5-11)	SS waits for an IMS emergency registration and call setup.

12-19) Void.

20-21)	Having reached the active state, the call is cleared by the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  |  | MO call is initiated on the UE by dialling a “non emergency” number. |
| 2 |  |  | INVITE | UE sends INVITE. Request-URI of the INVITE request matches with the “non emergency” number dialled. |
| 3 |  |  | 380 Alternative Service | The SS responds with a 380 Alternative Service |
|  |  |  | EXCEPTION: Step 4 and Steps 5-13 may occur in parallel |  |
| 4 |  |  | ACK | The UE acknowledges the receipt of 380 response for INVITE. |
| 5-13 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1, steps 9-15, for a UE with E-UTRA support, see NOTE 2. |
| 14-19 |  |  |  | Void |
| 20-24 |  |  | Steps defined in Annex C.32 | The UE releases the call |

NOTE 1:	The default messages contents in annex A are used with condition "IMS security".

NOTE 2: 	After step 4, an RRC connection already exists. Therefore the UE will directly send PDN CONNECTIVITY REQUEST for emergency bearer without service request procedure. Hence, 36.508 table 4.5A.4.3-1 steps 9-15 will be executed.

Specific Message Contents

INVITE (Step 2 of Annex C.21)

Use the default message “INVITE” in annex A.2.1. for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec shall be present |

380 Alternative Service (Step 3)

Use the default message “380 Alternative Service” in annex A.4.1 with the following exception.

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Contact |  |  |  |
| addr-spec | urn:service:sos.ambulance |  |  |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <ims-3gpp version="1"> <alternative-service> <type>emergency</type> <reason/> <action>emergency-registration</action> </alternative-service> </ims-3gpp> |  |  |

ACK (Step 4)

Use the default message "ACK" in annex A.2.7

INVITE (Step 9)

Use INVITE as specified for step 1 in annex C.22 with condition A8 in addition and with the following exceptions:

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |
| Request-URI |  | urn:service:sos.ambulance |  |  |

<End of modification>

<Start of modification>

#### 19.3.4.4	Method of test

Initial conditions

UE contains ISIM and USIM applications or only USIM application on UICC. UE has activated EPS bearers, discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration.

Test procedure

1-15)	Emergency registration followed by an emergency call set-up

16-17)	The emergency call is terminated by the UE

18)	MO call is initiated on the UE by dialling a non emergency number.

19)	SS waits the UE to send an INVITE request with Request-URI that matches the non emergency number dialled.

20)	SS responds to the INVITE request with a 380 Alternative Service.

21)	SS waits for the UE to send an ACK to acknowledge receipt of the 380 Alternative Service.

22-25)	Void

26-37)	SS waits for IMS Emergency Call Setup procedure

38-39)	Having reached the active state, the call is cleared by the UE.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-9 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 10-15 |  |  |  | Void |
| 16-17A |  |  | Steps 1-3 of Annex C.32 | The UE releases the call |
| 18 |  |  |  | MO call is initiated on the UE by dialling a “non emergency” number. |
| 19 |  |  | INVITE | UE sends INVITE. Request-URI of the INVITE request matches with the “non emergency” number dialled. |
| 20 |  |  | 380 Alternative Service | The SS responds with a 380 Alternative Service |
| 21 |  |  | ACK | The UE acknowledges the receipt of 380 response for INVITE. |
| 22-25 |  |  |  | Void |
| 26-30 |  |  | Steps defined in annex C.22 | IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 31-37 |  |  |  | Void |
| 38-42 |  |  | Steps defined in Annex C.32 | The UE releases the call |

NOTE: The default messages contents in annex A are used with condition “IMS security“.

Specific Message Contents

INVITE (Step 19)

Use the default message “INVITE” in annex A.2.1. for MO Call” in annex A.2.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: AMR codec shall be present |

380 Alternative Service (Step 20)

Use the default message “380 Alternative Service” in annex A.4.1 with the following exception.

| Header/param | Value/remark | Rel | Reference |
| --- | --- | --- | --- |
| Message-body | <?xml version="1.0" encoding="UTF-8"?> <ims-3gpp version="1"> <alternative-service> <type>emergency</type> <reason/> <action>emergency-registration</action> </alternative-service> </ims-3gpp> |  |  |

ACK (Step 21)

Use the default message "ACK" in annex A.2.7

INVITE (step 1 of Annex C.22)

Use INVITE as specified for step 1 in annex C.22 with condition A8 in addition if the UE is capable of obtaining location information.

<End of modification>

<Start of modification>

#### 19.4.1.4	Method of test

Initial conditions

The UE is Switched OFF and contains no ISIM or USIM.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-19) UE executes the procedures described in TS 36.508 [94] table 4.5A.5.3-1 steps 1 to 19 for EPS emergency bearer context activation and subsequent IMS emergency speech call.

Expected sequence:

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-5 |  |  | Steps defined in annex C.22 | Generic test procedure for setting up emergency speech call. Referred from 36.508 [94] table 4.5A.5.3-1 for a UE with E-UTRA support. |
| 6-7 |  |  | Void |  |
| 8-12 |  |  | Steps defined in annex C.32 | UE releases the emergency call using generic test procedure for MO release of IMS call |

Specific Message Contents

INVITE (Step 1)

Use INVITE as specified for step 1 in annex C.22 with condition A6 instead of A7 and with condition A8 in addition if the UE is capable of obtaining location information.

180 Ringing (Step 3)

Use 180 Ringing as specified for step 3 in annex C.22 with condition A7 instead of A8.

200 OK for INVITE (Step 4)

Use 200 OK as specified for step 4 in annex C.22 with condition A7 instead of A6.

BYE (Step 9)

Use the default message “BYE” in annex A.2.8 with condition A6.

<End of modification>

<Start of modification>

#### 19.4.2.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC. The UE is initially IMS registered in cell A and made to select cell B. The Tracking area update procedure is rejected with cause #15, No suitable cells in tracking area in cell B, thus ensuring that the UE is in state EMM-REGISTERED.LIMITED-SERVICE.

The SS is configured with the IMSI within the USIM application, the home domain name, the SS is listening to SIP default port 5060 for both UDP and TCP protocols. The SS supports EMM emergency attach procedure and emergency bearers.

The SS configures two cells as below:

EUTRA cell A and Cell B as in TS 36.508

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

-	the IMS emergency call is initiated on the UE.

-	UE executes the procedures described in TS 36.508 [94] table 4.5A.5.3-1 steps 1 to 19 for EMM Emergency registration, EPS emergency bearer context activation, IMS emergency speech call establishment with PSAP.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | User initiates an emergency call |  |
| 2-6 |  |  | Steps defined in annex C.22 | Generic test procedure for setting up emergency speech call. Referred from 36.508 [94] table 4.5A.5.3-1 for a UE with E-UTRA support. |
| 6A-C |  |  | Steps 1-3 of Annex C.32 | The UE releases the emergency call using steps 1-3 of Annex C.32 |
| 7 |  |  | void |  |
| 8 |  |  | void |  |
|  |  |  | EXCEPTION: Either step 9 or steps 10-11 are performed, depending on UE behaviour: if UE detaches, step 9 is performed. Otherwise, steps 10-11 are performed. |  |
| 9 |  |  | Detach procedure initiated by the UE | Generic test procedure for UE initiated detach according to TS 36.508 [94] table 6.4.3.14-1 may happen within 5 seconds. |
| 10-11 |  |  | Steps 4-5 of Annex C.32 | If step 9 was not performed steps 4-5 of Annex C.32 are performed |

Specific Message Contents

INVITE (step 1 of procedure in annex C.22)

Use INVITE as specified for step 1 in annex C.22 with condition A6 instead of A7.

180 Ringing (Step 3)

Use 180 Ringing as specified for step 3 in annex C.22 with condition A7 instead of A8.

200 OK for INVITE (Step 4)

Use 200 OK as specified for step 4 in annex C.22 with condition A7 instead of A6.

BYE (Step 9)

Use the default message “BYE” in annex A.2.8 with condition A6.

<End of modification>

<Start of modification>

#### 19.4.5.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is  registered to IMS services, by executing the generic test procedure in Annex C.2 and  it is attached to the HPLMN E-UTRA service as provided by SS. In the attach SS has indicated that the cell supports E-UTRA emergency bearers.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols.

Test environment shall be set up to send in response to Emergency REGISTER message a 401 Unauthorized such that UE will not be able to establish temporary set of security associations.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	IMS emergency call is initiated on the UE.

2-13)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 (parallel behaviour steps 1) for EPS emergency bearer context activation, IMS emergency speech call establishment with PSAP

14)	UE sends initial REGISTER message.

15)	The SS responds to the initial REGISTER request with a valid 401 Unauthorized response.

16)	The SS waits for the UE to set up a temporary set of security associations and to send another REGISTER request, over those security associations.

17)	The SS responds REGISTER message with 403 Forbidden and ignores any further REGISTER message reception.

18-22)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 (parallel behaviour steps 6-10) IMS emergency speech call establishment with PSAP.

23-23F)	The Call is released on the UE using annex C.32a procedure which includes the Emergency Bearer context deactivation

24-25)	Void

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | User initiates an emergency call |  |
| 2-13 |  |  | EPS emergency bearer context activation by the UE. | Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. Steps 2-10 of the parallel behaviour in Table 4.5A.4.3-2 is replaced by below steps 14-22. |
| 14 |  |  | REGISTER | The UE sends initial IMS emergency registration |
| 15 |  |  | 401 Unauthorized |  |
| 16 |  |  | REGISTER | The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
|  |  |  |  | Note: From this point onward the SS shall ignore any Registration message sent by the UE. |
| 17 |  |  | 403 Forbidden The following messages are exchanged on non protected port. | The SS sends this message to get the UE in a stable state. |
| 18-22 |  |  | Steps defined in annex C.22 | IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 23-23F |  |  | Steps defined in annex C.32a | The UE releases the call |
| 24-25 |  |  | Void. |  |

Specific Message Contents

REGISTER (Step 14)

Use the default message “REGISTER” in annex A.1.1 with condition A1.

REGISTER (Step 16)

Use the default message “REGISTER” in annex A.1.1 with conditions A2 and A17.

INVITE (Step 18)

Use INVITE as specified for step 1 in annex C.22 with condition A6 instead of A7.

180 Ringing for INVITE (Step 20)

Use 180 Ringing as specified for step 3 in annex C.22 with condition A7 instead of A8.

200 OK for INVITE (Step 21)

Use 200 OK as specified for step 4 in annex C.22 with condition A7 instead of A6.

BYE (Step 23A)

Use the default message “BYE” in annex A.2.8 with condition A6.

<End of modification>

<Start of modification>

#### 19.4.6.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the NW is VPLMN and the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)-12) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 and parallel behaviour steps 1 for EPS emergency bearer context activation,

13) SS waits for the UE to send an initial REGISTER request containing “sos” SIP URI parameter in the Contact header field.

14) The SS responds to the REGISTER request with a 403 Forbidden response,

15) The SS waits for the UE to send an INVITE request.

16)-18A) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 and parallel behaviour Steps 2-5 defined in annex C.22 of TS 34.229-1 for IMS Emergency call for EPS is established,

19)-23B) The UE releases the call as defined in annex C.32a which includes the Emergency Bearer context deactivation.

24)-25) Void.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-12 |  |  | Steps defined in TS 36.508 [94] table 4.5A.4.3-1 | EPS Bearer Activation procedure and IP address allocation according TS 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 13 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 14 |  |  | 403 Forbidden The following messages are exchanged on non protected port. | The SS responds with a failure. |
| 15 |  |  | INVITE | UE sends INVITE request without emergency registration. |
| 16-18A |  |  | Steps 2 to 5 defined in annex C.22 | IMS emergency call setup with PSAP |
| 19-23B |  |  | Steps defined in annex C.32a | The UE releases the call |
| 24-25 |  |  | Void. |  |

Specific Message Contents

REGISTER (Step 13)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”

403 Forbidden for REGISTER (Step 14)

Use the default message “403 FORBIDDEN” in annex A.3.2 with condition A1 “IMS emergency registration for an  anonymous emergency call”.

INVITE (Step 15)

Use INVITE as specified for step 1 in annex C.22 with condition A6 instead of A7.

180 Ringing for INVITE (Step 17)

Use 180 Ringing as specified for step 3 in annex C.22 with condition A7 instead of A8.

200 OK for INVITE (Step 18)

Use 200 OK as specified for step 4 in annex C.22 with condition A7 instead of A6.

BYE (Step 20)

Use the default message “BYE” in annex A.2.8 with condition A6.

<End of modification>

<Start of modification>

#### 19.4.7.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

1)-12) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 12 and parallel behaviour steps 1 for EPS emergency bearer context activation,

13) SS waits for the UE to send an initial REGISTER request containing “sos” SIP URI parameter in the Contact header field.

14) The SS responds to the REGISTER request with a 420 Bad Extension response,

15) The SS waits for the UE to send an INVITE request.

16)-18A) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 13 to 15 and parallel behaviour Steps 2-5 defined in annex C.22 of TS 34.229-1 for IMS Emergency call for EPS is established,

19)-23B) The UE releases the call as defined in annex C.32a which includes the Emergency Bearer context deactivation.

24)-25) Void.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-12 |  |  | Steps defined in TS 36.508 [94] table 4.5A.4.3-1 | EPS Bearer Activation procedure and IP address allocation according TS 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 13 |  |  | REGISTER | UE sends initial registration for IMS services. |
| 14 |  |  | 420 Bad Extension The following messages are exchanged on non protected port. | The SS responds with a failure. |
| 15 |  |  | INVITE | UE sends INVITE request without emergency registration. |
| 16-18A |  |  | Steps 2 to 5 defined in annex C.22 | IMS emergency call setup with PSAP |
| 19-23B |  |  | Steps defined in annex C.32a | The UE releases the call |
| 24-25 |  |  | Void. |  |

Specific Message Contents

REGISTER (Step 13)

Use the default message “REGISTER” in annex A.1.1 with condition A1 "Initial unprotected REGISTER" and A7 “Initial unprotected or subsequent REGISTER for emergency registration”

420 Bad Extension for REGISTER (Step 14)

Use the default message “420 Bad Extension for REGISTER” in annex A.1.8 with condition A1 “IMS emergency registration for an anonymous emergency call.”

INVITE (Step 15)

Use INVITE as specified for step 1 in annex C.22 with condition A6 instead of A7.

180 Ringing for INVITE (Step 17)

Use 180 Ringing as specified for step 3 in annex C.22 with condition A7 instead of A8.

200 OK (Step 18)

Use 200 OK as specified for step 4 in annex C.22 with condition A7 instead of A6.

BYE (Step 20)

Use the default message “BYE” in annex A.2.8 with condition A6.

<End of modification>

<Start of modification>

#### 19.5.1.4	Method of test

Initial conditions

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by having executed the generic test procedure in Annex C.2 up to the last step. UE has registered to IMS emergency services, by having executed the generic test procedure in Annex C.20 up to the last step. Thereafter the UE has initiated an emergency call by having executed the generic test procedure in Annex C.22 up to the last step. The emergency call is released.

An IP address re-allocation is triggered by executing a network initiated detach procedure with detach type indication "re-attach required". The UE then triggers an Attach procedure. The SS indicates support of emergency bearers and allocates an IP address different from the one used in the attach procedure in the preamble.

Test procedure

applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-15) UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation, IMS emergency registration and subsequent IMS emergency speech call establishment with PSAP

16) Call is released on the UE. SS waits for the UE to send a BYE request.

17) SS responds to the BYE request with valid 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | User initiates an emergency call |  |
| 2-10 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 11-12 |  |  | Void |  |
| 13-17 |  |  | Steps defined in annex C.32 | Make the UE release the call including EPS Bearer Deactivation procedure according to TS 36.508 [94] subclause 4.5A.15. |

Specific Message Contents

INVITE (step 1 of Annex C.22)

Use INVITE as specified for step 1 in annex C.22 with condition A8 in addition if the UE is capable of obtaining location information.

<End of modification>

<Start of modification>

#### 19.5.7.4	Method of test

Initial conditions

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

UE contains either ISIM and USIM applications or only USIM application on UICC. In the E-UTRA attach SS has indicated to the UE that the cell supports E-UTRA emergency bearers. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

Test procedure

Expected sequence, procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1)	Emergency call is initiated on the UE as described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation and subsequent IMS emergency registration. The UE registers to IMS emergency services, by executing the generic test procedure in Annex C.20 up to the last step with the exception that the SS sets the expiration time to 30 seconds in Step 1 of Expected Sequence.

2)	After completing the IMS emergency registration UE starts the process of initiating an emergency call, by executing the generic test procedure in steps 1-3 of Annex C.22.

2A-2D)	The UE re-registers for IMS emergency services.

3)	The SS accepts the call.

4)	The UE acknowledges call acceptance.

5)	The UE is made to release the call.

6)	The SS answers the call release.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Steps defined in annex C.20 | EPS emergency bearer context activation and subsequent IMS emergency registration by the UE. SS sets the expiration time of emergency registration to 30 seconds. |
| 2 |  |  | Steps 1-3 defined in annex C.22. | IMS emergency call setup with PSAP using preconditions |
| 2A |  |  | REGISTER | UE re-registers to the emergency services in a time span between half of the expiration time and the full expiration time. Note: in this test case, the re-registration time is set to an untypically short value of 30 seconds. As there are no requirements on the duration of a re-registration procedure it is only checked that the re-registration procedure starts between half of the expiration time and the full expiration time. |
| 2B |  |  | 401 Unauthorized | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. |
| 2C |  |  | REGISTER | UE completes the security negotiation procedures, sets up a new temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials. |
| 2D |  |  | 200 OK | The SS responds with 200 OK. |
| 3 |  |  | 200 OK | Response for INVITE sent in step 2 Note: 200 OK will be sent using previous socket connection before using old SA |
| 4 |  |  | ACK | Response from UE to confirm the dialog |
| 5-9 |  |  | Steps defined in Annex C.32 | MO call release |

Specific Message Contents

REGISTER (Step 2A)

Use the default message “REGISTER” in annex A.1.1. with condition A2 "Subsequent REGISTER sent over security associations” and A17 "UE initiated IMS re-registration or de-registration" and the following exceptions applying:

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | SIP URI with IP address or FQDN and protected server port of UE. The SIP URI shall contain the sos URI parameter. |
| Security-Client |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous REGISTER |

401 Unauthorized for REGISTER (Step 2B)

Use the default message “401 Unauthorized for REGISTER” in annex A.1.2 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Security-Server |  |
| spi-c | new SPI number of the inbound SA at the protected client port |
| spi-s | new SPI number of the inbound SA at the protected server port |
| port-c | new protected client port needed for the setup of new pairs of security associations |
| port-s | Same value as in the previous Security-Server headers |
| WWW-Authenticate |  |
| nonce | Base 64 encoding of a new RAND and AUTN |

REGISTER (Step 2C)

Use the default message “REGISTER” in annex A.1.1 like in step 1 above. The only difference is that the response value within Authorization header shall have been recalculated based on the nonce received from SS within 401 response.

200 OK for REGISTER (Step 2D)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition A3 “Response for an emergency registration” and the expires parameter of Contact header set to 1200.

<End of modification>

<Start of modification>

#### 19.5.9.4	Method of test

Initial conditions

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities (including the public emergency user identity allocated for the user) together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is registered to IMS services, by executing the generic test procedure in Annex C.2 up to the last step.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-15)	UE executes the procedures described in TS 36.508 [94] table 4.5A.4.3-1 steps 1 to 15 for EPS emergency bearer context activation, IMS emergency registration and subsequent IMS emergency speech call

16)	SS sends a SIP NOTIFY request in order to terminate the non-emergency IMS registration.

17)	UE responds the NOTIFY request with 200 OK response. The emergency call remains unaffected on the UE.

18)	Emergency call is terminated manually on the UE. Consequently the UE sends SIP BYE request.

19)	SS responds the BYE request with 200 OK response.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-15 |  |  | Steps defined in annex C.20 followed by the steps defined in annex C.22 | IMS emergency registration by the UE followed by IMS emergency call setup with PSAP. Referred from 36.508 [94] table 4.5A.4.3-1 for a UE with E-UTRA support. |
| 16 |  |  | NOTIFY | The SS sends a NOTIFY for registration event package, containing partial registration state information, with all previously registered non-emergency public user identities as "terminated" and "rejected" |
| 17 |  |  | 200 OK | The UE responds the NOTIFY with 200 OK |
| 18-22 |  |  | Steps defined in Annex C.32 | MO call release |

Specific Message Contents

NOTIFY (Step 16)

Use the default message “NOTIFY for reg-event package” in annex A.1.6 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| CSeq |  |
| Value | 2 |
| Message-body | <?xml version=”1.0” encoding="UTF-8"?> <reginfo xmlns=”urn:ietf:params:xml:ns:reginfo” version=”1” state=”partial”> <registration aor=”PublicUserIdentity2 (NOTE 1)” id=”a102” state=”terminated”> <contact id=”980” state=”terminated” event=”rejected”> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> <registration aor=”AssociatedTelUri(NOTE 1)” id=”a101” state=”terminated”> <contact id=”981” state=”terminated” event=”rejected”> <uri>same value as in Contact header of REGISTER request</uri> </contact> </registration> </reginfo> |

NOTE 1:	The public user ids and the associated TEL URI are as returned to the UE in the P-Associated-URI header of the 200 (OK) response to the REGISTER request;
PublicUserId1 is the default public user id i.e. the first one contained in P-Associated-URI;
AssociatedTelUri is the same as used in P-Associated-URI
PublicUserId2 and PublicUserId3 are the remaining IMPUs of the P-Associated-URI header

200 OK for NOTIFY (Step 17)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1

<End of modification>

<Start of modification>

### G.19.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has activated the IP-CAN to the Evolved Packet Core (EPC) via Wireless Local Access Network (WLAN) with SS. The UE has registered to IMS over WLAN according to procedure C.2c.

Test procedure

0)	UE executes the procedure described in TS 36.508 [94] Table 4.5A.24.3-1.

1-4)	UE performs IMS emergency registration and IMS emergency speech call.

10-12)	Call is released on the UE.

12A)	The UE initiates a disconnection from the IPsec tunnel set up for the emergency session as defined in 36.508 [94] Table 4.5A.23A.3-1.

Expected sequence:

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-4 |  |  | Steps defined in clause C.20 | IMS emergency registration by the UE |
| 5-9 |  |  | Steps defined in clause C.22 | IMS emergency call setup with PSAP |
| 10-12 |  |  | Steps 1-3 defined in clause C.32 | The UE releases the call |
| 12A |  |  |  | IPsec tunnel disconnection procedure according to TS 36.508 [94] subclause 4.5A.23A.3 |

Specific Message Contents

None.

<End of modification>