3GPP TSG-RAN5 Meeting #106	R5-250350

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1573 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in 17.1, C.10 and C.38 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There are still places where "Session in progress" is used instead of "Session progress". |  |  |  |  |  |  |  |  |
| Summary of change: |  | - "Session in progress" replaced by "Session progress". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.1.4, C.10, C.38 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 17.1.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF, registered to IMS services and set up the MO call, by executing annex C.21.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration and MO call.

Test procedure

1)	Adding video to the voice call is initiated on the UE.

2-10)	UE executes the procedure described in TS 36.508 [94] table 4.5A.11.3-1, steps 2 to 7. In detail the following steps are done in IMS.

2)	UE to sends a re-INVITE request to the SS.

3)	SS responds to the re-INVITE request with a 100 Trying response.

4)	SS responds to the re-INVITE request with a 183 Session Progress response.

5)	SS waits for the UE to send a PRACK request possibly containing the second SDP offer for update of precondition state.

6)	SS responds to the PRACK request with a valid 200 OK response.

7)	SS waits for the UE to optionally send an UPDATE request containing the final SDP offer. UE will not send the UPDATE request if the PRACK in step 4 already contained the final offer with preconditions met.

8)	SS responds to the UPDATE request (if UE sent one) with a valid 200 OK response.

9)	SS responds to the re-INVITE request with a valid 200 OK response.

10)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

10A)	The UE is triggered to remove the video stream from the multimedia call

11)	SS waits the UE to send a re-INVITE request with a SDP offer indicating the removal of the video stream.

12)	SS responds to the re-INVITE request with a 100 Trying response.

12A)	SS deactivates the EPS bearer corresponding to the video stream and releases the associated radio resources by applying the procedure described in TS 36.508 [94] clause 4.5A.15

13)	SS responds to the re-INVITE request with a valid 200 OK response.

14)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for re-INVITE.

15-19)	MO Call release according to procedure C.32.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make UE add video to the voice call. |  |
| 2 |  |  | INVITE | UE sends re-INVITE with an SDP offer containing media lines for both voice and video |
| 3 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 4 |  |  | 183 Session Progress | SS responds with an SDP answer indicating that SS has not reserved its resources for video. |
| 5 |  |  | PRACK | UE acknowledges the receipt of 183 response with PRACK and optionally offers second SDP to indicate the changed precondition status. |
| 6 |  |  | 200 OK | The SS responds PRACK with 200 OK and answers the second SDP (if any) with mirroring its contents. |
| 7 |  |  | UPDATE | UE sends an UPDATE after having reserved the resources for video if meeting the preconditions was not already indicated in step 2 or 5. |
| 8 |  |  | 200 OK | The SS responds UPDATE, if received, with 200 OK and indicates having reserved the resources |
| 9 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 10 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 10A |  |  | Make UE release video from the media call |  |
| 11 |  |  | INVITE | UE sends re-INVITE with a SDP offer indicating that the video component is removed from the call |
| 12 |  |  | 100 Trying | The SS responds with a 100 Trying provisional response |
| 12A |  |  | SS deactivates the EPS bearer for video |  |
| 13 |  |  | 200 OK | The SS responds re-INVITE with 200 OK |
| 14 |  |  | ACK | The UE acknowledges the receipt of 200 OK for re-INVITE |
| 15-19 |  |  | Steps defined in annex C.32 | MO Call release |

NOTE:	The default messages contents in annex A are used with condition “IMS security“ or “GIBA” when applicable

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog), A11 (INVITE for creating a video call) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 4] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) or RTP/AVP (fmt) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=tcap:1 RTP/AVPF [Note 2] - a=pcfg:1 t=1 [Note 2] - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: The tcap/pcfg attributes are present if RTP/AVP is present on the m line. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. |

183 Session Progress (Step 4)

Use the default message "183 Session Progress" in annex A.2.3 with condition A15 (183 sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Require |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - s=- - c=IN (addrtype) (connection-address for SS) - b=AS:30 Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000/1 [Note 1] - a=fmtp: (format) mode-change-capability=2; max-red=220 [Note 1] - a=ptime:20 - a=maxptime:240 Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) [Note 1] - b=AS: (bandwidth-value) [Note 1] - b=RS: (bandwidth-value) [Note 1] - b=RR: (bandwidth-value) [Note 1] Attributes for media: - a=acfg:1 t=1 [Note 2] - a=rtpmap: (payload type) H264/90000 [Note 1] - a=fmtp: (format) (format specific parameters) [Note 1] Attributes for preconditions: - a=curr:qos local none - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos mandatory remote sendrecv - a=conf:qos remote sendrecv Note 1: The value for fmt, bandwidth, payload type, format and format specific parameters copied from step 2. Note 2: Present if tcap/pcfg attributes were included in step 2. |

PRACK (Step 5)

Use the default message “PRACK” in annex A.2.4 with the exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported option-tag | precondition [Note 4] |
| Message-body | Header optional Contents if present: The following SDP types and values shall be present. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t=0 0 Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video (transport port) RTP/AVPF (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR: (bandwidth-value) Attributes for media: - a=rtpmap: (payload type) H264/90000 - a=fmtp: (format) profile-level-id= (att-field) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote none - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: Shall be present if message-body present. |

200 OK for PRACK (Step 6)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | Header present if Prack (step 5) contained SDP. Contents if present: SDP body of the 200 response copied from the received PRACK and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (video): - a=curr:qos remote sendrecv |

UPDATE (Step 7)

Use the default message “UPDATE” in annex A.2.5 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Supported | Same contents as specified in step 5. |
| Message-body | Same contents as specified in step 5. |

200 OK for UPDATE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition (shall be present if SDP message-body present) |
| Content-Type | Header optional Contents if present: |
| media-type | application/sdp |
| Content-Length | Contents if header Content-Type is present: |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received UPDATE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and transport port on "m=" lines changed to indicate to which IP address and port the UE should start sending the media; Attributes for preconditions (video): - a=curr:qos remote sendrecv |

200 OK for INVITE (Step 9)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog).

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with condition A5.

INVITE (Step 11)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5 (re-INVITE within a dialog) and the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Supported |  |
| option-tag | precondition |
| Message-body | The following SDP types and values. Session description: - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) [Note 2] - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) RTP/AVP (fmt) - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) - b=RS: (bandwidth-value) - b=RR:(bandwidth-value) Attributes for media: - a=rtpmap: (payload type) AMR-WB/16000 [Note 3] - a=fmtp: (format) Attributes for preconditions: - a=curr:qos local sendrecv - a=curr:qos remote sendrecv - a=des:qos mandatory local sendrecv - a=des:qos optional remote sendrecv or a=des:qos mandatory remote sendrecv Media description: - m=video 0 RTP/AVPF (fmt) [Note 4] Note 1: At least one "c=" field shall be present. Note 2: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one. Note 3: The AMR channel number shall be “/1” or omitted. Note 4: There are no requirements on b-, c- or a-lines on the video stream. |

200 OK for INVITE (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A23 (Response sent for re-INVITE within an established dialog) and the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Require option-tag | precondition |
| Content-Type |  |
| media-type | application/sdp |
| Content-Length |  |
| Value | length of message-body |
| Message-body | SDP body of the 200 response copied from the received INVITE and modified as follows: - "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one - IP address on "c=" line and, for audio, transport port on "m=" line changed to indicate to which IP address and port the UE should start sending the media; |

ACK (Step 14)

Use the default message “ACK” in annex A.2.7 with condition A5.

<End of modification>

<Start of modification>

# C.10	Generic test procedure for MTSI conference creation - EPS

The generic test procedure for creating MTSI conference may be performed after successful IMS or early IMS registration

Test procedure

1)	UE attempts to make conference call

2-7a) UE creates the voice conference. The same message sequence as in steps 2 - 8 of Annex C.21 are used to create the conference into the conference focus and negotiate the media.

8)	SS responds to the INVITE request with valid 200 OK response.

9)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

10)	SS waits the UE to optionally subscribe to the conference event package with a SUBSCRIBE message

11)	If UE sent SUBSCRIBE, SS responds to it with 200 OK response.

12)	If UE sent SUBSCRIBE, SS sends a NOTIFY for the conference event package to the UE.

13) If SS sent a NOTIFY, SS waits the UE to respond the NOTIFY with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS Conference call |  |
| 2-7a |  |  | Steps 2-8 of Annex C.21 | The same messages as in steps 2 - 8 of Annex C.21 |
| 8 |  |  | 200 OK | The SS responds INVITE with 200 OK and gives the final conference URI within the response |
| 9 |  |  | ACK | The UE acknowledges the receipt of 200 OK for INVITE |
|  |  |  | EXCEPTION: steps 10 – 13 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 10 |  |
| 10 |  |  | SUBSCRIBE | UE subscribes the conference event |
| 11 |  |  | 200 OK | SS responds to the subscription |
| 12 |  |  | NOTIFY | SS sends the initial state of the conference event to the UE |
| 13 |  |  | 200 OK | UE responds to the NOTIFY |

NOTE:	The default messages contents in annex A are used with condition “IMS security“ or “early IMS security” when applicable

Specific Message Contents

The specific message contents for steps 1 – 7a is otherwise identical to what have been specified in Annex C.21, but with the exceptions as below:

INVITE (Step 2)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |

183 Session Progress for INVITE (Step 4)

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | sip:temporary@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | isfocus |
| Record-Route |  |
| rec-route | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |

200 OK for INVITE (Step 8)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Record-Route |  |
| rec-route | Same value as in the 183 response |
| Contact |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | Isfocus |

ACK (Step 9)

Use the default message “ACK” in annex A.2.7 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |

SUBSCRIBE (Step 10)

Use the default message “SUBSCRIBE for conference event package” in annex A.5.1

200 OK (Step 11)

Use the default message “200 OK for SUBSCRIBE” in annex A.5.2

NOTIFY (Step 12)

Use the default message “MT NOTIFY for conference event package” in annex A.5.3 with condition A3

200 OK (Step 13)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>

<Start of modification>

# C.38	Generic test procedure for MTSI Video conference creation - EPS

Test procedure

1-8) UE creates the video conference. The same message sequence as in steps 1 - 8 of Annex C.25 are used to create the conference into the conference focus and negotiate the media.

9)	SS responds to the INVITE request with valid 200 OK response.

10)	SS waits for the UE to send an ACK to acknowledge receipt of the 200 OK for INVITE.

11)	SS waits the UE to optionally subscribe to the conference event package with a SUBSCRIBE message

12)	If UE sent SUBSCRIBE, SS responds to it with 200 OK response.

13)	If UE sent SUBSCRIBE, SS sends a NOTIFY for the conference event package to the UE.

14) If SS sent a NOTIFY, SS waits the UE to respond the NOTIFY with 200 OK.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt an IMS video conference call |  |
| 2-8 |  |  | Steps 2-8 of Annex C.25 | The same messages as in steps 2 - 8 of Annex C.25 |
| 9 |  |  | 200 OK | The SS responds INVITE with 200 OK and gives the final conference URI within the response |
| 10 |  |  | ACK | The UE acknowledges the receipt of 200 OK for INVITE |
|  |  |  | EXCEPTION: steps 11 – 14 describe optional behaviour depending on UE configuration. The SS shall wait up to 3s for the SUBSCRIBE of step 10 |  |
| 11 |  |  | SUBSCRIBE | UE subscribes the conference event |
| 12 |  |  | 200 OK | SS responds to the subscription |
| 13 |  |  | NOTIFY | SS sends the initial state of the conference event to the UE |
| 14 |  |  | 200 OK | UE responds to the NOTIFY |

NOTE:	The default messages contents in annex A are used with condition “IMS security“ or “GIBA” when applicable

Specific Message Contents

The specific message contents for steps 1 – 8 is otherwise identical to what have been specified in Annex C.25, but with the exceptions to steps 1 and 3 as below:

INVITE (Step 2)

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |
| To |  |
| addr-spec | sip:mmtel@conf-factory appended with px_IMS_HomeDomainName |

183 Session Progress for INVITE (Step 4)

| Header/param | Value/remark |
| --- | --- |
| Contact |  |
| addr-spec | sip:temporary@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | isfocus |
| Record-Route |  |
| rec-route | <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |

200 OK for INVITE (Step 9)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Record-Route |  |
| rec-route | Same value as in the 183 response |
| Contact |  |
| addr-spec | sip:final@conf-factory. appended with px_IMS_HomeDomainName |
| feature-param | Isfocus |

ACK (Step 10)

Use the default message “ACK” in annex A.2.7 with the following exceptions:

| Header/param | Value/remark |
| --- | --- |
| Request-Line |  |
| Request-URI | sip:final@conf-factory. appended with px_IMS_HomeDomainName |

SUBSCRIBE (Step 11)

Use the default message “SUBSCRIBE for conference event package” in annex A.5.1.

200 OK for SUBSCRIBE (Step 12)

Use the default message “200 OK for SUBSCRIBE” in annex A.5.2.

NOTIFY (Step 13)

Use the default message “MT NOTIFY for conference event package” in annex A.5.3 with condition A5.

200 OK for NOTIFY (Step 14)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1.

<End of modification>