3GPP TSG-RAN5 Meeting #106	R5-250355

# Athens, Greece, 17th - 21st Feb 2025

Agenda Item:	6.6.6.6

Source:	MCC TF160

Title:	Further issues and inconsistencies with conditions in SIP default message contents

Document for:	Endorsement

# 1	Introduction

In R5-244439 proposals have been made to resolve inconsistencies in the use of conditions in references to default message contents. The proposals have been implemented by several CRs presented at RAN5#105 with basically no TTCN impact:

- R5-246632	Corrections and clarifications for conditions in SIP default message contents

- R5-246633	Corrections and clarifications for use of conditions in annex C

- R5-247607	Corrections and clarifications for use of conditions in test cases

- R5-247610	Corrections and clarifications for use of conditions in annex A

- R5-246636	Corrections and clarifications for use of conditions in test cases 10.X

- R5-247611	Corrections and clarifications for use of conditions in test cases 6.X

- R5-246639	Corrections and clarifications for use of conditions in test cases 7.X

- R5-247612	Corrections and clarifications for use of conditions in test cases 8.X

- R5-246641	Corrections and clarifications for use of conditions in test cases 9.X

In addition, further changes are needed in 36.523-1 and 38.523-1 which have not been handled yet and there are issues with emergency call establishment and eCall.

# 2	Issues

### 2.1	Emergency call establishment

### 2.1.1	Call establishment with and without registration

There are several conditions for the SIP messages concerning emergency call establishment:

- INVITE for MO Call Setup (34.229-1 A.2.1)

| Condition | Explanation |
| --- | --- |
| A6 | INVITE for creating an emergency session in case of no registration |
| A7 | INVITE for creating an emergency session within an emergency registration using IMS security |
| A19 | INVITE for creating an emergency session within an emergency registration using GIBA |

- 180 Ringing for INVITE (34.229-1 A.2.6)

| Condition | Explanation |
| --- | --- |
| A4 | 180 sent by the SS when setting up an emergency call or a non-UE detectable emergency Call |
| A7 | Response sent by SS for emergency call without emergency registration |
| A8 | Response sent by SS for emergency call with emergency registration or a non-UE detectable emergency call |

- 200 OK for INVITE (34.229-1 A.3.1)

| Condition | Explanation |
| --- | --- |
| A6 | Response sent by SS for INVITE for emergency call or non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |

For a normal emergency call (with registration) the following conditions are needed:

| INVITE | A7 (or A19) |
| --- | --- |
| 180 Ringing | A4, A8 |
| 200 OK | A6 |

In case of limited service it is

| INVITE | A6 |
| --- | --- |
| 180 Ringing | A4, A7 |
| 200 OK | A7 (NOTE 1) |

NOTE 1:	According to the explanation of conditions A6 and A7 in A.3.1 for an emergency call without registration both conditions would be applicable but for the Record-Route header field A6 and A7 need to be used mutually exclusive.

Proposal 2.1.1-1: Clarification to be added to condition A6 of 34.229-1 of A.3.1 that it is for emergency call with registration  A6 and A7 get mutual exclusive.

The procedures for emergency call establishment (34.229-1 C.22, 34.229-5 A.6) specify for the INVITE neither A6 nor A7, for the 180 Ringing neither A7 nor A8 but for 200 OK A6 is specified (normal emergency call). 
 Where C.22 or A.6 is used specific message contents would be needed to specify the conditions for normal emergency call or limited service. But this is not done in most cases.

Proposal 2.1.1-2: Procedures C.22 and A.6 shall be modified to fully specify the case of normal emergency call establishment (i.e. with prior emergency registration).

Proposal 2.1.1-3: Specific message contents shall be added/completed in test cases and test procedures for limited service.

### 2.1.2	P-Asserted-Identity in SIP responses

The general use of the P-Asserted-Identity header field is specified in RFC 3325. 24.229 describes the special case of the P-Asserted-Identity added to SIP responses in DL at emergency call establishment. Clauses 5.1.2A.1.1 and 5.1.6.10 specify the UE behaviour (mainly for the case of non-UE detectable emergency call).

Clause 5.11.2 specifies for the E-CSCF that

"When the E-CSCF receives any 1xx or 2xx response related to a UE-originated dialog or standalone transaction, the E-CSCF shall remove any P-Preferred-Identity header field and P-Asserted-Identity header field, and insert a P-Asserted-Identity header field with the digits that can be recognized as a valid emergency number if dialled as a tel URI representing the number, before forwarding the message."

At emergency call establishment basically all 1xx or 2xx responses should have a P-Asserted-Identity header field. Nevertheless, there are inconsistencies in 34.229 annex A:

- P-Asserted-Identity in 200 OK for INVITE in limited service (without emergency registration)
Currently in 34.229-1 A.3.1 the P-Asserted-Identity is not specified for condition A7 (emergency call without emergency registration). This is in contrast to what is specified for the 180 Ringing in A.2.6 where the P-Asserted-Identity is included for any kind of emergency call (condition A4). Furthermore, in TTCN the P-Asserted-Identity header field is included for limited-service test cases too.

Proposal 2.1.2-1: Prose to be aligned with TTCN by adding condition A7 to the P-Asserted-Identity in A.3.1.

- P-Asserted-Identity in 200 OK for INVITE for eCall
34.229-1 C.47 and 34.229-5 A.23 specify condition A6 to be used for the 200 OK for INVITE at eCall establishment. Nevertheless, in TTCN the P-Asserted-Identity is missing in the 200 OK for INVITE at eCall establishment.

Proposal 2.1.2-2: RAN5 to confirm that 200 OK for INVITE shall have P-Asserted-Identity even in case of eCall. TTCN CR to be raised accordingly.

- P-Asserted-Identity in 200 OK for UPDATE
There are two test cases using the procedures of normal MO call establishment to establish the emergency call (non-UE detectable emergency call): LTE test case 19.3.1 using C.21 (34.229-1) and NR5GC test case 10.4 using A.4.1a (34.229-5). NR5GC test case 10.4 specifies to use condition A6 for the 200 OK for UPDATE as needed for the Route header field (see issue 2 in 2.1.4) but this is missing in 19.3.1. Assuming that condition A6 shall be added to the 200 OK for UPDATE in 19.3.1, for both test cases the 200 OK for UPDATE should have a P-Asserted-Identity but this is currently not the case in TTCN.

Proposal 2.1.2-3: RAN5 to confirm that 200 OK for UPDATE in case of non-UE detectable emergency call shall have a P-Asserted-Identity as according to condition A6 of A.3.1.

- P-Asserted-Identity in 200 OK for PRACK
Test cases 19.3.1 and 10.4 for non-UE detectable emergency call do not specify a specific message content for the 200 OK responses for PRACK at call establishment  According to the current prose the 200 OK for PRACK responses don't have a P-Asserted-Identity - in contrast to clause 5.11.2 of 24.229.

Proposal 2.1.2-4: RAN5 to decide whether or not the 200 OK for PRACK responses at establishment of a non-UE detectable emergency call test case shall have a P-Asserted-Identity. If so, specific message contents need to be added to the test cases 19.3.1 and 10.4 specifying the P-Asserted-Identity (NOTE: When the 200 OK for PRACK shall have a P-Asserted-Identity then this cannot be specified by using condition A6 of A.3.1 as this would imply that the 200 OK for PRACK gets a Record-Route header field too).

### 2.1.3	Conflicting conditions in 200 OK and 183 Session Progress

At emergency call establishment the 200 OK for INVITE and 183 Session Progress for non-UE detectable emergency call need specific conditions for emergency call which conflict with conditions for normal call establishment in specification of the Record-Route header field:

- 200 OK for INVITE:
Conditions A1/A3 (Response sent by SS for INVITE/UPDATE) and A7/A8 (Response sent by SS for INVITE for emergency call with/without emergency registration) for the 200 OK in 34.229-1 A.3.1 need to be used mutually exclusive as otherwise there is a conflict for the Record-Route header field. But when A1/A3 are not applicable together with A7/A8 then the Contact header field is not specified for emergency call.

Proposal 2.1.3-1: Conditions A6 and A7 to be added to the Contact header field in A.3.1. Clarification to be added to NOTE 1.

- 183 Session Progress for non-UE detectable emergency call
In case of non-UE detectable emergency call the 183 Session Progress needs condition A5 which conflicts with A1/A3 in specification of the Record-Route header field
 A1/A3 and A5 need to be used mutually exclusive and A5 is needed for Contact, Rseq and Content-Length (as done for the To-tag already).

Proposal 2.1.3-2: Condition A5 to be added to the Contact, Rseq and Content-Length header fields in A.2.3. Clarification to be added to NOTE 4.

### 2.1.4	Route set of non-UE detectable emergency call

In contrast to the dialog of a normal call the dialog created in LTE test case 19.3.1 and NR5GC test case 10.4 for non-UE detectable emergency call has the route set of an emergency call which is set by the Record-Route of the 183 Session Progress (E-CSCF, P-CSCF; condition A5 in 34.229-1 A.2.3).  all further requests sent by the UE (PRACK, UPDATE, ACK, BYE) shall use this route set in the Route header field and the SS shall use it in the Record-Route header field of the 180 Ringing and the 200 OK responses for INVITE and UPDATE (The 200 OK responses for PRACK don't have a Record-Route header field).

Issues:

- Route header field of PRACK, UPDATE
34.229-1 A.2.4 and A.2.5 specify the Route header field to be the "URIs of the Record-Route header field as specified in A.2.3 (183 Session Progress) for condition A1 and A3 in reverse order"
 This is not applicable for the emergency call (due to references to A1 and A3).
On the other hand the UE has to use exactly the Route set as provided by the SS which can be expressed in A.2.4 and A.2.5 without references to any conditions (as done for e.g. ACK and BYE already).

Proposal 2.1.4-1: Route header field of PRACK (A.2.4) and UPDATE (A.2.5) to be changed to 
"MO call (dialog creating INVITE sent by the UE):
URIs of the Record-Route header field as sent to the UE in the 183 Session Progress.
MT call (dialog creating INVITE sent by the SS):
URIs of the Route header field as sent to the UE in the dialog creating INVITE."

- Record-Route header field of 200 OK for UPDATE
The 200 OK for UPDATE needs condition A6 to achieve the correct Route header field, but according to the explanation in A.3.1 A6 is applicable for INVITE only. Nevertheless, there is specific message content using A6 for the 200 OK for UPDATE in NR5GC test case 10.4 already but not in LTE test case 19.3.1

Proposal 2.1.4-2a: Clarification to be added to A.3.1 that in case of non-UE detectable emergency call condition A6 is applicable for a 200 OK for UPDATE too.

Proposal 2.1.4-2b: Specific message content for 200 OK for UPDATE to be added to LTE test case 19.3.1.

- Record-Route header field of 200 OK for INVITE
The 200 OK for INVITE needs condition A6 to achieve the correct Record-Route header field but in contrast to LTE test case 19.3.1 there is no specific message content in NR5GC test case 10.4.

Proposal 2.1.4-3: Specific message content for 200 OK for INVITE to be added to NR5GC test case 10.4.

- Record-Route header field of 180 Ringing
The 180 Ringing needs condition A8 to get the correct Record-Route header field, but in contrast to LTE test case 19.3.1 NR5GC test case 10.4 does not have a specific message content.

Proposal 2.1.4-4: Specific message content for 180 Ringing to be added to NR5GC test case 10.4.

### 2.2	Emergency call release

At MO call release of an emergency call without emergency registration condition A6 shall be specified for the BYE.

Proposal 2.2-1: Specific content for the BYE request to be added to test cases for emergency call without emergency registration.

In eCall test cases and some of the NR5GC emergency test cases the call release is mobile terminated: The BYE is sent by the SS and needs to have a Via header field consistent with the route set of the emergency call which is different from a normal call. In fact, the Via header field shall be the same as specified for the MT INFO in 34.229-1 A.2.20.

Proposal 2.2-2: New condition A9 in A.2.8 to specify the MT BYE for emergency call (Via header field).

Proposal 2.2-3: Condition A9 to be specified in specific message contents for the BYE in eCall or emergency test cases with MT call release.

### 2.3	eCall

### 2.3.1	Conditions used for particular flavours of eCalls

Basically, an eCall can be started manually or automatically. This is reflected in conditions A20 and A21 for the INVITE in A.2.1 of 34.229-1. As none of the procedures C.47 in 34.229-1 or A.23 in 34.229-5 or the procedures in 36.508 (4.5A.26, 4.5A.27) using C.47 and in 38.508-1 (4.9.11, 4.9.29) using A.23 specify which of A20 and A21 to be used, this needs to be done in the test cases.

Proposal 2.3.1-1: Test cases directly or indirectly referring to 34.229-1 C.47 or 34.229-5 A.23 shall specify which of conditions A20 and A21 shall be used.

In general, the 200 OK for INVITE at eCall establishment shall have condition A12 indicating the eCall and either condition A13 (ACK element = TRUE) or A14 (ACK element = FALSE). But it seems that A14 is not used by any test case.

Proposal 2.3.1-2: In the specific message content of the 200 OK for INVITE in 34.229-1 C.47 and 34.229-5 A.23 condition A13 shall be specified to be used. If there are use cases for condition A14 then this shall be specified at test case level.

In case of MSD update the INFO sent by the UE at step 6 in 34.229-1 C.47 and 34.229-5 A.23 uses either condition A1 (eCall over IMS was started manually) or A2 (eCall over IMS was started automatically) and either condition A3 (UE able to provide an updated MSD) or A4 (UE not able to provide an updated MSD) which basically should be specified at test case level. Nevertheless, in general condition A1/A2 can be taken over from condition A20/A21 for the INVITE (step 1) and condition A4 seems to be a special, test case specific case.

Proposal 2.3.1-3: Clarification to be added to 34.229-1 C.47 and 34.229-5 A.23 that condition A1 shall be used for the MO INFO at step 6 when condition A20 is specified for the INVITE at step 1 and condition A2 shall be used for the MO INFO at step 6 when condition A21 is specified for the INVITE at step 1.

Proposal 2.3.1-4: Condition A3 to be used per default for the MO INFO (step 6) of 34.229-1 C.47 and 34.229-5 A.23.

Proposal 2.3.1-5: Specific message contents of eCall test cases in 34.229-1, 34.229-5, 36.523-1 and 38.523-1 to aligned to the changes proposed by proposal 2.3.1-2, proposal 2.3.1-3 and proposal 2.3.1-4.

### 2.3.2	Test eCall

There are test cases 11.3.2 in 36.523-1 and 11.5.2 in 38.523-1 with test eCall where condition A25 (instead of A20/A21) is used for the INVITE for which neither condition A6 nor condition A7 shall be applied and condition A12 is specified for the 200 OK for INVITE but A13/A14 is missing and A7 shall not be applied. In addition, there is specific message content specifying a Call-Info, SDP and MSD for the INVITE and SDP and MSD for the 200 OK for INVITE.

Basically, the INVITE and its 200 OK use the routing of a normal Call but SDP and MSD of an eCall.

Proposal 2.3.2-1: Condition A25 to be added to the Call-Info in 34.229-1 annex A.2.1.

Proposal 2.3.2-2: Specific message content of test cases 11.3.2 (LTE) and 11.5.2 (NR5GC) to be reduced to refer to steps 1 and 2 of C.47/A.23 with condition A25 (instead of A20/A21) for the INVITE, conditions A12 and A13 for the 200 OK for INVITE and not using A7 for the INVITE and A6 for the 200 OK for INVITE.

### 2.3.3	MO INFO for eCall over IMS (34.229-1 A.2.19)

There is no Route header field specified for the MO INFO of A.2.20 even though according to RFC 3261 clause 12.2.1.1

"If the route set is not empty, and the first URI in the route set contains the lr parameter ..., the UAC ... MUST include a Route header field containing the route set values in order, including all parameters."

The Route header field is mandatory for the MT INFO of eCall test cases.

Proposal 2.3.3-1: The Route header field to be added to A.2.19.

# 3	Summary

### 3.1	Proposals

Proposal 2.1.1-1: Clarification to be added to condition A6 of 34.229-1 of A.3.1 that it is for emergency call with registration  A6 and A7 get mutual exclusive.

Proposal 2.1.1-2: Procedures C.22 and A.6 shall be modified to fully specify the case of normal emergency call establishment (i.e. with prior emergency registration).

Proposal 2.1.1-3: Specific message contents shall be added/completed in test cases and test procedures for limited service.

Proposal 2.1.2-1: Prose to be aligned with TTCN by adding condition A7 to the P-Asserted-Identity in A.3.1.

Proposal 2.1.2-2: RAN5 to confirm that 200 OK for INVITE shall have P-Asserted-Identity even in case of eCall. TTCN CR to be raised accordingly.

Proposal 2.1.2-3: RAN5 to confirm that 200 OK for UPDATE in case of non-UE detectable emergency call shall have a P-Asserted-Identity as according to condition A6 of A.3.1.

Proposal 2.1.2-4: RAN5 to decide whether or not the 200 OK for PRACK responses at establishment of a non-UE detectable emergency call test case shall have a P-Asserted-Identity. If so, specific message contents need to be added to the test cases 19.3.1 and 10.4 specifying the P-Asserted-Identity (NOTE: When the 200 OK for PRACK shall have a P-Asserted-Identity then this cannot be specified by using condition A6 of A.3.1 as this would imply that the 200 OK for PRACK gets a Record-Route header field).

Proposal 2.1.3-1: Conditions A6 and A7 to be added to the Contact header field in A.3.1. Clarification to be added to NOTE 1.

Proposal 2.1.3-2: Condition A5 to be added to the Contact, Rseq and Content-Length header fields in A.2.3. Clarification to be added to NOTE 4.

Proposal 2.1.4-1: Route header field of PRACK (A.2.4) and UPDATE (A.2.5) to be changed to 
"MO call (dialog creating INVITE sent by the UE):
URIs of the Record-Route header field as sent to the UE in the 183 Session Progress.
MT call (dialog creating INVITE sent by the SS):
URIs of the Route header field as sent to the UE in the dialog creating INVITE."

Proposal 2.1.4-2a: Clarification to be added to A.3.1 that in case of non-UE detectable emergency call condition A6 is applicable for a 200 OK for UPDATE too.

Proposal 2.1.4-2b: Specific message content for 200 OK for UPDATE to be added to LTE test case 19.3.1.

Proposal 2.1.4-3: Specific message content for 200 OK for INVITE to be added to NR5GC test case 10.4.

Proposal 2.1.4-4: Specific message content for 180 Ringing to be added to NR5GC test case 10.4.

Proposal 2.2-1: Specific content for the BYE request to be added to test cases for emergency call without emergency registration.

Proposal 2.2-2: New condition A9 in A.2.8 to specify the MT BYE for emergency call (Via header field).

Proposal 2.2-3: Condition A9 to be specified in specific message contents for the BYE in eCall or emergency test cases with MT call release.

Proposal 2.3.1-1: Test cases directly or indirectly referring to 34.229-1 C.47 or 34.229-5 A.23 shall specify which of conditions A20 and A21 shall be used.

Proposal 2.3.1-2: In the specific message content of the 200 OK for INVITE in 34.229-1 C.47 and 34.229-5 A.23 condition A13 shall be specified to be used. If there are use cases for condition A14 then this shall be specified at test case level.

Proposal 2.3.1-3: Clarification to be added to 34.229-1 C.47 and 34.229-5 A.23 that condition A1 shall be used for the MO INFO at step 6 when condition A20 is specified for the INVITE at step 1 and condition A2 shall be used for the MO INFO at step 6 when condition A21 is specified for the INVITE at step 1.

Proposal 2.3.1-4: Condition A3 to be used per default for the MO INFO (step 6) of 34.229-1 C.47 and 34.229-5 A.23.

Proposal 2.3.1-5: Specific message contents of eCall test cases in 34.229-1, 34.229-5, 36.523-1 and 38.523-1 to aligned to the changes proposed by proposal 2.3.1-2, proposal 2.3.1-3 and proposal 2.3.1-4.

Proposal 2.3.2-1: Condition A25 to be added to the Call-Info in 34.229-1 annex A.2.1.

Proposal 2.3.2-2: Specific message content of test cases 11.3.2 (LTE) and 11.5.2 (NR5GC) to be reduced to refer to steps 1 and 2 of C.47/A.23 with condition A25 (instead of A20/A21) for the INVITE, conditions A12 and A13 for the 200 OK for INVITE and not using A7 for the INVITE and A6 for the 200 OK for INVITE.

Proposal 2.3.3-1: The Route header field to be added to A.2.19.

### 3.2	Change requests

| R5-250339 | 34.229-1 | Correction of annex A.2.1 | Proposal 2.3.2-1 |
| --- | --- | --- | --- |
| R5-250340 | 34.229-1 | Correction of annex A.2.19 | Proposal 2.3.3-1 |
| R5-250341 | 34.229-1 | Correction of annex A.2.3 | Proposal 2.1.3-2 |
| R5-250343 | 34.229-1 | Correction of annex A.2.8 | Proposal 2.2-2 |
| R5-250344 | 34.229-1 | Correction of annex A.3.1 | Proposal 2.1.1-1, Proposal 2.1.2-1, Proposal 2.1.3-1, Proposal 2.1.4-2a |
| R5-250345 | 34.229-1 | Correction of annex C.22 | Proposal 2.1.1-2 |
| R5-250346 | 34.229-1 | Correction of annex C.47 | Proposal 2.3.1-2, Proposal 2.3.1-3, Proposal 2.3.1-4 |
| R5-250347 | 34.229-1 | Correction of eCall test cases | Proposal 2.3.1-1/5, Proposal 2.2-3 |
| R5-250348 | 34.229-1 | Correction of emergency test cases | Proposal 2.1.1-2/3, Proposal 2.2-1, Proposal 2.1.4-2b |
| R5-250351 | 34.229-5 | Correction of eCall test cases | Proposal 2.3.1-1/5, Proposal 2.2-3 |
| R5-250352 | 34.229-5 | Correction of emergency test cases | Proposal 2.1.1-2/3, Proposal 2.2-3, Proposal 2.1.4-3/4 |
| R5-250353 | 34.229-5 | Corrections of annex A.23 | Proposal 2.3.1-2, Proposal 2.3.1-3, Proposal 2.3.1-4 |
| R5-250354 | 34.229-5 | Correction of annex A.6 | Proposal 2.1.1-2 |
| R5-250333 | 36.508 | Corrections of clause 4.5A.5 | Proposal 2.1.1-3 |
| R5-250336 | 36.523-1 | Corrections of emergency call test cases for limited service | Proposal 2.2-1 |
| R5-250334 | 36.523-1 | Corrections of eCall test case 8.2.4.28 | Proposal 2.3.1-1, Proposal 2.3.1-2, Proposal 2.3.1-3, Proposal 2.2-3 |
| R5-250335 | 36.523-1 | Corrections of eCall test cases 11.3.x | Proposal 2.3.1-1, Proposal 2.3.1-2, Proposal 2.3.1-3, Proposal 2.2-3 |
| R5-250313 | 38.508-1 | Correction of clause 4.9.12B | Proposal 2.2-3 |
| R5-250314 | 38.508-1 | Corrections of clauses 4.9.11 and 4.9.12 | Proposal 2.1.1-2/3 |
| R5-250321 | 38.523-1 | Corrections of emergency call test cases for limited service 11.4.x | Proposal 2.2-1, Proposal 2.1.1-3 |
| R5-250324 | 38.523-1 | Corrections of emergency call test cases for limited service in Idle Mode | Proposal 2.2-1, Proposal 2.1.1-3 |
| R5-250322 | 38.523-1 | Corrections of eCall test cases 11.5.x | Proposal 2.3.1-1/5, Proposal 2.3.2-2, Proposal 2.1.1-3, Proposal 2.2-3 |
| R5-250328 | 38.523-1 | Corrections of eCall test case 8.1.4.1.10 | Proposal 2.3.1-1/5, Proposal 2.3.2-2, Proposal 2.1.1-3, Proposal 2.2-3 |
| R5-250319 | 38.523-1 | Correction of test case 11.4.13 | Proposal 2.2-3 |
| R5-250320 | 38.523-1 | Correction of test case 11.4.14 | Proposal 2.1.1-3 |

### 3.3	Open issues

- Proposal 2.1.2-2: P-Asserted-Identity in 200 OK for INVITE for eCall
Needs a TTCN CR when confirmed by RAN5, otherwise further prose changes are needed.

- Proposal 2.1.2-3: P-Asserted-Identity in 200 OK for UPDATE (non-UE detectable emergency call)
When confirmed by RAN5 the prose R5-250344 (Correction of annex A.3.1, addressing Proposal 2.1.4-2a) can be used for changes in TTCN, otherwise further prose changes are needed.

- Proposal 2.1.2-4: P-Asserted-Identity in 200 OK for PRACK (non-UE detectable emergency call)
If RAN5 decides that a 200 OK for PRACK does not need to have a P-Asserted-Identity no changes are needed; otherwise, the test cases need specific message contents for the 200 OK for PRACK responses and TTCN needs to be changed accordingly.

### 3.4	TTCN impact:

- Via header field in BYE for MT release of emergency call (Proposal 2.2-2)

- Route header field for MO INFO (eCall: MSD update; Proposal 2.3.3-1)

- Depending on RAN5 decision:
P-Asserted-Identity in 200 OK for INVITE for eCall and in 200 OK for UPDATE and PRACK for non-UE detectable emergency call (proposals 2.1.2-2/3/4)