3GPP TSG-RAN5 Meeting #106	R5-250357

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0004 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of references to Table 5.5.12.1.1-1 in clauses 5.3C.2, 5.3C.3 and 5.3C.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Incorrect references to Table 5.5.12.1.1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - References corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3C.2, 5.3C.3, 5.3C.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

### 5.3C.2	CO MCData Call Establishment

5.3C.2.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3C.2.2	Void

5.3C.2.3	Procedure

Table 5.3C.2.3-1: CO MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP INVITE requesting the establishment of an MCData call? | --> | SIP INVITE | - | P |
| - | EXCEPTION: In parallel to the steps below a dedicated bearer gets established as described in clause 5.4.3. | - | - | - | - |
| 3 | The SS sends a SIP 100 Trying | <-- | SIP 100 (Trying) | - | - |
| 4 | The SS (MCData server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | Check: Does the UE (MCData client) send a SIP ACK to acknowledge the session establishment/modification? | --> | SIP ACK | - | P |
| 6 | The UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 8 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint |  |  |  |  |  |

5.3C.2.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.2.4-1: MSRP SEND (Step 7, Table 5.3C.2.3-1)

Derivation Path: Table 5.5.12.1.1-1, condition EMPTY_SEND_REQ

<End of modification>

<Start of modification>

### 5.3C.3	CT MCData Call Establishment

5.3C.3.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is in RRC_IDLE state.

5.3C.3.2	Void

5.3C.3.3	Procedure

Table 5.3C.3.3-1: CT MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure 'MCX CT communication' as described in clause 5.4.4 is started to establish an RRC connection and a dedicated bearer. | - | - | - | - |
| 2 | The SS (MCX Server) sends a SIP INVITE requesting the establishment of an MCData call. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 3a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 4 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 5 | The SS (MCX server) sends a SIP ACK | <-- | SIP ACK | - | - |
| - | EXCEPTION: Steps 6a1 - 6b3 describe behaviour that depends on which role of an endpoint the UE (MCData client) has chosen in its SDP answer sent at step 4 | - | - | - | - |
| 6a1 | IF the UE (MCData client) acts as passive endpoint (NOTE 1) THEN the SS connects to the TCP server at the UE side to establish an MSRP connection | - | - | - | - |
| 6a2 | The SS sends an empty MSRP SEND request to bind the TCP connection to the MSRP session. | <-- | MSRP SEND | - | - |
| 6a3 | Check: Does the UE (MCData client) send an MSRP 200 (OK) response? | --> | MSRP 200 (OK) | - | P |
| 6b1 | ELSE (NOTE 2) the UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection | - | - | - | - |
| 6b2 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 6b3 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: The MCData client indicates to act as passive endpoint by setting the a=setup attribute of the SDP answer at step 4 to "passive" (according to RFC 4145 [119]) NOTE 2: The MCData client indicates to act as active endpoint by setting the a=setup attribute of the SDP answer at step 4 to "active" (according to RFC 4145 [119]) |  |  |  |  |  |

5.3C.3.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.3.4-1: MSRP SEND (Step 6a2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.2-1, condition EMPTY_SEND_REQ

Table 5.3C.3.4-2: MSRP SEND (Step 6b2, Table 5.3C.3.3-1)

Derivation Path: Table 5.5.12.1.1-1, condition EMPTY_SEND_REQ

<End of modification>

<Start of modification>

### 5.3C.12	CO MCData call establishment using a pre-established session

5.3C.12.1	Initial conditions

The procedure can be used in any test configuration for on-network UE testing described in clause 5.2.2.

The UE is either in RRC_IDLE state or in RRC_CONNECTED state.

5.3C.12.2	Void

5.3C.12.3	Procedure

Table 5.3C.12.3-1: CO MCData Call Establishment

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Step 1a1 describes behaviour that depends on the UE's RRC state. | - | - | - | - |
| 1a1 | IF the UE is in RRC_IDLE state THEN the procedure 'MCX CO communication' as described in clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send a SIP REFER message to request the establishment of an MCPTT call using a pre-established session? | --> | SIP REFER | - | P |
| 3 | The SS (MCData server) responds with a SIP 200 (OK) message indicating that the MCPTT call has been established | <-- | SIP 200 (OK) | - | - |
| 4 | The SS (MCX Server) sends a SIP re-INVITE to verify that the MCData call has been established. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 5a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying). | - | - | - | - |
| 5a1 | The UE (MCX client) sends a SIP 100 (Trying) | --> | SIP 100 (Trying) | - | - |
| 6 | Check: Does the UE (MCX client) respond to the SIP re-INVITE with SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS (MCX server) sends a SIP ACK in response to the SIP 200 (OK) message. | <-- | SIP ACK | - | - |
| 8 | The UE (MCData client) connects to the TCP server at the SS side to establish an MSRP connection. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCData client) send an empty MSRP SEND request to bind the TCP connection to the MSRP session? | --> | MSRP SEND | - | P |
| 10 | The SS (MCData server) sends an MSRP 200 (OK) response. | <-- | MSRP 200 (OK) | - | - |
| NOTE 1: According to TS 24.282 [87] clauses 9.2.3.4.2, 9.2.4.4.2 and 10.2.5.4.2 the SS sets the a=setup attribute set to "passive" (see table 5.5.3.1.2-3) The UE's MCData client has the role of the active endpoint |  |  |  |  |  |

5.3C.12.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure, with the following clarifications:

Table 5.3C.12.4-1: SIP re-INVITE from the SS (step 4, Table 5.3C.12.3-1)

| Derivation Path: Table 5.5.2.5.2-1, condition MCDATA_SDS, re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI | tsc_MCX_SessionID_B | session identity of the pre-established session | TS 24.282 [87] clause 9.2.5.2.2.1 |  |
| Message-body |  |  |  |  |
| MIME body part | not present | SDP message |  |  |
| MIME body part |  | MCData Info |  |  |
| MIME-part-body | MCData-Info message as described in Table 5.3C.12.4-2 |  |  |  |
| Editor’s Note: TS 24.282 [87] clause 9.2.5.1.2 does not clearly specify the header fields of the INVITE and therefore the default header fields are used |  |  |  |  |

Table 5.3C.12.4-2: MCData-Info (Table 5.3C.12.4-1)

| Derivation Path: Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | not present |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| anyExt |  |  |  |  |
| mcdata-communication-state | "establish-success" |  | TS 24.282 [87] clause 9.2.5.1.2 |  |

Table 5.3C.12.4-3: SIP 200 (OK) from the UE (step 6, Table 5.3C.12.3-1)

| Derivation Path: Table 5.5.2.17.1.1-1, condition INVITE-RSP, MCDATA_SDS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | not present |  |  |  |
| Message-body | not present |  |  |  |

Table 5.3C.12.4-4: MSRP SEND (Step 9, Table 5.3C.12.3-1)

Derivation Path: Table 5.5.12.1.1-1, condition EMPTY_SEND_REQ

<End of modification>