3GPP TSG-RAN5 Meeting #106	R5-250359

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-1 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections in 5.5.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Borders and formats in tables of clause 5.5.11 are not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Borders and formats corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial issues remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.11	Default MCVideo Transmission Control Messages and other Information Elements

#### 5.5.11.0	General

The following conditions apply throughout clause 5.5.11:

Table 5.5.11.0-1: Conditions

| Condition | Explanation |
| --- | --- |
| FA | IE for when an active Functional Alias is used |
| ACK | Message requests a Transmission control Ack |
| UPLINK | The message is sent from the UE |
| DOWNLINK | The message is sent from the SS |
| NOTE: For further conditions see table 5.5.1-1 |  |

For MCVideo media plane control different SSRCs (Synchronization SouRCes) need to be distinguished. Table 5.5.11.0-2 specifies the SSRCs as used in the default MCVideo media plane control messages for the case that there is no multiplexing of media plane control channels.

NOTE 1:	Multiplexing of media plane control channels has been introduced in Rel-18 of TS 24.281 [86] and TS 24.581 [88] and may be specified in Rel-18 and above test cases.

NOTE 2:	In contrast to Rel-18 where there are distinct SSRCs for the audio and video stream, in Before-Rel-18 releases there is still only one SSRC identifying a media stream. In Transmission Control Messages this Media SSRC effectively is the same as the Audio SSRC of Rel-18 as the Field ID is the same, but in Before-Rel-18 releases there is no field for a Video SSRC.

NOTE 3:	In test cases Before-Rel-18 behaviour is applied unless specified otherwise in the test case.

Table 5.5.11.0-2: SSRCs in MCVideo media plane control messages
(No multiplexing of media plane control channels)

| SSRC (NOTE 1) | Description | Value |
| --- | --- | --- |
| Media/Audio SSRC of the client (NOTE 1) | Before-Rel18: SSRC identifying the media stream of the client (Client A) | Arbitrarily selected by the SS and assigned to the client when the transmission is granted by sending a Transmission Granted message (NOTE 2) |
|  | Rel-18 and later: SSRC identifying the audio stream of the client (Client A) |  |
| Video SSRC of the client (NOTE 1) | Before-Rel18: Not applicable - there is no Video SSRC in transmission control messages and no SDP fmtp attribute indicating such SSRC and the SSRC used in RTP packets of the video stream is not specified. | Before-Rel18: Not present in DL, not checked in UL |
|  | Rel-18 and later: SSRC identifying the audio stream of the client (Client A) | Rel-18 and later: Arbitrarily selected by the SS and assigned to the client when the transmission is granted (NOTE 2) |
| Media/Audio SSRC of a remote client (NOTE 1) | Before-Rel18: SSRC identifying the media stream of a remote client (Client B, C) | Arbitrarily selected by the SS (NOTE 2) |
|  | Rel-18 and later: SSRC identifying the audio stream of a remote client (Client B, C) |  |
| Video SSRC of a remote client (NOTE 1) | Before-Rel18: Not applicable - there is no Video SSRC in transmission control messages and no SDP fmtp attribute indicating such SSRC and the SSRC used in RTP packets of the video stream is not specified. | Before-Rel18: Not present in DL, not checked in UL |
|  | Rel-18 and later: SSRC identifying the video stream of a remote client (Client B, C) | Rel-18 and later: Arbitrarily selected by the SS (NOTE 2) |
| RTCP SSRC of the client (NOTE 1) | SSRC used by the client (Client A) in the RTCP header of the MCVideo media plane control messages sent to the SS | The client may use any value, value is not checked by the SS (NOTE 4). |
| RTCP SSRC of the SS (NOTE 1) | SSRC used by the SS in the RTCP header of the MCPTT media plane control messages sent to the client | Arbitrarily selected by the SS (NOTE 4) |
| NOTE 1: The terms "RTCP SSRC", "Audio SSRC" and "Video SSCR" have been introduced in Rel-18 of TS 24.581 [88]. "Media SSRC" is used as in Rel-14 .. Rel-17 there is no "Audio SSRC" and "Video SSRC" yet in media plane control messages, but only a single SSRC value identifying the transmitter of a media stream. NOTE 2: Different SSRC values shall be selected by the SS for media streams from different clients. Nevertheless it is not clear for Before-Rel-18 releases how the client uses the SSRC value provided by the server in the Transmission Granted message and there is no way to provide SSRC value(s) in case of implicit transmission grant In general collisions according to IETF RFC 3550 [76] may occur but collision resolution is out of scope of this document. NOTE 3: For Before-Rel-18 releases the Media SSRC can only be assigned by sending a Transmission Granted message as there are no "mc_ssrc", "mc_audio_ssrc" or "mc_video_ssrc" fmtp attributes in Before-Rel-18 releases (nevertheless there can still be implicit grants for Before-Rel-18 releases even though in this case the server has no control over the SSRC values used by the client). From Rel-18 onward the Audio and Video SSRCs can be assigned with an implicit grant using fmtp attributes "mc_audio_ssrc" and "mc_video_ssrc" or by sending a Transmission Granted message with Audio and Video SSRC ( The server has control over the Audio and Video SSRCs used by the client). NOTE 4: In clause 4.3.3.1 TS 24.581 [88] clarifies in Rel-18 that "the SSRC of the RTCP header is used to enable multiplexing of media plane control channels"; in clauses 14.2.7 and 14.3.8 it is clarified that the "mc_transmission_ssrc" fmtp attribute is used to indicate support of multiplexing and to exchange of the SSRC values to be used in the RTCP header. It is assumed that RTCP SSRC values have no meaning in case of no multiplexing. |  |  |

#### 5.5.11.1	Transmission Control Specific Messages Sent by the Transmission Participant

##### 5.5.11.1.1	Transmission Request

Table 5.5.11.1.1-1: Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.4-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00000” | Transmission Request | TS 24.581 [88] clause 9.2.4 and Table 9.2.2.1-1 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| Transmission Priority | If present |  | TS 24.581 [88] clause 9.2.3.2 |  |
| Transmission Priority Value | Any allowed value | If present, a value between '0' and '255' where '0' is the lowest priority and '255' is the highest priority. If the Transmission Priority field is not included in the message the default priority is used as the Transmission Priority value. The value of the default priority is '0'. The default priority is sometimes referred to as normal priority. |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.1.2	Transmission Release

Table 5.5.11.1.2-1: Transmission Release

| Derivation Path: TS 24.581 [88] Table 9.2.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP-header |  |  |  |  |
| Subtype | “00010” | Transmission Release | TS 24.581 [88] clause 9.2.7 and Table 9.2.2.1-1 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo ID of the transmission participant sending the Transmission Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  |  |  |
| Transmission Indicator | "1000000000000000" | Normal call | TS 24.581 [88] clause 9.2.3.11 |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.1.3	Queue Position Request

Table 5.5.11.1.3-1: Queue Position Request

| Derivation Path: TS 24.581 [88] Table 9.2.11-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Queue Position Request | TS 24.581 [88] clause 9.2.11 and 9.2.2.1-1 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76], |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |

##### 5.5.11.1.4	Receive Media Request

Table 5.5.11.1.4-1: Receive Media Request

| Derivation Path: TS 24.581 [88] Table 9.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Receive Media Request | TS 24.581 [88] clause 9.2.14 and 9.2.2.1-1 |  |
|  | “10100” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field is used to carry the identity of the user who is requesting the reception of the media. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| User Id of the Transmitting User | any value if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of the Transmitting User (client B) as provided by the SS in the Media Transmission Notification |  | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | any value if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Reception Priority | if present | Describes the level of reception priority requested in a Reception Request message or granted in a Reception Granted message. The max reception priority that can be requested in a Reception Request message is negotiated between the transmission control participant and the transmission control server | TS 24.581 [88] clause 9.2.3.19 and 6.2.5.3.3 |  |
| Reception Priority value | any allowed value | The reception priority (0 to 255) where 0 is the lowest reception priority and 255 is the highest reception priority. If the Reception Priority field is not included in the message the default reception priority is used as the Reception Priority value. The value of the default reception priority is 0. The default reception priority is sometimes referred to as normal reception priority. |  |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |

##### 5.5.11.1.5	Void

##### 5.5.11.1.6	Remote Transmission Request

Table 5.5.11.1.6-1: Remote Transmission Request

| Derivation Path: TS 24.581 [88] Table 9.2.22-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Remote Transmission Request | TS 24.581 [88] clause 9.2.22 and Table 9.2.2.1-1 |  |
|  | “10111” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| Remote ID |  | Carries the identity of the user who remotely initiated the media transmission of another user. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |
| User ID |  | Carries the identity of the user whose media transmission is requested. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |

##### 5.5.11.1.7	Remote Transmission Cancel Request

Table 5.5.11.1.7-1: Remote Transmission Cancel Request

| Derivation Path: TS 24.581 [88] Table 9.2.24-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Remote transmission cancel request | TS 24.581 [88] clause 9.2.24 and Table 9.2.2.1-1 |  |
|  | “11000” |  |  | ACK |
| SSRC | RTCP SSRC of the client |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV0 |  |  |  |
| User ID |  | Carries the identity of the user whose media transmission is requested for cancellation. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |

#### 5.5.11.2	Transmission Control Specific Messages Sent by the Transmission Control Server

##### 5.5.11.2.1	Transmission Granted

Table 5.5.11.2.1-1: Transmission Granted

| Derivation Path: TS 24.581 [88] Table 9.2.5-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00000” | Transmission granted | TS 24.581 [88] clause 9.2.5 and 9.2.2.1-2 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator |  | OFF-NETWORK |
| name | MCV1 | Transmission Control messages sent by the transmission control server and transmission control participant |  |  |
| Duration |  |  | TS 24.581 [88] clause 9.2.3.3 |  |
| Duration | "00000000 10000000" | 128 sec (an arbitrary value) |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC which should be used by the client in the header of RTP packets | Before-Rel-18: SSRC indentifying the media stream Rel-18 and later: SSRC to be used by the client in the audio stream | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission priority | Not present | If the Transmission Priority field is not included in the message the default priority (='0') is used as the Floor Priority value | TS 24.581 [88] clause 9.2.3.2 |  |
| User ID | Not present |  |  |  |
| User ID |  |  | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Queue Size | Not present |  |  |  |
| Queue Size | "0" | the number of queued MCVideo clients in the MCVideo call | TS 24.581 [88] clause 9.2.3.15 | OFF-NETWORK |
| SSRC of queued floor participant | Not present |  |  |  |
|  | The SSRC of queued transmission participant |  | IETF RFC 3550 [76] | OFF-NETWORK |
| Queued User ID | Not present |  |  |  |
|  | px_MCVideo_ID_User_C | MCVideo ID of the transmission participant in the queue | TS 24.581 [88] clause 9.2.3.14 | OFF-NETWORK |
| Queue Info | Not present |  |  |  |
| Queue Info |  | queue position and granted transmission priority in the queue | TS 24.581 [88] clause 9.2.3.5 | OFF-NETWORK |
| queue position info | "00000001" |  |  |  |
| queue priority level | "00000000" |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] Table 9.2.3.11-2 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.2	Transmission Rejected

Table 5.5.11.2.2-1: Transmission Rejected

| Derivation Path: TS 24.581 [88] Table 9.2.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission rejected | TS 24.581 [88] clause 9.2.6 and 9.2.2.1-2 |  |
|  | “10001” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Reject Cause |  | Includes the reason for the rejecting the transmission request and can be followed by a text-string explaining why the transmission request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause | "255" | Th<Reject cause> value set to '255' indicates that the transmission control server does not grant the transmission request due to the transmission control server local policy. | TS 24.581 [88] clause 9.2.6.2 |  |
| Reject Cause Phrase | “Other reason” | A text string encoded the text string in the SDES item CNAME. | IETF RFC 3550 [76] |  |
| User ID | Not present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the requesting transmission participant to which the Transmission Rejected message is sent. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.3	Transmission Arbitration Taken

Table 5.5.11.2.3-1: Transmission Arbitration Taken

| Derivation Path: TS 24.581 [88] Table 9.2.8-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | Transmission Arbitration Taken | TS 24.581 [88] clause 9.2.8 and 9.2.2.1-2 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User |  | MCVideo Id of the user who has been granted the right to transmit media. | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause9.2.3.7 |  |
| Permission to Request the Transmission | "1" | Coded as follows: 0 The receiver is not permitted to request transmission. 1 The receiver is permitted to request transmission |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Taken message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Arbitration Taken message, if any, increased by 1 | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call. |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of the Transmitting User (client B) | Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |

##### 5.5.11.2.4	Transmission Arbitration Released

Table 5.5.11.2.4-1: Transmission Arbitration Released

| Derivation Path: TS 24.581 [88] Table 9.2.9-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Transmission Arbitration Release | TS 24.581 [88] clause 9.2.9 and 9.2.2.1-2 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Granted Party's Identity |  | Identifies the MCVideo user that is granted to send media. | TS 24.581 [88] clause 9.2.3.6 |  |
| Granted Party's Identity | px_MCVideo_ID_User_A |  |  |  |
| User ID | Not Present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID carries the MCVideo ID of the transmission participant sending the Transmission Arbitration Release message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Arbitration Release message, if any, increased by 1 | The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again. |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| SSRC of Granted Transmission Participant | Media SSRC of the intended recipient of the message | Notation in accordance with clause 5.5.11.0. | IETF RFC 3550 [76] |  |

##### 5.5.11.2.5	Transmission Revoked

Table 5.5.11.2.5-1: Transmission Revoked

| Derivation Path: TS 24.581 [88] Table 9.2.10-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Revoked | TS 24.581 [88] clause 9.2.10 and 9.2.2.1-2 |  |
|  | “10100” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Reject Cause |  | Message includes <Reject Cause> cause value in the Reject Cause field explaining why the transmission control server wants the transmission participant to stop sending media and can be followed by additional information. Therefore the length of the packet can vary depending on the value of the rejection cause. | TS 24.581 [88] clause 9.2.3.4 |  |
| Reject Cause Value | 7 | The <Reject Cause> value set to 7 indicates that the MCVideo client's permission to send a media is being queued. No additional information is included. | TS 24.581 [88] clause 9.2.10.2 |  |
| Reject Cause Phrase | "Queue the transmission" | A text string encoded the text string in the SDES item CNAME. | TS 24.581 [88] clause 9.2.10.2 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.6	Queue Position Info

Table 5.5.11.2.6-1: Queue Position Info

| Derivation Path: TS 24.581 [88] Table 9.2.12-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00101” | Queue Position Info | TS 24.581 [88] clause 9.2.12 and 9.2.2.1-2 |  |
|  | “10101” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID | Not present |  |  |  |
| User ID |  | The User ID field is used in off-network only. The User ID field carries the MCVideo user ID of the transmission participant sending the Queue Position Info message. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| SSRC of Queued Transmission Participant | Not present |  |  |  |
| SSRC of Queued Transmission Participant | The SSRC of the queued transmission participant | Applicable only in off-network and shall carry the SSRC of the queued transmission participant. | IETF RFC 3550 [76]. | OFF-NETWORK |
| Queued User ID | Not present |  |  |  |
| Queued User ID | px_MCVIDEO_ID_User_B | Used in off-network only. The Queued User ID field carries the MCVideo ID of the queued transmission control participant. | TS 24.581 [88] clause 9.2.3.8 | OFF-NETWORK |
| Queue Info |  | Defines the queue position and granted transmission control priority in the queue. | TS 24.581 [88] clause 9.2.3.5 |  |
| Queue Position Info | "1" | value is a binary value |  |  |
| Queue Priority Level | "0" | value consists of 8 bit parameter giving the transmission priority. The value of the default priority is ‘0’. The default priority is sometimes referred to as normal priority. | TS 24.581 [88] clause 9.2.3.2 |  |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Transmission Control Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.7	Media Transmission Notification

Table 5.5.11.2.7-1: Media Transmission Notification

| Derivation Path: TS 24.581 [88] Table 9.2.13-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00110” | Media Transmission Notification | TS 24.581 [88] clause 9.2.13 and 9.2.2.1-2 |  |
|  | “10110” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User (NOTE 1) |  | User ID of the user transmitting the media | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (Client B) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Permission to Request the Transmission |  | Indicates whether receiving parties are allowed to request the transmission. | TS 24.581 [88] clause 9.2.3.7 |  |
| Permission to Request the Transmission value | 1 | The receiver is permitted to request transmission |  |  |
|  | 0 | The receiver is not permitted to request transmission |  | BROADCAST-CALL |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal Call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |
| Track Info | Not present | The MCVideo call does not involve a non-controlling MCVideo function | TS 24.581 [88] clause 9.2.3.13 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88]clause 9.2.3.21 | FA |
| Reception Mode |  |  | TS 24.581 [88]clause 9.2.3.22 |  |
| Reception Mode value | 1 | The receiver is not granted permission to automatically receive media |  |  |
|  | 0 | The receiver is granted permission to automatically receive media |  | EMERGENCY-CALL, IMMPERIL-CALL, BROADCAST-CALL |
| NOTE 1: Before Rel-18 the "User Id" field (field ID 006) has been used instead of "User Id of the Transmitting User" (field ID 004). Nevertheless, it is assumed that the client can cope with unexpected fields and with missing of the User Id. |  |  |  |  |

##### 5.5.11.2.8	Receive Media Response

Table 5.5.11.2.8-1: Receive Media Response

| Derivation Path: TS 24.581 [88] Table 9.2.15-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00111” | Receive Media Response | TS 24.581 [88] clause 9.2.15 and 9.2.2.1-2 |  |
|  | “10111” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76], |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| Result |  | Indicates whether media reception is possible as per the request | TS 24.581 [88] clause 9.2.3.17 |  |
| Result | "1" | 0 - The receiver is not permitted (rejected) to receive the media transmission. 1 - The receiver is permitted (granted) to receive the media transmission. |  |  |
| Reject Cause | Not present | Includes the reason for the rejecting the media receive request and can be followed by a text-string explaining why the media receive request was rejected. Therefore the length of the packet will vary depending on the size of the application dependent field |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 |  |
| Audio SSRC of the Transmitting User | Same value as in the corresponding Receive Media Request | SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.2.9	Media Reception Notification

Table 5.5.11.2.9-1: Media Reception Notification

| Derivation Path: TS 24.581 [88] Table 9.2.16-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01000” | Media Reception Notification | TS 24.581 [88] clause 9.2.16 and 9.2.2.1-2 |  |
|  | “11000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76] |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID |  | The User ID field is used to carry the identity of the user who is receiving the media | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| Functional Alias | Not present |  |  |  |
|  | px_MCVideo_ID_FA_B | functional alias URI of the transmitting user | TS 24.581 [88] clause 9.2.3.21 | FA |

##### 5.5.11.2.10	Void

##### 5.5.11.2.11	Transmission Cancel Request Notify

Table 5.5.11.2.11-1: Transmission Cancel Request Notify

| Derivation Path: TS 24.581 [88] Table 9.2.19-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01010” | Transmission Cancel Request Notify | TS 24.581 [88] clause 9.2.19 and 9.2.2.1-2 |  |
|  | “11010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator |  | OFF-NETWORK |
| name | MCV1 | Transmission Control messages sent by the transmission control server and transmission control participant |  |  |

##### 5.5.11.2.12	Remote Transmission Response

Table 5.5.11.2.12-1: Remote Transmission Response

| Derivation Path: TS 24.581 [88] Table 9.2.23-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01011” | Remote Transmission Response | TS 24.581 [88] clause 9.2.23 and 9.2.2.1-2 |  |
|  | “11011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |

##### 5.5.11.2.13	Remote Transmission Cancel Response

Table 5.5.11.2.13-1: Remote Transmission Cancel Response

| Derivation Path: TS 24.581 [88] Table 9.2.25-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01100” | Remote Transmission Cancel Response | TS 24.581 [88] clause 9.2.25 and 9.2.2.1-2 |  |
|  | “11100” |  |  | ACK |
| SSRC | RTCP SSRC of the SS |  | IETF RFC 3550 [76]. |  |
| name | MCV1 |  |  |  |

##### 5.5.11.2.14	Media Reception Override Notification

Table 5.5.11.2.14-1: Media Reception Override Notification

| Derivation Path: TS 24.581 [88] Table 9.2.28-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01101” | Media Reception Override Notification | TS 24.581 [88] clause 9.2.28 and 9.2.2.1-2 |  |
|  | “11101” |  |  | ACK |
| SSRC | RTCP SSRC of the SS |  | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User ID |  | Carries the identity of the user who is requesting the reception of the media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_A |  |  |  |
| SSRC of transmitter | Same value as sent to the client in the Receive Media Response (Media SSRC of client B) | The SSRC of transmitter field carries the SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | IETF RFC 3550 [76]. |  |
| Overriding ID |  | Carries the identity of the user of the overriding media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_C |  |  |  |
| Overridden ID |  | Carries the identity of the user of the overridden media. | TS 24.581 [88] clause 9.2.3.8 |  |
| User ID | px_MCVideo_ID_User_B |  |  |  |

##### 5.5.11.2.15	Transmission End Notify

Table 5.5.11.2.15-1: Transmission End Notify

| Derivation Path: TS 24.581 [88] Table 9.2.29-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01110” | Transmission End Notify | TS 24.581 [88] clause 9.2.29 and 9.2.2.1-2 |  |
|  | “11110” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server. | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender |  |  | OFF-NETWORK |
| name | MCV1 |  |  |  |
| User Id of the Transmitting User (NOTE 1) |  | Carries the identity of the user whose media transmission has been released | TS 24.581 [88] clause 9.2.3.6 |  |
| User Id of the Transmitting User | px_MCVideo_ID_User_B |  |  |  |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (Client B) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 |  |
| NOTE 1: Before Rel-18 the "User Id" field (field ID 006) has been used instead of "User Id of the Transmitting User" (field ID 004). Nevertheless, it is assumed that the client can cope with unexpected fields and with missing of the User Id. |  |  |  |  |

##### 5.5.11.2.16	Transmission Idle

Table 5.5.11.2.16-1: Transmission Idle

| Derivation Path: TS 24.581 [88] Table 9.2.30-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “01111” |  | TS 24.581 [88] clause 9.2.2.1-2 |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server | IETF RFC 3550 [76]. |  |
|  | The SSRC of the message sender | The SSRC of the transmission arbitrator. |  | OFF-NETWORK |
| name | “MCV1” | Transmission Control messages sent by the Transmission Control Server and the Transmission Control Participant. |  |  |
| Message Sequence Number |  |  | TS 24.581 [88] clause 9.2.3.9 |  |
| Message Sequence Number | The value sent in the previous Transmission Idle message, if any, increased with 1 | value is a binary value. The <Message Sequence Number> value can be between '0' and '65535'. When the '65535' value is reached, the <Message Sequence Number> value starts from '0' again |  |  |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

#### 5.5.11.3	Transmission control specific messages sent by both the transmission control server and transmission control participant

##### 5.5.11.3.1	Transmission End Request

Table 5.5.11.3.1-1: Transmission End Request

| Derivation Path: TS 24.581 [88] Table 9.2.20-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00000” | Transmission End Request | TS 24.581 [88] clause 9.2.20 and 9.2.2.1-3 |  |
|  | “10000” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76]. | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User (NOTE 1) |  | identity of the user whose media transmission is requested to be terminated. | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| User Id of the Transmitting User | if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | UPLINK |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| User Id | Not present |  |  | DOWNLINK |
| User Id | if present | Before Rel-18 |  | UPLINK |
| User Id | px_MCVideo_ID_User_A |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Reject Cause |  | Includes the reason explaining why the transmission control server wants the transmission participant to stop sending media | TS 24.581 [88] clause 9.2.3.4 | DOWNLINK |
| Reject Cause Value | 8 |  |  |  |
| Reject Cause Phrase | "No receiving participant" |  |  |  |
| Reject Cause | not present |  |  | UPLINK |
| NOTE 1: Before Rel-18 the "User Id" field (field ID 006) has been used instead of "User Id of the Transmitting User" (field ID 004). Nevertheless, it is assumed that the client can cope with unexpected fields and with missing of the User Id. |  |  |  |  |

##### 5.5.11.3.2	Transmission End Response

Table 5.5.11.3.2-1: Transmission End Response

| Derivation Path: TS 24.581 [88] Table 9.2.21-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00001” | Transmission End Response | TS 24.581 [88] clause 9.2.21 and 9.2.2.1-3 |  |
|  | “10001” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76]. | DOWNLINK |
|  | RTCP SSRC of the client |  |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User (NOTE 1) |  | identity of the user whose media transmission is requested to be terminated. | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| User Id of the Transmitting User | if present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | UPLINK |
| User Id of the Transmitting User | px_MCVideo_ID_User_A |  |  |  |
| User Id | Not present |  |  | DOWNLINK |
| User Id | if present | Before Rel-18 |  | UPLINK |
| User Id | px_MCVideo_ID_User_A |  |  |  |
| Audio SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.16 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| NOTE 1: Before Rel-18 the "User Id" field (field ID 006) has been used instead of "User Id of the Transmitting User" (field ID 004). Nevertheless, it is assumed that the client can cope with unexpected fields and with missing of the User Id. |  |  |  |  |

##### 5.5.11.3.3	Media Reception End Request

Table 5.5.11.3.3-1: Media Reception End Request

| Derivation Path: TS 24.581 [88] Table 9.2.26-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00010” | Media Reception End Request | TS 24.581 [88] clause 9.2.26 and 9.2.2.1-3 |  |
|  | “10010” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client as provided in Media transmission notification message sent to the UE | SSRC of the user transmitting the media Notation in accordance with clause 5.5.11.0. | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Transmission Indicator |  |  | TS 24.581 [88] clause 9.2.3.11 |  |
| Transmission Indicator | "1000000000000000" | Normal call |  |  |
|  | "0100000000000000" | Broadcast group call |  | BROADCAST-CALL |
|  | "0001000000000000" | Emergency call |  | EMERGENCY-CALL |
|  | "0000100000000000" | Imminent peril call |  | IMMPERIL-CALL |

##### 5.5.11.3.4	Media Reception End Response

Table 5.5.11.3.4-1: Media Reception End Response

| Derivation Path: TS 24.581 [88] Table 9.2.27-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00011” | Media Reception End Response | TS 24.581 [88] clause 9.2.27 and 9.2.2.1-3 |  |
|  | “10011” |  |  | ACK |
| SSRC | RTCP SSRC of the SS | The SSRC of the transmission control server | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| User Id of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.6 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |
| Audio SSRC of the Transmitting User | Media/Audio SSRC of remote client (same value as in the corresponding Media Reception End Request) | SSRC of the user transmitting the media | TS 24.581 [88] clause 9.2.3.16 |  |
| Video SSRC of the Transmitting User | Not present | Rel-18 and later | TS 24.581 [88] clause 9.2.3.23 | DOWNLINK |
|  | Any value if present |  |  | UPLINK |

##### 5.5.11.3.5	Transmission Control Ack

Table 5.5.11.3.5-1: Transmission Control Ack

| Derivation Path: TS 24.581 [88] Table 9.2.31-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| RTCP |  |  |  |  |
| Subtype | “00100” | Transmission Control Ack | TS 24.581 [88] clause 9.2.31 and 9.2.2.1-3 |  |
| SSRC | RTCP SSRC of the SS | The SSRC of the Transmission Control server for on-network and transmission arbitrator for off-network. | IETF RFC 3550 [76] | DOWNLINK |
|  | RTCP SSRC of the client | The SSRC of the transmission control participant |  | UPLINK |
| name | MCV2 |  |  |  |
| Source |  |  | TS 24.581 [88] clause 9.2.3.12 |  |
| Source | "2" | the controlling MCVideo function is the sender of the message |  | DOWNLINK |
|  | “0” | the transmission participant is the sender of the message |  | UPLINK |
| Message name |  |  | TS 24.581 [88] clause 9.2.3.18 |  |
| Message Name | Message Name of the transmission control messages which requested the acknowledgement | value is as coded as an ascii name field of the RTCP APP packet containing the message to be acknowledged |  |  |
| Message type |  |  | TS 24.581 [88] clause 9.2.3.10 |  |
| Message Type | ’0000xxxx’ with ‘xxxx’ being the lower four bits of the subtype of the message to be acknowledged | Message Type of the transmission control messages which requested the acknowledgement |  |  |