3GPP TSG-RAN5 Meeting #106	R5-250363

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0075 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify output power dynamics related test cases test frequency rang requirements to be aligned with regular LTE UE requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | While LTE NB-IoT NTN is used in smart phones for mission critical services, LTE NB-IoT NTN output power dynamics test procedures should be tested the same frequency ranges as regular LTE UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following test configuration tables are modified to have the same test frequency ranges as regular LTE UEs - Table 6.3B.1.4.1-1 - Table 6.3B.3.1.4.1-1 - Table 6.3B.3.2.4.1-1 - Table 6.3B.4.1.4.1-1 - Table 6.3B.4.2.4.1-1 - Table 6.3B.4.3.4.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The existing test frequency ranges are not aligned with regular LTE UE test frequency range requirements for LTE NB-IoT NTN in smart phones |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

Table 6.3B.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions Note 2: only applicable for low range Note 3: only applicable for high range |  |  |  |  |

<skipped unchanged sections>

Table 6.3B.3.1.4.1-1: Test Configuration Table for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 15kHz |

<skipped unchanged sections>

Table 6.3B.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 | Normal |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 | Mid range |
| NPRACH preamble format | 0 |
|  | 1 |

<skipped unchanged sections>

Table 6.3B.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Mid range |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | N/A |  | QPSK | 1@0 | 3.75 kHz |
| 2 |  |  | QPSK | 1@0 | 15 kHz |
| 3 (NOTE 1) |  |  | QPSK | 12@0 | 15 kHz |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |  |

<skipped unchanged sections>

Table 6.3B.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 | Low range |
| NPRACH preamble format | 0 |

<skipped unchanged sections>

Table 6.3B.4.3.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Mid range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones, start position | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 15 |
| 2 |  | QPSK | 1@11 | 15 |
| 3 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

<End of changes>