3GPP TSG-RAN5 Meeting #106	R5-250430

# Athens, Greece, Feb 17-21, 2025

Title:	Discussion on REFSENS testing for DL CA with single UL

Source:	Huawei, HiSilicon, OPPO

Document for:	Endorsement

# Background

In RAN plenary meeting#106, LS [1] was sent to RAN5, which is about RAN4 agreements on HPUE in DL CA with 1UL CC configured scenario as below table. This paper is to discuss about the necessary changes in RAN5 as per the RAN4 agreements.

- As an exception, UE is allowed to indicate higher power class than default power class for a CA configuration with a single UL CC with DL CA and applicability note for the supported power class is not present in RAN4 specification. In this case, the UE shall be verified with default power class MSD requirements with Tx power being configured with default power class - For UEs already on the market before the HPUE MSD requirements are introduced into RAN4 specification, they are not required to meet the HPUE MSD requirements. - Considering UEs already under development may confront the situation that they were not verified against the HPUE MSD requirements to be defined in the future, it was agreed that a 6 months transition period is applied. The transition period is defined as the moment that the HPUE MSD requirement of a band combination with 1UL CC is added into RAN4 specification to the end of the 6 months period following the introduction. During this transition period, the MSD requirements for UEs shall be verified with the default power class.

# Discussion

## Uplink transmit power configuration in tests

Previously, UE can transmit higher power class than default PC3 on one single uplink carrier only when the HPUE applicability notes present in the second column of the Tables in TS 38.101-1 clause 5.5A.3.1. For example:

Table 5.5A.3.1-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) NR CA configuration; Uplink CA configuration or single uplink carrier10; NR Band; Channel bandwidth (MHz) (NOTE 3); Bandwidth combination set. CA_n1A-n3A; n38 CA_n1A-n3A; n1; 5, 10, 15, 20; 0. n3; 5, 10, 15, 20, 25, 30. n1; 5, 10, 15, 20, 25, 30, 40, 50; 1. n3; 5, 10, 15, 20, 25, 30, 40. n1; 5, 10, 15, 20; 2. n3; 5, 10, 15, 20, 25, 30, 35, 40. n1; n1 channel bandwidths in Table 5.3.5-1; 4 and 5. n3; n3 channel bandwidths in Table 5.3.5-1. ……. CA_n1A-n41A; n418,9 CA_n1A-n41A8; n1; 5, 10, 15, 20; 0. n41; 10, 15, 20, 40, 50, 60, 80, 90, 100. n1; 5, 10, 15, 20, 25, 30, 40, 50; 1. n41; 10, 15, 20, 30, 40, 50, 60, 80, 90, 100. ……. NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 10: Only single uplink carriers with power class other than PC3 are listed.

- If these notes exist, UE could support the PC2 or PC1.5 MSD requirements for this DL CA with 1UL CC configuration.

- Without these notes, UE should only support default PC3 MSD requirements for this DL CA with 1UL CC configuration.

Observation 1: Previously, in RAN4 the HPUE can be supported only when the applicability notes were added into the band combination tables in TS 38.101-1.

In RAN4#113, CRs [2,3] were agreed which allow UEs to enable higher power class before the corresponding HPUE MSDs are defined. This is a big change to how HPUE is enabled and tested today.

- Currently, when testing the MSD requirements for DL CA with 1UL CC configuration in RAN5 test case 7.3A.1_1 Reference sensitivity power level for 2DL CA exceptions, UE needs to indicate the power class of the single UL carrier in TS 38.508-2 Table A.4.3.2A.4.1-3 based on the TS 38.101-1 Table 5.5A.3.1-1 and select applicable test requirement tables.

- The test requirement of test case 7.3A.1_1 is described as below:

7.3A.1_1.5 Test requirement For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

And table A.4.3.2A.4.1-3 in TS 38.508-2 is extracted as below:

Table A.4.3.2A.4.1-3: Supported configurations for NR Inter-band CA within FR1 and two bands NR FR1 Inter-band CA configuration / Item (Note 1, 9); Release; Supported; Supported power class for single uplink carrier (Note 12); Supported CA Bandwidth Class(es) in UL (Note 2,5); Supported Bandwidth Combination Set(s) (Note 3); Supported 1Tx-2Tx ULTxSwitch-ing Band Pair (Note 7, 8); Supported 2Tx-2Tx ULTxSwitching Band Pair (Note 7, 8); Supported uplinkTx Switching-DL-Interruption-r16 (Note 10); Supported simultaneousRxTx (Note 11). CA_n1A-n3A; Rel-16. CA_n1A-n5A; Rel-17. Unchanged rows skipped. Note 12: The UE supplier shall indicate the supported single uplink carrier with power class other than PC3, as per TS 38.101-1 [23] Table 5.5A.3.1-1. For this release of specification valid choices are ’-’, ‘nX PC2’, ‘nY PC2’, where both nX and nY are the NR bands. For example, for CA_n1A-n78A, ‘-’ would mean only supports PC3 single uplink carrier, ‘n1 PC2’ would mean supports single carrier PC2 on band n1, ‘n78 PC2’ would mean supports single carrier PC2 on band n78..

As per the new RAN4 agreements, UE could transmit UL PC2 even though the HPUE applicability notes are not present and HPUE MSD requirements are absent. When testing the MSD requirements for a DL CA with single UL without the HPUE applicability notes in clause 5 of TS 38.521-1, two cases may occur:

- UE indicates and transmits PC2:	Table 7.3A.1_1.5-1a is selected, but there’s no applicable PC2 test requirements for this configuration

- UE indicates default PC3 but actually transmits PC2:	Table 7.3A.1_1.5-1 is selected, but the PC3 test requirements are not applicable in this case

To avoid these cases, p-max limitation should be introduced into test case 7.3A.1_1.

- For the CA configurations with HPUE applicability notes, UE could indicate PC2 and apply the PC2 test requirement in table 7.3A.1_1.5-1a. No need to limit the uplink power class and the p-max could be configured as “Not Present”.

- As for the CA configurations without HPUE applicability notes, UE should be verified with default PC3 MSD requirements with Tx power being configured with default PC3. In this case the p-max should be configured as 23.

Observation 2: Enabling HPUE before the HPUE MSDs are defined has impact to RAN5 UE test configurations and requirements to be verified due to potential different UE behaviours.

Proposal 1:	For CA configuration with single uplink carrier testing in 7.3A.1_1, configure P-max = Not Present when the HPUE applicability notes exist in clause 5 of TS 38.521-1, otherwise configure the P-max = 23.

## Transition period definition

According to [4], it can be seen that the purpose of introducing transition period is to handle the tests of UEs which are already on the market or already under development based on the PC3 MSD requirements. RAN4 agreed that for these two kinds of UEs they will only be verified with PC3 MSD requirements. And a 6 months transition period therefore was introduced for the newly introduced HPUE MSD requirements as shown in figure 1 copied from [4].

Figure 1 Transition period for HPUE MSD requirement application [4]

As can be seen, in RAN4 agreements, the transition period is defined as the moment that the HPUE MSD requirement of a band combination with 1UL CC is added into RAN4 specification to the end of the 6 months period following the introduction.

But in RAN5 group, the introduction of new HPUE tests for CA combinations are depending on the industry needs. When there’s a need to introduce HPUE tests for a CA configuration, it’s difficult for RAN5 group to identify when the requirements were introduced into RAN4 specification. Checking the 6 months transition period defined in RAN4 brings many workloads to RAN5. To achieve RAN4 transition period purpose and also keep RAN5 workload in a reasonable level, RAN5 needs to think carefully on how the 6 months transition period is implemented in test specification or work management.

Observation 3: Checking the 6 months transition period defined in RAN4 brings many workloads to RAN5. To achieve RAN4 transition period purpose and also keep RAN5 workload in a reasonable level, RAN5 needs to consider how the 6 months transition period is implemented in test specification and work management.

Considering the fact that in RAN4, new band combos RF requirements won’t be introduced into frozen release specifications. The transition period could be considered to be defined as:

- If the HPUE MSD requirements for a DL CA with 1UL CC configuration was defined in frozen releases in RAN4, and that release has been frozen for more than 6 months:

  - It applies to Rel-18 and earlier requirements, considering currently Release-18 and earlier releases have been frozen for more than 6 months in RAN4, no additional changes need to be done in RAN5.

  - RAN5 can introduce the tests any time as long as there are industry needs and these requirements should be verified;

- Otherwise, for the open releases (Rel-19 and forward) in RAN4:

  - the transition period is counted from the moment that the HPUE MSD of a band combination with 1UL CC is introduced into RAN5 specification to the end of the 6 months period following the introduction. During this transition period, the MSD requirements for UEs shall be verified with the default power class. This can be captured in RAN5 specifications

  - Besides, in RAN5, the Release-19 WI for HPUE band combinations hasn’t started yet. The description and applicable rules for transition period could be considered to be included in the WID document for R19 RAN5 WI for HPUE band combinations in the future.

Observation 4: In RAN4, RF requirements of new band combinations won’t be introduced into frozen releases, and 6 months have passed since the frozen of Rel-18 in RAN4, there is no impact to RAN5 for the Rel-18 and earlier releases.

Observation 5: For the open releases (Rel-19 and forward) in RAN4, the transition period can be transferred into RAN5 based transition period, i.e., it is counted from the moment that the HPUE MSD of a band combination with 1UL CC is introduced into RAN5 specification.

Proposal 2:	The following transition period definition be included in RAN5 R19 WID for HPUE band combination tests in the future:

- For a band combination with 1UL CC which has defined new HPUE MSD requirements in RAN4 parent WI, a transition period of 6 months will be applied for this HPUE MSD requirements unless the RAN4 parent WI has been completed over 6 months. The 6 months transition period is defined as starting from the time the HPUE MSD requirements being introduced into RAN5 specification. During this transition period, the UEs shall be verified with default power class MSD requirements.

To better understand the meaning of Proposal 2, below figure 2 and 3 can be referred.

Fig 2. Case 1: RAN5 introduce the HPUE MSD when RAN4 WI is closed more than 6 months

Fig 3. Case 2: RAN5 introduce the HPUE MSD when RAN4 WI is active or been closed less than 6 months

# Conclusion

This paper discussed the RAN LS on the HPUE for DL CA with 1UL CC, and has got below observations and proposals.

Observation 1: Previously, in RAN4 the HPUE can be supported only when the applicability notes were added into the band combination tables in TS 38.101-1.

Observation 2: Enabling HPUE before the HPUE MSDs are defined has impact to RAN5 UE test configurations and requirements to be verified due to potential different UE behaviours.

Observation 3: Checking the 6 months transition period defined in RAN4 brings many workloads to RAN5. To achieve RAN4 transition period purpose and also keep RAN5 workload in a reasonable level, RAN5 needs to consider how the 6 months transition period is implemented in test specification and work management.

Observation 4: In RAN4, RF requirements of new band combinations won’t be introduced into frozen releases, and 6 months have passed since the frozen of Rel-18 in RAN4, there is no impact to RAN5 for the Rel-18 and earlier releases.

Observation 5: For the open releases (Rel-19 and forward) in RAN4, the transition period can be transferred into RAN5 based transition period, i.e., it is counted from the moment that the HPUE MSD of a band combination with 1UL CC is introduced into RAN5 specification.

Proposal 1:	For CA configuration with single uplink carrier testing in 7.3A.1_1, configure P-max = Not Present when the HPUE applicability notes exist in clause 5 of 38.101-1, otherwise configure the P-max = 23.

Proposal 2:	The following transition period definition be included in RAN5 R19 WID for HPUE band combination tests in the future:

- For a band combination with 1UL CC which has defined new HPUE MSD requirements in RAN4 parent WI, a transition period of 6 months will be applied for this HPUE MSD requirements unless the RAN4 parent WI has been completed over 6 months. The 6 months transition period is defined as starting from the time the HPUE MSD requirements being introduced into RAN5 specification. During this transition period, the UEs shall be verified with default power class MSD requirements.

*	Proposal 1 is implemented in CR R5-250431

# Annex

[1] RP‑243308 	LS on HPUE MSD application for DL CA with 1UL CC, RAN->R5/4 OPPO, RAN#106, 2024-12

[2] R4-2420520 	(R17) CR to TS 38.101-1: DL CA HPUE, Qualcomm, RAN4#113 2024-11

[3] R4-2420524 	(R18) CR to TS 38.101-1: DL CA HPUE, Qualcomm, RAN4#113 2024-11

[4] RP-242788 	RAN task on HPUE with 1UL and DL CA, OPPO, Huawei, RAN#106 2024-12