3GPP TSG-RAN5 Meeting #106	R5-250434

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3190 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating PC2 AMPR testing for NS_43 and NS_43U for band n8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_NR_FR1_FDD_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR updates the PC2 AMPR testing for NS_43 and NS_43U. NS_43: In 6.2.3 AMPR test case, looking at the NS_43 PC2 AMPR requirements in table 6.2.3.3.6-1 and table 6.2.3.3.6-2a, we’ll find that: - For 10MHz CBW, both A4 and A5 apply for QPSK and 16QAM, and both A2 and A3 apply for 64QAM. - When the PC2 UE is tested at 10MHz CBW + QPSK/16QAM + Outer_Full: both A4 and A5 could apply and A5 has larger AMPR value - When the PC2 UE is tested at 10MHz CBW + 64QAM + Outer_Full: both A2 and A3 could apply and A3 has larger AMPR value In current TS 38.521-1 table 6.2.3.4.1-6a, test IDs 6/7, 11/12, 16/17, 23/24, 28/29, 33/34 have exactly the same test configurations but with different test AMPR test requirements due to above reasons. For these test points, the larger AMPR value should be applied in the test requirements. Thus, test IDs 6, 11, 16, 23, 28 and 33 should be removed. NS_43U: As per the minimum requirements, the applicable PC2 AMPR regions are same for NS_43 and NS_43U. The test points in table 6.2.3.4.1-6a for NS_43 also apply for NS_43U PC2 AMPR testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove test IDs 6, 11, 16, 23, 28 and 33 in Table 6.2.3.3.6-2a, and delete corresponding test requirements. - Updating the minimum requirements for NS_43U PC2 AMPR, and adding test configuration table 6.2.3.4.1-7a and test requirement table 6.2.3.5-11a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 UE can not correctly test the AMPR of NS_43. The PC2 AMPR testing for NS_43U are absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0985 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-250435 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 6.2.3	UE additional maximum output power reduction

### <Unchanged Skipped>

6.2.3.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.3.6-1: A-MPR regions for NS_43 (Power class 3 and 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.3.6-2: A-MPR for NS_43 (Power class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  | N/A |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.3.6-2a: A-MPR for NS_43 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) | A7 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 12 | ≤ 4 | 3 |
|  | QPSK | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 4 | 3 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  | ≤ 3.5 |  | ≤ 5.5 |  | ≤ 12 | ≤ 5 | 4 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 4.5 |  | ≤ 5 |  |  |  |  |  | ≤ 12 | ≤ 5.5 | 5.5 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 5.5 |
| CP-OFDM | QPSK | ≤ 4 |  |  |  |  |  | ≤ 4 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 4.5 |
|  | 16 QAM | ≤ 4 |  |  |  |  |  | ≤ 4.5 |  | ≤ 6 |  | ≤ 12 | ≤ 6 | 5 |
|  | 64 QAM | ≤ 4 |  | ≤ 5 |  | ≤ 6.5 |  |  |  |  |  | ≤ 12 | ≤ 6.5 | 6 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 12 | ≤ 7 | 7 |

Table 6.2.3.3.6-3: Void

For power class 3 operation, when NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.3.6-4 applies.

Table 6.2.3.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

For power class 2 operation, when NS_43U is signalled, the larger one between the PC2 A-MPR for NS_100 defined in Table 6.2.3.1-2 and the PC2 A-MPR for NS_43 defined in this clause applies.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.6.

### <Unchanged Skipped>

Table 6.2.3.4.1-6a: Test Configuration table for NS_43 for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth (Ch BW) as specified in test parameters |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A for A-MPR | DFT-s--OFDM | Pi/2 BPSK | Outer_Full (A1) |
| 2 | 910 | 10 |  |  |  | Pi/2 BPSK | Outer_Full (A2) |
| 3 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Outer_Full (A6) |
| 4 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Edge_1RB_Left (A6) |
| 53 | 902.5 | 5 |  |  |  | QPSK | Outer_Full (A1) |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 8 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 93 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 10 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 11 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 12 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 133 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 143 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 15 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 16 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 17 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 18 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| 193 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full (A1) |
| 20 | 910 | 10 |  |  |  | QPSK | Outer_Full (A5) |
| 21 | 907.5 | 15 |  |  |  | QPSK | Outer_Full (A6) |
| 22 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left (A6) |
| 233 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full (A1) |
| 24 | 910 | 10 |  |  |  | 16QAM | Outer_Full (A5) |
| 25 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full (A6) |
| 26 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left (A6) |
| 273 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full (A1) |
| 283 | 910 | 10 |  |  |  | 64QAM | Outer_Full (A3) |
| 29 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full (A6) |
| 30 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left (A6) |
| 31 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full (A6) |
| 32 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

Table 6.2.3.4.1-7: Test Configuration table for NS_43U for PC3 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15 kHz |  |
| A-MPR test parameters for NS_43U |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | 15 | N/A for A-MPR | DFT-s--OFDM | PI/2 BPSK | Outer_Full |
| 2 | 910 | 10 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 3 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Edge_1RB_Left |
| 4 | 907.5 | 15 | 15 |  |  | PI/2 BPSK | Outer_Full |
| 5 | 902.5 | 5 | 15 |  |  | QPSK | Outer_Full |
| 6 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 7 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 8 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 9 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 10 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 11 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 12 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 13 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 14 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 15 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 16 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 17 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 18 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 19 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 20 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| 21 | 902.5 | 5 | 15 |  | CP-OFDM | QPSK | Outer_Full |
| 22 | 910 | 10 | 15 |  |  | QPSK | Outer_Full |
| 23 | 907.5 | 15 | 15 |  |  | QPSK | Edge_1RB_Left |
| 24 | 907.5 | 15 | 15 |  |  | QPSK | Outer_Full |
| 25 | 902.5 | 5 | 15 |  |  | 16 QAM | Outer_Full |
| 26 | 910 | 10 | 15 |  |  | 16 QAM | Outer_Full |
| 27 | 907.5 | 15 | 15 |  |  | 16 QAM | Edge_1RB_Left |
| 28 | 907.5 | 15 | 15 |  |  | 16 QAM | Outer_Full |
| 29 | 902.5 | 5 | 15 |  |  | 64 QAM | Outer_Full |
| 30 | 910 | 10 | 15 |  |  | 64 QAM | Outer_Full |
| 31 | 907.5 | 15 | 15 |  |  | 64 QAM | Edge_1RB_Left |
| 32 | 907.5 | 15 | 15 |  |  | 64 QAM | Outer_Full |
| 33 | 902.5 | 5 | 15 |  |  | 256 QAM | Outer_Full |
| 34 | 910 | 10 | 15 |  |  | 256 QAM | Outer_Full |
| 35 | 907.5 | 15 | 15 |  |  | 256 QAM | Edge_1RB_Left |
| 36 | 907.5 | 15 | 15 |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-7a: Test Configuration table for NS_43U for PC2 UE

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Use channel bandwidth (Ch BW) as specified in test parameters |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_43 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | 902.5 | 5 | Default | N/A for A-MPR | DFT-s--OFDM | Pi/2 BPSK | Outer_Full |
| 2 | 910 | 10 |  |  |  | Pi/2 BPSK | Outer_Full |
| 3 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Outer_Full |
| 4 | 907.5 | 15 |  |  |  | Pi/2 BPSK | Edge_1RB_Left |
| 53 | 902.5 | 5 |  |  |  | QPSK | Outer_Full |
| 6 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 7 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 8 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 93 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full |
| 10 | 910 | 10 |  |  |  | 16QAM | Outer_Full |
| 11 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full |
| 12 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left |
| 133 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full |
| 143 | 910 | 10 |  |  |  | 64QAM | Outer_Full |
| 15 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full |
| 16 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left |
| 17 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full |
| 18 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left |
| 193 | 902.5 | 5 |  |  | CP-OFDM | QPSK | Outer_Full |
| 20 | 910 | 10 |  |  |  | QPSK | Outer_Full |
| 21 | 907.5 | 15 |  |  |  | QPSK | Outer_Full |
| 22 | 907.5 | 15 |  |  |  | QPSK | Edge_1RB_Left |
| 233 | 902.5 | 5 |  |  |  | 16QAM | Outer_Full |
| 24 | 910 | 10 |  |  |  | 16QAM | Outer_Full |
| 25 | 907.5 | 15 |  |  |  | 16QAM | Outer_Full |
| 26 | 907.5 | 15 |  |  |  | 16QAM | Edge_1RB_Left |
| 273 | 902.5 | 5 |  |  |  | 64QAM | Outer_Full |
| 283 | 910 | 10 |  |  |  | 64QAM | Outer_Full |
| 29 | 907.5 | 15 |  |  |  | 64QAM | Outer_Full |
| 30 | 907.5 | 15 |  |  |  | 64QAM | Edge_1RB_Left |
| 31 | 907.5 | 15 |  |  |  | 256QAM | Outer_Full |
| 32 | 907.5 | 15 |  |  |  | 256QAM | Edge_1RB_Left |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Not applicable when SCS=60kHz |  |  |  |  |  |  |  |

### <Unchanged Skipped>

Table 6.2.3.5-10a: UE Power Class 2 test requirements (NS_43)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0.5 | 1 | 0 | 25 | 2 | 3 | 28+TT | 22-TT |
| 2 | 26 | 0.5 | 1.5 | 0 | 24.5 | 2 | 3 | 28+TT | 21.5-TT |
| 3 | 26 | 0.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 4 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 5 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 6 |  | 26 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 7 |  | 26 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 8 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 9 |  | 26 |  | 2 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 10 |  | 26 |  | 2 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 11 |  | 26 |  | 2 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 12 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 13 |  | 26 |  | 2.5 |  | 3.5 |  | 0 |  | 22.5 |  | 2 |  | 3 |  | 28+TT |  | 19.5-TT |  |
| 14 |  | 26 |  | 2.5 |  | 5 |  | 0 |  | 21 |  | 2 |  | 3 |  | 28+TT |  | 18-TT |  |
| 15 |  | 26 |  | 2.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 16 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 17 |  | 26 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 18 |  | 26 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 19 |  | 26 |  | 3 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 20 |  | 26 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 21 |  | 26 |  | 3 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 22 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 23 |  | 26 |  | 3 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 24 |  | 26 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 25 |  | 26 |  | 3 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 26 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 27 |  | 26 |  | 3.5 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 28 |  | 26 |  | 3.5 |  | 6.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 29 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 30 |  | 26 |  | 3.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 31 |  | 26 |  | 6.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 32 |  | 26 |  | 6.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-11: UE Power Class 3 test requirements (NS_43U)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 2 | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 4 | 23 | 0.5 | 9 | 0 | 14 | 5 | 2 | 25+TT | 9-TT |
| 5 | 23 | 1.0 | 2 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 6 | 23 | 1.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 7 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 8 | 23 | 1.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 9 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 10 | 23 | 2.0 | 2.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 11 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 12 | 23 | 2.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 13 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 14 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 15 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 16 | 23 | 2.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 17 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 4.5 | 4.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 20 | 23 | 4.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 21 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 24 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 25 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 26 | 23 | 3.0 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 28 | 23 | 3.0 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 29 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 31 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 32 | 23 | 3.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 33 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 34 | 23 | 6.5 | 6.5 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |
| 35 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| 36 | 23 | 6.5 | 9 | 0 | 14 | 5.0 | 2 | 25+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-11a: UE Power Class 2 test requirements (NS_43U)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 2 | 26 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 3 | 26 | 0.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 4 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 5 | 26 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 6 | 26 | 1 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 7 | 26 | 1 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 8 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 9 | 26 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 2 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 11 | 26 | 2 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 12 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 13 | 26 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 14 | 26 | 2.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 15 | 26 | 2.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 16 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 17 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 18 | 26 | 4.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 19 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 20 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 22 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 23 | 26 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 24 | 26 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 3 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 26 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 27 | 26 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 28 | 26 | 3.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 29 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 30 | 26 | 3.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 31 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| 32 | 26 | 6.5 | 12 | 0 | 14 | 5 | 3 | 28+TT | 9-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

### <End of Changes>