Title:	Discussion on test point selection for NS_43U A-MPR in FR1

Document for:	Discussion and approval

# Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_43U. The intention with NS_43U is to protect the downlink band of n18/n19 (DL 860-890MHz) as indicated in Table 1 below.

Table 1: Additional requirement for "NS_43" (from [4])

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15 MHz |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

# Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] for FR1. For NS_43U the upper part of operating band n8 and n81 (UL 880-915 MHz) has specific A-MPR requirements to protect the downlink band of n18/n19 (DL 860-890 MHz), see Table 2.

Table 2: A-MPR regions for NS_43U (from [4])

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 2.7 MHz/12/SCS | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  | ≥ 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/CS | A7 |
|  |  |  |  |  | ≥ 6.12 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2 and Table 6.2.3.6-2a. NOTE 2: Void NOTE 3: Void |  |  |  |  |  |  |  |

The A-MPR requirements in Table 2 are split into 2 A-MPR regions for 5, 10 and 15 MHz channel bandwidths. The UL RB allocation for testing of these A-MPR regions are allocated to the upper part of operating band n8 and n81. The start of the UL RB allocation and number of RB’s depends on both carrier center frequency and channel bandwidth. Table 2 is identical for NS_43 and NS_43U. However, the A-MPR requirement differs. Table 3 shows the A-MPR requirement for NS_43U for PC3 UE.

Table 3: A-MPR requirement for NS_43U for PC3 (from [4])

For power class 3 operation, when NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.6-4 applies. Table 6.2.3.6-4: A-MPR for NS_43U Modulation/Waveform; 15 MHz. Outer / Inner (dB). DFT-s-OFDM; Pi/2 BPSK; ≤ 9. QPSK; ≤ 9. 16 QAM; ≤ 9. 64 QAM; ≤ 9. 256 QAM; ≤ 9. CP-OFDM; QPSK; ≤ 9. 16 QAM; ≤ 9. 64 QAM; ≤ 9. 256 QAM; ≤ 9. For power class 3 operation, when NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.6-4 applies.

For PC3 UE, 5 and 10 MHz channel bandwidths the A-MPR requirements for NS_100 (table 6.2.3.1-2 in [4]) applies. But with the exception for DFT-s-OFDM QPSK modulation where A-MPR ≤ 2.5 dB when >5.4 MHz/12/SCS. Table 3a below summarizes the PC3 A-MPR requirement when NS_43U is signaled.

Table 3a: Overview of the A-MPR requirement for NS_43U for PC3

For power class 2 operation, the AMPR requirements when NS_43U is signalled are specified in Table 4:

Table 4: A-MPR requirement for NS_43U for PC2 (from [4])

For power class 2 operation, when NS_43U is signalled, the larger one between the PC2 A-MPR for NS_100 defined in Table 6.2.3.1-2 and the PC2 A-MPR for NS_43 defined in this clause applies. PC2 AMPR requirements for NS_43: Table 6.2.3.6-2a: A-MPR for NS_43 (Power Class 2) Modulation/Waveform; A1 (dB); A2 (dB); A3 (dB); A4 (dB); A5 (dB); A6 (dB); A7 (dB). Outer; Inner; Outer; Inner; Outer; Inner; Outer; Inner; Outer; Inner; Outer / Inner; Outer; Inner. DFT-s-OFDM; Pi/2 BPSK; ≤ 1; N/A; ≤ 1.5; N/A; N/A; N/A; ≤ 12; ≤ 4; 3. QPSK; ≤ 3.5; ≤ 3.5; ≤ 5.5; ≤ 12; ≤ 4; 3. 16 QAM; ≤ 3.5; ≤ 3.5; ≤ 5.5; ≤ 12; ≤ 5; 4. 64 QAM; ≤ 3.5; ≤ 4.5; ≤ 5; ≤ 12; ≤ 5.5; 5.5. 256 QAM; ≤ 12; ≤ 6.5; 5.5. CP-OFDM; QPSK; ≤ 4; ≤ 4; ≤ 6; ≤ 12; ≤ 6; 4.5. 16 QAM; ≤ 4; ≤ 4.5; ≤ 6; ≤ 12; ≤ 6; 5. 64 QAM; ≤ 4; ≤ 5; ≤ 6.5; ≤ 12; ≤ 6.5; 6. 256 QAM; ≤ 12; ≤ 7; 7.

As for PC2, the NS_43U AMPR requirements is applying the PC2 AMPR requirements of max(NS_100, NS_43). Considering following aspects:

- As specified in Table 2, in the NS_43 PC2 AMPR regions, the larger one between the PC2 A-MPR for NS_100 and NS_43 defined in this clause applies. For the areas without NS_43 PC2 AMPR defined, NS_100 requirements apply.

- NS_43U is tested for band n8 band n81. Meanwhile NS_100 also applies for n8 and n81. The areas not covered by NS_43(U) in Table 2 will be covered by NS_100 PC2 AMPR testing.

- Too much testing time and unnecessary double testing should be avoided

It's suggested to select test points based on the NS_43(U) AMPR regions in Table 2 and apply applicable AMPR requirements. Same test configurations as NS_43 PC2 AMPR testing should be selected for NS_43U PC2 AMPR testing.

The PC2 AMPR requirements for NS_43U could be summarized in Table 4a and RB allocations of NS_43U could be interpreted in Table 4b:

Table 4a: Overview of the A-MPR requirement for NS_43U for PC2

| Ch BW (MHz) |  |  | Region A |  |  |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fc (MHz) | SCS | RBstart | LCRB | LCRB@RBstart | Proposal | RBstart | LCRB | LCRB@RBstart | Proposal |
| 5 | 902.5 | 15 | - | >15 | Min16@0 Max 25@0 | Outer_Full (A1) | - | - | - | - |
|  |  | 30 | - | >7.5 | Min 8@0 Max 11@0 | Outer_Full (A1) | - | - | - | - |
|  |  | 60 | - | >3.75 | - | - | - | - | - | - |
|  |  | 15 | - | >40 | Min 41@0 Max 52@0 | Outer_Full (A2) | - | >30 | Min 31@0 Max 52@0 | Outer_Full (A4) |
|  |  | 30 | - | >20 | Min 21@0 Max 24@0 | Outer_Full (A2) | - | >15 | Min 16@0 Max 24@0 | Outer_Full (A4) |
|  |  | 60 | - | >10 | 11@0 | Outer_Full (A2) | - | >7.5 | Min 8@0 Max 11@0 | Outer_Full (A4) |
| 10 | 910 | 15 | - | >45 | Min 46@0 Max 52@0 | Outer_Full (A3) | - | >40 | Min 41@0 Max 52@0 | Outer_Full (A5) |
|  |  | 30 | - | >22.5 | Min 23@0 Max 24@0 | Outer_Full (A3) | - | >20 | Min 21@0 Max 24@0 | Outer_Full (A5) |
|  |  | 60 | - | >11.3 | - | - | - | >10 | 11@0 | Outer_Full (A5) |
|  |  | 15 |  |  | Min 1@0 Max 79@0 | Edge_1RB_Left + Outer_Full (A6) | >10 <34 | >40 |  | Skip test (not low edge) (A6) |
|  |  | 30 | <5 | >0 | Min 1@0 Max 38@0 | Edge_1RB_Left + Outer_Full (A6) | >5 <17 | >20 | 32@6 |  |
|  |  | 60 |  |  | Min 1@0 Max 18@0 | Edge_1RB_Left + Outer_Full (A6) | >2.5 <8.5 | >10 | 15@3 |  |
|  |  | 15 | >68 | >0 | 10@69 | Skip test (not low edge) (A6) | >10 <34 | >40 | 1@78 | Skip test (not low edge) (A7) |
| 15 | 907.5 | 30 | >34 | >0 | 3@35 |  | >5 <17 | >20 | 1@37 |  |
|  |  | 60 | >17 | >0 | - |  | >2.5 <8.5 | >10 | 1@17 |  |
|  |  | 15 | - | - | - | - | >34 <40 | >30 | 44@35 | Skip test (not low edge) (A7) |
|  |  | 30 | - | - | - | - | >17 <20 | >15 | 20@18 |  |
|  |  | 60 | - | - | - | - | >8.5 <10 | >7.5 | 9@9 |  |

Table 4b: Overview of the A-MPR requirement for NS_43U for PC2

|  |  | 5MHz CBW AMPR(dB) | 10MHz CBW AMPR(dB) | 15MHz CBW AMPR(dB) |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2 (NS_100) | 2 (NS_100) | 12 (A6) |
|  | QPSK | 3.5 (A1) | 3.5 (A4), 5.5 (A5) | 12 (A6) |
|  | 16QAM | 3.5 (A1) | 3.5 (A4), 5.5 (A5) | 12 (A6) |
|  | 64QAM | 3.5 (A1) | 4.5 (A2), 5 | 12 (A6) |
|  | 256QAM | NA | NA | 12 (A6) |
| CP-OFDM | QPSK | 4 (A1) | 4 (A4), 6 (A5) | 12 (A6) |
|  | 16QAM | 4 (A1) | 4.5 (A4), 6 (A5) | 12 (A6) |
|  | 64QAM | 4 (A1) | 5 (A2), 6.5 | 12 (A6) |
|  | 256QAM | NA | NA | 12 (A6) |

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_43U.

## Test Frequencies

UL test carrier centre frequencies are defied in Table 2 for the NS_43U A-MPR regions. Select 902.5 MHz as test frequency for 5 MHz channel bandwidth, since this frequency is closest to the DL band to be protected.

PROPOSAL 2: Select 902.5 MHz, 910 MHz and 907.5 MHz as test frequencies for NS_43U A-MPR tests.

## Test Channel Bandwidth

The minimum requirements for NS_43 apply for all channel bandwidths between 5 MHz to 15 MHz.

PROPOSAL 3: If supported, select 5, 10 and 15 MHz channel bandwidth.

## Test Subcarrier Spacing

All requirements in Table 2 are specified for only 15 kHz SCS before Rel-18. That means that only the lowest SCS shall be tested. And considering spectrum utilization and minimum guard band, it is reasonable to test the Lowest SCS.

PROPOSAL 4: Select lowest subcarrier spacing.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_43U A-MPR in FR1.

## Uplink RB allocation

The same RB allocation can be used for NS_43U and NS_43. The NS_43 RB allocation will be repeated in this subclause.

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full. These common uplink configurations are listed in [5].

A-MPR for NS_43 has some Region specific RB allocations in Table 2, where the ranges for the RB allocation is specified in terms “absolute values” or in terms of RBend × 12 × SCS and LCRB × 12 × SCS. Table 5 shows the corresponding RB ranges and proposed allocation expressed as LCRB@RBstart.

Table 5: UL RB allocations for NS_43 and NS_43U

| Ch BW (MHz) |  |  | Region A |  |  |  | Region B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fc (MHz) | SCS | RBstart | LCRB | LCRB@RBstart | Proposal | RBstart | LCRB | LCRB@RBstart | Proposal |
| 5 | 902.5 | 15 | - | >15 | Min16@0 Max 25@0 | Outer_Full (A1) | - | - | - | - |
|  |  | 30 | - | >7.5 | Min 8@0 Max 11@0 | Outer_Full (A1) | - | - | - | - |
|  |  | 60 | - | >3.75 | - | - | - | - | - | - |
|  |  | 15 | - | >40 | Min 41@0 Max 52@0 | Outer_Full (A2) | - | >30 | Min 31@0 Max 52@0 | Outer_Full (A4) |
|  |  | 30 | - | >20 | Min 21@0 Max 24@0 | Outer_Full (A2) | - | >15 | Min 16@0 Max 24@0 | Outer_Full (A4) |
|  |  | 60 | - | >10 | 11@0 | Outer_Full (A2) | - | >7.5 | Min 8@0 Max 11@0 | Outer_Full (A4) |
| 10 | 910 | 15 | - | >45 | Min 46@0 Max 52@0 | Outer_Full (A3) | - | >40 | Min 41@0 Max 52@0 | Outer_Full (A5) |
|  |  | 30 | - | >22.5 | Min 23@0 Max 24@0 | Outer_Full (A3) | - | >20 | Min 21@0 Max 24@0 | Outer_Full (A5) |
|  |  | 60 | - | >11.3 | - | - | - | >10 | 11@0 | Outer_Full (A5) |
|  |  | 15 |  |  | Min 1@0 Max 79@0 | Edge_1RB_Left + Outer_Full (A6) | >10 <34 | >40 |  | Skip test (not low edge) (A6) |
|  |  | 30 | <5 | >0 | Min 1@0 Max 38@0 | Edge_1RB_Left + Outer_Full (A6) | >5 <17 | >20 | 32@6 |  |
|  |  | 60 |  |  | Min 1@0 Max 18@0 | Edge_1RB_Left + Outer_Full (A6) | >2.5 <8.5 | >10 | 15@3 |  |
|  |  | 15 | >68 | >0 | 10@69 | Skip test (not low edge) (A6) | >10 <34 | >40 | 1@78 | Skip test (not low edge) (A7) |
| 15 | 907.5 | 30 | >34 | >0 | 3@35 |  | >5 <17 | >20 | 1@37 |  |
|  |  | 60 | >17 | >0 | - |  | >2.5 <8.5 | >10 | 1@17 |  |
|  |  | 15 | - | - | - | - | >34 <40 | >30 | 44@35 | Skip test (not low edge) (A7) |
|  |  | 30 | - | - | - | - | >17 <20 | >15 | 20@18 |  |
|  |  | 60 | - | - | - | - | >8.5 <10 | >7.5 | 9@9 |  |

As can be seen in Table 5 Region A has restrictions on either the RBstart or LCRB. The proposals for LCRB@RBstart in Table 5 are all based on allocating as many RBs as possible close to the upper band to be protected (DL 860-890 MHz) as it can be considered a worst-case scenario for spurious emissions towards the DL band. Two RB allocations for 15 MHz band within Table 5 can be skipped since they are not close to the DL band. Worst case for 15 MHz channel bandwidth is Edge_1RB_Left and Outer_Full since they are closer to the DL band to be protected.

As specified in table 4b, for PC2 requirements for 10MHz CBW + Outer_Full RB allocation: both A4 and A5 apply for QPSK and 16QAM, and both A2 and A3 apply for 64QAM. Under this test configuration, A3 has larger AMPR value than A2, and A5 has larger AMPR value than A4.

- When the PC2 UE is tested at 10MHz CBW + QPSK + Outer_Full: A5 AMPR requirements (5.5dB for DFT-s-OFDM and 6dB for CP-OFDM) applies

- When the PC2 UE is tested at 10MHz CBW + 16QAM + Outer_Full: A5 AMPR requirements (5.5dB for DFT-s-OFDM and 6dB for CP-OFDM) applies

- When the PC2 UE is tested at 10MHz CBW + 64QAM + Outer_Full: A3 AMPR requirements (5dB for DFT-s-OFDM and 6.5dB for CP-OFDM) applies

PROPOSAL 6: Select Outer_Full for all tests, with addition of Edge_1RB_Left for 15 MHz Region A.

2.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_43U:

| Environment conditions | Number SCS | Number of ChBW | Maximum Number of Modulations | Maximum Number of Frequencies | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 9 | 23 | 36 for PC3 32 for PC2 |

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_43U A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select 902.5 MHz, 910 MHz and 907.5 MHz as test frequencies for NS_43U A-MPR tests.

PROPOSAL 3: If supported, select 5, 10 and 15 MHz channel bandwidth.

PROPOSAL 4: Select lowest i.e. 15 kHz subcarrier spacing.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_43U A-MPR in FR1.

PROPOSAL 6: Select Outer_Full for all tests, with addition of Edge_1RB_Left for 15 MHz Region A.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V16.1.0 (2019-09), Table 6.2.3.1-2

- 3GPP TS 38.521-1, V16.1.0 (2019-09), Table 6.1-1