3GPP TSG-RAN5 Meeting #106	R5-250451

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3942 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NITZ testcase 12.2.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Step-14 and Step-24, test case provides LocalTimeZone and DST fields only in the GMM INFORMATION message. The universal time elements information is not included. Due to this, the output of AT command +CTZE does not show universal time parameters, as expected by test case at Step-16 and Step-26. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include universal time elements in Step-14 and Step-24 similar to Step-6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case unfairly fails a conformant device (a module) where output of +CTZE is the only method to verify Step-16 and Step-24. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 12.2.1.13	NITZ / GMM/ Time zone, Time and DST Handling

12.2.1.13.1	Definition

To check that the UE correctly handles time zone, time and DST.

12.2.1.13.2	Conformance requirement

The feature Network Identities and Timezone shall make it possible for a serving PLMN to transfer its current identity, universal time, DST and LTZ to MSs, and for the MS to store and use this information. Each one of these elements is optional. The feature significantly enhances roaming as it enables the accurate indication of network identities that are either newer than the ME or have changed their name since the ME was manufactured or sold. Additionally time and timezone information can be utilised by MEs as desired.

When using the default character set (see 3GPP TS 23.038 [3]), the serving PLMN shall make both a "short" and a "long" name available to the MS. As an alternative or, in addition, to the default character set, the serving PLMN can make a name available in UCS2. The MS shall be free to choose one of these names depending upon its own characteristics and/or limitations, such as those of its display.

NOTE:	Guidance is sought, particularly from non-European operators, as to whether long and short name is required in UCS2 format.

The Network Operator may change the network identity at any time. However the change of network identity need not force immediate transfer of information to the MS.

As a network option, it shall be possible to send universal time (UT) by the network. Time information shall include: Year, Month, Day, Hour, Minute, Second, Timezone and DST. The expected accuracy of the time information is in the order of minutes.

NOTE:	Universal time indicates the time at which this information element (see 3GPP TS 24.008 [2]) may have been sent by the network. Thus it can be assumed that the accuracy of the time information when it arrives at the MS is usually within a couple minutes.

The serving PLMN shall make Local Time Zone (LTZ) available to the MS as an offset from Universal Time in units of 15 minutes.

When the LTZ is compensated for DST (summertime), the serving PLMN shall provide a DST parameter to indicate this. The adjustment for DST can be +1h or +2h.

The TP-Service-Centre-Time-Stamp field is given in semi-octet representation, and represents the local time in the following way:

|  | Year: | Month: | Day: | Hour: | Minute: | Second: | Time Zone |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Digits: (Semi-octets) | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT. In the first of the two semi-octets, the first bit (bit 3 of the seventh octet of the TP-Service-Centre-Time-Stamp field) represents the algebraic sign of this difference (0: positive, 1: negative).

The Service-Centre-Time-Stamp, and any other times coded in this format that are defined in the present document, represent the time local to the sending entity.

If the MS has knowledge of the local time zone, then any time received (e.g. Service-Centre-Time-Stamp) at the MS may be displayed in the local time rather than the time local to the sending entity. Messages shall be stored as received without change to any time contained therein.

The Time Zone code enables the receiver to calculate the equivalent time in GMT from the other semi-octets in the Service-Centre-Time-Stamp, or indicate the time zone (GMT, GMT+1H etc.), or perform other similar calculations as required by the implementation. The value contained in the Time Zone field must take into account daylight saving time, such that when the sending entity changes from regular (winter) time to daylight saving (summer) time, there is a change to the value in the Time Zone field, for example in the UK the winter setting is 00000000 and the summer setting is 01000000.

If the MS receives a non-integer value in the SCTS, it shall assume that the digit is set to 0 but shall store the entire field exactly as received.

If the local time zone has been adjusted for Daylight Saving Time, the network shall indicate this by including the IE Network Daylight Saving Time.

References

3GPP TS 22.042 sub clause 4

3GPP TS 23.040 sub clause 9.2.3.11

3GPP TS 24.008 sub clause 9.4.19.4

12.2.1.13.3	Test purpose

To verify that a UE supporting any of the NITZ Time related feature (Local Time Zone, Universal Time and DST IE (and thus GMM Information) is able to handle them properly.

12.2.1.13.4	Method of test

Related ICS/IXIT statements

Support of FDD Yes/No.

Support of PS bearer service Yes/No.

-	Support of Switch on off button Yes/No.

-	Support of Automatic GPRS attach procedure at switch on or power on Yes/No.

-	Support of NITZ DST Yes/No.

-	Support of NITZ Universal Time for PLMN Yes/No.

-	Support of NITZ Local Time Zone for PLMN Yes/No.

Initial conditions

System Simulator:

Two cells operating in network operation mode I, cell A in MCC1/MNC1/LAC1/RAC1 (RAI-1), cell B in MCC1/MNC1/LAC1/RAC2 (RAI-4).

User Equipment:

-	The UE is "idle updated" on cell A.

-	The UE is switched off with a valid USIM inserted.

Test procedure

Following the GPRS attachment on Cell A, SS sends its local time and date (on GMT+1, Winter Time) using the GMM INFORMATION Message to the UE. The test operator verifies then the parameters and/or the time and date stored in the MS.

The UE is then moved to Cell B, and after a ROUTING AREA UPDATE procedure, the time is changed to “Summer Time” with the DST IE using a GMM INFORMATION Message. The test operator verifies then the parameters and/or the time and date stored in the UE and switches Off the UE

The UE is then moved to Cell A, and after a ROUTING AREA UPDATE procedure the Time Zone is then changed, no DST present (GMT+2, Winter Time), using GMM INFORMATION message. The operator verifies then the parameters and/or the time stored on the UE.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | UE |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non Suitable neighbour cell". The UE is powered up or switched on and initiates an attach (see PICS). The following messages are sent and shall be received on cell A |
| 2 | -> |  | ATTACH REQUEST |  |
| 3 | SS |  |  | The SS starts integrity protection. |
| 4 | <- |  | ATTACH ACCEPT | Attach result = Attach type in step 2 Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 5 | -> |  | ATTACH COMPLETE |  |
| 6 | <- |  | GMM INFORMATION | Universal Time IE is included: “< Current Year >/12/31 04:15:00” for Local Time “+ 1 hour” for Timezone No DST or Local Time Zone IE included See specific message content |
| 7 | SS |  |  | The SS releases the RRC connection |
| 8 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time: Year: < Current Year > Month: December Day: 31st Hour: 5 Hours Minute: 15 Minutes Timezone: GMT+1 Local Time Zone: Not sent DST: Daylight Saving Time not in use (i.e. winter time) (See Note) |
| 9 | SS |  |  | Set the cell type of cell B to the "Serving cell". Set the cell type of cell A to the "Suitable neighbour cell". The following messages are sent and shall be received on cell B |
| 10 | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-2 signature Routing area identity = RAI-1 |
| 11 | SS |  |  | The SS starts integrity protection. |
| 12 | <- |  | ROUTING AREA UPDATING ACCEPT | Update result = Update type in step 10 Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-4 |
| 13 | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 14 | <- |  | GMM INFORMATION | Universal Time IE is included: “< Current Year >/12/31 04:15:00” for Local Time Local Time Zone IE and DST IE are included: “+ 2 hours” for Timezone (including DST) “+ 1 hour” for DST See specific message content |
| 15 | SS |  |  | The SS releases the RRC connection |
| 16 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time Year: < Current Year > Month: December Day: 31st Hour: 6 Hours Minutes: 15 Minutes Timezone: GMT+2 Local Time Zone Timezone: GMT+2 (DST included) DST: Daylight Saving Time in use (i.e. “summer time”) (See Note) |
| 17 | SS |  |  | Set the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Suitable neighbour cell". The following messages are sent and shall be received on cell A |
| 18 | -> |  | ROUTING AREA UPDATING REQUEST | P-TMSI-1 signature Routing area identity = RAI-4 |
| 19 | SS |  |  | The SS starts integrity protection. |
| 20 | <- |  | ROUTING AREA UPDATING ACCEPT | Update result = Update type in step 18 Mobile identity = P-TMSI-2 P-TMSI-2 signature Routing area identity = RAI-1 |
| 21 | -> |  | ROUTING AREA UPDATING COMPLETE |  |
| 22 | <- |  | GMM INFORMATION | Universal Time IE is included: “< Current Year >/12/31 04:15:00” for Local Time Local Time Zone IE is included: “+ 2 hours” for Timezone No DST IE included See specific message content |
| 23 | SS |  |  | The SS releases the RRC connection |
| 24 | UE |  |  | Operator Action: The use of the supported Fields is checked: Universal Time: Year: < Current Year > Month: December Day: 31st Hour: 6 Hours Minutes: 15 Minutes Timezone: GMT+2 Local Time Zone: Timezone: GMT+2 DST: Daylight Saving Time not in use (i.e. winter time) (See Note) |
| Note: In step 6, 16 and 24 the “minute” is not so relevant and can be higher than “15” depending on operator’s action time. The check of Timezone and DST is done implicitly by checking the time only in case UE does not support the display of these two fields. Current Year is derived by the SS Steps 9-16 resp. 17-24 are only performed if the UE supports pc_LocalTimeZone. In Steps 8, 16, 24 Local Time Zone is checked. DST is to be checked only if the UE supports pc_DaylightSavingTime. |  |  |  |  |

Specific message contents:

GMM Information on step 6:

| Information element | Value/remark |
| --- | --- |
| Universal Time IE | 47 |
| Year | < Current Year > |
| Month | 21 - December |
| Day | 13 - 31 |
| Hour | 40 - 4 hours |
| Minute | 51 - 15 Minutes |
| Second | 00 - 0 second |
| Time Zone | 40 - GMT+1 (4*15 minutes+ 0*15 minutes DST) |

GMM Information on step 14:

| Information element | Value/remark |
| --- | --- |
| Universal Time IE Year Month Day Hour Minute Second Time Zone Local Time Zone IE | 47 < Current Year > 21 - December 13 - 31 40 - 4 hours 51 - 15 Minutes 00 - 0 second 80 - GMT+1+1 (4*15 minutes+ 4*15 minutes DST) 46 |
| Time Zone | 80 - GMT+1+1(4*15 minutes+ 4*15 minutes DST) |
| Daylight Saving Time IE | 49 |
| Length of DST Content | 1 |
| Value | 1 - + 1 hour (summer time) |

GMM Information on step 22:

| Information element | Value/remark |
| --- | --- |
| Universal Time IE Year Month Day Hour Minute Second Time Zone Local Time Zone IE | 47 < Current Year > 21 - December 13 - 31 40 - 4 hours 51 - 15 Minutes 00 - 0 second 80 - GMT+2 (8*15 minutes+ 0*15 minutes DST) 46 |
| Time Zone | 80 - GMT+2 (8*15 minutes+ 0*15 minutes DST) |

12.2.1.13.5	Test requirements

In step 6, 16 and 24 the UE populates the supported fields with the correct values.

<End of modified section>